3GPP TSG-RAN WG3 Meeting #126	R3-247226

Orlando, US, Nov. 18th – Nov. 22nd, 2024

Agenda Item:	12.2

Source: 	Canon Research Centre France

Title:	Discussion on enhancements for WAB

Document for:	Discussion

# Introduction

In this paper, we discuss some aspects related to Wireless Access Backhaul (WAB), based on the TR 38.799 on WAB [2], the WID on additional topological enhancements [3] and on agreements/notes from the previous RAN3 meeting [1].

# Discussion

## Multi-hop prevention

At RAN3 #125bis, RAN3 agreed that the multi-hop WAB topology scenario where a WAB-gNB serves a WAB-MT(s), even though not being precluded, should be preventable by means of different standard based solutions, with a preference for a RAN-based solution to solve this issue.

In this respect, a WAB-gNB cell may broadcast a WAB indicator in SIB (e.g., SIB1), which would be used to bar a WAB-MT. For instance, if a WAB-gNB broadcasts a WAB cell indication in SIB1, the WAB-MT of a WAB node in the vicinity may avoid selecting this WAB cell when performing cell (re)selection. This solution relies on basic non-ambiguous signalling while introducing limited stage-3 impact.

Also, the WAB-MT of a WAB node in the vicinity may refrain from performing measurements and associated reporting of WAB cells broadcasting a WAB indicator to its BH RAN node.

Proposal 1a: Select Solution 3 for preventing multi-hop WAB upon cell (re)selection, i.e., WAB-gNB-cells broadcast a new indicator in SIB to bar WAB-MT, and the WAB-MT avoids (re)selection of cells broadcasting this indicator.

In addition, during WAB node handover, a (WAB-aware) target BH RAN node may rely on the presence of a WAB indication in the handover request received from the source BH RAN node to accept or reject the handover request. For multi-hop WAB prevention, a WAB-gNB can reject the handover request based on this WAB indication.

In order to allow for multi-hop WAB prevention upon WAB node handover while relying on legacy source BH RAN nodes, the WAB indication carried in the handover request may be based on an existing IE, e.g., UE capability, S-NSSAI. Indeed, the WAB indication may be provided through a new WAB indicator in UE capabilities or through some slice-related information (e.g., a list of S-NSSAIs dedicated to WAB).

Proposal 1b: Select Solution 5 for preventing multi-hop WAB upon WAB handover, i.e., in case of handover for a WAB-node, the new WAB-node indication is included in the HO request, then the target BH-RAN node can perform access control for this WAB-node.

Proposal 1c: The new WAB-node indication included in the HO request may be based on UE capabilities or on a list of S-NSSAIs.

Regarding the use of a list of S-NSSAIs for access control, SA2 added a note in the CR [4] stating that based on operator configuration, certain S-NSSAIs and DNNs can be dedicated to support the wireless backhaul links established by PDU sessions of the MWAB-UE. Therefore, it is not guaranteed that a WAB-MT will always be able to connect using a WAB dedicated slice ID. Before selecting an option for multi-hop prevention involving the use of a list of S-NSSAIs, RAN3 should therefore assess that WAB dedicated slices will always be configured.

Observation 1: Before selecting an option for multi-hop prevention involving the use of a list of S-NSSAIs, RAN3 should assess that WAB dedicated slices will always be configured.

## Xn Interface

RAN3 agreed that the establishment of Xn connections between a WAB-gNB and the BH RAN node serving the WAB-MT or between a WAB-gNB and surrounding RAN nodes is supported. It can follow legacy procedures, but addition of new information in these legacy procedures is not precluded.

Xn Connection Setup between a WAB-gNB and the BH RAN node serving the WAB-MT

For instance, to support coordination for resource multiplexing between a WAB-gNB and a (WAB-aware) BH RAN node serving the WAB-MT, the BH RAN node should have the knowledge of which WAB-MT is collocated with the WAB-gNB while the WAB-gNB should know the BH RAN node serving the WAB-MT it is collocated with.

Thus, the Xn setup procedure may be enhanced so that the XN SETUP REQUEST message sent by a WAB-gNB includes information on its co-located WAB-MT.

However, the Xn setup procedure may be initiated either by the WAB-gNB or by the BH RAN node serving the WAB-MT. Thus, the XN SETUP RESPONSE message sent by a WAB-gNB should also include the information on its co-located WAB-MT.

Observation 2: The Xn setup procedure may be initiated either by the WAB-gNB or by the BH RAN node serving the WAB-MT.

Proposal 2a: The XN SETUP REQUEST and XN SETUP RESPONSE messages sent by a WAB-gNB to the BH RAN node serving the WAB-MT include information on its co-located WAB-MT.

Besides, a WAB-gNB may already have established a Xn connection with a NG-RAN node before this NG-RAN node becomes the BH RAN node serving the co-located WAB-MT (e.g., before the handover of the WAB-MT toward this NG-RAN node). Therefore, a WAB-gNB should be able to provide co-location information to a BH RAN node while a Xn connection is already established with this BH RAN node. For this purpose, the WAB-gNB may use the NG-RAN Node Configuration Update procedure.

Proposal 2b: The NG-RAN NODE CONFIGURATION UPDATE message sent by a WAB-gNB to the BH RAN node serving the WAB-MT may include information on its co-located WAB-MT.

Xn Connection Setup between WAB-gNBs

In the context of two WAB-gNBs on top of two different vehicles having completely different trajectories, the interest of establishing Xn connection between these WAB-gNBs would be limited. In this respect, the objectives of Rel-19 WID [3] include how to avoid setting up Xn between WAB-gNBs.

However, when considering vehicles embedding several WAB nodes – e.g., a cruise ship or a train – establishing Xn connections between the WAB-gNBs bound to a same vehicle would become relevant. For instance, for the served UEs moving inside the vehicle, the handover from one WAB-gNB to another could be facilitated by having Xn connection between the source and target WAB-gNBs. Thus, it seems interesting not to preclude the establishment of Xn connections between WAB-gNBs as such connections may be relevant for some WAB deployments.

Proposal 2c: An Xn connection may be setup between any two WAB-gNBs.

To allow or prevent the setup of an Xn connection between two WAB-gNBs, it is proposed to have an optional WAB indication shared between WAB-gNBs during the Xn setup procedure.

Proposal 2d: An optional WAB indication may be included in the XN SETUP REQUEST and the XN SETUP RESPONSE message.

Sharing of neighbour NG-RAN nodes

In order to ease up some further Xn connection establishment, a WAB-gNB as other NG-RAN nodes, can share neighbour information to update the Neighbour Cell Relation Table (NCRT). In particular, the shared neighbour information may include the identification of neighbour WAB cells / WAB-gNBs.

Proposal 2e: The neighbour information shared between NG-RAN nodes may include the identification of neighbour WAB cells / WAB-gNBs.

Xn Connection Rejection

A WAB-gNB may, for some WAB specific reasons (e.g., preventing Xn connection between WAB gNBs, mobility constraints …), reject an Xn setup request received from another NG-RAN node, by issuing an XN SETUP FAILURE message, which includes the cause of rejection.

Proposal 2f: A new cause value is introduced, to be used by a WAB-gNB in the XN SETUP FAILURE message to reject a Xn setup request for WAB specific reason.

Xn Connection Removal

In case the benefit of maintaining an Xn connection is considered as not sufficient (e.g., Xn link quality is too low, WAB-gNB is moving away, WAB-node authorization status has changed …) by a WAB-gNB or a BH RAN node or a surrounding NG-RAN node, this WAB-gNB / BH RAN node / surrounding NG-RAN node may initiate the removal of the Xn connection by sending an XN REMOVAL REQUEST message. In such case, a cause value information indicating the reason for initiating the Xn connection removal may be included in the XN REMOVAL REQUEST message. The existing cause value “Action Desirable for Radio Reasons” may be appropriate when the Xn link quality is too low or when the WAB-node is moving away, but a new cause value may be introduced, for instance “WAB not authorized”, when the reason is that the WAB-node authorization has changed.

Proposal 2g: The XN REMOVAL REQUEST message may include a cause value indicating the reason for requesting the Xn connection removal.

## NG interface

NG Connection Setup

A WAB-gNB may advertise to the Core Network that it actually belongs to a WAB node by sending some WAB indication to AMFs during the NG Setup procedure, e.g., by adding a WAB indication in the NG SETUP REQUEST message.

An AMF may confirm, during the NG setup procedure, that it actually supports WAB features by sharing some WAB support information with the WAB-gNB, e.g., by adding some WAB support information in the NG SETUP RESPONSE message.

Proposal 3a: A WAB indication is added to the NG SETUP REQUEST message.

Proposal 3b: A WAB support information is added to the NG SETUP RESPONSE message.

NG Connection Removal

A WAB-gNB may, at some point, decide to remove an NG connection with the Core Network (e.g., due to a change of AMFa change of authorization status, the co-located WAB-MT entering RRC_IDLE / RRC_INACTIVE state …). When initiating NG removal process, a WAB-gNB may further include in the NG Removal Request message sent to the AMF a cause value indicating the reason for requesting the NG connection removal.

Proposal 3c: a WAB-gNB may include in the NG Removal Request message a cause value indicating the reason for requesting the NG connection removal.

## Backhaul link degradation management

Unlike the typical 5G NR scenario where the NG and Xn interfaces are operated over wired links, the WAB architecture considered in [2] would imply that part of the NG/Xn links is wireless (i.e., the BH link between the WAB-MT and the BH RAN node is operated over the wireless Uu interface).

Considering that the NG/Xn protocols have been designed to be operated over wired links, some countermeasures should be considered to guarantee the reliability of these interfaces when operated over a versatile wireless link, which is likely to experience link degradation or congestion.

Proposal 4: RAN3 to study solutions for mitigating backhaul link degradation.

# Conclusion

In this contribution, we have discussed some WAB-related subjects about multi-hop prevention, Xn connection management, NG connection management. Finally, we have made the following observation and proposals:

Observation 1: Before selecting an option for multi-hop prevention involving the use of a list of S-NSSAIs, RAN3 should assess that WAB dedicated slices will always be configured.

Proposal 1a: Select Solution 3 for preventing multi-hop WAB upon cell (re)selection, i.e., WAB-gNB-cells broadcast a new indicator in SIB to bar WAB-MT, and the WAB-MT avoids (re)selection of cells broadcasting this indicator.

Proposal 1b: Select Solution 5 for preventing multi-hop WAB upon WAB handover, i.e., in case of handover for a WAB-node, the new WAB-node indication is included in the HO request, then the target BH-RAN node can perform access control for this WAB-node.

Proposal 1c: The new WAB-node indication included in the HO request may be based on UE capabilities or on a list of S-NSSAIs.

Observation 2: The Xn setup procedure may be initiated either by the WAB-gNB or by the BH RAN node serving the WAB-MT.

Proposal 2a: The XN SETUP REQUEST and XN SETUP RESPONSE messages sent by a WAB-gNB to the BH RAN node serving the WAB-MT include information on its co-located WAB-MT.

Proposal 2b: The NG-RAN NODE CONFIGURATION UPDATE message sent by a WAB-gNB to the BH RAN node serving the WAB-MT may include information on its co-located WAB-MT.

Proposal 2c: An Xn connection may be setup between any two WAB-gNBs.

Proposal 2d: An optional WAB indication may be included in the XN SETUP REQUEST and the XN SETUP RESPONSE message.

Proposal 2e: The neighbour information shared between NG-RAN nodes may include the identification of neighbour WAB cells / WAB-gNBs.

Proposal 2f: A new cause value is introduced, to be used by a WAB-gNB in the XN SETUP FAILURE message to reject a Xn setup request for WAB specific reason.

Proposal 3a: A WAB indication is added to the NG SETUP REQUEST message.

Proposal 3b: A WAB support information is added to the NG SETUP RESPONSE message.

Proposal 3c: a WAB-gNB may include in the NG Removal Request message a cause value indicating the reason for requesting the NG connection removal.

Proposal 4: RAN3 to study solutions for mitigating backhaul link degradation.

# References

[1] Chair notes at RAN3#125bis meeting

[2] TR 38.799 V19.0.0

[3] RP-242395 - New WID on additional topological enhancements for NR

[4] S2-2411119 – CR 23.502 MWAB support