3GPP TSG-RAN WG3 Meeting #126                                                              R3-247228

Orlando, Florida, November 18 – 22, 2024

Agenda item: 		12.2

Source: 		Qualcomm Incorporated

Title: 	(TP to BL CR 38.401) Discussion of aspects related to WAB

Document for:		Discussion

# 1	Introduction

This contribution aims to make further progress on Wireless Access Backhaul based on the objectives defined by the WID on Additional Topological Enhancements for NR [1]. The discussion includes recent agreements by SA2 related to MWAB [2] as well as agreements on WAB from last RAN3 meeting [3].

# 2	Discussion

## 2.1	Authorization of WAB-node

According to the WID for RAN’s WI on Additional Topological Enhancements for NR, RAN3 should define the behaviour of the WAB-node in case the authorization of WAB-MT and/or WAB-gNB changes [1].

In the last meeting, SA2 agreed on WAB-node authorization details which were captured in a CR to 23.501 in S2-2411238 [2]. Based on this CR and in compliance with RAN3’s understanding from the study, WAB-node authorization includes the authorization of the WAB-MT to obtain a PDU session for backhauling, and the authorization for the WAB-gNB to provide services to UEs.

SA2 agreed that WAB-MT authorization is based on a WAB-dedicated S-NSSAI(s) and DNN. According to SA2’s agreement the WAB-MT authorization can further be time- and location-dependent. RAN3 should confirm these agreements.

Proposal 1-1: In alignment with SA2, RAN3 confirms that the WAB-MT is authorized to support a PDU session for backhauling based on WAB-specific S-NSSAI and DNN, and that this authorization can be time- and location-dependent.

SA2 further agreed that the WAB-gNB authorization is managed by the OAM system of the broadcasted PLMN/SNPN. The WAB-gNB’s authorization can also be time- and location-dependent.

Proposal 1-2: In alignment with SA2, RAN3 confirms that the WAB-gNB’s service authorization is based on OAM configuration.

In case the WAB-gNB’s authorization status changes from “authorized” to “not authorized”, the UEs served by the WAB-gNB should be handed over to other RAN nodes or released, and then the WAB-gNB’s NG and Xn connection(s) should be removed. This behaviour was agreed during the RAN3 study. This behaviour is also aligned with an equivalent behaviour defined for mobile IAB. SA2’s CR confirms aspects of this behaviour [2].

Proposal 1-3: In alignment with SA2, RAN3 confirms that when the WAB-gNB’s authorization status changes from “authorized” to “not authorized”, the UEs served by the WAB-gNB should be handed over to other RAN nodes or released, and then the WAB-gNB’s NG and Xn connection(s) should be removed.

In case the WAB-gNB’s authorization status changes from “not authorized” to “authorized”, the WAB-gNB to reestablish NG connection and potentially Xn connection(s), and to start serving UEs. This behaviour was agreed during the RAN3 study. This behaviour is also aligned with an equivalent behaviour defined for mobile IAB.

Proposal 1-4: When the WAB-gNB’s authorization status changes from “not authorized” to “authorized”, the WAB-gNB can reestablish NG connection and potentially Xn connection(s) and start serving UEs.

Since an authorized WAB-gNB cannot serve UEs without BH PDU session, WAB-gNB authorization needs to be time-aligned with WAB-MT. SA2’ CR captures this in the following NOTE [2]:

NOTE: It is expected the HPLMN/Subscribed SNPN network ensures that the BH PDU sessions authorization for MWAB UE does not interfere with the ability of the OAM of the MWAB Broadcasted PLMN/SNPN to manage the MWAB-gNB , e.g. the S-NSSAI(s) for the BH PDU session(s) of the MWAB-UE is(are) maintained long enough in the MWAB-UE subscription and are available for the MWAB-UE for a location and time that is sufficient for the MWAB-UE, so that the BH PDU sessions are available for the MWAB-gNB to be able to perform OAM-controlled shut down, which may include handing over the UEs it serves.

RAN3 should capture the spirit in clearer terms: The WAB-MT should be authorized to support BH PDU sessions while the collocated WAB-gNB is serving UEs and has backhaul connections established. RAN3 assumes that this time alignment can be achieved based on configuration.

Proposal 1-5: The WAB-MT should be authorized to support BH PDU sessions while the collocated WAB-gNB is serving UEs and has backhaul connections established. RAN3 assumes that this time alignment can be achieved based on configuration.

## 2.2	Avoiding Xn establishment between WAB-gNBs

According to the WID for RAN’s WI on Additional Topological Enhancements for NR, RAN3 should define the support of Xn interface(s) by the WAB-gNB with the WAB-MTs serving BH RAN node and with other surrounding gNBs, including how to avoid setting up Xn between WAB-gNBs [1].

Since the WAB-gNB supports the functionality of a legacy gNB, it can establish Xn interface with BH RAN and with other WAB-gNBs. Further, neighbour discovery and the resolution of the neighbour RAN-node’s TNL address can use legacy procedures.

Proposal 2-1: Since the WAB-gNB supports the functionality of a legacy gNB, it can establish Xn connections with BH gNB’s and WAB-gNBs, and it can use the legacy procedures to discover neighbour gNBs and resolve their TNL addresses.

During the study, RAN3 discussed whether WAB-gNBs should be permitted to establish mutual Xn connections. The establishment of such mutual Xn between WAB-gNBs may have benefits in some scenarios, e.g., when the WAB-nodes are stationary and in close vicinity, but it may have shortcomings in others, e.g., when the WAB-nodes move with high relative speed. The establishment of Xn between WAB-gNBs should therefore be allowed, if desirable, but it should be preventable in case it is not desirable.

To prevent the Xn establishment between WAB-gNBs, a WAB-gNB needs to be made aware whether a neighbour node is a WAB-gNB or another RAN node. The following options can be considered:

Option A: A dedicated frequency and/or PCI value is assigned to the WAB-gNB cell. A WAB-gNB can reject the Xn SETUP REQUEST in case it contains a served cell using such dedicated frequency and/or PCI value. It can remove an Xn connection in case such cell is included in the Xn SETUP RESPONSE.

Option B: WAB-gNBs includes a “WAB-cell” indicator in the served-cell information contained in the Xn Setup Request. The WAB-gNB can reject Xn SETUP REQUEST messages in case they contain the “WAB” indicator in the served cell information. It can remove an Xn connection in case such cell is included in the Xn SETUP RESPONSE.

Option A has no stage-3 impact, but it requires allocating a dedicated frequencies and/or PCI values to WAB-gNB cells. Option B does have stage-3 impact, but it allows using same frequencies and PCI values for WAB-gNB cells as for BH RAN cells. Option B, if supported, should not have adverse impact on the Xn establishment between a WAB-gNB and a BH RAN node in case the BH RAN node does not support Rel-19 WAB enhancements.

Proposal 2-2: The WAB-gNB can prevent Xn establishment with a peer WAB-gNB based on implementation.

Proposal 2-3: The WAB-gNB can discover that a peer RAN node is a WAB-gNB based on a WAB-specific frequency, a WAB-specific PCI value and/or a WAB-specific indicator provided for a served cell in the Xn Setup procedure.

Proposal 2-4: WAB-related enhancements to Xn are ignored by a BH RAN node that does not support WAB.

## 2.3	Change of AMF for UEs served by WAB-node

According to the WID for RAN’s WI on Additional Topological Enhancements for NR, RAN3 should define support for the UE’s AMF change for UEs connected to, or camped on, a WAB-gNB [1].

During the study, RAN3 discussed procedures to support the UE’s AMF change for UEs connected to, or camped on, a WAB-gNB. RAN3 agreed that such AMF change can be conducted via a two-logical-gNB solution. RAN3 further contemplated the following options that only use a single logical WAB-gNB:

Option 1: Single WAB-gNB with a single cell using mobility registration update due to TAC change

Option 2: Single WAB-gNB with two cells with different TACs, using NG-based HO

Option 3: Single WAB-gNB with single cell without TAC change

In all three options, the WAB-gNB is concurrently connected to both AMFs. In options 1 and 2, the WAB-gNB reports a different TAC to each AMF. In Option 3, the WAB-gNB reports the same TAC to both AMFs.

Problem with Option 1:

CM-IDLE UE:

When the CM-IDLE UE observes the change of TAC in SI, it sends an MRU. The WAB-gNB will direct the MRU to the new AMF. The new AMF can pull the UE’s context from the old AMF. No problem here.

CM-CONNECTED UE:

When the CM-CONNECTED UE observes the change of TAC in SI, it sends an MRU. Since the UE is already CM-CONNECTED with context in the initial AMF, the WAB-gNB directs the MRU including the initial TAC to the initial AMF. The initial AMF has no reason to perform AMF reallocation since the initial TAC is still registered for the cell the UE is connected to. Therefore, enhancements would be necessary to RAN/CN procedures and/or AMF behavior to enable Option 1 for CM-CONNECTED UEs.

Observation 1: Enhancements are needed to RAN/CN procedures and/or AMF behavior to enable Option 1 for CM-CONNECTED UEs.

Problem with Option 2:

CM-IDLE UE:

When the initial cell is removed from the air interface, the CM-IDLE UE will reselect the new cell. Since the TAC has changed, the UE will send an MRU. The WAB-gNB will direct the MRU to the new AMF. The new AMF can pull the UE’s context from the old AMF. No problem here.

CM-CONNECTED UE:

The WAB-gNB will initiate an NG handover for the CM-CONNECTED UE from the initial cell to the new cell. When the initial AMF receives the HO Required message, it will not perform an AMF reallocation since the target cell belongs to the same gNB. Therefore, enhancements would be necessary to RAN/CN procedures and/or AMF behavior to enable Option 2 for CM-CONNECTED UEs.

Observation 2: Enhancements are needed to the AMF behavior to support Option 2 for CM-CONNECTED UEs.

Problem with Option 3:

CM-IDLE UE:

Since neither the cell nor TAC change during the AMF change, the CM-IDLE UE will not send an MRU, and therefore, its context will be retained in the initial AMF. This implies that when the WAB-node moves out of the initial AMF’s coverage area, the CM-IDLE UE cannot be paged anymore. Significant enhancements to existing RAN/CN procedures would be necessary to support Option 3 for CM-IDLE UEs.

CM-CONNECTED UE:

As for the CM-IDLE UE, the CM-CONNECTED UE will not invoke an MRU at the time of the AMF change. With only single cell, the WAB-gNB cannot cannot hand it over, and therefore, its context will be retained in the initial AMF. This implies that when the WAB-node moves out of the initial AMF’s coverage area, the CM-CONNECTED UE is losing AMF connectivity. Significant enhancements to existing RAN/CN procedures would be necessary to support Option 3 for CM-CONNECTED UEs.

Observation 3: Significant enhancements to existing RAN/CN procedures are necessary to support Option 3 for CM-IDLE and CM-CONNECTED UEs.

Based on observations 1, 2 and 3, all options considered for AMF change with a single logical WAB-gNB require enhancements to AMF and/or RAN/CN procedures. Since such enhancements are not needed for AMF change with two logical WAB-gNBs, the single-logical-WAB-gNB solutions should be deprioritized.

Proposal 3-1: When the UEs’ AMF changes for UEs connected to the WAB-node, the WAB-node to establish a second logical WAB-gNB and perform NG handover of these UEs to the second logical WAB-gNB as described in TR 38.799.

Proposal 3-2: Deprioritize solutions to facilitate AMF change using single-WAB-gNB since these solutions require enhancements to AMF and/or RAN/CN procedures which are not needed when using two logical WAB-gNBs.

## 2.4	Handling of WAB-gNB’s traffic during PDU session change

According to the WID for RAN’s WI on Additional Topological Enhancements, RAN3 should define the handling of the WAB-gNB’s traffic (including Xn, NG and OAM traffic) during WAB-node mobility, including the case where the WAB-MT’s BH PDU session changes [1].

The only critical scenario to be considered is when the WAB-MT’s PDU session changes. In this case, the IP address(es) of the WAB-MT change(s), too. NG-, Xn- and OAM traffic need to cope with the change of IP addresses.

The following solutions can be considered:

- IP address change for NG-C: According to TS 38.412, the WAB-gNB can establish a new SCTP association with the AMF using one of the new IP addresses and conduct the RAN CONFIGURATION UPDATE procedure as the first procedure via this SCTP association.

- IP address change for Xn-C: According to TS 38.422, the WAB-gNB can establish a new SCTP association with the peer gNB using one of the new IP addresses and conduct the RAN CONFIGURATION UPDATE procedure as the first procedure via this SCTP association.

- IP address change for NG-U: According to TS 38.413, the WAB-gNB can migrate the NG-U GTP-U tunnels to the new IP address(es) via the NGAP PDU session Resource Modify Indication procedure.

- IP address change for Xn-U: Xn-U is used for data forwarding during handover and for DC (split bearers, MN-terminated SCG bearer, SN-terminated MCG bearer). Data forwarding during handover usually covers only a short time frame. In case data forwarding is interrupted due to change of tunnel end point IP addresses, it may be easier to retransmit the packets on upper layers. For DC, the GTP-U tunnels can be migrated via the S-NG-RAN node MODIFICATION PROCEDURES.

- IP address change for OAM traffic: The impact of IP address change on OAM traffic is out-of-scope. Also, an equivalent scenario was already supported for IAB, where OAM traffic can be exchanged via the backhaul, whose IP addresses can change.

Proposal 4-1: When the WAB-MT changes the WAB-MT’s PDU session, the following legacy procedures can be used to migrate the WAB-gNB’s interfaces to the new IP addresses:

- NG-C and Xn-C can be migrated via legacy procedures defined in TS 38.412 and TS 38.422, respectively.

- NG-U GTP-U tunnels can be migrated via the NGAP PDU session Resource Modify Indication procedure.

- Xn-U GTP-U tunnels used for DC can be migrated via the Xn S-NG-RAN NODE MODIFICATION PROCEDURES.

- Xn-U GTP-U tunnels used during UE handover do not need to be migrated since short-lived.

Proposal 4-2: When the WAB-MT changes the WAB-MT’s PDU session, the migration of OAM traffic to the new IP address(es) is out of scope.

## 2.5	PCI configuration and collision avoidance

According to the WID for RAN’s WI on Additional Topological Enhancements for NR, RAN3 should define the handling of PCI collision avoidance [1]. The PCI configuration for gNB cells is defined in TS 38.401, clause 7.8. This clause further defines the following procedure for PCI collision avoidance for mobile IAB:

For mobile IAB deployments, the legacy mechanisms can be reused for PCI collision detection. The PCI space can be partitioned between mobile IAB cells and stationary cells by implementation.

Based on NOTE 1 in WID [1], these procedures should also be applied to handle PCI configuration and collision avoidance of WAB-gNB cells.

Proposal 5: PCI configuration and PCI collision avoidance to follow the same procedure as defined for mobile IAB in TS 38.401, clause 7.8.

## 2.6	Stage-2 specification

RAN3 to capture agreements on the above proposals on stage 2.

Proposal 6-1: RAN3 to capture the above proposals, if agreed, on stage-2.

In the last meeting, RAN3 discussed on how to split stage-2 definitions of WAB between TS 38.300 and TS 38.401 [3].

Since the WI on Additional Topological Enhancements for NR includes WAB and NR Femto, they both should be captured in the same stage-2 spec(s). Presently, RAN3 has endorsed CRs to TS 38.300 for NR Femto. Therefore, TS 38.300 should also include an introduction to WAB. All the details of WAB (authorization, network integration, WAB-node mobility, etc.) can then be captured in TS 38.401. This approach is also well aligned with the specification for mobile IAB.

Proposal 6-2: As for NR Femto, the introduction of WAB to be included in TS 38.300. Specification of WAB details to be included in TS 38.401 following endorsed BL CR 0439 to TS 38.401 in R3-245840.

A BL draft CR to 38.300 has been submitted separated.

In the Annex below, please find a TP for the BL CR to TS 38.401.

Proposal 6-3: RAN3 to approve TP for BL CR to S 38.401 in the Annex.

# 3	Conclusion

This contribution discussed the aspects of WAB. The following observations and proposals have been made:

Authorization of WAB-node

Proposal 1-1: In alignment with SA2, RAN3 confirms that the WAB-MT is authorized to support a PDU session for backhauling based on WAB-specific S-NSSAI and DNN, and that this authorization can be time- and location-dependent.

Proposal 1-2: In alignment with SA2, RAN3 confirms that the WAB-gNB’s service authorization is based on OAM configuration.

Proposal 1-3: In alignment with SA2, RAN3 confirms that when the WAB-gNB’s authorization status changes from “authorized” to “not authorized”, the UEs served by the WAB-gNB should be handed over to other RAN nodes or released, and then the WAB-gNB’s NG and Xn connection(s) should be removed.

Proposal 1-4: When the WAB-gNB’s authorization status changes from “not authorized” to “authorized”, the WAB-gNB can reestablish NG connection and potentially Xn connection(s) and start serving UEs.

Proposal 1-5: The WAB-MT should be authorized to support BH PDU sessions while the collocated WAB-gNB is serving UEs and has backhaul connections established. RAN3 assumes that this time alignment can be achieved based on configuration.

Avoiding Xn between WAB-gNBs

Proposal 2-1: Since the WAB-gNB supports the functionality of a legacy gNB, it can establish Xn connections with BH gNB’s and WAB-gNBs, and it can use the legacy procedures to discover neighbour gNBs and resolve their TNL addresses.

Proposal 2-2: The WAB-gNB can prevent Xn establishment with a peer WAB-gNB based on implementation.

Proposal 2-3: The WAB-gNB can discover that a peer RAN node is a WAB-gNB based on a WAB-specific frequency, a WAB-specific PCI value and/or a WAB-specific indicator provided for a served cell in the Xn Setup procedure.

Proposal 2-4: WAB-related enhancements to Xn are ignored by a BH RAN node that does not support WAB.

Change of AMF for UEs served by WAB-node

Observation 1: Enhancements are needed to RAN/CN procedures and/or AMF behavior to enable Option 1 for CM-CONNECTED UEs.

Observation 2: Enhancements are needed to the AMF behavior to support Option 2 for CM-CONNECTED UEs.

Observation 3: Significant enhancements to existing RAN/CN procedures are necessary to support Option 3 for CM-IDLE and CM-CONNECTED UEs.

Proposal 3-1: When the UEs’ AMF changes for UEs connected to the WAB-node, the WAB-node to establish a second logical WAB-gNB and perform NG handover of these UEs to the second logical WAB-gNB as described in TR 38.799.

Proposal 3-2: Deprioritize solutions to facilitate AMF change using single-WAB-gNB since these solutions require enhancements to AMF and/or RAN/CN procedures which are not needed when using two logical WAB-gNBs.

Handling of WAB-gNB’s traffic during PDU session change

Proposal 4-1: When the WAB-MT changes the WAB-MT’s PDU session, the following legacy procedures can be used to migrate the WAB-gNB’s interfaces to the new IP addresses:

- NG-C and Xn-C can be migrated via legacy procedures defined in TS 38.412 and TS 38.422, respectively.

- NG-U GTP-U tunnels can be migrated via the NGAP PDU session Resource Modify Indication procedure.

- Xn-U GTP-U tunnels used for DC can be migrated via the Xn S-NG-RAN NODE MODIFICATION PROCEDURES.

- Xn-U GTP-U tunnels used during UE handover do not need to be migrated since short-lived.

Proposal 4-2: When the WAB-MT changes the WAB-MT’s PDU session, the migration of OAM traffic to the new IP address(es) is out of scope.

PCI configuration and collision avoidance

Proposal 5: PCI configuration and PCI collision avoidance to follow the same procedure as defined for mobile IAB in TS 38.401, clause 7.8.

Stage-2 specification

Proposal 6-1: RAN3 to capture the above proposals, if agreed, on stage-2.

Proposal 6-2: As for NR Femto, the introduction of WAB to be included in TS 38.300. Specification of WAB details to be included in TS 38.401 following endorsed BL CR 0439 to TS 38.401 in R3-245840.

Proposal 6-3: RAN3 to approve TP for BL CR to S 38.401 in the Annex.

# Reference

[1]  RP-242395, New WID on additional topological enhancements for NR, 3GPP TSG RAN Meeting #105, Melbourne, Australia, September 2024

[2] S2-2411238, CR5688 to TS 23.501, 3GPP TSG SA WG2#165, Hyderabad, India, October 2024

[3]  Chair notes, 3GPP TSG RAN WG3 Meeting #126, Hefei, China, October 2024

# Annex 2: TP for BL CR to 38.401

START OF CHANGE

### 6.1.x	Wireless Access Backhaul architecture

A WAB-node consists of a WAB-gNB and a WAB-MT. The WAB-gNB is based on the gNB functionality specified in TS 38.300 [2] and serves UEs by means of a terrestrial NR Uu radio link.

The WAB-MT is served by a BH-RAN-node. The WAB-gNB traffic, including NG, Xn and OAM traffic is transported via backhaul PDU session(s) of the WAB-MT.

NOTE: The use of other types of backhaul, e.g. non-3GPP backhaul, is up to implementation.

The WAB-gNB and the WAB-MT may connect to the same PLMN or to different PLMNs.

Figure 6.1.x-1 shows the WAB architecture for 5GS.

Figure 6.1.x-1: The WAB architecture

In in-band scenarios, backhaul and access of the WAB-node use terrestrial radio links. In out-of-band scenarios, the backhaul can use a terrestrial or a non-terrestrial radio link, while the access uses terrestrial radio link.

The WAB-MT may connect to a public PLMN or an SNPN.

The WAB-gNB may connect to a public PLMN or an SNPN.

Figure 6.1.x-2 shows protocol stacks for NG Control plane and NG User plane transport via the wireless backhaul.

Figure 6.1.x-2: Protocol stacks for NG Control plane and NG User plane transport

Figure 6.1.x-3 shows protocol stacks for Xn Control plane and Xn User plane transport for WAB-node.

The WAB-gNB can establish Xn connections with the BH gNB and/or with other surrounding RAN nodes. The WAB-gNB can use legacy procedures to discover a neighbour RAN-node and resolves its TNL address.

The WAB-gNB can prevent establishment of Xn connections with other WAB-gNBs based on implementation. The WAB-gNB can detect the WAB functionality of another RAN-node via WAB-specific frequency, PCI value or WAB-indicator contained in the served-cell information of the Xn SETUP REQUEST or Xn SETUP REQUEST messages.

= = = = > SKIP

## X.Y	WAB-node Authorization

The WAB-MT needs to be authorized to support PDU sessions for backhauling. This authorization is based on WAB-specific S-NSSAI(s) and DNN as defined in TS 23.501[3]. This authorization can be time- and location-dependent.

The WAB-gNB needs to be authorized to provide services to UEs. This authorization is based on OAM configuration as defined in TS 23.501[3].

When the WAB-gNB’s authorization status changes from “authorized” to “not authorized”, the UEs served by the WAB-gNB should be handed over to other RAN nodes or released. Then, the WAB-gNB’s NG and Xn connection(s) should be removed.

When the WAB-gNB’s authorization status changes from “not authorized” to “authorized”, the WAB-gNB can reestablish NG connection and potentially Xn connection(s), and then start serving UEs.

The authorization of WAB-gNB and collocated WAB-MT should be time-aligned so that the WAB-MT can provide backhaul support while the WAB-gNB has backhaul connections established and is serving UEs. This time alignment is achieved via implementation as defined in TS 23.501[3].

## X.Y	WAB-node mobility

= = = = > SKIP

### X.Y.Z+1	Change of the AMF(s) for UEs served by the WAB-node

Due to WAB-node movement, the AMF(s) of the UE(s) served by the WAB-node may have to be changed, e.g., based on the WAB-node’s current location and/or additional criteria.

The procedure contains the following steps:

1.	The WAB-node instantiates a new logical WAB-gNB, and it establishes NG connection(s) towards one or more new AMF(s). The new logical WAB-gNB may also establish Xn connection(s) with the BH RAN or with other surrounding RAN nodes. The WAB-node may obtain the configuration parameters needed to establish the connection to the UE’s new AMF(s) via OAM.

2.	The new logical WAB-gNB may activate one or more new cells with cell configuration parameters related to the WAB-gNB’s current location. The TAC broadcasted by the new cell(s) is different from the TAC broadcasted by the old cell(s).

3.	The UEs are handled as follows:

-	A UE in RRC_CONNECTED state is handed over between an old cell served by the old logical WAB-gNB and a new cell served by the new logical WAB-gNB via NG-based handover with AMF relocation, as defined in TS 23.502 [7]. After all UEs served by the WAB-node have been handed over, the old cell(s) are removed from service.

-	When the old cell(s) are removed from service, a UE in RRC_IDLE or RRC_INACTIVE state that has been camping on an old cell reselects a new cell and performs a Mobility Registration Update procedure as defined in TS 23.502 [7]. The Mobility Registration Update is triggered by the change of TAC between old and new cell.

4.	The NG connection(s) between the old logical WAB-gNB and the initial AMF(s), and all Xn connection(s) between the old logical WAB-gNB and the BH RAN and/or other RAN nodes are removed, and the WAB-gNB is removed from service.

### X.Y.Z+2 Handling of WAB-gNB’s traffic during PDU session change

When the WAB-MT’s PDU session used for backhauling changes, the WAB-gNB needs to update the IP addresses used for its backhaul traffic. The following procedures can be used:

- IP address change for NG-C: According to TS 38.412, the WAB-gNB can establish a new SCTP association with the AMF using one of the new IP addresses and conduct the RAN CONFIGURATION UPDATE procedure as the first procedure via this SCTP association.

- IP address change for Xn-C: According to TS 38.422, the WAB-gNB can establish a new SCTP association with the peer gNB using one of the new IP addresses and conduct the RAN CONFIGURATION UPDATE procedure as the first procedure via this SCTP association.

- IP address change for NG-U: According to TS 38.413, the WAB-gNB can migrate the NG-U GTP-U tunnels to the new IP address(es) via the NGAP PDU SESSION RESOURCE MODIFY INDICATION procedure.

- IP address change for Xn-U: Xn-U is used for data forwarding during handover and for DC (split bearers, MN-terminated SCG bearer, SN-terminated MCG bearer). Data forwarding during handover usually covers only a short time frame. In case data forwarding is interrupted due to change of tunnel end point IP addresses, it may be easier to retransmit the packets on upper layers. For DC, the GTP-U tunnels can be migrated via the S-NG-RAN NODE MODIFICATION procedures.

- IP address change for OAM traffic: The impact of IP address change on OAM traffic is out-of-scope. Also, an equivalent scenario was already supported for IAB, where OAM traffic can be exchanged via the backhaul, whose IP addresses can change.

= = = = > SKIP

## 7.8	PCI Optimisation Function

The PCI Optimization Function in non-split gNB case is specified in TS 38.300 [2].

In split gNB architecture, the OAM configures a PCI for each NR cell to the gNB-DU.

For centralized PCI assignment in split gNB architecture, the gNB-CU detects PCI conflict of NR cells and reports the NR cells suffering PCI conflict to OAM directly. The OAM is in charge of reassigning a new PCI for the NR cell subject to PCI conflict.

For distributed PCI assignment in split gNB architecture, the OAM assigns a list of PCIs for each NR cell and sends the configured PCI list to the gNB-CU. If the gNB-CU detects PCI conflict, the gNB-CU may select a new PCI value from the preconfigured PCI list for the NR cell and send it to the gNB-DU by either F1 Setup procedure or gNB-CU configuration update procedure.

For mobile IAB and/or WAB deployments, the legacy mechanisms can be reused for PCI collision detection. The PCI space can be partitioned between mobile IAB cells, WAB cells and stationary cells by implementation.

END OF CHANGE