3GPP TSG-RAN WG3 Meeting #126                                                            R3-247230

Orlando, Florida, USA, November 18 - 22, 2024

Agenda item: 		17.3

Source: 		Qualcomm Incorporated

Title: 	Aspects of on-demand SIB1 for NES enhancements

Document for:		Discussion

# 1	Introduction

In the last TSG RAN meeting, the WID for enhancements of network energy savings for NR was updated [1]. The updated WID includes the following objectives for OD SIB1:

- Specify support for on-demand SIB1 for UEs in idle/inactive mode [RAN1/2/3] - Specify procedures and signaling method(s) for Case 2 [RAN1/2] - Case 2: UE obtains UL WUS configuration from Cell A, UE transmits UL WUS on NES Cell, UE receives on-demand SIB1 from NES Cell - Triggering method by UL WUS using PRACH - Specify inter NG-RAN node signalling at least for the configuration of UL WUS [RAN3] - Note 1: No modification of SSB will be discussed under this objective - Note 2: - UL WUS: Uplink wake-up signal - Cell A: A cell that is periodically transmitting at least its own SIB1 - NES Cell: A cell that may transmit SIB1 transmission in response to UL WUS from a UE - Note 3: - RAN1 strives to minimize impact to legacy UE - RAN1 specification impact to support this feature should be minimized

The chair notes of RAN3#125bis include the following agreements and to-be-continued topics on OD SIB1 [1]:

The UL WUS configuration will be transferred over Xn via a new class1 defined procedure. Introduce the start/stop mechanism from NES cell to Cell A for UL WUS configuration broadcast. Further check on first activation of UL WUS configuration broadcast in Cell A. Cell A gNB can decide and signal to NES Cell gNB that it stops the UL WUS configuration broadcast in its SIB. To be continued...

This contribution aims to make further progress for the Case-2 OD-SIB1 scenario.

# 2	Discussion

## 2.2	Xn signaling related to UL WUS configuration

RAN3 agreed that a new class-1 procedure is introduced for the NES-cell-hosting CU to send the UL WUS configuration to the Cell-A-hosting CU.

RAN3 further agreed that the Cell-A-hosting CU can decide and signal to the NES-cell-hosting gNB that it stops transmission of the UL WUS configuration.

In an offline meeting, RAN3 also discussed on whether there should be a (re)start mechanism from NES cell to Cell A for UL WUS configuration broadcast, but no agreements were reached. In this offline, it was also discussed whether potential delay between the confirmation of the UL WUS transmission and start of the execution of this transmission would require additional signaling messages.

Moving forward, RAN3 might be able to make faster progress by first converging on the high-level information exchange between gNBs related to OD SIB1 before discussing detailed procedure or stage-3 enhancements.

Proposal 1-0: RAN3 to first converge on the high-level information exchange related to OD-SIB1 that should occur over Xn before discussing detailed procedures or stage-3 enhancements.

Rel-19 NES cells can only be used by Rel-19 UEs. It may be beneficial for a gNB to be informed about the peer-gNB-cell’s OD SIB1 functionality so that it does not consider this cell as target for handover of legacy UEs.

Proposal 1-1: The NES-cell-hosting gNB to inform each peer gNB about the OD SIB1 functionality of its NES cells so that the peer gNB does not consider these cells for handover of legacy UEs.

RAN3 agreed that the NES-cell-hosting gNB needs to send the NES-cell’s UL WUS configuration to the Cell-A-hosting gNB, so that Cell-A can transmit this configuration when needed. The conveyance of the UL WUS configuration does not need an acceptance/rejection indication by the Cell-A-hosting gNB since the information does not contain a request.

Proposal 1-2: RAN3 confirms that the NES-cell-hosting gNB informs the Cell-A-hosting gNB about the relevant UL WUS configurations so that Cell-A can transmit these configurations when needed. No acceptance/rejection reply needed.

The Cell-A-hosting gNB should know the cells that are using the UL WUS configuration it received. This allows the Cell-A-hosting gNB to assess the benefit of transmitting the configuration. This assessment may be based, e.g., on inter-cell overlap, which may be derived from the handover rate from a Cell-A to a NES cell.

Proposal 1-3: The NES-cell hosting gNB to include with the UL WUS configuration the cells that use this configuration, so that the Cell-A-hosting gNB can assess the benefit of transmitting the configuration, e.g., based on inter-cell overlap.

Before Cell-A transmits the UL WUS configuration of a NES-cell, the NES-cell may already assume operation using SIB1-broadcast mode. Also, at a later point in time, the NES-cell may decide to switch between OD-SIB1 and SIB1-broadcast mode. When the NES-cell is in SIB1-broadcast mode for a longer period of time, Cell-A should not be mandated to transmit the UL WUS configuration on behalf of this NES-cell, e.g., to enable energy savings for Cell A.

Proposal 1-4: To support energy savings for Cell-A, Cell-A should not be mandated to broadcast UL WUS configuration on behalf of a NES cell that is in SIB1-broadcast mode for an extended period of time.

Therefore, the NES-cell-hosting gNB needs to be able to request from the Cell-A-hosting gNB to start/stop the transmission of an UL WUS configuration. The NES-cell-hosting gNB may use the start functionality, e.g., when the NES cell intends to switch from SIB1-broadcast to OD SIB1 mode. It may use the stop functionality, e.g., when the NES cell intends to switch from OD SIB1 to SIB1-broadcast mode.

The Cell-A-hosting gNB should send an acceptance/rejection in response to a start request. It need not, but could, provide an acceptance/rejection indication in response to a stop request.

Proposal 1-5: The NES-cell-hosting gNB can request from the Cell-A-hosting gNB to (re)start/stop transmission of an UL WUS configuration, and the Cell-A-hosting gNB should accept or reject this request for this specific configuration.

In analogue manner, the Cell-A-hosting gNB should be able to inform the NES-cell-hosting gNB to stop/(re)start the transmission of the UL WUS configuration. The Cell-A-hosting gNB may use the stop functionality, e.g., in case it wants to save resources and/or energy due to high load conditions on Cell-A. The Cell-A-hosting gNB should be able to inform the NES-cell-hosting gNB that it (re)starts the transmission of the UL WUS configuration, e.g., in case the high load conditions have been alleviated. The NES-cell-hosting gNB need not provide an acceptance/rejection indication in response.

Proposal 1-6: The cell-A-hosting gNB can inform the NES-cell-hosting gNB that it will stop or restart transmission of an UL WUS configuration. No acceptance/rejection reply needed.

When the NES-cell-hosting gNB receives from the Cell-A-hosting gNB a stop (or restart) indication for an UL WUS configuration, the affected NES cell(s) can be switched from OD-SIB1 to SIB1-broadcast mode (or from SIB1-broadcast to OD SIB1 mode).

Proposal 1-7: RAN3 assumes that when the NES-cell-hosting gNB receives a stop (or restart) indication for an UL WUS configuration, the affected NES cell(s) can be switched from OD-SIB1 to SIB1-broadcast mode (or from SIB1-broadcast to OD-SIB1 mode).

## 2.2	F1 signaling related to UL WUS configuration

RAN3 agreed that the UL WUS configuration should be sent from NES-cell DU to NES-cell CU via F1. The question arises whether an existing procedure can be used for this purpose, or whether a new procedure should be introduced.

Since the UL WUS configuration is part of the NES-cell DU configuration, it makes sense to have the NES-cell DU include this configuration in the F1 SETUP REQUEST message and send updates via the F1 gNB-DU CONFIGURATION UPDATE message.

Proposal 2-1: Since OD SIB1 represents a semi-static (and not dynamic) feature of the NES-cell-hosting DU, the NES-cell-hosting DU to include the UL WUS configuration in the F1 SETUP REQUEST message and send updates via the F1 gNB-DU CONFIGURATION UPDATE message.

When the NES-cell-hosting DU has provided its served-cell NES cell information to its CU, the CU can activate these cells and the cells can start service. Since negotiation with Cell-A-hosting gNBs for the transmission of the NES-cell’s UL WUS configuration may take some time and it is not even guaranteed to succeed, the activation of the NES cell should not permit the NES cell to enter OD SIB1 mode. Instead, the NES-cell-hosting DU should only enter OD SIB1 mode unless it has been explicitly permitted by its CU.

Proposal 2-2: Since the transmission of the UL WUS configuration via relevant Cell-A(s) needs confirmation and is not always guaranteed, the NES-cell-hosting DU to only apply OD SIB1 mode when explicitly permitted by its CU.

It can be expected that the NES-cell-hosting CU will take some time to forward UL WUS configurations to Cell-A-hosting peers and obtain confirmation about the transmission of this configuration. Also, activation of the transmission of the UL WUS configuration may take time even after the Cell-A-hosting peer has confirmed the transmission. For these reasons, the NES-cell-hosting CU should provide the permission to enter OD SIB1 mode in an asynchronous manner, i.e., via separate message. For this purpose, the GNB-DU CONFIGURATION UPDATE message can be used. In case the CU wishes to revoke OD SIB1 mode it can do so via the same message.

Proposal 2-3: The CU to provide/revoke permission for OD SIB1 mode to the NES-cell-hosting DU via the F1 GNB-CU CONFIGURATION UPDATE message.

# 3	Conclusion

This contribution aimed to make further progress for the Case-2 OD-SIB1 scenario. The following observations and proposals have been made:

Xn signaling related to UL WUS configuration

Proposal 1-0: RAN3 to first converge on the high-level information exchange related to OD-SIB1 that should occur over Xn before discussing detailed procedures or stage-3 enhancements.

Proposal 1-1: The NES-cell-hosting gNB to inform each peer gNB about the OD SIB1 functionality of its NES cells so that, e.g., the peer gNB does not consider these cells for handover of legacy UEs.

Proposal 1-2: RAN3 confirms that the NES-cell-hosting gNB informs the Cell-A-hosting gNB about the relevant UL WUS configurations so that Cell-A can transmit these configurations when needed. No acceptance/rejection reply needed.

Proposal 1-3: The NES-cell hosting gNB to include with the UL WUS configuration the cells that use this configuration, so that the Cell-A-hosting gNB can assess the benefit of transmitting the configuration, e.g., based on inter-cell overlap.

Proposal 1-4: To support energy savings for Cell-A, Cell-A should not be mandated to broadcast UL WUS configuration on behalf of a NES cell that is in SIB1-broadcast mode for an extended period of time.

Proposal 1-5: The NES-cell-hosting gNB can request from the Cell-A-hosting gNB to (re)start/stop transmission of an UL WUS configuration, and the Cell-A-hosting gNB should accept or reject this request for this specific configuration.

Proposal 1-6: The cell-A-hosting gNB can inform the NES-cell-hosting gNB that it will stop or restart transmission of an UL WUS configuration. No acceptance/rejection reply needed.

Proposal 1-7: RAN3 assumes that when the NES-cell-hosting gNB receives a stop (or restart) indication for an UL WUS configuration, the affected NES cell(s) can be switched from OD-SIB1 to SIB1-broadcast mode (or from SIB1-broadcast to OD-SIB1 mode).

F1 signaling related to UL WUS configuration

Proposal 2-1: Since OD SIB1 represents a semi-static (and not dynamic) feature of the NES-cell-hosting DU, the NES-cell-hosting DU to include the UL WUS configuration in the F1 SETUP REQUEST message and send updates via the F1 gNB-DU CONFIGURATION UPDATE message.

Proposal 2-2: Since the transmission of the UL WUS configuration via relevant Cell-A(s) needs confirmation and is not always guaranteed, the NES-cell-hosting DU to only apply OD SIB1 mode when explicitly permitted by its CU.

Proposal 2-3: The CU to provide/revoke permission for OD SIB1 mode to the NES-cell-hosting DU via the F1 GNB-CU CONFIGURATION UPDATE message.

# Reference

[1]  RP-242354, TSG RAN meeting #105, Melbourne, Australia, September 2024

[1]  Chair notes, RAN WG3 meeting #125bis, Hefei, China, October 2024