3GPP TSG-RAN WG3 Meeting #126	R3-247232

Orlando, USA, Nov 18th - Nov 22nd, 2024

Agenda item:	13.2

Source:	Qualcomm Incorporated, Reliance JIO, Vodafone, NTT DOCOMO, Sony

Title:	Comparison of RRC/PDCP anchor and anchor switch-based solutions for inter-gNB LTM

WID/SID:	NR_Mob_Ph4-Core – Release 19

Document for:	Discussion and decision

# Introduction

In the last RAN3 meeting (RAN3 #125-bis [1]), progress was achieved in developing the procedures of inter-CU LTM considering the case when the gNB-CU can be switched at LTM cell switch.

RAN3 #125-bis agreements:

Current SSB information in Xn Setup and Configuration Update procedures can be reused for LTM preparation phase.

WA: For inter-CU LTM mobility, a separate LTM request message (i.e. HANDOVER REQUEST message) is used for each candidate cell.

The error handling of multiple UE associations need to be considered.

WA: The source gNB-CU sends the CSI resource configuration of candidate cells to other candidate gNB-CUs via Handover Request message, and the candidate gNB-CU sends the CSI report configuration to the source gNB-CU via Handover Request ACK message.

The LTM Configuration IDs are allocated by the source CU.

WA: Reuse the existing XnAP UE CONTEXT RELEASE message at the source gNB if no LTM candidate cell(s) exist in the source gNB.

Follow F1AP, the source gNB-CU sends the CSI resource configuration of candidate cells to candidate gNB-CUs via Handover Request message for subsequent LTM, and the candidate gNB-CU sends the CSI report configuration to the source gNB-CU via Handover Request ACK message.

How the source gNB-CU sends the reference configuration to all candidate gNBs is pending on RAN2 progress.

Confirm the name of the new procedure as “LTM Configuration Update”.

Introduce a new procedure for candidate gNB-initiated LTM cancellation.

Allow UE association in-between candidate CUs (in case the Xn connectivity existed) for subsequent LTM. When and how to establish the UE association is FFS.

The RRC/PDCP anchor solution for Rel-19 inter-CU LTM handover is characterized by the fact that the UE remains connected to the source gNB-CU after every inter-gNB LTM cell switch and has been discussed in our contributions in previous RAN3 meetings, which highlight its many advantages; see, e.g., [2], [3]. Until now, there was no technical comparison analysis done for different solutions and RAN3 started Stage-3 detailed discussions directly for anchor switch-based solution without discussing the pros and cons of different solutions.

In this contribution, we provide a comparative analysis of the RRC/PDCP anchor and anchor switch-based solutions for inter-gNB LTM, considering the following primary aspects.

- Compliance with the WID,

- NG-RAN architecture aspects,

- Signalling and standards impact: LTM preparation phase,

- Signalling and standards impact: Other signalling aspects,

- Impact on the UE,

- RRC anchor tracking,

- Race conditions and RLF.

# 2 Compliance with the WID

| RRC/PDCP anchor solution | Anchor switch-based solution |
| --- | --- |
| The RRC/PDCP anchor solution does not require security update upon an inter-gNB cell switch. This solution is thus compliant with the WID, since RRC reconfiguration message is not required to be sent to the UE upon a cell switch in inter-CU LTM. Observation 1a. The RRC/PDCP anchor solution is compliant with the WID, because for subsequent inter-gNB LTM HO, RRC reconfiguration message is not required to be sent to the UE upon a cell switch. | For subsequent inter-gNB LTM, the anchor switch-based solution requires security update upon an inter-gNB cell switch. This requirement is not aligned with the following WID objective: “Specify support for subsequent LTM mobility procedures aiming to avoid RRC configuration between cell switches as per Rel-18 LTM - Coordination with SA3 needed with respect to security key handling” Observation 1b. The anchor switch-based solution is not compliant with the current WID IF the solution requires, for subsequent inter-gNB LTM, an RRC reconfiguration upon an inter-gNB cell switch, which provides the new security update, or at any LTM re-preparation. |

# 3 NG-RAN architecture aspects

| RRC/PDCP anchor solution | Anchor switch-based solution |
| --- | --- |
| “We believe that such abovementioned facts are a violation to the current NG-RAN architecture.” Contributions in previous RAN3 meetings (see, e.g., [4]) make an incorrect observation that the PDCP anchor solution violates the current NG-RAN architecture: This observation assumes that inter-gNB F1 interfaces are necessary for the PDCP anchor solution, which is incorrect. Figure 1: Inter-gNB LTM with fixed RRC/PDCP anchor is transparent to gNB-DUs. No inter-gNB F1 interface is needed. Figure 1 illustrates the scenario of two UEs: - UE1 is initially connected to gNB1-DU and performs inter-gNB LTM cell switch to gNB2-DU but remains RRC/PDCP-connected to gNB1-CU - UE2 has always been connected to gNB2-DU and RRC-connected to gNB2-CU. Each of gNB1-DU and gNB2-DU establish a single F1 connection to gNB1-CU and gNB2-CU, respectively. In the scenario of Figure 1, gNB2-DU is not aware that UE1 and UE2 are RRC-connected to different gNB-CUs. Instead, gNB2-DU forwards all signaling of UE1 and UE2 to gNB2-CU. gNB2-CU figures out that RRC signaling of UE1 shall be forwarded to gNB1-CU, whereas RRC signaling of UE2 terminates at gNB2-CU. This is feasible based on information exchanged during LTM preparation b/w the two gNB-CUs, where gNB1-CU informs gNB2-CU that UE1 will stay anchored at gNB1-CU in case an inter-gNB cell switch to gNB2-DU occurs in the future. The same principle was followed for R17 SDT with partial UE context transfer and DC based MN terminated SCG and SN terminated MCG bearers, which is already compliant with 5G RAN Architecture and same applies for RRC/PDCP Anchor based solution as well. Observation 2a. Inter-gNB LTM with RRC/PDCP anchor is compliant with the current NG-RAN architecture principles, which is similar as R17 SDT with partial UE context transfer and NR-DC MN/SN terminated SCG/MCG bearer concept. Observation 2b. No cross-gNB F1-C interface is needed between the source gNB-CU and target gNB-DU after LTM cell switch. | The anchor switch-based solution complies with the existing NG-RAN architecture, so there is nothing to note here. Observation 2c. Inter-gNB LTM with anchor switch-based solution is also compliant with NG-RAN Architecture principles. |

# 4 Signalling and standards impacts: LTM preparation phase

| RRC/PDCP anchor solution | Anchor switch-based solution |
| --- | --- |
| For the RRC/PDCP anchor solution, the existing Xn Handover Request procedure can be reused during the LTM preparation phase. We briefly highlight some fundamental aspects regarding the use of the Xn Handover Request procedure during LTM preparation; for more details, please see our contribution [5]. - An explicit indication is included in the Handover Request message to indicate that the source gNB-CU will remain as the anchor upon inter-gNB LTM cell switch. - The source gNB-CU determines and includes the mapping from QoS flows of the PDU sessions to DRBs for the candidate cell configuration and includes it in a DRB To Be Setup List IE (similar as the existing IE in the UE Context Setup Request message). - The mandatory UE Context Information > RRC Context IE is included in the Handover Request message and information in this IE such as UE capabilities and source cell configuration need to be forwarded by the candidate gNB-CU to the candidate gNB-DU, which is legacy behavior. Observation 3a. For the RRC/PDCP anchor solution, the existing Xn Handover Request procedure can be reused during LTM preparation. - An explicit indication is included in the Handover Request message to indicate to a candidate gNB-CU that the source gNB-CU will remain as the anchor upon LTM cell switch. - The source gNB-CU determines and includes the mapping from QoS flows of the PDU sessions to DRBs for the candidate cell configuration and includes it in a DRB To Be Setup List IE, included in Handover Request. - The mandatory UE Context Information > RRC Context IE is included in Handover Request and information in this IE such as UE capabilities and source cell configuration need to be forwarded by the candidate gNB-CU to the candidate gNB-DU, which is legacy behavior. | For the anchor switch-based solution, specification work is currently ongoing in RAN3 to define the signaling support for the following main procedures in LTM preparation. - LTM candidate cell configuration, - Early TA RACH configuration, - TA Information Transfer, - LTM CSI configuration and TCI state configuration. In our contribution [5], we provide the signalling details of these procedures for the RRC/PDCP anchor solution. The signalling overhead is found to be similar as the anchor switch-based solution, and majority of the steps in the procedures are the same for the two solutions. Observation 3b. The Stage-2 signalling procedures for inter-gNB LTM preparation for both the anchor switch-based solution and the RRC/PDCP anchor solution are similar, and there exist some Stage-3 differences. - LTM candidate cell configuration, - Early TA RACH configuration, - TA Information Transfer, - LTM CSI configuration and TCI state configuration. |

# 5 Signalling and standards impacts: Other signalling aspects

| RRC/PDCP anchor solution | Anchor switch-based solution |
| --- | --- |
| The RRC/PDCP anchor solution involves significantly less network signalling overhead compared with the anchor switch-based solution because: - There is no security update needed after each inter-gNB LTM cell switch. - There is no need of data forwarding or path switch after each LTM cell switch. - There is no need to establish new UE associations between the source gNB and the other candidate gNBs after an LTM cell switch. Observation 4a. For supporting subsequent LTM in case of the RRC/PDCP anchor solution, the backhaul signalling overhead is significantly less compared with the anchor switch-based solution because: - There is no security update needed after each inter-gNB LTM cell switch. - There is no need of data forwarding or path switch after each LTM cell switch. - There is no need to establish new UE associations between the source gNB and the other candidate gNBs after each LTM cell switch. In our contributions in the previous RAN3 meetings (RAN3 #125 [6], RAN3 #125-bis [7]), signalling solutions were provided for the following aspects. It was shown that no significant signalling overhead is expected to support these aspects. - Support for UE associated F1-AP and/or NG-AP procedures when the anchor gNB-CU communicates with the target gNB-DU after LTM cell switch. - Handling of NAS messages. Observation 4b. For the RRC/PDCP anchor solution, signalling solutions were provided for the following cases, and it was shown that additional overhead is not significant to support these cases. - Support for UE associated F1-AP and/or NG-AP procedures when the anchor gNB-CU communicates with the target gNB-DU after LTM cell switch. - Handling of NAS messages. | For subsequent LTM, the anchor switch-based solution requires security update upon an inter-gNB cell switch. Consequently, after completion of an inter-gNB LTM cell switch, the new serving gNB needs to share with each of the candidate gNBs the AS security key that needs to be applied at the next inter-gNB LTM cell switch. This exchange of AS security information is part of the UE context information and needs to be repeated after each inter-gNB cell switch for both horizonal and vertical key derivation. Furthermore, for subsequent LTM, the following also contribute significantly to the signalling overhead. - Exchange of data forwarding information, e.g., tunnel address information, between each gNB and every other gNB, as explained in [9]. - Signalling overhead due to the need to establish pairwise UE associations between the new serving gNB and each of the candidate gNBs for signalling purposes, after each inter-gNB LTM cell switch. This was discussed in RAN3#125-bis [1] and adds additional complexity. “Allow UE association in-between candidate CUs (in case the Xn connectivity existed) for subsequent LTM. When and how to establish the UE association is FFS.” Observation 4c. For supporting subsequent LTM in case of anchor switch-based solution, the backhaul preparation/signaling overhead incurred is significantly large because of the following reasons. - Transmission of updated UE context information, including new AS security information, between the new serving gNB and each of the candidate gNBs after each inter-gNB LTM cell switch. - Exchange of data forwarding information, e.g., tunnel address information, between each gNB and every other gNB. - Signalling overhead due to the need to establish pairwise UE associations between the new serving gNB and each of the candidate gNBs, after each inter-gNB LTM cell switch. |

# 6 Impact on the UE

| RRC/PDCP anchor solution | Anchor switch-based solution |
| --- | --- |
| In case of the RRC/PDCP anchor solution, for subsequent LTM, there is no security update at the UE nor is there any RRC reconfiguration towards the UE, at an LTM cell switch. This enables the UE to have similar performance as Rel-18 LTM during LTM cell switch. Observation 5a. In case of the RRC/PDCP anchor solution, for subsequent LTM, the complexity and user plane performance impact at the UE during LTM cell switch is minimal because: - There is no security update during LTM cell switch, and hence there is no PDCP re-establishment, - There is no RRC reconfiguration towards the UE during inter-gNB LTM cell switch. | In the case of the anchor switch-based solution, for subsequent inter-gNB LTM, the UE will need to support the following upon an inter-gNB cell switch. - Support PDCP reestablishment procedure, - Support RRC reconfiguration involving security key update, - Maintain awareness of intra-gNB versus inter-gNB mobility. The UE will also experience at every inter-gNB cell switch: - Increased data interruption due to RRC reconfiguration message processing, - Increased data interruption due to PDCP reestablishment. The handover timeline will also be impacted because of the above reasons and will be worse than the Rel-18 LTM cell switch timeline, thus undermining the benefits of inter-gNB LTM when compared with legacy L3 HO. Observation 5b. In case of the anchor switch-based solution, for subsequent LTM, there is significant complexity of handling and inferior performance at the UE upon every inter-gNB cell switch, because of the following reasons. - PDCP reestablishment, - RRC reconfiguration involving security key update, - Maintaining awareness of intra-gNB versus inter-gNB mobility, - Increased data interruption due to RRC reconfiguration message processing, - Increased data interruption due to PDCP reestablishment. This will lead to inferior data interruption than Rel-18 LTM cell switch, thus undermining the benefits of inter-gNB LTM. |

# 7 RRC anchor tracking

| RRC/PDCP anchor solution | Anchor switch-based solution |
| --- | --- |
| Procedures such as modification or cancellation of LTM candidate cell configurations need to be developed for both the solutions and are needed for subsequent inter-gNB LTM as well. In case of the RRC/PDCP anchor solution, since the source gNB-CU remains the anchor, these procedures are simplified and do not require any additional signalling for RRC anchor tracking. Observation 6a. In case of the RRC/PDCP anchor solution, procedures for modification or cancellation of LTM candidate cell configurations are simplified as they do not require any additional signalling for RRC anchor tracking. | All candidate gNBs must always be aware of the current serving RRC anchor of the UE. This is necessary in case any candidate gNB needs to modify the UE’s RRC configuration, such as initiating cancellation or modification of the LTM candidate cells it manages. This adds additional Xn co-ordination signalling after each RRC anchor switch. Observation 6b. In case of anchor switch-based solution, for subsequent LTM, after each inter-gNB cell switch, additional inter-gNB co-ordination signalling needs to be introduced to notify all candidate gNBs about the new serving gNB and adds additional network signaling. |

# 8 Race conditions and RLF

| RRC/PDCP anchor solution | Anchor switch-based solution |
| --- | --- |
| For the PDCP anchor solution, we have not found any possible race conditions in the procedures, that need to be resolved. Observation 7a. In case of the RRC/PDCP anchor solution, no additional race-conditions are expected, and UE performance will be same for both Intra-gNB and Inter-gNB LTM HO. | In our previous RAN3 #125-bis contribution [8], we discussed a couple of race conditions associated with the anchor switch-based solution. - Consider the case where while there is an ongoing Xn procedure between the serving gNB and the other candidate gNBs for transmission of AS security information, UE’s connection to the serving gNB-DU deteriorates. However, since the Xn procedure is ongoing, serving gNB-DU cannot trigger LTM cell switch for the UE, which may then experience RLF. - It was pointed out in [10] that during an ongoing path switch procedure between the target gNB and the CN, if the UE’s connection to the target gNB-DU deteriorates, then the target gNB-DU cannot trigger LTM cell switch for the UE until the path switch procedure completes. Delaying transmitting the LTM cell switch command due to slow network signalling results in UE declaring RLF, which negatively impacts inter-gNB LTM performance especially for dense urban cell deployment scenario. Observation 7b. In the anchor switch-based solution, in subsequent inter-gNB LTM, there are possible race conditions that need to be resolved. E.g.: - Between Xn LTM preparation before the cell switch and MAC-CE-based triggering of the cell switch. - Between MAC-CE-based triggering of the cell switch and the NG path switch procedure. |

# 9 Conclusion

Based on the above discussion, below is summary of comparison table for RRC/PDCP anchor based solution and Anchor switch-based solution for Inter-gNB LTM HO.

| Discussion point | RRC/PDCP Anchor based solution | Anchor switch-based solution |
| --- | --- | --- |
| 1. Impact towards UE during inter-gNB LTM procedure | 1a. In case of the RRC/PDCP anchor solution, for subsequent LTM, the complexity and user plane performance impact at the UE during inter-gNB LTM cell switch is minimal because: - There is no security update during LTM cell switch, and hence there is no PDCP re-establishment, - There is no RRC reconfiguration towards the UE during inter-gNB LTM cell switch. This solution enables support for both intra-gNB and inter-gNB LTM for Rel-18 LTM UEs. | 1b. In case of the anchor switch-based solution, for subsequent inter-gNB LTM, there is significant complexity of handling and inferior performance at the UE upon every inter-gNB cell switch, because of the following reasons. - PDCP reestablishment procedure, - RRC reconfiguration involving security key update, - Maintaining awareness of intra-gNB versus inter-gNB mobility, - Increased data interruption due to RRC reconfiguration message processing, - Increased data interruption due to PDCP reestablishment. This will lead to inferior data interruption than Rel-18 LTM cell switch, thus undermining the benefits of inter-gNB LTM. This inter-gNB solution works with Rel-19 LTM capable UEs. |
| 2. Compliance with the WID objective | 2a: The RRC/PDCP anchor solution is compliant with the WID, because for subsequent inter-gNB LTM HO, RRC reconfiguration message is not required to be sent to the UE upon a cell switch. | 2b: The anchor switch-based solution is not compliant with the current WID, IF the solution requires, for subsequent inter-gNB LTM, an RRC reconfiguration upon an inter-gNB cell switch, which provides the new security update, or at any LTM re-preparation. |
| 3. 5G RAN Architecture compliance | 3a. Inter-gNB LTM with RRC/PDCP anchor is compliant with the current NG-RAN architecture principles, which is similar as R17 SDT with partial UE context transfer and NR-DC MN/SN terminated SCG/SCG bearer concept. 3b. No cross-gNB F1-C interface is needed between the source gNB-CU and target gNB-DU after LTM cell switch. | 3c. Inter-gNB LTM with anchor switch-based solution is also compliant with NG-RAN Architecture principles. |
| 4. Signaling during LTM preparation phase | 4a. For the RRC/PDCP anchor solution, the existing Xn Handover Request procedure can be reused during LTM preparation. - An explicit indication is included in the Handover Request message to indicate to a candidate gNB-CU that the source gNB-CU will remain as the anchor upon LTM cell switch. - The source gNB-CU determines and includes the mapping from QoS flows of the PDU sessions to DRBs for the candidate cell configuration and includes it in a DRB To Be Setup List IE, included in Handover Request. - The mandatory UE Context Information > RRC Context IE is included in Handover Request and information in this IE such as UE capabilities and source cell configuration need to be forwarded by the candidate gNB-CU to the candidate gNB-DU, which is legacy behavior. | 4b. The Stage-2 signalling procedures for inter-gNB LTM preparation for both the anchor switch-based solution and the PDCP anchor solution are similar and there exist some Stage-3 differences. - LTM candidate cell configuration, - Early TA RACH configuration, - TA Information Transfer, - LTM CSI configuration and TCI state configuration. |
| 5. Network signaling overhead to support subsequent inter-gNB LTM | 5a. For supporting subsequent LTM in case of the RRC/PDCP anchor solution, the backhaul signalling overhead is significantly less compared with the anchor switch-based solution because: - There is no security update needed after each inter-gNB LTM cell switch. - There is no need of data forwarding or path switch after each LTM cell switch. - There is no need to establish new UE associations between the source gNB and the other candidate gNBs after each LTM cell switch. | 5c. For supporting subsequent inter-gNB LTM in case of anchor switch-based solution, the backhaul preparation/signaling overhead incurred is significantly large because of the following reasons. - Transmission of updated UE context information, including new AS security information, between the new serving gNB and each of the candidate gNBs after each inter-gNB LTM cell switch. - Exchange of data forwarding information, e.g., tunnel address information, between each gNB and every other gNB. - Signalling overhead due to the need to establish pairwise UE associations between the new serving gNB and each of the candidate gNBs, after each inter-gNB LTM cell switch. There are other network signalling overhead issues that are not highlighted above. |
|  | 5b. For the RRC/PDCP anchor solution, signalling solutions were provided for the following cases, and it was shown that additional overhead is not significant to support these cases. - Support for UE associated F1-AP and/or NG-AP procedures when the anchor gNB-CU communicates with the target gNB-DU after LTM cell switch. - Handling of NAS messages. |  |
| 6. RRC Anchor Node tracking | 6a. In case of the RRC/PDCP anchor solution, procedures for modification or cancellation of LTM candidate cell configurations are simplified as they do not require any additional inter-gNB coordination signalling for RRC anchor tracking. | 6b. In case of anchor switch-based solution, for subsequent inter-gNB LTM, after each inter-gNB cell switch, additional inter-gNB co-ordination signalling needs to be introduced to notify all candidate gNBs about the new serving gNB. |
| 7. Race conditions and RLF issues | 7a. In case of the RRC/PDCP anchor solution, no additional race-conditions are expected, and UE performance will be same for both Intra-gNB and Inter-gNB LTM HO. | 7b. In the anchor switch-based solution, in subsequent inter-gNB LTM, there exist race conditions as mentioned below that negatively impact the performance of subsequent inter-gNB LTM and does not add any additional benefit compared to legacy L3 HO procedure. E.g.: - Between Xn LTM preparation before the cell switch and MAC-CE-based triggering of the cell switch. - Between MAC-CE-based triggering of the cell switch and the NG path switch procedure. |
| 8. Deployment scenario suitability | 8a. The RRC/PDCP anchor solution provides efficient signalling mechanism for various types of deployment scenarios 1, 2, and 3 as discussed in the contribution [11], by avoiding frequent RRC reconfiguration to the UE, avoiding ping-pong inter-gNB LTM HO signaling, and mitigating excessive inter-gNB and RAN-CN coordination signaling. | 8b. The anchor switch-based solution for deployment scenarios 1, 2, and 3 as discussed in the contribution [11], involves frequent RRC reconfiguration to the UE, ping-pong inter-gNB LTM HO signaling, and excessive inter-gNB and RAN-CN coordination signaling. For commercial subsequent inter-gNB LTM, this solution does not fulfill operator requirements as described in [11]. |

Based on the pros and cons discussed in the above summary table, we propose that RAN3 agrees to support the RRC/PDCP anchor-based solution.

Proposal 1. Based on the comparative analysis, RAN3 agrees to support the RRC/PDCP anchor-based solution.

# References

[1] Chair notes, RAN3 #125-bis meeting.

[2] R3-245314, Enabling subsequent inter-gNB LTM by decoupling of cell and anchor changes, RAN3 #125-bis meeting contribution, Qualcomm Incorporated, NTT DOCOMO, Vodafone, Bharti AirTel (India), Sony.

[3] R3-245313, Signalling enhancements for inter-CU LTM HO, RAN3 #125-bis meeting contribution, Qualcomm Incorporated, Vodafone, Bharti Airtel (India).

[4] R3-245201, Discussion on the PDCP anchor based solution, RAN3 #125-bis meeting contribution, Huawei, Ericsson, CATT, China Telecom, CMCC, Samsung.

[5] R3-247235, Signalling enhancements for inter-CU LTM HO, RAN3 #126 meeting contribution, Qualcomm Incorporated, Reliance JIO, Vodafone, Sony.

[6] R3-244646, Response to R3-244482, RAN3 #125 meeting contribution, Qualcomm Incorporated.

[7] R3-245676, Response to R3-245201, RAN3 #125-bis meeting contribution, Qualcomm Incorporated.

[8] R3-245315, Impact analysis of coupling LTM cell switch and CU anchor changes on subsequent inter-gNB LTM, RAN3 #125-bis meeting contribution, Qualcomm Incorporated, Vodafone, NTT DOCOMO, Sony, Bharti Airtel (India).

[9] R3-244055, Discussion on inter-CU LTM, RAN3 #125 meeting contribution, ZTE Corporation.

[10] R2-2407073, On inter-CU aspects for LTM, RAN2 #127 meeting contribution, Nokia.

[11] R2-247239, Inter-gNB/CU LTM typical deployment scenarios and operational requirements, Reliance JIO, RAN3 #126 meeting contribution.