3GPP TSG-RAN WG3 Meeting #126	R3-247237

Orlando, US, 18 - 22 Nov, 2024

Title:	Signaling enhancements for L3 measurement based R18 LTM

Agenda Item:	8.1

Source:	Qualcomm Inc, Reliance JIO, Bharti Airtel

Document for:	Discussion/Agreement

# 1	Introduction

In RAN3#125bis, RAN3 received an LS [1] from RAN2 about support for L3 measurement based LTM, and is given shown below:

1. Overall Description: In the late phase of Rel-18 LTM definition, it was decided, that LTM can be triggered based on Layer 3 measurements by the UE. Consequently, Layer 3 measurements are delivered via RRC to the CU and the CU needs to request the DU to send MAC CE to trigger LTM execution. During RAN2 discussion, it was recognised, that there is no message over F1 which would be sent from CU to DU requesting to trigger MAC CE for the case that the LTM mobility is based on L3 measurements. It was also recognised that such a message would be needed from Release 18 onwards. 2. Actions: RAN2 kindly asks RAN3 to introduce F1 support for the case where the CU needs to request the DU to send MAC CE to trigger LTM execution based on UE Layer 3 measurements.

During RAN3#125bis online discussion, some companies proposed CU to decide about triggering of L3 measurements based LTM and CU will inform DU to trigger MAC-CE based cell switch command. But some companies argued that it is enough for CU to provide L3 measurements to DU and then DU will make decisions about when to trigger Early TA Acquisition and MAC-CE based cell switch command etc.

Couple of companies argued that RAN2 LS does talk about any Early TA Acquisition for L3 measurements based LTM and it is not applicable. Which is not aligned with many other companies understanding and as a result another LS R3-245710 was sent from RAN3 to RAN2 asking for clarification about whether Early TA Synchronization is applicable for L3 measurement based LTM or not.

RAN WG3 discussed the introduction of F1 support for the case where LTM is based on UE layer 3 measurements and acknowledge that this is not supported in Rel-18.

During the discussion, some companies raised the concern whether the early synchronization should be supported or not for L3 measurements based LTM.

RAN2 sent another reply LS R2-2409373 to RAN3 and given below.

RAN WG2 would like to thank RAN WG3 for their LS in R2-2409382. RAN WG2 understands that early synchronisation is also applicable to L3 measurements based LTM. It is also RAN WG2 understanding that for L3 measurements based LTM, the CU decides early synchronisation according to the results of L3 measurements and indicates that decision with the corresponding beam information to a DU and then the DU triggers early synchronisation.

Note that it is not RAN2 realm to make decision about whether CU or DU to make decisions about whether L3 measurements based LTM decisions like when to trigger early TA synchronization, MAC-CE based TCI state and MAC-CE based cell switch command. It is purely RAN3 to make such decision and necessary F1 enhancements needed.

In this contribution, we discuss reasons for why it is DU to make decision and associated F1 signalling enhancement to support L3 measurement based LTM.

# 2	Discussion

In Rel-18, the main motivation for introducing LTM is to reduce the handover interruption latency and ability to switch cells/beams quickly by DU based on L1 filtered measurements received from UE.

Due to some initial development phase, some vendors claims that they may not be able to support L1 measurements processing and some UEs may not be able to support L1 measurements reporting for all scenarios. Thus, to enable initial deployments, L3 measurements based LTM was also agreed in RAN4 and RAN2.

It is important to note that L3 measurements based LTM was agreed in RAN4 primarily for FR1 only and is not intended for FR2 case.

Observation 1 : L3 measurement based R18 LTM is supported only for FR1.

Overtime, it is possible that multiple RAN infra vendors supporting both L1 and L3 measurement based LTM configuration and different UEs based on capability are expected to support L1 or L3 measurement based LTM for FR1. In such mixed deployment scenario, it is desirable to have one entity gNB entity should control LTM HO triggering for both L1 and L3 measurements based LTM.

Proposal 1 : In split gNB Architecture, only one network node shall process and trigger LTM HO for both L1 and L3 measurement based R18 LTM  UEs for both FR1 and FR2.

Lets us first try to understand what is difference between L1 measurement reports and L3 measurement reports from UE. Following Figure from TS 38.300 depicts UE processing of L1 and L3 measurements.

In case of L3 based measurements, UE does L3 based filtering which is long term average of reporting radio metrics (i.e., RSRP, RSRQ and SINR) and reports using RRC measurement reports. For L1 based measurements, UE does L1 based filtering (which is quick) and reports to gNB via either PUCCH or PUSCH in UCI. R18 Layer 1 LTM measurements are based on SSB only and UE reports L1-RSRP which is directly received by DU.

Between L1-RSRP and L3 RSRP, the only difference is what filter is being used by UE and these metric values meaning remains same. Example : RSRP of -90 dBm value is same whether it is L1 or L3 filtered and receiving node does not have to perform any additional decoding.

Observation 2 : The difference between L1 vs L3 measurements is only about usage of L1 filter  vs L3 filter by the UE. At receiving RAN node, there is no additional de-filtering needed and meaning of absolute value of L1-RSRP Vs L3-RSRP remains same. DU will understand radio measurement very well.

DU also hosts control plane part which can understand RRC IEs and in many cases DU prepared RRC IEs including lower layer configuration and send to CU via “DU to CU RRC Information IE”. CU sends multiple RRC IEs  to DU in “CU to DU RRC Information IE” in many F1-AP messages. It is not correct to argue that DU does not understand L3 filtered measurement report and only CU can understand them.

In legacy L3 HO procedures, in existing F1-AP signalling messages CU to DU RRC Information IE -> Handover Preparation Information -> RRM-Config -> MeasResultList2NR is already sent from CU to DU and is not done for first time.

Observation 3 : control plane part of DU can understand RRC IEs and in many existing F1-AP signalling messages RRC IEs are already exchanged by CU and DU. In fact DU gets MeasResultList2NR IE within CU to DU RRC Information IE -> Handover Preparation Information -> RRM-Config -> MeasResultList2NR.

CU also forwards several SON reports such as RLF Report, RA Report, SHR, SPR as “RRC containers” in Access and Mobility Indication message over F1AP as shown below. This is based on the assumption that DU can also understand RRC containers, otherwise there would be no reason to forward these SON reports as “RRC containers” to DU.

Examples:

| >>RA Report Container | M |  | OCTET STRING | Includes the RA-ReportList-r16 IE as defined in subclause 6.2.2 in TS 38.331 [8]. | - |  |
| --- | --- | --- | --- | --- | --- | --- |

| >>NR UE RLF Report Container | M |  | OCTET STRING | Includes the nr-RLF-Report-r16 IE contained in the UEInformationResponse message defined in TS 38.331 [8]. | - |  |
| --- | --- | --- | --- | --- | --- | --- |

| >>Successful HO Report Container | M |  | OCTET STRING | Includes the SuccessHO-Report IE as defined in subclause 6.2.2 in TS 38.331 [8]. | - |  |
| --- | --- | --- | --- | --- | --- | --- |

| >>Successful PSCell Change Report Container | M |  | OCTET STRING | Includes the SuccessPSCell-Report IE as defined in TS 38.331 [8]. | - |  |
| --- | --- | --- | --- | --- | --- | --- |

Observation 4: CU forwards several SON reports such as RLF Report, RA Report, SHR, SPR as “RRC containers” in Access and Mobility Indication message over F1AP. This is based on the assumption that DU can understand RRC containers, otherwise there would be no reason to forward these SON reports as “RRC containers” to DU.

Key question is for FR1 L3 measurement based LTM, which network node has to make decision about LTM procedure triggers. i.e CU vs DU.

Following are 2 possible options.

Option 1 : Based on L3 measurements received from UE, CU makes LTM decision and informs DU to trigger LTM procedures.

Option 2 : CU forwards L3 measurements received from UE to DU via F1 signalling and DU makes all LTM decisions similar to L1 based measurements.

For R18 LTM, following procedures are used at high level.

- LTM candidate cell pre-configuration via RRC

- Early DL/UL synchronization

  - MAC-CE based TCI state activation for DL synchronization.

  - PDCCH order tiggered early UL TA synchronization

- LTM cell switch execution upon receiving MAC-CE based cell switch command

- LTM cell switch completion.

R18 LTM HO latency reduction benefits will come primarily due to RACH-less LTM which requires early DL/UL synchronization before network sending MAC-CE based cell switch command and is depicted in below figure from TS 38.300, Annex G.

Option 1 Analysis: CU trigger based solution

DU is responsible for PHY/MAC scheduling operations and in case of R18 LTM, DU schedular is responsible when to trigger MAC-CE based TCI state activation, PDCCH order trigger and MAC-CE based cell switch command as well.

If CU makes LTM HO decision based on L3 measurements received from UE, there are multiple aspects need to be considered and decision should be based on careful technical analysis rather decision should not be based on legacy L3 HO decisions are made by CU and therefore follow same for L3 measurements based LTM as well. This is not correct thing to do.

If CU has to make decision about when to trigger MAC-CE based TCI activation, PDCCH order for UL TA Acquisition and MAC-CE based cell switch command, basically CU is taking role of DU’s PHY/MAC scheduling decisions which is violation of CU-DU functional split. Additionally CU has to send multiple F1-AP signalling messages independently to DU using F1-AP signalling message based triggers for these 3 events and adds additional delay and it is not role of CU.

Observation 5 : CU triggering multiple F1-AP signalling messages towards DU for MAC-CE based TCI state activation, PDCCH order and MAC-CE Cell switch command adds additional F1 signalling overhead and complexity without any real benefit. DU shall be responsible for PHY/MAC scheduling decisions and CU taking control of DU scheduling functions is violation of CU-DU functional split.

Another possibility is that CU sends one single F1-AP signalling message to tell DU to trigger DL/UL synchronization and MAC-CE based LTM command. This means that after receiving single F1-AP message containing cell ID, beam ID, TCI state etc, DU has to trigger these 3 events in sequence. This defeats the purpose of early DL/UL synchronization.

In FR1 LTM, handover can be either Intra Freq or Inter Freq. early DL/UL synchronization caused interruption or delay is dependent on whether it is intra or inter frequency LTM HO.

For MAC-CE based TCI activation for DL synchronization typically takes a minimum of 23ms. For intra frequency UL TA pre-synchronization, UE side interruption may be very small. For inter frequency UL TA pre-synchronization (assuming UE UL RACH preamble transmission to candidate cell is not within configured UE UL BWP), it can add approximately 15-20ms interruption at UE side. Additionally, for both intra-frequency and inter-frequency cases, candidate cell DU has to send estimated UL TA value to CU and CU has to forward back to source cell DU. This latency may be variable depending on deployment dependent F1 delay and processing delay of candidate DU, CU forwarding delay and source DU processing delay as well.

Upon receiving F1-AP signalling message, if DU triggers DL/UL synchronization and then sends MAC-CE cell switch command then it adds to additional delay to trigger MAC-CE based cell switch command because MAC-CE based DL TCI state activation delay , upon triggering PDCCH order, UE has to send CFRA preamble to target cell/DU and target DU/cell has to transfer TA information to source cell/DU via non-UE associated TA Information Transfer messages as described above. Thus, performing these 3 steps in sequence causes DU to delay MAC-CE based cell switch command, increased LTM HO interruption and defeats the purpose of introducing early DL/UL synchronization for LTM and it may even be longer than L3 based HO interruption due to involvement of network side UL TA information exchange delay. If DU skips early DL/UL synchronization to speed up MAC-CE based cell switch command, then UE has to perform RACH-based LTM, which causes additional LTM HO interruption and LTM latency reduction gains will be lost.

Observation 6 : CU triggering single F1-AP signalling messages towards DU for triggering MAC-CE based TCI state activation, PDCCH order and MAC-CE Cell switch command causes additional delay for DU to trigger MAC-CE based cell switch command and defeats the purpose of early DL/UL synchronization for UE.

Option 2 Analysis: DU trigger based solution

If CU simply forwards L3 measurements received from UE to DU, it is simple for DU to process L3 filtered measurement reports and DU schedular autonomously can make decisions whether and when to trigger MAC-CE based TCI state activation, PDCCH order based early UL TA acquisition and MAC-CE based cell switch command.  This simplifies both CU and DU implementation and DU will keep control of when to trigger these 3 events for UE. As we already explained above, DU should be able to process RRC based L3 measurements and DU is not required to perform any additional de-filtering for L3 filtered radio measurements. i.e RSRP, RSRQ and SINR. Based on RRC configuration, it is already possible for UE to report both beam level and cell level measurements, which DU will better understand than CU. Note that for L1 measurements, UE reports measurements at beam level for a number of cells and is controlled by UE capability.

Observation 7 : CU forwarding L3 filtered measurements to DU simplifies F1 signalling design and DU schedular can autonomously make decision about when to trigger DL/UL pre-synchronization and MAC-CE based cell switch command.

Based on above discussion, we do not see any benefits for option 1 CU based solution and we strongly believe option 2 approach aligns with CU-DU functional split and original design spirit of DU triggered LTM will be retained. Additionally, option 2 approach avoids different network nodes making decisions for different scenarios. Thus, we propose

Proposal 2: For L3 measurements based R18 LTM , DU makes decision about when to trigger DL/UL pre-synchronization and MAC-CE based Cell Switch Command. (i.e., CU will not instruct DU when to trigger DL/UL pre-synchronization and MAC-CE based cell switch command).

Proposal 3: CU forwards L3 fileted measurements received from UE to DU and DU will process L3 measurements for LTM events decision.

# 3	Conclusion

In this contribution, we discussed how to support L3 measurement based LTM and following are key observations and proposals for discussion.

Observation 1 : L3 measurement based R18 LTM is supported only for FR1.

Observation 2 : The difference between L1 vs L3 measurements is only about usage of L1 filter  vs L3 filter by the UE. At receiving RAN node, there is no additional de-filtering needed and meaning of absolute value of L1-RSRP Vs L3-RSRP remains same. DU will understand radio measurement very well.

Observation 3 : control plane part of DU can understand RRC IEs and in many existing F1-AP signalling messages RRC IEs are already exchanged by CU and DU. In fact DU gets MeasResultList2NR IE within CU to DU RRC Information IE -> Handover Preparation Information -> RRM-Config -> MeasResultList2NR.

Observation 4: CU forwards several SON reports such as RLF Report, RA Report, SHR, SPR as “RRC containers” in Access and Mobility Indication message over F1AP. This is based on the assumption that DU can understand RRC containers, otherwise there would be no reason to forward these SON reports as “RRC containers” to DU.

Observation 5: CU triggering multiple F1-AP signalling messages towards DU for MAC-CE based TCI state activation, PDCCH order and MAC-CE Cell switch command adds additional F1 signalling overhead and complexity without any real benefit. DU shall be responsible for PHY/MAC scheduling decisions and CU taking control of DU scheduling functions is violation of CU-DU functional split.

Observation 6: CU triggering single F1-AP signalling message towards DU for triggering MAC-CE based TCI state activation, PDCCH order and MAC-CE Cell switch command causes additional delay for DU to trigger MAC-CE based cell switch command and defeats the purpose of early DL/UL synchronization for UE.

Observation 7 : CU forwarding L3 filtered measurements to DU simplifies F1 signalling design and DU schedular can autonomously make decision about when to trigger DL/UL pre-synchronization and MAC-EC based cell switch command.

Thus, we propose

Proposal 1 : In split gNB Architecture, only one network node shall process and trigger LTM HO for both L1 and L3 measurement based R18 LTM  UEs for both FR1 and FR2.

Proposal 2: For L3 measurements based R18 LTM , DU makes decision about when to trigger DL/UL pre-synchronization and MAC-CE based Cell Switch Command. (i.e., CU will not instruct DU when to trigger DL/UL pre-synchronization and MAC-CE based cell switch command).

Proposal 3: CU forwards L3 fileted measurements received from UE to DU and DU will process L3 measurements for LTM events decision.

# 4	Reference

[1] R2-2407606	L3 measurement based LTM support over F1

[2] R2-2409373	L3 measurement based LTM support over F1

[3] R3-245710    Reply LS on L3 measurement based LTM support over F1