3GPP TSG RAN WG3 126	R3-247238

Orlando, USA, Nov 18th –22nd, 2024

Title: 	Discussion on Timing Aspects for AIML support in CCO

Source: 	Jio

Agenda item:	11.3

Document Type:	Discussion

# 1. Introduction

At the most recent RAN3#125-bis meeting, which focused on the Study Item (SI) regarding AI/ML applications in RAN Release 19, participants discussed and summarized several key agreements concerning the proactive approach to Coverage and Capacity Optimization (CCO) as outlined in TR 38.743 [1].

Following the 3GPP TS 38.401[2] in sub-clause 7.9.3: “In case of split gNB architecture, CCO detection function is located at the gNB-CU. The gNB-CU signals to the gNB-DU the CCO issue and the affected cells and beams. The gNB-DU resolves the CCO issue concerning own served cell by local action within the OAM configured limits. The gNB-DU may also take into account information received for other cells when adopting the CCO configuration. The gNB-DU informs the gNB-CU of the new coverage states adopted.” the CCO reactively detection function is located in gNB CU.

Possible solutions for F1 and Xn signalling following agreements are discussed based on open issues captured at last RAN3 meeting for timing aspects.

# 2. Discussion

In the recent RAN3#125-bis meeting it has been discussed and agreed for AI/ML based CCO that the task of generating proactively predictions for potential CCO issues will be handled by gNB for non-split architecture as well by the gNB-CU in split architecture, according to two different time references, as following:

Time for future coverage state: The point in time when the future coverage state will be applied.

Time for predicted CCO issue: The point in time when the CCO issue is predicted to happen

Following [3] once the gNB-CU identifies a predicted CCO issue, it relays this information to the gNB-DU (step1) with a conditional future CCO assistance state as to be further discussed as well in this meeting. Additionally, during the latest RAN3 meeting, it was confirmed that after receiving the predicted CCO issue, the gNB-DU will determine the future coverage state corresponding to that issue (step2). This determination can be made based on whether the implementation follows AI/ML-based methods or the traditional, legacy approaches. The gNB-DU’s calculation of the future coverage state will be send back to gNB CU (step3) to be shared with neighbouring NG-RAN nodes (step 4). This last step 4 exchange of information serves two key purposes:

- it assists with Mobility Robustness Optimization (MRO),

- it enables neighbouring nodes to match their coverage configurations with the new future coverage state,

as described in TS 38.300 sub-clause 15.5.5.3, “the coverage state indicator may also be used at the receiving NG-RAN node to adopt coverage configurations matching with neighbouring cells' coverage configurations.” This mechanism ensures that the coverage transitions across multiple NG-RAN nodes are smooth and coordinated, preventing degradation of performance and maintaining network robustness. In this way, neighbouring nodes can adjust their coverage to match and coordinate with the changes made in the originating node.

## Local gNB CU predicted CCO issue timing info

Jio believes that the timing information in gNB CU is critical indeed for ensuring the accuracy, relevance, and effectiveness of the gNB-CU’s CCO predictions, especially before passing them to the gNB-DU for the and future state execution decision and execution, managing to capture any variations in the traffic load, user mobility, and interference conditions.

Observation 1: Considering a timing execution framework in gNB CU, before sending the predicted CCO via F1 interface to gNB DU is important to account for highly predicted traffic load and mobility conditions.

According to 3GPP TS 38.300 in sub-clause 16.20.2

“For the deployment of AI/ML for NG-RAN the following scenarios may be supported:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the NG-RAN node;

-	AI/ML Model Training and AI/ML Model Inference are both located in the NG-RAN node.

gNB-CU is able to analyse the expected rate of change in the channel conditions, traffic variations, and user mobility patterns, thus enforcing the timing info to ensure that the gNB-DU processes relevant time frame predictions and time frame future state execution based on real-time or near-future conditions.

Observation 2: The predicted CCO issue may become irrelevant or out of date if there is a significant delay between prediction time and future state execution time.

Following figure 1 description, gNB CU after predicting a specific CCO coverage issue should spend some time to collect information, activate the AI/ML inference and estimate a timing info before sending the predicted CCO issue via F1 interface signalling F1AP gNB CU CONFIGURATION UPDATE to gNB DU, as described in the 3GPP TS 38.473[4] sub-clause 8.2.5.2 “The gNB-CU initiates the procedure by sending a GNB-CU CONFIGURATION UPDATE message including the appropriate updated configuration data to the gNB-DU. The gNB-DU responds with a GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE message to acknowledge that it successfully updated the configuration data”. The gNB CU CONFIGURATION UPDATE message includes the legacy CCO Assistance Information IE, which already includes the CCO issue as ENUMERATED value, and a list of affected cells and beams. However there is no reference or IE for the timing info which has to be added in order to fully support this new functionality.

Observation 3: F1AP gNB CU CONFIGURATION UPDATE signalling from the gNB-CU to the gNB-DU for predicted CCO issue along with the time frame needs to include prediction time information along with the predicted CCO issue

Figure 1: The timing info in the gNB CU predicted CCO coverage issue framework

The information to be collected as input to the AI/ML inference for the timing info estimation would potentially include Traffic Load Projections (i.e. Predicted data traffic demand in the cell or sector), User Mobility Profiles (i.e. Information about the speed, direction and trajectory, and density of mobile users), Resource Block Scheduling Window (i.e. Time granularity in which resource allocations are made in the gNB-DU), Interference Projections (i.e. Predictions of future interference, especially from neighbouring cells or external sources) and perhaps Neighbouring Cell Synchronization (i.e. Status and potential handoff synchronization with adjacent cells that may be impacted). By considering these factors, the gNB-CU can accurately predict not only the future CCO issue status but also when it will be most effective for the gNB-DU to activate it. This leads to more precise timing, ensuring minimal disruption and maximizing network efficiency.

Proposal 1: To activate the gNB CU AI/ML inference in order to estimate the proper timing info for the resolution of the predicted CCO coverage issue by the gNB DU further actions.

In a next step (figure 2) gNB DU will consider the predicted CCO coverage issue, generate the future CCO state along with the corresponding future CCO time frame and update the gNB CU via F1 interface signalling F1AP gNB DU CONFIGURATION UPDATE, as described in the 3GPP TS 38.473 sub-clause 8.2.4.2 “The gNB-DU initiates the procedure by sending a GNB-DU CONFIGURATION UPDATE message to the gNB-CU including an appropriate set of updated configuration data that it has just taken into operational use. The gNB-CU responds with GNB-DU CONFIGURATION UPDATE ACKNOWLEDGE message to acknowledge that it successfully updated the configuration data”.

All the necessary info is included within the Coverage Modification Notification IE. gNB DU then executes the future CCO status taking into consideration the timing info reference framework.

Proposal 2: For AI/ML based CCO, introduce time information to both predicted CCO issue and future status coverage issue between the local gNB CU and gNB DU via F1 interface.

When signaling a predicted CCO issue from gNB-CU to gNB-DU, including timing information, it is essential to clarify whether the CCO issue reported by the gNB-CU is a current (detected) issue or a future (predicted) one. To ensure this is conveyed in a straightforward manner, we propose reusing the existing F1AP message used for legacy CCO. However, to support AI/ML-based CCO, we anticipate that new Information Elements (IEs) or extensions of the current IEs will be required in the signaling mechanisms, as follows:

- In F1AP signalling messages (3GPP 38.473 gNB CU CONFIGURATION UPDATE and gNB DU CONFIGURATION UPDATE) :

  - new IE is needed to indicate a reference prediction time corresponding to a predicted CCO issue,

  - There is a need to differentiate predicted CCO issue with current (detected) CCO issue.

  - There is a need to differentiate time for predicted CCO issue from time for future coverage state.

Proposal 3: F1AP gNB CU CONFIGURATION UPDATE to include new IEs for the predicted and future CCO issue timing info.

Figure 2: The timing info consideration during the gNB DU future CCO state execution

# 3. Conclusions

Based on the discussion in this paper, we derived the following observations and proposals:

Observation 1: Considering a timing execution framework in gNB CU, before sending the predicted CCO via F1 interface to gNB DU is important to account for highly predicted traffic load and mobility conditions.

Observation 2: The predicted CCO issue may become irrelevant or out of date if there is a significant delay between prediction time and future state execution time.

Observation 3: F1AP gNB CU CONFIGURATION UPDATE signalling from the gNB-CU to the gNB-DU for predicted CCO issue along with the time frame needs to include prediction time information along with the predicted CCO issue

Proposal 1: To activate the gNB CU AI/ML inference in order to estimate the proper timing info for the resolution of the predicted CCO coverage issue by the gNB DU further actions.

Proposal 2: For AI/ML based CCO, introduce time information to both predicted CCO issue and future status coverage issue between the local gNB CU and gNB DU via F1 interface.

Proposal 3: F1AP gNB CU CONFIGURATION UPDATE to include new IEs for the predicted and future CCO issue timing info.

# References

[1] 3GPP TR 38.743 V2.0.0 (2024-08)

[2] 3GPP TS 38.401

[3] R3-244698 Summary of offline Discussion (SoD) for AIRAN2_CCO

[4] 3GPP TS 38.473