3GPP TSG-RAN WG3 Meeting #126	             R3-247243

Orlando, USA, 18th – 22nd Nov 2024

Agenda Item:	20.2

Source:	CMCC

Title:	Support Assisted AI/ML Positioning

Document for:	Discussions & Approval

# Introduction

In the RAN 3 125 Bis meeting, the following agreement for Case 3a has been made [1]:

Agreement (RAN3#125 Bis) - The AI/ML model (inference and training functions) is at the gNB. The existing LCS framework is re-used for AI/ML positioning. Case 3a agreements: - LMF provides ground truth label and related data (i.e., part B information as called by RAN1) to NG-RAN. FFS on the content of the ground truth label and related data pending on RAN1 progress. FFS on the signalling design. - How the LMF obtains the ground truth label and related data information is outside of RAN3 scope. - NG-RAN obtains measurements from TRP (i.e., part A information as called by RAN1). The details of the measurements are still pending and subject to further discussion in RAN1. - WA: RAN3 will proceed with Option A: NG-RAN node performs monitoring metric calculation for its own model.

# Discussions on Assisted AI/ML Positioning

AI/ML-assisted positioning utilizes AI/ML models to produce intermediate measurement statistics that play a critical role in determining location. These statistics can include metrics such as the probability of a signal being Line-of-Sight (LOS) or Non-Line-of-Sight (NLOS), the angle of arrival (AoA) or departure (AoD) of signals, and time-of-arrival data (ToA). These measurements are essential for refining positioning accuracy, especially in complex environments. In this document, we mainly focus on the top priority case of Assisted AL/ML Positioning: Case 3a. The Case 3a is illustrated in Figure 1. The AI/ML model (inference and training functions) is at the NG-RAN Node.

Figure 1: Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning

Regarding Case 3a, in the past RAN1 meetings, the following agreements have been made:

### Model Training

As agreed from the last RAN3 meeting, the NG-RAN Node obtains measurements from TRP and LMF provides ground truth label and related data to the NG-RAN Node, with the latter part having RAN3 impact.

﻿Quoted from TS 38.305 [2], an NRPPa transaction is currently only initiated by the LMF, which is realized by the execution of one procedure defined as a request and a response (class 1), followed by one or several procedures initiated by the NG-RAN Node, each of which is defined as a single message (class 2).

Quoted from TS 38.305 For possible extensibility, the protocol is considered to operate between a generic "access node" (e.g. gNB, ng-eNB) and a "server" (e.g. LMF). An NRPPa transaction is only initiated by the server. In the NAPPa protocol, ﻿the described transaction may be realized by the execution of one procedure defined as a request and a response, followed by one or several procedures initiated by the NG-RAN node (each procedure defined as a single message) to realize the additional responses.

In our view, the LMF should be provisioned or even decide on which NG-RAN Nodes perform the assisted positioning model training and inference. Thus, it is not required for the NG-RAN Node to indicate the LMF that the measurement data and ground truth labels are needed for the assisted positioning model training.

The LMF should be provisioned or even determine on which NG-RAN Nodes perform the assisted positioning model training and inference.

The NG-RAN Node can indicate the LMF that the measurement data and ground truth labels are needed for the assisted positioning model training.

The NG-RAN Node could initiate the data collection procedure, including channel measurement based on SRS received from PRUs/UEs, and corresponding ground truth labels and related data from the LMF. The NG-RAN Node initiates a Data Collection Request to the LMF, which will return a Data Collection Response to the NG-RAN Node, containing the intermediate measurement statistics as ground truth labels derived from the PRUs/UEs locations by the LMF.

Since the NRPPa is only initiated by the server, in the above procedure, we can consider the NG-RAN Node is the Server, which hosts the AI/ML assisted model with training and inference to support UE positioning at the LMF.

The NG-RAN Node sends Data Collection Request to the LMF, the LMF shall reply with a Data Collection Response, containing the intermediate measurement statistics data as ground truth labels.

In the CU-DU split architecture, AI/ML model training can be hosted within the gNB-CU, aligning with the principles outlined in the AI/ML for NG-RAN framework. The above procedures would have F1 interface impact as well.

For the CU-DU split architecture, AI/ML model training is hosted within the gNB-CU for Case 3a. The input data for the AI/ML model training is sent from the gNB-DU to the gNB-CU by reusing the existing positioning measurements procedures.

### Model Inference

It was agreed in RAN1 118 meeting [5], that the existing procedures can be reused in terms of SRS configuration for model training, monitoring and inference. Regarding the model inference output, RAN1 116 meeting [4] agreed that at least LOS/NLOS indicator and/or timing information are supported for reporting from the NG-RAN Node to the LMF. If reporting the LOS/NLOS indicator, it can be provided as a soft or hard indicator, as specified in TS 38.214 [7]. If reporting timing information, it includes at minimum the UL RTOA or the gNB Rx-Tx time difference, as specified in TS 38.215 [8]. However, the details of the reporting are pending further discussions in RAN1.

Agreement (RAN1#116) Agreement For AI/ML assisted positioning Case 3a, at least LOS/NLOS indicator and/or timing information are supported for reporting. - If LOS/NLOS indicator is reported, the indicator can be reported as soft indicator or hard indicator as defined in 38.214. - If timing information is reported, the timing information at least can be reported via UL RTOA or gNB Rx-Tx time difference as defined in 38.215. - Note: details of the report are pending further discussion.

Agreement (RAN1#118 Bis) [6] Agreement From RAN1 perspective, when timing information is reported for Rel-19 AI/ML positioning Case 3a, at least the following are mandatorily or optionally supported in a measurement report from gNB to LMF: - (Mandatory) timing information; - (Optional) Quality of the timing information; - Existing IE “Timing Measurement Quality” can be reused. - (Mandatory) Time stamp. FFS: LOS/NLOS indicator. Note: The final decision of “mandatory” or “optional” presence of each field is up to RAN3.

In our view, the model inference at the NG-RAN side in Case 3a can reuse the current NRPPa measurement procedures [10]. Legacy measurement IEs can still be used as part of traditional positioning methods, while additional AI/ML-based information elements may be introduced. The LMF can then calculate the UE's position using either legacy measurements or AI/ML-based measurements, incorporating both traditional and new IEs as needed.

Model inference at the NG-RAN side in Case 3a can reuse the current NRPPa measurement procedures.

For CU/DU Split architecture, since the model training is performed at the gNB-CU, it is reasonable to think that the gNB-CU also performs the model inference. The input data for the model inference comes from TRPs, which are located in the gNB-DU. According to TS 38.470 [9], the positioning function of F1 interface enables the gNB-CU to request positioning measurements from gNB-DU, and gNB-DU response to gNB-CU with the positioning measurements.

For CU-DU split architecture, AI/ML model inference is hosted within the gNB-CU for Case 3a. The input data for the AI/ML model inference is sent from the gNB-DU to the gNB-CU by reusing the existing positioning measurements procedures.

### Model Monitoring

As agreed in the RAN3 125 Bis meeting [1], we have the WA that: RAN3 will proceed with Option A: NG-RAN Node performs monitoring metric calculation for its own model. However, the LMF is the entity that gathers data from multiple NG-RAN nodes and calculates the UE’s position in the final step. Therefore, it may be advantageous for the LMF to provide assistance to the NG-RAN Node in calculating the monitoring metric or making the monitoring decision. What assistance information could be provided from the LMF to the NG-RAN node is for further discussion.

For Case 3a, the LMF can provide assistance information to the NG-RAN Node for the monitoring metric calculation and monitoring decision making.

# Conclusions

In this submission, we discussed the direct AI/ML positioning cases and potential RAN 3 impact. We presented the following proposals:

- The LMF should be provisioned or even determine on which NG-RAN Nodes perform the assisted positioning model training and inference.

The NG-RAN Node can indicate the LMF that the measurement data and ground truth labels are needed for the assisted positioning model training.

The NG-RAN Node sends Data Collection Request to the LMF, the LMF shall reply with a Data Collection Response, containing the intermediate measurement statistics data as ground truth labels.

For the CU-DU split architecture, AI/ML model training is hosted within the gNB-CU for Case 3a. The input data for the AI/ML model training is sent from the gNB-DU to the gNB-CU by reusing the existing positioning measurements procedures.

Model inference at the NG-RAN side in Case 3a can reuse the current NRPPa measurement procedures.

For CU-DU split architecture, AI/ML model inference is hosted within the gNB-CU for Case 3a. The input data for the AI/ML model inference is sent from the gNB-DU to the gNB-CU by reusing the existing positioning measurements procedures.

For Case 3a, the LMF can provide assistance information to the NG-RAN Node for the monitoring metric calculation and monitoring decision making.

# References

draft_MeetingReport_RAN3#125-bis_vOct25

3GPP TS 38.305, Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN (Release 18)

3GPP TS 23.273, 5G System (5GS) Location Services (LCS); Stage 2 (version 18.7.0 Release 18)

Final_Minutes_report_RAN1#116_v100

Final_Minutes_report_RAN1#118_v100

Draft_Minutes_report_RAN1#118b_v010

﻿3GPP TS 38.214, “Physical layer procedures for data,” (version 18.4.0 Release 18)

﻿3GPP TS 38.215, “Physical layer measurements,” (version 18.3.0 Release 18)

﻿3GPP TS 38.470, “F1 general aspects and principles,” (version 18.3.0 Release 18)

3GPP TS 38.455, “NR Positioning Protocol A (NRPPa),” V18.3.0 (2024-09)