3GPP TSG-RAN WG3 Meeting #126	             R3-247244

Orlando, USA, 18th – 22nd Nov 2024

Agenda Item:	20.2

Source:	CMCC

Title:	(TP to BL CR for TS 38.455) Support Direct AI/ML Positioning

Document for:	Discussions & Approval

# Introduction

In the RAN 3 125 Bis meeting, the following agreement for Case 3b has been made [1]:

Agreement (RAN3#125 Bis) - Re-using the Measurement procedure defined in NRPPa to support requesting and reporting the related measurements to LMF for Case 3b. The details of the related measurements are still pending and subject to further discussion in RAN1. - NG-RAN gets measurements (i.e., part A information as called by RAN1) from TRP and provides them to LMF. The details of the measurements are still pending and subject to further discussion in RAN1.

# Discussions on Direct AI/ML Positioning

In this document, we mainly focus on the top priority case of Direct AL/ML Positioning: Case 3b. The Case 3b is illustrated in Figure 1.

Figure 1: Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning

### Model Training

Agreement (RAN1#116) [2] Agreement For AI/ML based positioning case 3b, at least the following types of time domain channel measurements are supported for reporting: - timing information; - paired timing information and power information. Agreement In Rel-19 AI/ML based positioning, regarding the time domain channel measurements, RAN1 investigate the following alternatives: - Alternative (a). Sample-based measurements, where the timing information is an integer multiple of sampling periods. - Alternative (b). Path-based measurements, where the timing information is according to the detected path timing and may not be an integer multiple of sampling periods.

Agreement (RAN1#116 bis) [3] Agreement For training data generation of AI/ML based positioning Case 3a and 3b, the measurement and its related data (e.g., timestamp) are generated by TRP/gNB. Agreement For training data collection of AI/ML based positioning, the collected data sample can include the following components: Part A: - channel measurement - quality indicator of channel measurement - time stamp of channel measurement Part B: - ground truth label (or its approximation) - quality indicator of label - time stamp of label Note: "Part A" and "Part B" terminologies are only for RAN1 discussion purpose, and may not be used in specification. Note: contents in Part A and Part B may or may not be generated by different entities. Note: Part A and/or Part B, and their contents may or may not apply for each case FFS: detailed definition of channel measurement

Agreement (RAN1#117)[4] Agreement Sample-based measurement is defined as: - The measurement is composed of Nt' samples of the estimated channel response in time domain. The timing information for the Nt' samples are reported with a timing granularity T, where T=2kxTc. k represents the timing reporting granularity factor. Tc is the basic time unit for NR. - The corresponding measurement (e.g., power if reported) corresponds to the measurement for the reported Nt' samples. - Nt' and k can be signalled - FFS: the value range of Nt'; the value range of integer k for the timing granularity T. - The timing information is defined relative to a reference time Further discussion is expected on the determination of Nt' and k (including signaling) , and a rule to be introduced for selecting Nt' samples. Note: It doesn’t imply the definition of Sample-based measurement will be captured into the spec. Agreement Path-based measurement refers to the measurement in the existing specifications (up to Rel-18) including measurement reporting, with potential enhancements on the number of reported paths (if needed).

Agreement (RAN1#118) [5] Agreement For training data collection of AI/ML based positioning case 3b, for time stamp of channel measurement, - For channel measurement generated by TRP/gNB, existing IE “Time Stamp” in TS 38.455 can be reused from RAN1 perspective. Note: Purpose, such as above “training data collection", will not necessarily be specified in RAN 1 specifications. Agreement For the definition of sample-based measurement, select Nt’ samples out of a list of Nt consecutive samples, - The Nt samples have timing granularity T. - FFS: the starting time of the list of Nt samples. - FFS: the value range of Nt For the sample-based measurement (if accepted in Rel-19), - For measurement by TRP/gNB, the Nt’ selected samples are expected to be those with the highest power. - Note: Choice of the maximum value of Nt, Nt’ should take into account the need to preserve proprietary implementation. Agreement For AI/ML positioning Case 2b and 3b, regarding the power information for determining the model input, - For downlink power measurement, use DL PRS-RSRPP defined in TS 38.215 as a starting point. - For measurement report of DL PRS-RSRPP, use the existing measurement report mapping table for PRS-RSRPP in 38.133 as a starting point. - For uplink power measurement, use UL SRS-RSRPP defined in TS 38.215 as a starting point. - For measurement report of UL SRS-RSRPP, use the existing measurement report mapping table for SRS-RSRPP in 38.133 as a starting point. Agreement For Rel-19 AI/ML based positioning Case 3b, regarding sample-based measurement (if supported), from RAN1 perspective, - LMF can signal parameter values of Nt, Nt', k to gNB via NRPPa.

Agreement (RAN1#118 Bis) [6] Agreement For training data collection of AI/ML based positioning, the quality indicator of timing information in Part A when reported is: - When applicable, the existing IE for timing quality, i.e., NR-TimingQuality in 37.355 and IE “Timing Measurement Quality” in 38.455; - FFS: details on how to associate quality indicator to timing information Agreement From RAN1 perspective, for model inference of AI/ML positioning Case 3b, at least the following are mandatorily or optionally supported in a measurement report from gNB to LMF: - (Mandatory) Channel measurement; - (Optional) Quality of the channel measurement; - FFS: details of the quality - (Mandatory) Time stamp of the channel measurement.

Based on the agreements made by RAN 1 past meetings that are quoted and highlighted above, for Case 3b, the measurements are obtained by the NG-RAN Node from TRPs and reported from the NG-RAN Node to the LMF. The measurements might be:

- Alternative (a).  Sample-based measurements, where the timing information is an integer multiple of sampling periods.

- Alternative (b).  Path-based measurements, where the timing information is according to the detected path timing and may not be an integer multiple of sampling periods.

And for each measurement, at least the following are mandatorily or optionally supported in a measurement report from gNB to LMF:

- (Mandatory) Channel measurement.

- (Optional) Quality of the channel measurement.

  - FFS: details of the quality

- (Mandatory) Time stamp of the channel measurement. For time stamp of channel measurement, for channel measurement generated by TRP/gNB, existing IE “Time Stamp” in TS 38.455 [7] can be reused from RAN1 perspective.

For sample-based measurement (if supported), from RAN1 perspective, LMF can signal parameter values of Nt, Nt', k to gNB via NRPPa:

- Nt is the window size over which to collect samples.

- Nt' is the number of samples to report

- k is the resolution of the window, indicating the interval between samples.

For path-based measurement, it refers to the measurement in the existing specifications (up to Rel-18) including measurement reporting, with potential enhancements on the number of reported paths (if needed).

For both sample-based measurement and path-based measurements, the time stamp associated with the measurement needs to be reported with the measurement, while the quality is optionally. And the existing IE “Time Stamp” defined in TS 38.455 can be utilized, the quality definition is upon RAN1 progress.

For sample-based measurement, the parameter values of Nt, Nt', k need to be signalled from the LMF to the NG-RAN Node via NRPPa.

For path-based measurements, RAN3 to wait further RAN1 progress on whether enhancement on the number of reported paths is needed.

Measurement procedure defined in NRPPa is reused to support the channel measurement request and report for Case 3b to support direct model training in LMF.

If CU-DU split architecture is considered, Positioning Measurement procedure defined in F1AP is reused to support the channel measurement request and report for Case 3b to support direct model training in LMF.

### Model Inference

For LMF-side model inference, the UE or gNB can generate the necessary input data while the termination point for this input data lies within the LMF where the inference process is performed [8].

If the necessary input data is generated from NG-RAN Node for LMF-side model inference, Measurement procedure defined in NRPPa is reused to support the channel measurement request and report.

If CU-DU split architecture is considered, Positioning Measurement procedure defined in F1AP is reused to support the channel measurement request and report for Case 3b to support direct model inference in LMF.

### Model Monitoring

In Case 3b, the LMF should be the entity responsible for deriving the monitoring metric, as outlined in [2]. The LMF gather measurements and estimated positions as labels, it is logical for the LMF to make the monitoring decisions. However, it is likely that NG-RAN node could provide further measurements to the LMF for the model monitoring purpose.

For Case 3b, LMF is the entity that is responsible for model monitoring.

NG-RAN node might provide further measurements to the LMF for the model monitoring purpose. RAN 3 should wait for the RAN 1 progress.

# Conclusions

In this submission, we discussed the direct AI/ML positioning cases and potential RAN 3 impact. We presented the following observations and proposals:

- For both sample-based measurement and path-based measurements, the time stamp associated with the measurement needs to be reported with the measurement, while the quality is optionally. And the existing IE “Time Stamp” defined in TS 38.455 can be utilized, the quality definition is upon RAN1 progress.

- For sample-based measurement, the parameter values of Nt, Nt', k to gNB need to be signalled from the LMF to the NG-RAN Node via NRPPa.

For path-based measurements, RAN3 to wait further RAN1 progress on whether enhancement on the number of reported paths is needed.

Measurement procedure defined in NRPPa is reused to support the channel measurement request and report for Case 3b to support direct model training in LMF.

If CU-DU split architecture is considered, Positioning Measurement procedure defined in F1AP is reused to support the channel measurement request and report for Case 3b to support direct model training in LMF.

If the necessary input data is generated from NG-RAN Node for LMF-side model inference, Measurement procedure defined in NRPPa is reused to support the channel measurement request and report.

If CU-DU split architecture is considered, Positioning Measurement procedure defined in F1AP is reused to support the channel measurement request and report for Case 3b to support direct model inference in LMF.

For Case 3b, LMF is the entity that is responsible for model monitoring.

NG-RAN node might provide further measurements to the LMF for the model monitoring purpose. RAN 3 should wait for the RAN 1 progress.

# References

draft_MeetingReport_RAN3#125-bis_vOct25

Final_Minutes_report_RAN1#116_v100

Final_Minutes_report_RAN1#116b_v100

Draft_Minutes_report_RAN1#117_v030

Final_Minutes_report_RAN1#118_v100

Draft_Minutes_report_RAN1#118b_v010

3GPP TS 38.455 V18.3.0 (2024-09), “NR Positioning Protocol A (NRPPa).”

3GPP TR 38.843, “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface.”

# Text Proposal to TS 38.455

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

### 9.1.4	Messages for Measurement Information Transfer Procedures

#### 9.1.4.1	MEASUREMENT REQUEST

This message is sent by the LMF to request the NG-RAN node to configure a positioning measurement.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| LMF Measurement ID | M |  | INTEGER (1..65536, …) |  | YES | reject |
| TRP Measurement Request List |  | 1 |  |  | YES | reject |
| >TRP Measurement Request Item |  | 1..<maxnoofMeasTRPs> |  |  | - |  |
| >>TRP ID | M |  | 9.2.24 |  | - |  |
| >>Search Window Information | O |  | 9.2.26 |  | - |  |
| >>Cell ID | O |  | NR CGI 9.2.9 | The Cell ID of the TRP identified by the TRP ID IE. | YES | ignore |
| >>AoA Search Window Information | O |  | UL-AoA Assistance Information 9.2.66 |  | YES | ignore |
| >>Number of TRP Rx TEGs | O |  | ENUMERATED (2, 3, 4, 6, 8, …) |  | YES | ignore |
| >>Number of TRP RxTx TEGs | O |  | ENUMERATED (2, 3, 4, 6, 8, …) |  | YES | ignore |
| Report Characteristics | M |  | ENUMERATED (OnDemand, Periodic, ...) |  | YES | reject |
| Measurement Periodicity | C-ifReportCharacteristicsPeriodic |  | ENUMERATED (120ms, 240ms, 480ms, 640ms, 1024ms, 2048ms, 5120ms, 10240ms, 1min, 6min, 12min, 30min, 60min,…, 20480ms, 40960ms, extended) | The codepoint 120ms, 240ms, 480ms, 1024ms, 2048ms, 1min, 6min, 12min, 30min, and 60min are not applicable | YES | reject |
| TRP Measurement Quantities |  | 1 |  |  | YES | reject |
| >TRP Measurement Quantities Item |  | 1 .. <maxnoPosMeas> |  |  | - |  |
| >>TRP Measurement Type | M |  | ENUMERATED (gNB-RxTxTimeDiff, UL-SRS-RSRP, UL-AoA, UL-RTOA,…, Multiple UL-AoA, UL SRS-RSRPP, UL-RSCP) | The UL-RSCP measurement is applicable only when the UL-RTOA and/or gNB-RxTxTimeDiff measurement(s) is also requested. | - |  |
| >>Timing Reporting Granularity Factor | O |  | INTEGER (0..5) | Value (0..5) corresponds to (k0..k5) TS 38.133 [16]. This IE is ignored when the Timing Reporting Granularity Factor Extended IE is included. | - |  |
| >>Timing Reporting Granularity Factor Extended | O |  | INTEGER (-6..-1, …) | Value -6 corresponds to kminus6, value -5 corresponds to kminus5 and so on, see TS 38.133 [16] | - |  |
| >>Sample-based Measurement Parameters | O |  |  |  |  |  |
| >>>Nt |  |  |  | Window size for measurement collection. |  |  |
| >>>Nt’ |  |  |  | Number of reported measurement samples. |  |  |
| >>>k |  |  |  | Interval between measurement samples. |  |  |
| SFN initialisation Time | O |  | Relative Time 1900 9.2.36 | If this IE is not present, the TRP may assume that the value is same as its own SFN initialisation time. | YES | ignore |
| SRS Configuration | O |  | 9.2.28 |  | YES | ignore |
| Measurement Beam Information Request | O |  | ENUMERATED (true,...) | This IE is ignored when the Measurement Characteristics Request Indicator IE is included. | YES | ignore |
| System Frame Number | O |  | INTEGER(0..1023) |  | YES | ignore |
| Slot Number | O |  | INTEGER(0..79) |  | YES | ignore |
| Measurement Periodicity Extended | C-ifMeasPerExt |  | ENUMERATED (160ms, 320ms, 1280ms, 2560ms, 61440ms, 81920ms, 368640ms, 737280ms, 1843200ms, …) |  | YES | reject |
| Response Time | O |  | 9.2.68 | This IE is ignored when the Report Characteristics IE is set to “periodic”. | YES | ignore |
| Measurement Characteristics Request Indicator | O |  | 9.2.81 |  | YES | ignore |
| Measurement Time Occasion | O |  | ENUMERATED (o1, o4, …) |  | YES | ignore |
| Measurement Amount | O |  | ENUMERATED (0, 1, 2, 4, 8, 16, 32, 64) | This IE is ignored if the Report Characteristics IE is set to ‘OnDemand’. Value 0 represents an infinite number of periodic reporting. | YES | ignore |
| Time Window Information Measurement List | O |  | 9.2.91 |  | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifReportCharacteristicsPeriodic | This IE shall be present if the Report Characteristics IE is set to the value "Periodic". |
| ifMeasPerExt | This IE shall be present if the Measurement Periodicity IE is set to the value "extended". |

| Range bound | Explanation |
| --- | --- |
| maxnoPosMeas | Maximum no. of measured quantities that can be configured and reported with one positioning measurement message. Value is 16384. |
| maxnoofMeasTRPs | Maximum no. of TRPs that can be included within one message. Value is 64. |

<<<<<<<<<<<<<<<<<<<< The end of changes >>>>>>>>>>>>>>>>>>>>