3GPP TSG-RAN WG3 Meeting #126	R3-247264

Orlando, US, 18 - 22 Nov, 2024

Title:	(TP for MDT BLCRs for TS38.413, TS38.423, TS37.320) SON for Intra-NTN mobility

Agenda Item:	10.3.1

Source:	Huawei

Document for:	other

1	Introduction

The following progress were made at RAN3 #124 meeting for SON for intra-NTN mobility.

Take the connection failure scenarios in §15.5.2.2.2 of TS 38.300 as starting point with necessary update, if needed, in MRO for intra-NTN mobility.

MRO for intra-NTN mobility shall consider the following trigger conditions:

- Measurement-based trigger condition;

- A time-based trigger condition;

- A location-based trigger condition;

- A time-based and Measurement-based combined trigger condition;

- A location-based and Measurement-based combined trigger condition;

Do not consider MRO for feeder link switch.

MRO for intra-NTN mobility shall consider both conditional and non-conditional intra-NTN handover.

FFS on whether there are any enhancements to RLF report due to support for multiple trigger conditions in intra-NTN mobility.

FFS on whether to consider any enhancement for MDT configuration e.g., area scope and trigger condition for NTN.

Discuss MRO for RACH-less HO of NTN, only for those different issues identified compared with MRO for LTM.

Time information when RLF occurs needs to be included in the RLF report, other information to be included can be checked as well in next meeting.

Further progress was made at RAN3 #125 meeting,

MRO for CHO with a time-based trigger condition:

Case 1 Too Late CHO Execution: RLF happens in the source cell before UE executes CHO (before or after T1 threshold starts), and the UE re-connects the cell other than the source cell.

- Distinguish if the RLF happens before or after T1

Case 2 Too Early CHO Execution/CHO Execution to Wrong Cell: HOF happens or RLF happens shortly after successful CHO, and the UE re-connects in the source cell or a third cell.

- FFS on whether it matters if RLF happens within the duration or after the duration period.

The RLF report includes for single time-based trigger condition:

-      Absolute time (measured UTC time) when RLF occurred

-      Measured UTC time when CHO is executed

Whether to consider the fallback issue from RACH-less HO to RACH-based HO for NTN?

Whether to introduce any enhancement for the Area Scope of logged MDT for NTN over NG interface?

Progress at RAN3 #125bis meeting:

For time based trigger condition:

WA: down-select among below three alternatives.

Alt 1: the RLF report includes for single time-based trigger condition:

-      Absolute time (measured UTC time) when RLF occurred

Alt 2: a flag to indicate whether the RLF happens before T1 or after T1

Alt 3: based on existing timers in RLF report.

Further check the granularity of the current timers.

For location based trigger condition:

Too late CHO Execution: RLF happens in the source cell before CHO execution. The UE re-establishes in a cell other than the source cell.

Too Early CHO Execution/CHO Execution to Wrong Cell: HOF happens or RLF happens shortly after successful CHO, and the UE re-connects in the source cell or a third cell.

For earth fixed case, there is no need to report the measured distance from the UE.

FFS on the earth moving case.

For joint trigger condition:

Agree on the following definition of the use cases and agreement:

- Case 1: Too Late CHO Execution: RLF happens in the source cell. The UE re-establishes in the cell other than the source cell.

- Case 2: Too Early CHO Execution/CHO Execution to Wrong Cell: HOF happens or RLF happens in the target cell short after successful HO. The UE re-establishes in the source cell or a third cell.

UE reports the fulfilled CHO trigger conditions before RLF occurs in case of “time and measurement based trigger condition” or “location and measurement based trigger condition”, it is up to RAN2 to decide to reuse the existing IE or not.

MDT for NTN:

New finer area scope for MDT:

Is this for signaling based MDT or for management-based MDT?

Introduce a geographical area (a reference location plus a radius) as the area scope for MDT or a mapped cell ID list?

In this contribution, we continue on this topic and present our further proposals.

2	Discussion

2.1	SON for intra-NTN mobility

2.1.1	Failure scenarios

Time based trigger condition:

The following WAs are made at last meeting:

WA: down-select among below three alternatives.

Alt 1: the RLF report includes for single time-based trigger condition:

-      Absolute time (measured UTC time) when RLF occurred

Alt 2: a flag to indicate whether the RLF happens before T1 or after T1

Alt 3: based on existing timers in RLF report.

Further check the granularity of the current timers.

According to TS 38.331, the event T1-threthold is in 10ms granularity in UTC form. And the T1 duration has a range of 1-6000, with each step representing 100ms as shown below.

t1-Threshold The field counts the number of UTC seconds in 10 ms units since 00:00:00 on Gregorian calendar date 1 January, 1900 (midnight between Sunday, December 31, 1899 and Monday, January 1, 1900).

duration This field is used for defining the leaving condition T1-2 for conditional HO event condEventT1. Each step represents 100ms.

Furthermore, the granularity of the timeSinceCHO-Reconfig in the RLF-report is 100ms. The network may not be able to identify whether the RLF is occurred before the event T1 is triggered or after, if the time between the time of RLF and the time when event T1 is triggered is less than 100ms.

Proposal 1:RAN3 to discuss and decide whether an absolute UTC time is needed or a flag is sufficient from the UE RLF report.

Location based trigger condition:

The following agreement and FFS are made at last meeting.

For earth fixed case, there is no need to report the measured distance from the UE.

FFS on the earth moving case.

During the discussion at last meeting, there were comments that the network can calculate the distance to the reference location when RLF occurs based on the location information in the UE RLF report, therefore there is no need any enhancement to the RLF report. While, there were also concerns on the uncertainty and the complexity to the network calculation based solution.

The following quotations shows the definition of distanceThreshFromReference and the EpochTime.

distanceThreshFromReference1, distanceThreshFromReference2 Distance from a fixed reference location configured with referenceLocation1 or referenceLocation2 for condEventD1. Distance from a moving reference location determined by the UE based on the serving cell movingReferenceLocation broadcast in SIB19 or referenceLocation and the corresponding epoch time and satellite ephemeris configured within the MeasObjectNR associated to the event for condEventD2. Each step represents 50m.

– EpochTime The IE EpochTime is used to indicate the epoch time for the NTN assistance information, and it is defined as the starting time of a DL sub-frame, indicated by a SFN and a sub-frame number signaled together with the assistance information. The reference point for EpochTime of the serving, target, or neighbour NTN payload ephemeris and Common TA parameters is the uplink time synchronization reference point when this field is provided in an NTN cell and the gNB when this field is provided in a TN cell. In case of handover or conditional handover, EpochTime is based on the timing of the target cell, i.e. the SFN and sub-frame number indicated refers to the SFN and sub-frame of the target cell. Otherwise, EpochTime is used based on the timing of the serving cell, i.e. the SFN and sub-frame number indicated refers to the SFN and sub-frame of the serving cell.

From above quotation, we may learn that the granularity of the UE measured distance to the reference location is 50m. And the granularity of the input epoch time is the starting time of a DL sub-frame which means a 1ms granularity.

Observation 1: The UE measured distance has a granularity of 50m with a 1ms granularity epoch time as input.

If we let the network to calculate, the complexity is shown in the following Figure 1. The uncertainty is  mainly introduced by the timeSinceCHO-Reconfig in the RLF report when calculating the UTC time of RLF (step 1 in Figure 1. Because the granularity of the timeSinceCHO-Reconfig is 100ms. The uncertainty will be enlarged in step2 when calculating the satellite location, as the satellite usually is moving quite fast.

According to Table 7.3.2.1.4-1 in TS 38.821, we may assume that the relative velocity of the satellite is 7.56 km/s, meaning 756m/100ms. It roughly indicates that the uncertainty introduced by the timeSinceCHO-Reconfig is 756m which is much higher than the granularity of UE measured distance, i.e, 50m.

Figure 1: distance calculation based on location info in RLF report by the network

Observation 2: The network based solution to calculate the distance to the reference location when RLF occurs has huge complexity and large uncertainty.

The second aspect is that although the RLF already includes the location information. This information is optional in the RLF report. Relying on the location information in RLF report to calculate the distance to reference location when RLF occurred may mandate the UE to always include the location information into the RLF report.

Observation 3: The network based solution to calculate the distance to the reference location when RLF occurs mandates the UE to include the location information in the RLF report.

On the other hand, collecting the measured distance when RLF occurs and including it in the RLF report by the UE bring very less cost to the UE.

Based on above observations, we propose:

Proposal 2:	In case of location based trigger condition, the RLF report includes the following information.

- An indicator to indicate which threshold is fulfilled, the distanceThreshFromReference1, the distanceThreshFromReference2, or neither.

- The measured distanceFromReference1 when the condition to the candidate cell is fulfilled, or the measured distanceFromReference2 when the condition to the source cell is fulfilled.

- The time duration between the two thresholds fulfilled.

Joint trigger condition:

The following agreement was made at last meeting.

UE reports the fulfilled CHO trigger conditions before RLF occurs in case of “time and measurement based trigger condition” or “location and measurement based trigger condition”, it is up to RAN2 to decide to reuse the existing IE or not.

- Based on the dicusson at last meeting, it was agreed to send a LS to RAN2 to inform the progress.

- Proposal 3: RAN3 to send a LS to inform RAN2 about the progress on this topic.

2.2	MDT for NTN

2.3.2	Area scope enhancement

- The need of a smaller area scope for MDT for NTT was discussed at last meeting, and the following FFSs are left.

Is this for signaling based MDT or for management-based MDT?

Introduce a geographical area (a reference location plus a radius) as the area scope for MDT or a mapped cell ID list?

We believe that both the management based MDT and signalling based MDT can benefit from the smaller area scope when the operators initiate MDT to the UEs in NTN.

Regarding the definition of the new area scope, we think that a geographical area (a reference location plus a radius) is needed. As the mapped cell ID list is not applicable for logged MDT.

Proposal 4: RAN3 to agree to introduce a geographical area (a reference location plus a radius) as the area scope for both management based MDT and signalling based MDT in NTN.

A LS to RAN2 and SA5 may be needed to inform them to update their spec accordingly.

Proposal 5: To send a LS to RAN2 and SA5 to inform the progress on MDT in NTN.

3	Conclusion

This document proposes:

Proposal 1:RAN3 to discuss and decide whether an absolute UTC time is needed or a flag is sufficient from the UE RLF report.

Observation 1: The UE measured distance has a granularity of 50m with a 1ms granularity epoch time as input.

Observation 2: The network based solution to calculate the distance to the reference location when RLF occurs has huge complexity and large uncertainty.

Observation 3: The network based solution to calculate the distance to the reference location when RLF occurs mandates the UE to include the location information in the RLF report.

Proposal 2:	In case of location based trigger condition, the RLF report includes the following information.

- An indicator to indicate which threshold is fulfilled, the distanceThreshFromReference1, the distanceThreshFromReference2, or neither.

- The measured distanceFromReference1 when the condition to the candidate cell is fulfilled, or the measured distanceFromReference2 when the condition to the source cell is fulfilled.

- The time duration between the two thresholds fulfilled.

Proposal 3: RAN3 to send a LS to inform RAN2 about the progress on this topic.

Proposal 4: RAN3 to agree to introduce a geographical area as the area scope for both management based MDT and signalling based MDT in NTN.

Proposal 5: To send a LS to RAN2 and SA5 to inform the progress on MDT in NTN.

The corresponding TPs and draft LSs are provided in the annex.

Annex	TP for MDT for 38.413

/*********************Start of changes************************************/

9.3.1.169	MDT Configuration-NR

This IE defines the MDT configuration parameters of NR.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| MDT Activation | M |  | ENUMERATED (Immediate MDT only, Logged MDT only, Immediate MDT and Trace, …) |  | - |  |
| CHOICE Area Scope of MDT | M |  |  |  | - |  |
| >Cell based |  |  |  | If PNI-NPN Area Scope of MDT IE is present, this IE covers non-CAG cells only, where non-CAG cells refer to cells that only provide public access. |  |  |
| >>Cell ID List for MDT |  | 1..<maxnoofCellIDforMDT> |  |  | - |  |
| >>>NR CGI | M |  | 9.3.1.7 |  | - |  |
| >TA based |  |  |  | If PNI-NPN Area Scope of MDT IE is present, this IE covers non-CAG cells only, where non-CAG cells refer to cells that only provide public access. |  |  |
| >>TA List for MDT |  | 1..<maxnoofTAforMDT> |  |  | - |  |
| >>>TAC | M |  | 9.3.3.10 | The TAI is derived using the current serving PLMN. | - |  |
| >PLMN wide |  |  | NULL |  |  |  |
| >TAI based |  |  |  | If PNI-NPN Area Scope of MDT IE is present, this IE covers non-CAG cells only, where non-CAG cells refer to cells that only provide public access. |  |  |
| >>TAI List for MDT |  | 1..<maxnoofTAforMDT> |  |  | - |  |
| >>>TAI | M |  | 9.3.3.11 |  | - |  |
| >PNI-NPN Based MDT |  |  |  |  | YES | ignore |
| >>CAG List for MDT |  |  | 9.3.3.65 |  | - |  |
| >SNPN Cell Based MDT |  |  |  |  | YES | ignore |
| >>SNPN Cell ID List for MDT |  | 1..<maxnoofCellIDforMDT> |  |  | - |  |
| >>>NR CGI | M |  | 9.3.1.7 |  | - |  |
| >>>NID | M |  | 9.3.3.42 | Identifies an SNPN together with the PLMN Identity in the NR CGI IE. | - |  |
| >SNPN TAI Based MDT |  |  |  |  | YES | ignore |
| >>SNPN TAI List for MDT |  | 1..<maxnoofTAforMDT> |  |  | - |  |
| >>>TAI | M |  | 9.3.3.11 |  | - |  |
| >>>NID | M |  | 9.3.3.42 | Identifies an SNPN together with the PLMN Identity in the TAI IE. | - |  |
| >SNPN Based MDT |  |  |  |  | YES | ignore |
| >>SNPN List for MDT |  | 1..<maxnoofMDTSNPNs> |  |  | - |  |
| >>>PLMN Identity | M |  | 9.3.3.5 |  | - |  |
| >>>NID | M |  | 9.3.3.42 | Identifies an SNPN together with the PLMN Identity IE. | - |  |
| >GeoLocation Based MDT |  |  |  |  | YES | ignore |
| >>Reference Location | M |  | OCTET STRING | This IE is used as a reference location and includes a Ellipsoid-Point IE as defined in TS37.355 [x]. | - |  |
| >> Distance Radius | M |  | INTEGER(0..65535) | Distance from the reference location. | - |  |
| CHOICE MDT Mode | M |  |  |  | - |  |
| >Immediate MDT |  |  |  |  |  |  |
| >>Measurements to Activate | M |  | BITSTRING (SIZE(8)) | Each position in the bitmap indicates a MDT measurement, as defined in TS 37.320 [41]. First Bit = M1, Second Bit= M2, Third Bit = M4, Fourth Bit = M5, Fifth Bit = M6, Sixth Bit = M7, Seventh Bit = logging of M1 from event triggered measurement reports according to existing RRM configuration, other bits reserved for future use. Value “1” indicates “activate” and value “0” indicates “do not activate”. | - |  |
| >>M1 Configuration | C-ifM1 |  | 9.3.1.171 |  | - |  |
| >>M4 Configuration | C-ifM4 |  | 9.3.1.172 |  | - |  |
| >>M5 Configuration | C-ifM5 |  | 9.3.1.173 |  | - |  |
| >>M6 Configuration | C-ifM6 |  | 9.3.1.174 |  | - |  |
| >>M7 Configuration | C-ifM7 |  | 9.3.1.175 |  | - |  |
| >>Bluetooth Measurement Configuration | O |  | 9.3.1.177 |  | - |  |
| >>WLAN Measurement Configuration | O |  | 9.3.1.178 |  | - |  |
| >>MDT Location Information | O |  | 9.3.1.176 |  | - |  |
| >>Sensor Measurement Configuration | O |  | 9.3.1.179 |  | - |  |
| >Logged MDT |  |  |  |  |  |  |
| >>Logging Interval | M |  | ENUMERATED (320ms, 640ms, 1280ms, 2560ms, 5120ms, 10240ms, 20480ms, 30720ms, 40960ms, 61440ms, infinity, …) | Corresponds to the LoggingInterval IE as defined in TS 38.331 [18]. | - |  |
| >>Logging Duration | M |  | ENUMERATED (10, 20, 40, 60, 90,120, …) | Corresponds to the LoggingDuration IE as defined in TS 38.331 [18]. Unit: [minute]. | - |  |
| >>CHOICE Report Type | M |  |  |  | - |  |
| >>>Periodical |  |  | NULL |  |  |  |
| >>>Event Triggered |  |  |  |  |  |  |
| >>>>Event Trigger Logged MDT Configuration | M |  | 9.3.1.180 |  | - |  |
| >>Bluetooth Measurement Configuration | O |  | 9.3.1.177 |  | - |  |
| >>WLAN Measurement Configuration | O |  | 9.3.1.178 |  | - |  |
| >>Sensor Measurement Configuration | O |  | 9.3.1.179 |  | - |  |
| >>Area Scope of Neighbour Cells | O |  | 9.3.1.182 |  | - |  |
| >>Early Measurement | O |  | ENUMERATED (true, ...) | This IE indicates whether the UE is allowed to log measurements on early measurement related frequencies in logged MDT as specified in TS 38.331 [18]. | YES | ignore |
| Signalling Based MDT PLMN List | O |  | MDT PLMN List 9.3.1.168 |  | - |  |
| PNI-NPN Area Scope of MDT |  | 0..1 |  | This IE is ignored if the Area Scope of MDT IE is set to "PLMN Wide" | YES | ignore |
| >CAG List for MDT | M |  | 9.3.3.65 |  | - |  |

/*********************End of changes************************************/

Annex	TP for MDT for 38.423

/*********************Start of changes************************************/

9.2.3.126	MDT Configuration-NR

The IE defines the MDT configuration parameters of NR.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| MDT Activation | M |  | ENUMERATED (Immediate MDT only, Immediate MDT and Trace, Logged MDT only, ...) |  | – |  |
| CHOICE Area Scope of MDT-NR | O |  |  |  | – |  |
| >Cell based |  |  |  | If PNI-NPN Area Scope of MDT IE is present, this IE covers non-CAG cells only, where non-CAG cells refer to cells that only provide public access. |  |  |
| >>Cell ID List for MDT-NR |  | 1 .. <maxnoofCellIDforMDT> |  |  | – |  |
| >>>NR CGI | M |  | 9.2.2.7 |  | – |  |
| >TA based |  |  |  | If PNI-NPN Area Scope of MDT IE is present, this IE covers non-CAG cells only, where non-CAG cells refer to cells that only provide public access. |  |  |
| >>TA List for MDT |  | 1 .. <maxnoofTAforMDT> |  |  | – |  |
| >>>TAC | M |  | 9.2.2.5 | The TAI is derived using the current serving PLMN. | – |  |
| >TAI based |  |  |  | If PNI-NPN Area Scope of MDT IE is present, it covers non-CAG cells only, where non-CAG cells refer to cells that only provide public access. |  |  |
| >>TAI List for MDT |  | 1 |  |  | – |  |
| >>>TAI List for MDT Item |  | 1 .. <maxnoofTAforMDT> |  |  | – |  |
| >>>>PLMN Identity | M |  | 9.2.2.4 |  | – |  |
| >>>>TAC | M |  | 9.2.2.5 |  | – |  |
| >PNI-NPN Based MDT |  |  |  |  | YES | ignore |
| >>CAG List for MDT |  |  | 9.2.3.191 |  | – |  |
| >SNPN Cell Based MDT |  |  |  |  | YES | ignore |
| >>SNPN Cell ID List for MDT |  | 1..<maxnoofCellIDforMDT> |  |  | – |  |
| >>>NR CGI | M |  | 9.2.2.7 |  | – | - |
| >>>NID | M |  | 9.2.2.65 | Identifies an SNPN together with the PLMN Identity in the NR CGI IE. | – | - |
| >SNPN TAI Based MDT |  |  |  |  | YES | ignore |
| >>SNPN TAI List for MDT |  | 1..<maxnoofTAforMDT> |  |  | – | - |
| >>>PLMN Identity | M |  | 9.2.2.4 |  | – | - |
| >>>TAC | M |  | 9.2.2.5 |  | – | - |
| >>>NID | M |  | 9.2.2.65 | Identifies an SNPN together with the PLMN Identity IE. | – | - |
| >SNPN Based MDT |  |  |  |  | YES | ignore |
| >> SNPN List for MDT |  | 1..<maxnoofMDTSNPNs> |  |  | – | - |
| >>>PLMN Identity | M |  | 9.2.2.4 |  | – | - |
| >>>NID | M |  | 9.2.2.65 | Identifies an SNPN together with the PLMN Identity IE. | – | - |
| >GeoLocation Based MDT |  |  |  |  | YES | ignore |
| >>Reference Location | M |  | OCTET STRING | This IE is used as a reference location and includes a Ellipsoid-Point IE as defined in TS37.355 [x]. | - |  |
| >> Distance Radius | M |  | INTEGER(0..65535) | Distance from the reference location. | - |  |
| CHOICE MDT Mode | M |  |  |  | – |  |
| >Immediate MDT-NR |  |  |  |  |  |  |
| >>Measurements to Activate | M |  | BITSTRING (SIZE(8)) | Each position in the bitmap indicates a MDT measurement, as defined in TS 37.320 [43]. First Bit = M1, Second Bit= M2, Fourth Bit = M4, Fifth Bit = M5, Sixth Bit = logging of M1 from event triggered measurement reports according to existing RRM configuration, Seventh Bit = M6, Eighth Bit = M7. Value "1" indicates "activate" and value "0" indicates "do not activate". This version of the specification does not use bits 3. | – |  |
| >>M1 Configuration | C-ifM1 |  | 9.2.3.128 |  | – |  |
| >>M4 Configuration | C-ifM4 |  | 9.2.3.129 |  | – |  |
| >>M5 Configuration | C-ifM5 |  | 9.2.3.130 |  | – |  |
| >>MDT Location Information | O |  | BITSTRING(SIZE(8)) | Each position in the bitmap represents requested location information as defined in TS 37.320 [43]. First Bit = GNSS Other bits are reserved for future use and are ignored if received. Value "1" indicates "activate" and value "0" indicates "do not activate". The eNB shall ignore the first bit unless the Measurements to Activate IE has the first bit or the sixth bit set to "1". | – |  |
| >>M6 Configuration | C-ifM6 |  | 9.2.3.131 |  | – |  |
| >>M7 Configuration | C-ifM7 |  | 9.2.3.132 |  | – |  |
| >>Bluetooth Measurement Configuration | O |  | 9.2.3.134 |  | – |  |
| >>WLAN Measurement Configuration | O |  | 9.2.3.135 |  | – |  |
| >>Sensor Measurement Configuration | O |  | 9.2.3.136 |  | – |  |
| >Logged MDT-NR |  |  |  |  |  |  |
| >>Logging interval | M |  | ENUMERATED (ms320, ms640, ms1280, ms2560, ms5120, ms10240, ms20480, ms30720, ms40960 , ms61440, infinity, ...) | Corresponds to information provided in the LoggingInterval IE as defined in TS 38.331 [10]. The value "infinity" represents one shot logging, i.e., only one log per event in the logged MDT report. | – |  |
| >>Logging duration | M |  | ENUMERATED (10, 20, 40, 60, 90, 120) | Corresponds to information provided in the LoggingDuration IE as defined in TS 38.331 [10]. Unit: [minute]. | – |  |
| >>CHOICE Report Type | M |  |  |  | – |  |
| >>>Periodical |  |  |  |  |  |  |
| >>>Event Triggered |  |  |  |  |  |  |
| >>>>Logged Event Trigger Config | M |  | 9.2.3.137 |  | – |  |
| >>Bluetooth Measurement Configuration | O |  | 9.2.3.134 |  | – |  |
| >>WLAN Measurement Configuration | O |  | 9.2.3.135 |  | – |  |
| >>Sensor Measurement Configuration | O |  | 9.2.3.136 |  | – |  |
| >>Area Scope of Neighbour Cells | O |  | 9.2.3.140 |  | – |  |
| >>Early Measurement | O |  | ENUMERATED (true, ...) | This IE indicates whether the UE is allowed to log measurements on early measurement related frequencies in logged MDT as specified in TS 38.331 [10]. | – |  |
| Signalling based MDT PLMN List | O |  | MDT PLMN List 9.2.3.133 |  | – |  |
| PNI-NPN Area Scope of MDT |  | 0..1 |  |  | YES | Ignore |
| >CAG List for MDT | M |  | 9.2.3.191 |  | – |  |

/*********************End of changes************************************/

Annex – TP for MDT BLCR for TS37.320

/*********************Start of changes************************************/

5.1.1.1.1	Configuration parameters

The logged measurement configuration consists of:

-	configuration of downlink pilot strength measurements logging for (E-)UTRA and NR.

-	configuration of MBSFN measurement logging for E-UTRA.

-	configuration of the triggering of logging events:

-	for (E-)UTRAN:

-	periodic measurement trigger is supported, for which the logging interval is configurable. The parameter specifies the periodicity for storing MDT measurement results. It should be configured in seconds in multiples of the applied IDLE mode DRX, i.e. multiples of 1.28s which is either a factor or multiple of the IDLE mode DRX. The UE behaviour is unspecified when the UE is configured with a DRX cycle larger than the logging interval.

-	for NR:

-	periodic measurement trigger is supported, for which the logging interval is configurable. The parameter specifies the periodicity for storing MDT measurement results.

-	for E-UTRAN and NR:

-	event-based trigger is supported, for which the logging interval is configurable, which determines periodical logging of available data (e.g. time stamp, location information), and the following two types of events are supported:

-	measurement quantity-based event L1, for which the event threshold, hysteresis, and time to trigger are configurable. If the configured time to trigger is not a multiple of the DRX cycle, then the UE uses the next multiple of DRX cycle duration that is larger than the time to trigger for evaluating the event L1;

-	out-of-coverage detection trigger.

NOTE:	The logging configuration for event-based and periodical DL pilot strength logged measurements can be configured independently. Only one type of event can be configured to the UE.

-	configuration of the logging duration. This configuration parameter defines a timer activated at the moment of configuration, that continues independent of state changes, RAT or RPLMN change. When the timer expires the logging is stopped and the configuration is cleared (except for the parameters that are required for further reporting e.g. network absolute time stamp, trace reference, trace recording session reference and TCE Id).

-	network absolute time stamp to be used as a time reference to UE.

-	Trace Reference parameter as indicated by the OAM configuration as specified in TS 32.422 [6].

-	Trace Recording Session Reference as indicated by the OAM configuration as specified in TS 32.422 [6].

-	TCE Id as indicated by the OAM configuration as specified in TS 32.422 [6].

-	(optionally) MDT PLMN List, indicating the PLMNs where measurement collection and log reporting is allowed. It is either the Management Based MDT PLMN List or the Signalling Based MDT PLMN List, depending on how the Logged MDT task was initiated (see 5.1.3).

-	(optionally) configuration of a logging area. A UE will log measurements as long as it is within the configured logging area. The scope of the logging area may consist of one of:

-	a list of up to 32 global cell identities for PLMN, and, for NR, additionally a list of up to 144 PNI-NPNs with a maximum of 12 different PLMN identities. If one or both of these lists are configured, the UE will only log measurements when camping in any of the cells belonging to the list of global cell identities, or in any of the cells belonging to the listed PNI-NPNs.

-	a list of up to 8 TAs or 8 LAs or 8 RAs for PLMN, and, for NR, additionally a list of up to 256 PNI-NPNs. If one or both of these lists are configured, the UE will only log measurements when camping in any cell belonging to the preconfigured TA/LA/RAs, or in any of the cells belonging to the listed PNI-NPNs.

-	for NR, a list of inter-frequency neighbouring cells per frequency.

-	for NR, a list of up to 144 PNI-NPNs with a maximum of 12 different PLMN identities.

-	for NR, a list of up to 16 SNPNs.

-	for NR, a list of up to 32 global cell identities for SNPN. If this list is configured, the UE will only log measurements when camping in any of these cells.

-	for NR, a list of up to 8 TAs for SNPN. If this list is configured, the UE will only log measurements when camping in any cell belonging to the configured TAs.

-	for NR, a Geographical location area. If this geographical location area is configured, the UE ill only log measurements when comping in any cell belongs to the configured location area.

-	The configured logging area can span one of:

-	PLMNs in the MDT PLMN List. If no area is configured, the UE will log measurements throughout the PLMNs of the MDT PLMN list.

-	Any configured SNPN area.

-	(optionally) for NR, configuration of a list of neighbouring frequencies and/or cells, indicating the UE to include neighbouring cell's measurements as indicated in the list in the logged MDT report.

-	(optionally) for E-UTRA, configuration of target MBSFN area(s) for MBSFN measurement logging. If target MBSFN area(s) is configured, UE applies it in addition to other restrictions such as the logging area. The UE will log measurements as long as it receives MBMS service from an indicated target MBSFN area and is within the configured logging area. The target MBSFN area(s) is defined by a list of up to 8 entries, where each entry indicates a carrier frequency and optionally indicates a specific MBSFN area on a carrier frequency.

-	(optionally) configuration of the WLAN access point names, indicating the UE to attempt to obtain WLAN measurements associated to these access points.

-	(optionally) configuration of the Bluetooth beacon names, indicating the UE to attempt to obtain Bluetooth measurements associated to these beacons.

-	(optionally) for NR, configuration of the sensor names, indicating the UE to attempt to obtain sensor measurements.

-	(optionally) for E-UTRA, configuration indicating the UE to attempt to obtain uncompensated barometric pressure measurements.

-	(optionally) for NR, the network can use a flag to indicate if an early measurement/idle mode configuration has relevance for logged measurement purposes, indicating the UE is allowed to log the measurement results related to early measurement frequencies in the logged MDT report.

-	(optionally) for NR and E-UTRA, logged MDT type flag, indicating the logged measurement configuration is the signalling based MDT (see 5.4.0).

Annex	LS to RAN2 on SON for NTN

Title:	[DRAFT] LS on SON for NTN

Response to:

Release:	Rel-19

Work Item:	NR_ENDC_SON_MDT_Ph4-Core

Source:	Huawei [will be RAN3]

To:	RAN2

Cc:

Contact person: 	Zhang Hongzhuo

zhanghongzhuo@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

- Overall description

RAN3 discussed the MRO for intra-NTN mobility and would like to inform RAN2 about the following agreement.

For location based CHO, the RLF report includes the following information:

- An indicator to indicate which threshold is fulfilled, the distanceThreshFromReference1, the distanceThreshFromReference2, or neither.

- The measured distanceFromReference1 when the condition to the candidate cell is fulfilled, or the measured distanceFromReference2 when the condition to the source cell is fulfilled.

- The time duration between the two thresholds fulfilled.

For joint trigger conditions based CHO:

- UE reports the fulfilled CHO trigger conditions before RLF occurs in case of joint trigger conditions based CHO.

- It is up to RAN2 to decide whether to reuse the existing IE or not.

# 2	Actions

To RAN2

ACTION: 	RAN3 kindly ask RAN2 to take the above RAN3 agreements into account.

# 3	Dates of next RAN3 meetings

Updated meeting schedule can be found at: https://portal.3gpp.org/?tbid=373&SubTB=381#/

RAN3#126	2024-11-18  – 2024-11-22 	Orlando, US

RAN3#127	2025-02-17  – 2024-02-21	Athens, GR

Annex	LS to RAN2  and SA5 on MDT for NTN

Title:	[DRAFT] LS on MDT for NTN

Response to:

Release:	Rel-19

Work Item:	NR_ENDC_SON_MDT_Ph4-Core

Source:	Huawei [will be RAN3]

To:	RAN2, SA5

Cc:

Contact person: 	Zhang Hongzhuo

zhanghongzhuo@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

- Overall description

RAN3 discussed the support of MDT in NTN. RAN3 would like to inform that RAN3 agrees to introduce a geographical area (a reference location plus a radius) as the area scope for both management based MDT and signalling based MDT in NTN.

# 2	Actions

To RAN2

ACTION: 	RAN3 kindly ask RAN2 and SA5 to take the above into account.

# 3	Dates of next RAN3 meetings

Updated meeting schedule can be found at: https://portal.3gpp.org/?tbid=373&SubTB=381#/

RAN3#126	2024-11-18  – 2024-11-22 	Orlando, US

RAN3#127	2025-02-17  – 2024-02-21	Athens, GR