3GPP TSG-RAN WG3 Meeting #126                                                                  R3-247276

Orlando, USA, Nov 18th – 22nd, 2024

Source:	CATT, ZTE, NEC

Title:	Support cell DTXDRX for NES

Agenda Item:	31.2

Document for:	Discussion and Decision

1 Introduction

In Rel-18 NES, the technique of cell DTXDRX is specified over air interface, however, the discussion in RAN3 has not finalized the specification for supporting cell DTXDRX over network interfaces.

This contribution provides analysis and proposals on impacts to network interfaces, i.e., F1 and Xn, for supporting cell DTXDRX.

2 Discussion

2.1 Cell DTXDRX configuration

GNB would use cell DTXDRX based on the resource usage. If the resource usage is low, e.g., lower than a threshold, the gNB can determine to use and configure cell DTX/DRX to UE. On the contrary, the gNB can determine release cell DTX/DRX if the resource usage is high enough.

In split architecture, both gNB-CU and gNB-DU have the UE capability information. But different to the gNB-CU, the gNB-DU knows the cell capacity, and it takes charge of DL/UL scheduling for each cell based on the QoS of traffic flow. The gNB-DU can control and thus be aware of the resource usage status of each cell. Hence, the gNB-DU has sufficient knowledge on determining the condition is fulfilled or not for using cell DTX/DRX. But the gNB-CU does not have the adequate information to derive the resource usage. Furthermore, the cell DTX/DRX configuration and activation/deactivation is performed by gNB-DU. As specified by RAN2, gNB is allowed to configure cell DTX to UEs which are configured with C-DRX configuration. To achieve a best efficiency of network energy saving, the gNB should ensures at least partial overlapping between UE C-DRX on-duration and cell DTX/DRX active time.

Clause 15.4.2.3 of TS 38.300 v18.0.0[1]:

Once the gNB recognizes there is an emergency call or public safety related service (e.g. MPS or MCS), the network should ensure that there is no impact to that service (e.g. it may release or deactivate cell DTX/DRX configuration). The network should also ensure that there is at least partial overlapping between UE's connected mode DRX on-duration and cell DTX/DRX active duration, i.e. the UE's connected mode DRX periodicity is a multiple of cell DTX/DRX periodicity or vice versa.

The gNB-CU cannot implement the configuration of cell DTX/DRX, the configuration of UE C-DRX, nor the alignment between cell DTX and UE C-DRX. According to the Rel-18 RRC spec[2], both UE C-DRX and cell DTXDRX are configured or released via CellGroupConfig in RRCReconfiguration message, which is produced by gNB-DU. GNB-DU should take the role to decide using cell DTX/DRX and the parameters for the cell DTX/DRX, and also decide releasing cell DTX/DRX.

Observation 1: gNB-DU has the sufficient information to decide using cell DTXDRX or not.

gNB-DU deciding to configure or release cell DTX/DRX for network energy saving is preferred.

Proposal 1-1: The gNB-DU decides to use cell DTX/DRX and parameters of cell DTX/DRX.

Proposal 1-2: The gNB-DU decides to release cell DTX/DRX.

2.2 F1 impacts

In split architecture, if the gNB-DU decides to apply cell DTX/DRX to a specific cell, the gNB-CU should be aware of that information, so as to carry out access control or handovers (or conditional handovers) to the unsuitable UEs. The gNB-CU may need to initiate handover to remove the unsuitable UEs from the NES cell if the NES cell serves as PCell for those UEs. The gNB-CU can configure NES specific CHO to those UEs supporting NES CHO, or the gNB-CU can carry out the legacy handover procedure for the UE not supporting NES CHO. If the NES cell serves as PSCell/SCell of the unsuitable UE, the CU can release the configuration of the NES cell to the unsuitable UEs.

At least cell DTXDRX configuration parameters including the on-duration timer and periodicity of cell DTXDRX are required by gNB-CU, that the gNB-CU can determine the unsuitable UEs using the parameters in conjunction with QoS information of traffic. The gNB-CU also need to get the cell DTX/DRX type (i.e., only DTX, only DRX or both DTX and DRX) since the determining unsuitable UEs would depend on the cell DTX/DRX type. For example, if cell DTX is configured, the gNB-CU would figure out the unsuitable UEs based on DL QoS information of UE’s traffic.

Observation 2: gNB-CU can remove the unsuitable UEs when it knows cell DTXDRX is configured and parameters of the cell DTXDRX configuration.

On the other hand, if the cell DTX/DRX information is delivered over F1, the gNB-CU can use it to derive the real PRB number used during the on-duration time of the NES cell in conjunction with the resource status report from the gNB-DU on the NES cell. Also, this information is beneficial on interference elimination between cells belonging to different gNB-Dus, for example, on-duration time of different cells can be configured without overlapped period.

Observation 3: The information of cell DTX/DRX is also beneficial for gNB-CU to derive the real resource status of the NES cell, or do interference elimination between different cells.

If the gNB-DU decides to stop using cell DTXDRX and release the cell DTXDRX configuration, it’s also beneficial for the gNB-CU to know that information.

The gNB-DU can send the parameters of cell DTX/DRX configuration when it applies or modifies cell DTX/DRX to a specific cell, or indicate cell DTX/DRX is released to a specific cell to the gNB-CU. That can be done by including a list of cell DTXDRX information in a F1 SETUP REQUEST message or GNB-DU CONFIGURATION UPDATE message.

Proposal 2: The gNB-DU sends the gNB-CU the parameters of cell DTX/DRX configuration when cell DTX/DRX is configured or modified, or indicate the gNB-CU that cell DTX/DRX is released via F1AP messages.

2.3 Xn impacts

When gNB applies cell DTX/DRX to a cell, it can indicate the information to the neighbor nodes via Xn interface so that the neighbor nodes can use the information in case of initiating UE’s handover. If the gNB does not send the information to the neighbor nodes, the neighbor node would not realize the cell is applying NES technique and thus initiate a Xn Handover Preparation procedure to the NES cell for a UE unsuitable of cell DTX/DRX, and that would result in the cell applying cell DTX/DRX terminating the NES operation (i.e., releasing cell DTX/DRX) or handover failure which is not desired. There are other aspects reaping the benefits from the cell DTX/DRX information delivery over Xn, e.g., the resource status for the on-duration time of the NES cell can be derived based on the information in conjunction with the resource status report from the gNB of the NES cell. Or the information can be used for avoiding/eliminating interference with the NES cell, e.g., by concentrating the DL/UL transmission on the non-active period of the NES cell.

For above motivations, the gNB applying cell DTX/DRX can send the cell DTXDRX configuration parameters to the neighbor nodes including at least the on-duration timer, periodicity of cell DTXDRX as well as the cell DTX/DRX type. Accordingly, if the cell DTX/DRX configuration is released, it’s also benefit for the gNB to indicate the information to the neighbor nodes.

The gNB can send the parameters of cell DTX/DRX configuration to the neighbor nodes when it applies or modifies cell DTX/DRX, or indicate the neighbor nodes that cell DTX/DRX is released to a specific cell. That can be done via including a list of cell DTXDRX information in a Xn SETUP REQUEST/RESPONSE message or NG-RAN NODE CONFIGURATION UPDATE message.

Proposal 3: The gNB sends neighbour nodes the parameters of cell DTX/DRX configuration when cell DTX/DRX is configured or modified, or indicates neighbor nodes that cell DTX/DRX is released via XnAP messages.

3 Conclusion

In the previous sections we made the following observations and proposals:

Proposal 1-1: The gNB-DU decides to use cell DTX/DRX and parameters of cell DTX/DRX.

Proposal 1-2: The gNB-DU decides to release cell DTX/DRX.

Proposal 2: The gNB-DU sends the gNB-CU the parameters of cell DTX/DRX configuration when cell DTX/DRX is configured or modified, or indicate the gNB-CU that cell DTX/DRX is released via F1AP messages.

Proposal 3: The gNB sends neighbour nodes the parameters of cell DTX/DRX configuration when cell DTX/DRX is configured or modified, or indicates neighbor nodes that cell DTX/DRX is released via XnAP messages.

4 Reference

- 3GPP TS 38.300, NR and NG-RAN Overall Description, Stage-2, v18.2.0 (2024-06)

- 3GPP TS 38.331, Radio Resource Control (RRC) protocol specification, v18.2.0 (2024-06)