3GPP TSG-RAN WG3 Meeting #126		R3-247282

Orlando, US, 18th – 22nd, Nov, 2024

Source:	CATT

Title:	Support of MBS broadcast service for NR NTN

Agenda Item:	14.2

Document for:	Discussion and Decision

Introduction

For MBS broadcast service topic, the NR NTN Phase 3 WID [1] content is as follows:

Specify signaling of the intended service area of a broadcast service (e.g. MBS broadcast) via NR NTN [RAN2, RAN3] - Specify SIB signaling to indicate the intended service area in case the satellite footprint covers a larger area. [RAN2] - Specify the necessary signaling between CN and NG-RAN. [RAN3]

The last RAN3 meeting has agreed that geographical service area information can also be transmitted from CN with the same format as defined in Uu. In this contribution, we will continue to discuss the issues related to MBS broadcast service topic.

Discussion

RAN3 takes the assumption that geographical service area information can be transmitted from CN with the same format as defined in Uu.

The last RAN3 meeting has agreed that the intended service area provided to the gNB can be Mapped Cell ID(s), TAI(s), or geographical service area information and send LS to SA2, RAN2 and CT4. RAN2 has already reached the following conclusions:

- The intended broadcast service area is defined by a geographical area represented by a (set of) referenceLocation and radius or by a (set of) polygon(s).

Observation 1: RAN2 has decided to define intended service area as a set of circles (reference location and radius) or a set of polygons.

Proposal 1: The format of geographical service area information waits for RAN2 progress.

In the previous RAN3 meetings, a few company mentioned that how to trigger MBS Broadcast Session Setup/Release procedure when the gNB on board enters/leaves a geographical area which is intended for MBS Broadcast.

The legacy procedure is that AMF sends Broadcast Session Setup/Release Request message to gNB. In our opinion, AMF could know when the satellite enters and leaves the MBS broadcast service area based on ephemeris information. Therefore, we understand that AMF could send Broadcast Session Setup Request message to an on-board gNB, when the satellite is about to enter the broadcast area. Which means the session setup could be performed a little time earilier than the satellite start to serve the intended broadcast area, details could be left to implementation.

Observation 2: MBS session setup could be performed a little time earilier than the satellite start to serve the intended broadcast area, details could be left to implementation.

Similarly, AMF could stop the MBS broadcast session by sending Broadcast Session Release Request message to the on-board gNB after the satellite leaves the intended MBS broadcast area.

Observation 3: MBS session stop could be performed a little time later than the satellite leaves the intended MBS broadcast area.

As above, when to setup/release the MBS session could be left to implementation, there is no need to enhance NGAP interface for the MBS Broadcast Session Setup/Release of NTN, the legacy procedure is sufficient.

Proposal 2: When to setup/release MBS broadcast session is up to implementation, no extra enhancement to NGAP is required.

Conclusion

According to the analysis in section 2, the following observations and proposals are provided:

Observation 1: RAN2 has decided to define intended service area as a set of circles (reference location and radius) or a set of polygons.

Proposal 1: The format of geographical service area information waits for RAN2 progress.

Observation 2: MBS session setup could be performed a little time earilier than the satellite start to serve the intended broadcast area, details could be left to implementation.

Observation 3: MBS session stop could be performed a little time later than the satellite leaves the intended MBS broadcast area.

Proposal 2: When to setup/release MBS broadcast session is up to implementation, no extra enhancement to NGAP is required.

Reference

- RP-240775 Rel-19 Moderator WID NR NTN phase 3

- RAN3_125bis_agenda_20241018_1430_EOM