3GPP TSG-RAN WG3 Meeting #126	R3-247312

Orlando, US, 18th – 22nd November 2024

Agenda item:	16.2

Source:	Xiaomi

Title:	Discussion on AIoT RAN architecture

Document for:	Discussion

# 1	Introduction

In this contribution, we’d like discuss possible architecture impacts for topology 1 and topology 2 based on previous agreements and progress of SA2.

# 2	Discussion

Topology 1

In Topology 1 the signalling transport for XXAP termination at the AIoT RAN node is SCTP/IP. Capture in the TP other transport layer options had been discussed in this study is not pursued.

According to the Interim Conclusion in SA2, new AIoTF is introduced and it can either directly or via an AMF connect to gMB.

The principles and aspects in this clause are agreed to support Topology 1: - The new core network function (AIOTF) is introduced to support Ambient IoT functionality, described in clause 8.1.1, with the following features for topology 1: - Communicate with BS Reader (i.e., AIoT RAN), either directly or via an AMF, to trigger e.g., inventory request and command request).

In topology 1, it is more efficient to establish a direct connection between AIoT RAN and AIoT AF without involving AMF for transferring AIoT related information, as non-UE associated operations are not required by AMF.

For indirect connection shown below, extend NG interface or introduce new interface can be considered.

Figure 1 direct connection for T1

For indirect connection shown below, NG interface can be enhanced, either to include SMF-like information or NRPPa-like message for A-IoT related information.

Figure 2 indirect connection for T1

Topology 2

According to the Interim Conclusion in SA2, only RRC-based solution and UP-based solution are captured.

Two options will be specified for Topology 2: - User-plane option as defined in clause 8.1.3.2 - RRC-based option as defined in clause 8.1.3.3 Editor’s note: Whether a NAS-based option is needed in addition is FFS.

For RRC-based solution, there’re also two types of connection captured in SA2, the direct connection architecture is shown in figure 3. Similar to T1, new interface or extend NGAP can be considered. There may some issues, how AIoTF find UE reader needs to be discussed.

Figure 3 direct connection for T2 RRC-based solution

The indirect connection architecture can is shown in figure 4. Similar to T1, introduce SMF-like information or NRPPa-like message in NGAP can be considered, the messages are UE-associated.

Figure 4 indirect connection for T2 RRC-based solution

For UP-based solution or NAS-based solution (if agreed by SA2), the architecture is shown in figure 5, how to allocate A-IoT resource should be discussed.

Figure 5 UP/NAS-based solution for T2

The following table provide a summary for each interface options for T1 and T2

|  | Options | Xx interface | Analysis |
| --- | --- | --- | --- |
| T1 | Direct connection | Extend NG or new interface | Most suitable option for T1 |
|  | Indirect connection | NGAP | Introduce SMF-like information or NRPPa-like message in NGAP |
| T2 | RRC-based direct connection | Extend NG or new interface | How AIoTF select UE needs to be solved. |
|  | RRC-based indirect connection | NGAP | Most suitable option for T2. |
|  | NAS-based (if agreed by SA2) | NGAP | How to allocate AIoT radio resource needs to be solved. |
|  | UP-based | NGAP | How to allocate AIoT radio resource needs to be solved. |

Observation 1, For topology 1 and RRC-based solution in Topology 2, there’re two types connection options between A-IoT RAN and AIoTF:

- for directly connection, extend NG interface or introduce new interface can be considered

- for indirectly connection, NG interface can include either SMF-like information message or NRPPa-like message for A-IoT related information

Observation 2, for UP/NAS-based solution in Topology 2, indirect connection between A-IoT RAN and AIoTF is used.

Observation 3, it’s more efficient to have direct communication between AIoT RAN and AIoT NF for T1

Observation 4, it’s issue-less to use indirect communication between AIoT RAN and AIoT NF for T2.

Proposal 1, RAN3 discuss the following options.

- option 1, use different interface options for T1 and T2, direct connection for T1 and indirect connection for T2

- option 2, use common interface for both T1 and T2, indirect connection for both T1 and T2 is preferred, as there may be some issues for direct connection.

# 3	Conclusion

In this contribution, we discussed possible architecture options and have the following proposals.

Observation 1, For topology 1 and RRC-based solution in Topology 2, there’re two types connection options between A-IoT RAN and AIoTF:

- for directly connection, extend NG interface or introduce new interface can be considered

- for indirectly connection, NG interface can include either SMF-like information message or NRPPa-like message for A-IoT related information

Observation 2, for UP/NAS-based solution in Topology 2, indirect connection between A-IoT RAN and AIoTF is used.

Observation 3, it’s more efficient to have direct communication between AIoT RAN and AIoT NF for T1

Observation 4, it’s issue-less to use indirect communication between AIoT RAN and AIoT NF for T2.

Proposal 1, RAN3 discuss the following options.

- option 1, use different interface options for T1 and T2, direct connection for T1 and indirect connection for T2

- option 2, use common interface for both T1 and T2, indirect connection for both T1 and T2 is preferred, as there may be some issues for direct connection.

# 4	Reference

[1]	 TR 23.700-13 V1.1.0, Study on Architecture support of Ambient power-enabled Internet of Things