3GPP TSG-RAN WG3 Meeting #126	R3-247318

Orlando, US, 18th – 22nd November 2024

Agenda item:	15.2

Source:	Xiaomi

Title:	Support of Store and Forward in IoT NTN

Document for:	Discussion

# 1	Introduction

RAN3 decides to work on Split MME architecture and Full CN on board architecture. Whether eNB only on board architecture is feasible or not can be discussed later.

Check progress in RAN1/2…

In this contribution, we’d like to discuss the LS [1] from RAN2 and discuss the scope of support store and forward in IoT NTN.

# 2	Discussion

### LS from RAN2 R3-247008

RAN2 send an LS to RAN3 indicate that RAN2 decide to support PWS for NB-IoT and wants to check whether RAN3 specifications need any updates to apply it.

RAN2 have started work on this and would like to inform working groups in receipt of this LS of the following agreements: - RAN2 confirms the understanding that this WID objective covers all PWS services, including ETWS and CMAS - Clarify in the spec (and at least in 36.300) that for NB-IoT the ETWS, CMAS, PWS requirement may not be met in some scenarios, e.g., when the UE is in eDRX (can come back to further clarify the specific cases) - The support for PWS introduced for NB-IoT NTN can also be made applicable for NB-IoT in TN, if this does not require additional NB-IoT TN specific changes.. RAN2 understand that this enhancement may have specification impact outside of RAN2 and will provide updates as the objective progresses. 2. Actions: To RAN3, CT1, SA1, SA2 ACTION: RAN2 invites RAN3, CT1, SA1, SA2 to take the above agreements into account and to consider whether any alignment will be needed.

We checked the TS 36.410 and TS 36.413, the specifications for PWS have no limitation description on the use cases, so we think existing RAN3 specifications can apply PWS for NB-IoT NTN, no enhancement is needed.  RAN3 can reply RAN2 that RAN3 specifications already apply PWS for NB-IoT NTN.

Observation 1, existing RAN3 specifications TS 36.410 and TS 36.413 has no limitation description on the use cases.

Proposal 1, RAN3 reply to RAN2 that existing RAN3 specifications can apply PWS for NB-IoT NTN.

### onboard eNB and onboard CN on one satellite

According to RAN3 agreement “RAN3 decides to work on Split MME architecture and Full CN on board architecture. Whether eNB only on board architecture is feasible or not can be discussed later.”, for split MME or full CN onboard architecture, if the gNB and CN part are deployed in a same satellite, the S1 interface signalling can be implementation, RAN3 can conclude that there’s no RAN3 impacts to support this scenario.

Observation 2, if gNB and CN part are deployed in a same satellite, the S1 interface is an internal interface, the signalling can be satellite implementation.

Proposal 2, RAN3 agree that there’s no RAN3 impact if eNB and CN part are deployed in a same satellite.

### onboard eNB and onboard CN on different satellites

One possible deployment option is that the onboard eNB and CN parts are deployed on different satellites, communicating via inter-satellite links as shown in Figure 1.

Figure 1 example arch for eNB and CN in different satellites

if the onboard eNB and onboard CN deployed in different satellites, we believe that the SA2’s solution can be used and there will be no additional SA2 impacts. One consideration is that the eNB needs to be aware of whether the MME on the other satellite has an available feeder link and whether store and forward (S&F) operation is supported, so that it can decide whether to serve UE or whether to broadcast the store and forward operation to UE.

The following scenarios may exist (assuming the eNB and CN can communicate via inter-satellite link):

Case 1: The eNB has no feeder link, while the MME has a feeder link. The eNB can provide normal service to the UE.

Case 2: Both the eNB and CN have no feeder link, and both support S&F. The eNB can broadcast an S&F indication to the UE.

Case 3: The eNB has a feeder link and can connect to the ground CN, allowing the eNB to provide normal service to the UE.

It is evident that the eNB’s behaviour may vary depending on the feeder link status of both the eNB and the MME. Therefore, it would be beneficial for the eNB to be aware of the feeder link status of the CN on other satellites.

Proposal 3, RAN3 discuss whether to support the deployment option that the onboard eNB and onboard CN are on different satellites.

Proposal 4, if the scenario that onboard eNB and onboard CN are deployed on different satellites is supported, eNB needs to be aware of the feeder link status of CN on other satellites.

# 3	Conclusion

Based on the discussion, we have the following observations and proposals.

### LS from RAN2 R3-247008

Observation 1, existing RAN3 specifications TS 36.410 and TS 36.413 has no limitation description on the use cases.

Proposal 1, RAN3 reply to RAN2 that existing RAN3 specifications can apply PWS for NB-IoT NTN.

### onboard eNB and onboard CN on one satellite

Observation 2, if gNB and CN part are deployed in a same satellite, the S1 interface is an internal interface, the signalling can be satellite implementation.

Proposal 2, RAN3 agree that there’s no RAN3 impact if eNB and CN part are deployed in a same satellite.

### onboard eNB and onboard CN in different satellites

Proposal 3, RAN3 discuss whether to support the deployment option that the onboard eNB and onboard CN are on different satellites.

Proposal 4, if the scenario that onboard eNB and onboard CN are deployed on different satellites is supported, eNB needs to be aware of the feeder link status of CN on other satellites.

# 4	Reference

[1] R3-247008, LS on PWS support for NB-IoT NTN, RAN2