3GPP TSG-RAN3 Meeting #126	R3-247326

Orlando, US, Nov 18-22, 2024

Agenda Item:	11.3

Source: 	Qualcomm Incorporated

Title: 	Discussion on AI/ML enabled CCO

Document for:	Discussion/Decision

# Background

In RAN3#125bis we made some agreements as below to use NG-RAN Config Update message used to carry the future CCO state and cause over Xn.

RAN3#125bis Transferring Future Coverage State and the associated Coverage Modification Cause in the NG-RAN NODE CONFIGURATION UPDATE message. The Coverage Modification Cause reflects the predicted CCO issue. WA: Reusing the CU configuration update procedure to transfer the predicted CCO issue and reusing the DU configuration update procedure to transfer the Future Coverage State over F1. Introducing separate time information for future coverage state and predicted CCO issue. Time for future coverage state: The point in time when the future coverage state will be applied. Time for predicted CCO issue: The point in time when the CCO issue is predicted to happen. Continue to work on the stage2/3 details…

In this paper we discuss our views on the stage 3 details for capturing the agreements made in CCO during study phase.

# Discussion

## Predicted CCO Issue and Future CCO State over F1

In the last meeting it was agreed to use NG-RAN Config Update message over Xn to send the Future Coverage State and the associated Coverage Modification Cause from source node to neighbouring nodes. Over F1 it was due to companies concern it was agreed as a Working Assumption to use the Config Update procedure over F1 to send the predicted CCO issue and future CCO state. Some companies proposed to use Data collection procedure to send the predicted CCO issue from CU to DU and DU Config Update message to send the future CCO state with associated modification cause.

Having different messages over different interfaces to transfer the similar information will cause confusion as to which message over Xn should trigger which message over F1. To avoid these ambiguities, it is easier to allow similar message to carry the same information content across interfaces.

Using different messages over Xn and F1 interface to carry future CCO state and associated coverage modification cause will cause ambiguity.

Confirm WA: Reusing the CU configuration update procedure to transfer the predicted CCO issue and reusing the DU configuration update procedure to transfer the Future Coverage State over F1.

There were proposals in the last meeting from companies to re-use the legacy CCO state IE for Future CCO state as well in Config Update message over Xn and F1. This might cause interoperability issue as it is not very clear at the receiving node whether the received information is a futuristic prediction or the legacy information. Hence new IEs would provide cleaner approach to exchange Future CCO State and associated cause value.

New IEs shall be introduced over Xn and F1 Config Update message to send Future CCO state and associated coverage modification cause value.

For feedback configuration to the neighbouring nodes, Data Collection Reporting initiation procedure can be used over Xn. NG-RAN Config Update message over Xn should include the Data Collection ID to indicate performance feedback is required.

Data Collection Reporting initiation procedure can be used to configure UE performance feedback after the new CCO state is applied and XnAP NG-RAN Config Update message should include the Data Collection ID to indicate UE Performance feedback reporting.

The associated time information for predicted CCO issue and future CCO state can be a relative time of certain duration in seconds calculated from the time of receiving the Config Update message over Xn or F1 interface.

The associated time information for predicted CCO issue and future CCO state can be a relative time of certain duration (e.g 0 to 3600) in seconds calculated at the receiving node from the time of receiving the Config Update message over Xn or F1.

## Future CCO State Assistance Information

A Note was added in the TR 38.743 to further discuss if gNB-CU-CP can generate the future CCO state as assistance information and send it to gNB-DU. On this Note, we are unclear on what are the input parameters which gNB-CU-CP will use to generate future CCO state. Moreover OAM provides the CCO state index configuration to gNB-DU. If gNB-CU-CP needs to generate the future CCO state, OAM also needs to configure gNB-CU-CP with CCO state index values which would involve changes to the configuration model and SA5 impact. This is also not a legacy behaviour. There could also be a race condiciton of CU suggesting a different CCO state and OAM informing DU a different CCO State. Hence we propose, that gNB-DU generates the future CCO state on its own and future CCO state need not be provided as assistance information from gNB-CU-CP.

The input parameters used by gNB-CU-CP to generate future CCO state is unclear. OAM provides the CCO state index configuration to gNB-DU. If gNB-CU-CP needs to generate the future CCO state, OAM also needs to configure gNB-CU-CP with CCO state index values which would involve changes to the configuration model and SA5 impact. Race condition at gNB-DU is also a possibility if CU and OAM suggest different CCO state to gNB-DU.

Future CCO state proposed to be generated by gNB-CU-CP as assistance information to gNB-DU is not needed as it involves changes to OAM to configure CCO index value to gNB-CU-CP similar to that of gNB-DU and duplicating functionality across gNB-CU-CP and gNB-DU and causing race condition.

## Assistance Information for Prediction of CCO Issue

It was agreed the gNB-CU will predict the CCO issue and forward it to the gNB-DU. gNB-CU predicts the CCO issue based on L3 measurements. gNB-DU has L1 measurements and beam level load information. Hence gNB-DU has a better visibility of the beam level information for detecting CCO issue. We therefore propose that gNB-DU can send affected beams based on L1 measurements as assistance information to gNB-CU.

gNB-CU uses L3 measurement information as one of the inputs to predict CCO issue. However gNB-CU lacks insight into the beam level information which is available in the gNB-DU. gNB-DU has better visibility of beam level information based on L1 measurements for detecting CCO issue.

For CCO issue prediction, gNB-DU can send assistance information to gNB-CU on affected beams based on L1 measurements. RAN3 to discuss including “Assistance Information on Affected beams” over F1 as input data for CCO issue prediction.

## Future CCO State Feedback

During the SI phase Predicted CCO issue and future CCO state feedback was not discussed due to time constraints. We therefore propose to discuss the feedback for Predicted CCO issue and future CCO state in normative phase.  Once the predicted coverage pattern is applied, the neighboring gNBs can send feedback on Cell performance like throughput, packet loss, packet delay, RLF reports etc to the source gNB to inform the effects of applying predicted coverage pattern. With only UE performance it is unclear which cell the UE belong to (whether it is the cell where the future CCO state was applied or a normal cell). Hence the neighbouring node can aggregate the UE performance at a cell level and send it over Xn or the neighbouring node also reports the NR-CGI where the UE belongs for an UE reporting its performance.

RAN3 to discuss the following options as feedback for the predicted CCO issue and future CCO state exchanged between neighbours over Xn.

- Legacy UE performance metrics including Cell ID where the UE belonged to.

- Reporting Cell level UL and DL throughput between neighbouring nodes

For coverage modification it will be beneficial to get feedback from cell edge UE. SON reports like RLF can also provide necessary feedback. RAN3 to discuss how to get UE performance metrics for cell edge UEs.

RAN3 to discuss how to get UE performance metrics for cell edge UEs.

# Summary

Based on the discussion above, we have following observations:

Observation 1.	Using different messages over Xn and F1 interface to carry future CCO state and associated coverage modification cause will cause ambiguity.

Observation 2.	The input parameters used by gNB-CU-CP to generate future CCO state is unclear. OAM provides the CCO state index configuration to gNB-DU. If gNB-CU-CP needs to generate the future CCO state, OAM also needs to configure gNB-CU-CP with CCO state index values which would involve changes to the configuration model and SA5 impact. Race condition at gNB-DU is also a possibility if CU and OAM suggest different CCO state to gNB-DU.

Observation 3.	gNB-CU uses L3 measurement information as one of the inputs to predict CCO issue. However gNB-CU lacks insight into the beam level information which is available in the gNB-DU. gNB-DU has better visibility of beam level information based on L1 measuremens for detecting CCO issue.

Based on these observations and discussion above, we have the following proposals:

Proposal 1:	Confirm WA: Reusing the CU configuration update procedure to transfer the predicted CCO issue and reusing the DU configuration update procedure to transfer the Future Coverage State over F1.

Proposal 2:	New IEs shall be introduced over Xn and F1 Config Update message to send Future CCO state and associated coverage modification cause value.

Proposal 3:	Data Collection Reporting initiation procedure can be used to configure UE performance feedback after the new CCO state is applied and XnAP NG-RAN Config Update message should include the Data Collection ID to indicate UE Performance feedback reporting.

Proposal 4:	The associated time information for predicted CCO issue and future CCO state can be a relative time of certain duration (e.g 0 to 3600) in seconds calculated at the receiving node from the time of receiving the Config Update message over Xn or F1.

Proposal 5:	Future CCO state proposed to be generated by gNB-CU-CP as assistance information to gNB-DU is not needed as it involves changes to OAM to configure CCO index value to gNB-CU-CP similar to that of gNB-DU and duplicating functionality across gNB-CU-CP and gNB-DU and causing race condition.

Proposal 6:	For CCO issue prediction, gNB-DU can send assistance information to gNB-CU on affected beams based on L1 measurements. RAN3 to discuss including “Assistance Information on Affected beams” over F1 as input data for CCO issue prediction.

Proposal 7:	RAN3 to discuss the following options as feedback for the predicted CCO issue and future CCO state exchanged between neighbours over Xn.

	Legacy UE performance metrics including NRCGI of the cell where the UE belonged to.

	Reporting Cell level UL and DL throughput between neighbouring nodes

Proposal 8:	RAN3 to discuss how to get UE performance metrics for cell edge UEs.