3GPP TSG-RAN WG3 Meeting #126															R3-247335

Orlando, USA, 18th – 22nd November 2024

Agenda item:	8.1

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on the UL RRC message segmentation for UECapabilityInformation

Document for:	Discussion and Decision

# 1	Introduction

RAN3 received a LS ([1]) from RAN2 on UL RRC message segmentation for UECapabilityInformation. This contribution analyses the RAN3 impact.

# 2	Discussion

In the RAN2 LS ([1]) on UL RRC message segmentation for UECapabilityInformation, the RAN3 related aspects are highlighted.

1. Overall Description: RAN2 standard currently supports UL RRC message segmentation for transferring UECapabilityInformation message. Each segment can be as large as the maximum PDCP SDU size and the RRC signalling supports up to 16 segments. RAN2 however came to the understanding that the existing network implementations do not always support the maximum number of RRC segments. RAN2 therefore agreed to introduce a new RRC signalling solution where the gNB indicates the maximum number of UL segments the UE is allowed to use, in UECapabilityEnquiry message. The UE then is required to generate UECapabilityInformation ensuring the size of the message does not exceed the maximum allowed size according to the gNB indication. It was pointed out in the RAN2 discussion that the network limitation mentioned above can reside in RAN or CN (given that the reported UE capability needs to be stored in RAN and CN). In the case that it is CN’s limitation, RAN2 believes that signalling from CN to gNB is necessary so that gNB can indicate the correct value in UECapabilityEnquiry message. RAN2 therefore would like to ask RAN3 to define a signalling solution where CN informs RAN about the maximum UE capability size that CN can accommodate. 2. Actions: To RAN1 group. ACTION: RAN2 asks RAN3 to take the above information into account and to define a signalling solution where CN informs RAN about the maximum UE capability size that CN can accommodate.

In a summary of the RAN2 LS, RAN2 agreed that the gNB can indicate the maximum number of UL segments the UE is allowed to use, in UECapabilityEnquiry message. The UE then is required to generate UECapabilityInformation ensuring the size of the message does not exceed the maximum allowed size according to the gNB indication. As indicated by operators, the limitation for maximum UE capability size can reside in RAN or CN. In case it is CN, RAN2 ask RAN3 to define a signaling solution to enable the CN inform RAN about the maximum UE capability size that CN can accommodate.

In current system, the UE radio capability information is stored in AMF (in case without RACS) or in UCMF (in case with RACS) for CM-IDLE/RM-REGISTERED state UE. The AMF shall if it is available, send its most up to date UE Radio Capability information to the RAN in the NGAP message, i.e. INITIAL CONTEXT SETUP REQUEST message or UE RADIO CAPABILITY CHECK REQUEST message. The UE radio capability information exchanged between the AMF and the gNB is transparent to the AMF.

With the RAN2 agreement, it allows the gNB to retrieve large size of UE radio capability based on the maximum UE capability size of RAN. Based on its maximum size, the gNB can determine the maximum number of RRC segments, so the UE only generate the UE capability information not exceeding the size limitation of the gNB.

The UE radio capability is also sent to CN node (e.g. AMF). The AMF may also have its own maximum UE capability size. However, the gNB is not aware of the maximum UE capability size of the AMF. It is possible that AMF may have a maximum UE capability size different to the RAN. When the AMF has a smaller maximum UE capability size than the gNB, the gNB may provide AMF a UE radio capability that exceeds the Maximum UE capability size in the AMF. This will make the UE/gNB enhancement to support RAN2 agreement less useful. The issue will remain. To address this issue, it is necessary for the gNB to know the maximum UE capability size of the AMF.

Observation 1: there is limitation in both RAN and CN for the Maximum UE capability size.

Observation 2: Without knowing the Maximum UE capability size of the CN node, the RAN node may provide CN a UE radio capability that exceed the Maximum UE capability size in the CN node.

Observation 3: it is necessary for the gNB to know the maximum UE capability size in the AMF.

We prefer to introduce a signaling based solution to enable the AMF to provide its Maximum Size of UE Radio Capability to the RAN, so the RAN can use it when need to retrieve the UE radio capability information from the UE.

For Stage-3, the NGAP enhancement need to consider following aspects:

- It is applicable to more than one UE.

- It is rare that CN may change it.

- It is only applicable to the new gNB that supports the RAN2 agreements.

Based on above reasons, we propose to introduce a new IE with Optional/Ignore in the NG SETUP RESPONSE message.

Proposal 1: Introduce a new IE in the NG SETUP RESPONSE message to allow the AMF provides the Maximum UE capability size to the gNB.

Draft NGAP CR can be found in ([3]).

Similar issue also exists in S1AP. It is unclear whether RAN2 will also discuss the similar enhancements for LTE Uu. We propose to ask RAN2 in the reply LS. Even in case the Uu enhancement is not introduced, it is still beneficial for eNB to know the Maximum UE capability size of the MME, in order to avoid the eNB sending a large UE capability exceeding the capability of the MME.

Observation 4: Similar issue may also exist in LTE system, i.e. LTE Uu and S1AP.

Proposal 2: send the Reply LS to ask RAN2 whether similar enhancement is needed for LTE.

Draft reply LS can be found in ([4])

# 3	Conclusion

In this contribution, we have analyzed the LS from RAN2. Our proposals are:

Observation 1: there is limitation in both RAN and CN for the Maximum UE capability size.

Observation 2: Without knowing the Maximum UE capability size of the CN node, the RAN node may provide CN a UE radio capability that exceed the Maximum UE capability size in the CN node.

Observation 3: it is necessary for the gNB to know the maximum UE capability size of the UE capability information in the AMF.

Proposal 1: Introduce a new IE in the NG SETUP RESPONSE message to allow the AMF provides the Maximum UE capability size to the gNB.

Draft NGAP CR can be found in ([3]).

Observation 4: Similar issue may also exist in LTE system, i.e. LTE Uu and S1AP.

Proposal 2: send the Reply LS to ask RAN2 whether similar enhancement is needed for LTE.

Draft reply LS can be found in ([4])

# References

- R3-247011/R2-2409313 LS on UL RRC message segmentation for UECapabilityInformation

- R3-185104, Reply LS on maximum size of UE Radio Capabilities and maximum Information Element size on network interfaces

- R3-247336, draft NGAP CR

- R3-247338, draft reply LS to RAN2