3GPP TSG-RAN WG3 Meeting #126														R3-247342

Orlando, USA, 18th – 22nd November 2024

Agenda item:	12.2

Source:	Nokia, Nokia Shanghai Bell

Title:	(TP for TS 38.401) Discussion on NG management and Xn management for WAB

Document for:	Discussion and Decision

# 1	Introduction

RAN#105 agreed WI on topological enhancements for NR [1]. The objectives for WAB are following, [1]:

The objectives of the Wireless Access Backhaul (WAB) work are as follows: - Specifications for the support of WAB including [RAN3]: - Support of a WAB-node including a WAB-gNB and a WAB-MT. - Support of backhauling of the WAB-gNB’s NG, Xn and OAM traffic over the WAB-MT’s PDU session(s). - Support of Xn interface(s) by the WAB-gNB with the WAB-MTs serving BH RAN node and with other surrounding gNBs, including how to avoid setting up Xn between WAB-gNBs. - Defining the behaviour of WAB-node in case the authorization status of WAB-MT and/or WAB-gNB changes. - Network integration procedures for WAB nodes. - Handling of WAB-gNB’s traffic (including Xn, NG and OAM traffic) during WAB-node mobility, including the case where the WAB-MT’s BH PDU session changes. - Support the UE’s AMF change for UEs connected to, or camped on, a WAB-gNB. - UE’s ULI that reflect the WAB node’s location. - The handling of: - PCI collision avoidance. - Reconfiguration of TAC and RANAC on WAB-gNBs. - Mechanisms to avoid multi-hop WAB topology. - Radio-resource coordination between access and backhaul links. - NG connection management. NOTE 1: For PCI collision avoidance and reconfiguration of TAC and RANAC on WAB-gNBs, follow the conclusion of mobile IAB. NOTE 2: NG connection management should take the NTN conclusion into account, avoiding parallel discussions. NOTE 3: No impact on the UE. NOTE 4: Coordination with other WGs (e.g. SA2, RAN2) when needed. NOTE 5: Backhaul link for WAB-MT can be TN or NTN. NOTE 6: Mobility procedures to be used for the UEs served by a WAB-gNB are legacy UE mobility procedures. Mobility of the WAB-MTs is based on legacy UE mobility procedures. NOTE 7: The interface between the WAB-MT and the co-located WAB-gNB is out-of-scope for the normative phase. NOTE 8: Split architecture of the WAB-gNB is out-of-scope for the normative phase. NOTE 9: RAN2 impact should be identified as early as possible, and should be minimal.

This contribution analyses the NG management, Xn management, and traffic mapping.

# 2	NG interface management

NR NTN WI have agreed the Stage-2/3 TPs for NG Removal procedure ([3][4]).

WAB is similar to the gNB in NR NTN Regenerative system.  It is proposed to adopt the same decision from NR NTN. Stage-2 TP is needed, e.g. to clarify the WAB-gNB can initiate the NG Removal procedure. Any Stage-3 impact can be discussed later after the BL CR for NR NTN is finalized.

Proposal 1: adopt the NR NTN decision for NG Removal.

The proposed Stage-2 TP can be found in Annex – TP for TS 38.401

# 3	Xn interface management

Previous meeting RAN3 agreed

Establishment of Xn Connections of the WAB-gNB with BH-RAN nodes, as well as with surrounding RAN nodes, is supported and can follow legacy procedures.

However, there is no TP captured in the TR 38.799 for Xn management (e.g. Xn establishment) for the WAB-gNB.

Xn Establishment

Xn establishment between WAB-gNB and the BH-gNB serving WAB-MT

When the WAB-MT connects to a BH-gNB serving WAB-MT, the WAB-MT knows the TAI/CGI of the serving cell. After the integration procedure (i.e. WAB-MT connected with the gNB serving WAB-MT, WAB-gNB setup NG with the AMF, BH PDU session establishment), the WAB-gNB can initiate the Xn-C TNL address discovery procedure towards the BH-gNB serving WAB-MT. The WAB-gNB may then initiate the TNL/Xn Setup with the BH-gNB serving WAB-MT.

Proposal 2-1: WAB-gNB can reuse existing Xn-C TNL address discovery procedure to know the Xn-C TNL address of BH-gNB serving WAB-MT, then setup Xn with BH-gNB serving WAB-MT.

Xn establishment between WAB-gNB and neighboring gNB

The surrounding RAN node can be a stationary gNB, or a WAB-gNB. There is different view on whether the Xn can be setup between the two WAB-gNBs.

In Rel-18, the mobile IAB acts as a gNB-DU/cell. The Xn between the IAB-donor and its neighboring gNB may be already established before mobile IAB connects with IAB-donor. The arrival/departure of a mobile IAB does not affect the Xn between the IAB-donor and its neighboring gNB, other than possibly informing neighbors about newly deployed/released IAB cell(s). From the perspective of a neighboring gNB, the arrival/departure of a mobile IAB can be considered to add/remove cell(s) of the serving IAB-donor.

The WAB brings a new challenge to the inter-gNB neighbor relation. Unlike a new Rel-18 mobile IAB appearing as a new gNB-DU/cell to neighboring gNB, WAB appears as a new gNB to the neighboring gNB. The arrival/departure of mobile WAB causes dynamic setup/release of the Xn between the WAB-gNB and its neighboring gNB(s).

In legacy system, the gNB can initiate the ANR procedure whenever the UE reports an unknown PCI, i.e. a PCI that is not in the NCRT. This can be similar for WAB, e.g. WAB initiate the ANR procedure whenever the UE reports an unknown PCI. However, WAB faces another challenge, for example, when the UE reports a known PCI, the WAB cannot know whether the PCI is related to an existing cell/entry in its NCRT, or the PCI belongs to a new neighboring cell that is not in the NCRT. This is due to the mobility nature of WAB and PCI reuse for cells in different geographical areas.

Figure 1: PCI reuse by cells of gNB1 and gNB3

An example is shown in above figure.

- When the WAB is at location A, it detects the new cell with PCI#1 (i.e. gNB1’s cell), WAB initiate ANR procedure to add gNB1’s cell (PCI#1) in its NCRT.

- Later, when WAB move to location B which is covered by gNB3’s cell with same PCI#1, WAB cannot know whether the PCI#1 corresponds to an existing entry in NCRT that is related to gNB1’s cell, or corresponds to a new cell.

So, it is necessary for WAB-gNB to know the neighboring cell, and a neighboring gNB to know WAB cell as early as possible. In legacy system, the Xn setup is either triggered by OAM, or by ANR based on the UE’s measurement report. However, in the mobile WAB case, the WAB-gNB or neighboring gNB may not be able to know the right time that it needs to ask UE to perform a measurement in order to detect the cell using a specific PCI is changed.

Observation 2-1: Due to PCI reuse, it may be difficult for WAB-gNB (or gNB) to determine the right time to ask UE to perform a measurement in order to detect the new neighbouring cell.

One option is to exchange the neighboring cell information over Xn interface, in order for WAB-gNB to know the neighboring cell, and a neighboring gNB to know WAB cell as early as possible.

The WAB-gNB can reuse the existing mechanism to detect the neighboring cell of a gNB. Since the WAB has a gNB functionality, it can initiate the ANR procedure based on the measurement report from a connected UE, or from the measurement report of the co-located WAB-MT. In addition, the WAB-gNB can also use the information received from BH-gNB serving WAB-MT over Xn.  In most of the scenarios, the WAB’s neighboring gNB is also a neighboring gNB to the BH-gNB. Since the BH-gNB may already setup Xn with the neighboring gNB, BH-gNB can provide the Xn-C TNL address of neighboring gNB to WAB-gNB, thus WAB-gNB can directly initiate the Xn setup procedure with neighboring gNB. Alternatively, during the Xn Setup or NG-RAN node configuration update procedure, the BH-gNB serving WAB-MT can provide its neighboring cell information to the WAB-gNB. The WAB-gNB can use the neighboring cell information received from the BH-gNB to update its NCRT, or initiate Xn-C TNL address discovery procedure for further TNL/Xn Setup with the neighboring gNB, without waiting for the measurement report from UE (or WAB-MT).

Proposal 2-2: BH-gNB can provide the Xn-C TNL address of neighboring gNB to WAB-gNB, so WAB-gNB can directly initiate Xn Setup with neighbour gNB.

Proposal 2-3: WAB-gNB can also use the neighboring cell information received from the BH-gNB to update its NCRT or initiate the Xn-C TNL address discovery procedure towards the neighboring gNB for further TNL/Xn Setup with the neighboring gNB, without waiting for the measurement report from UE (or WAB-MT).

Furthermore, in case Xn is to be avoided between WAB-gNBs (as proposed by some companies in the previous meeting due to the nature of WAB being mobile, which would lead frequent Xn setup and Xn release), this could even be avoided during TNL discovery procedure, i.e., as early as possible. For example, source WAB-gNB can inform the target gNB that it is a WAB during TNL discovery, and target can refrain sending TNL information in case the target itself is a WAB-gNB. That way, Xn establishment can be avoided at the earliest time possible and there would be no unnecessary additional signalling.

Proposal 2-4: If Xn is to be avoided among WAB-gNBs, TNL discovery procedure can be enhanced to avoid Xn establishment as early as possible among WAB-gNBs.

Xn Removal

When the WAB move away, the WAB may need to remove the Xn with previous surrounding gNBs. This can be supported by the existing Xn Removal procedure.

Observation 2-2: Existing Xn Removal procedure can be reused to remove the Xn between WAB-gNB and surround gNB.

Support for RRC INACTIVE

A UE connected to the WAB-gNB may transition to RRC INACTIVE, then move to the surrounding gNB’s coverage.  Similarly, a UE connected to the surrounding gNB may transition to RRC INACTIVE, then move to the WAB-gNB’s coverage. The RNA assigned to the UE may include the RAN area ID/cell ID of WAB-gNB and surrounding gNB.

Observation 2-3: A UE’s RNA can consist of RAN area ID(s)/cell ID(s) of WAB-gNB(s) and surrounding gNB(s).

Observation 2-4: An RRC_CONNECTED UE can transition to RRC_INACTIVE via WAB-gNB.

However, to support the Xn Retrieve UE Context procedure, the Xn Removal procedure should be carefully designed to avoid the scenario where UE context cannot be retrieved due to released Xn interface between the target gNB and the WAB-gNB. This is especially an issue considering WAB’s are moving devices, and Xn may be established and removed dynamically as the WAB moves. In our view, there are at least a couple of ways to tackle the issue:

- Avoiding “Too early Xn Removal”, i.e. avoiding that Xn is removed before Xn Retrieve UE Context Procedure is initiated. This could be, e.g., avoided by rejecting the Xn removal request.

- In case Xn is to be released between the WAB-gNB and another gNB, whose cell(s) are in the RNA of at least one of the UEs that WAB-gNB sent to RRC_INACTIVE, UE context of the corresponding UE(s) are provided to the gNB by the WAB-gNB.

Proposal 2-5: RAN3 discuss the enhancement to avoid the UE context retrieval problems when Xn is removed between a WAB-gNB and a surrounding gNB.

# 4	Conclusion

In this contribution, we discussed the architecture and other high-level aspects of WAB. Our proposals are:

Proposal 1: adopt the NR NTN decision for NG Removal.

Proposal 2-1: WAB-gNB can reuse existing Xn-C TNL address discovery procedure to know the Xn-C TNL address of BH-gNB serving WAB-MT, then setup Xn with BH-gNB serving WAB-MT.

Proposal 2-2: BH-gNB can provide the Xn-C TNL address of neighboring gNB to WAB-gNB, so WAB-gNB can directly initiate Xn Setup with neighbour gNB.

Proposal 2-3: WAB-gNB can also use the neighboring cell information received from the BH-gNB to update its NCRT or initiate the Xn-C TNL address discovery procedure towards the neighboring gNB for further TNL/Xn Setup with the neighboring gNB, without waiting for the measurement report from UE (or WAB-MT).

Proposal 2-4: If Xn is to be avoided among WAB-gNBs, TNL discovery procedure can be enhanced to avoid Xn establishment as early as possible among WAB-gNBs.

Proposal 2-5: RAN3 discuss the enhancement to avoid the UE context retrieval problems when Xn is removed between a WAB-gNB and a surrounding gNB.

The proposed Stage-2 TP can be found in Annex – TP for TS 38.401

# References

- RP-242395, New WID on additional topological enhancements for NR

- TR 38.799 v19.0.0

- R3-245862, (BL CR to 38.410) Introduce NG Removal procedure

- R3-245863, Support for Regenerative Payload and MBS broadcast in NR NTN

# Annex – TP for TS 38.401

## X.2	Configuration of WAB-node

### X.2.1	General

The following configurations of the WAB-node may need to be updated as the node moves:

-	The parameters that enable the WAB-gNB to select and connect to the AMF(s) that serve the UE(s).

-	The parameters that enable the WAB-gNB to connect to, and communicate with, the OAM system.

-	The parameters that the WAB-gNB should broadcast, e.g., the TAC(s), the cell ID(s), the RANAC(s).

A WAB-node may be provisioned with the parameters pertinent to different potential locations of the WAB-node.

Alternatively, the OAM can provision configuration parameters to the WAB-node based on the location of the node. In that case, the continuity of OAM connectivity needs to be ensured as the WAB-node moves.

### X.2.2	IP address configuration for the WAB-gNB

A WAB-MT obtains IP address(es) for the PDU sessions in the same manner as a legacy UE.

The IP address(es) of the WAB-MT can be used for the backhauling of WAB-gNB’s NG, Xn and OAM traffic.

In case the WAB-gNB uses the IPsec tunnel mode to protect the OAM, NG and/or Xn traffic, the IP address(es) of the WAB-MT is used as outer IP address(es), and the allocation of the inner tunnel IP address(es) is outside of 3GPP scope.

### X.2.3	TAC/RANAC (re-)configuration for a WAB-gNB’s cell

The TAC/RANAC of WAB-gNB’s cell is configured by the OAM, and it can be reconfigured by the OAM during the mobility of WAB-node. The TAC/RANAC of the WAB-gNB’s cell may be the same as, or different than, the TAC/RANAC of the co-located WAB-MT’s serving cell. The TAC/RANAC broadcast by the WAB-gNB’s cell can be changed in order to reflect the WAB-node’s physical location.

### X.y1	Management of NG interface for WAB-gNB

Based on the OAM configuration, the WAB-gNB can setup NG interface with an AMF. When disconnecting from an AMF is required due to e.g. the WAB-node move out of the serving area of the AMF, the WAB-gNB may request the removal of the NG interface by triggering the NG Removal procedure toward the AMF.

### x.y2	Management of Xn interface for WAB-gNB

The WAB-gNB may establish Xn interface with surrounding gNBs. The BH-gNB may provide the information of neighboring gNB (e.g. gNB ID, TNL address) to the WAB-gNB via Xn interface, which enables the WAB-gNB to establish Xn with the neighboring gNB(s).

The WAB-gNB may release the Xn interface with a neighboring gNB, e.g. when the WAB node move away from the neighboring gNB. The Xn removal should also consider the stored UE context for RRC INACTIVE UE(s), so the Xn interface removal will not cause issue to further XnAP Retrieve UE Context procedure.

### x.y3	WAB-gNB mobility without change of UE’s AMF(s)

During WAB-node movement, radio configuration parameters of the WAB-gNB may be changed (e.g., cell ID, PCI and/or TAC) without the change of the UE’s AMF(s). This change of radio configuration parameters may require the UE handling by means of, e.g., intra-gNB handover and/or Mobility Registration Update as defined in TS 23.502 [7].