3GPP TSG-RAN WG3 #126                                                                    R3-247350

Orlando, U.S.A., 18 - 22 November, 2024

Agenda item:	13.2

Source:	Samsung

Title:	Discussion on Inter-gNB-CU LTM

Document for:	Discussion & Decision

Introduction

RAN3 had some discussions on inter-CU LTM in previous meetings, and below agreements are achieved:

RAN3#124: Source gNB-CU initiates the handover preparation procedure for inter-gNB-CU LTM. Introduce a new procedure on Xn to transfer the TA information. A new XnAP class 2 procedure, namely LTM Cell Switch Notification is introduced on Xn to forward the target Cell ID and target TCI state ID(s) from the source gNB-CU to the target gNB-CU. Reuse Handover Success procedure over Xn for Rel-19 Inter-CU LTM, to tell the source CU that the UE has accessed to the target Cell. The Handover Cancel message is reused by the source gNB to release the reserved resource for LTM candidate cells in the candidate gNBs. Early data forwarding can be triggered before the Source gNB triggers a MAC CE Command to the UE to change cells, timing is left to implementation. Early sync configuration (TCI state and RACH configuration) can be obtained during the LTM preparation phase through the handover request and handover request acknowledge messages. A candidate gNB can initiate cancellation of configured LTM candidate cell(s) of its own. Details are FFS. FFS on the LTM modification procedures. FFS on whether to reuse the existing XnAP UE CONTEXT RELEASE message at the source gNB if no LTM candidate cell(s) exist in the source gNB. RAN3#125: Introduce a new UE associated Class-1 XnAP procedure to update the LTM configurations for subsequent LTM. Change the name of LTM Cell Switch Notification message to Cell Switch Notification message. Reuse the Early Status Transfer and SN Status Transfer message for inter-CU LTM. Adopt Class-2 procedure for candidate gNB-initiated LTM cancellation. Down select from Option1 and Option3 in the next meeting: Option 1: Reuse CHO Cancel Option 2: Rename CHO Cancel Option 3: Introduce new procedure The following documents are agreed R3-244836 (TP for LTM BLCR for TS38.300): Inter-CU LTM basic procedure RAN3#125bis: Current SSB information in Xn Setup and Configuration Update procedures can be reused for LTM preparation phase. WA: For inter-CU LTM mobility, a separate LTM request message (i.e. HANDOVER REQUEST message) is used for each candidate cell. The error handling of multiple UE associations need to be considered. The LTM Configuration IDs are allocated by the source CU. WA: Reuse the existing XnAP UE CONTEXT RELEASE message at the source gNB if no LTM candidate cell(s) exist in the source gNB. Follow F1AP, the source gNB-CU sends the CSI resource configuration of candidate cells to candidate gNB-CUs via Handover Request message for subsequent LTM, and the candidate gNB-CU sends the CSI report configuration to the source gNB-CU via Handover Request ACK message. How the source gNB-CU sends the reference configuration to all candidate gNBs is pending on RAN2 progress. Confirm the name of the new procedure as “LTM Configuration Update”. Introduce a new procedure for candidate gNB-initiated LTM cancellation. Allow UE association in-between candidate CUs (in case the Xn connectivity existed) for subsequent LTM. When and how to establish the UE association is FFS. Continue to work on the solutions comparison and details… The following documents are endorsed R3-245032 (BL CR to 38.300) Inter-CU LTM basic procedure The following documents are agreed R3-245841 (TP for LTM BLCR for TS38.300): Update on inter-CU LTM basic procedure R3-245842 (TP for TS 38.423) Xn support for inter-CU LTM

In this paper, we will further discuss the basic procedure for inter-gNB-CU LTM based on agreed BLCR in [1], [2] and [3], and check the FFS issues on Xn interface.

Discussion

To achieve the analysis for network interface impact for the scenario of inter-gNB-CU LTM, we draw the potential message flows of the whole inter-gNB-CU LTM process as shown in the below Fig.1 to provide the basic description for each involved message flows and our view for the issues raised in stage 2 BLCR in [1] and [2]. After that, we finally summarize our proposals.

Fig.1 Message Flows for Inter-gNB-CU LTM

The procedure for inter-gNB-CU LTM is as follows:

1.	The UE sends a MeasurementReport message (L3 measurement result) to the source gNB-DU containing measurements of inter-gNB neighbouring cells. The source gNB-DU sends an UL RRC MESSAGE TRANSFER message conveying the received MeasurementReport message to the gNB-CU.

2.	The source gNB-CU determines to initiate inter-gNB-CU LTM configuration.

3.	Source gNB-CU gNB requests inter-gNB LTM for one or more candidate cells belonging to one or more candidate gNB(s). The source gNB initiates a HANDOVER REQUEST message per candidate cell containing one candidate cell ID and the CSI resource configuration for subsequent LTM.

Editor’s Note:  Working Assumption: For inter-CU LTM mobility, a separate LTM request message (i.e. HANDOVER REQUEST message) is used for each candidate cell

4.  The candidate gNB-CU sends a UE CONTEXT SETUP REQUEST message to the candidate gNB-DU(s) for each candidate cell to request LTM configurations.

5.	If the candidate gNB-DU accepts the request of LTM configuration, it responds with a UE CONTEXT SETUP RESPONSE message including the LTM lower layer RRC configurations.

6.  If the candidate gNB-CU accepts the request of LTM configuration, it responds with a HANDOVER REQUEST ACKNOWLEDGE to the source gNB-CU including the generated RRC configurations for the accepted candidate cell.

7.	The source gNB-CU sends a UE CONTEXT MODIFICATION REQUEST message to the source gNB-DU including the information related to early sync and the LTM configuration IDs for the accepted target candidate cell(s) in other candidate gNB-CU(s).

8.	The source gNB-DU responds with a UE CONTEXT MODIFICATION RESPONSE message which includes an updated lower layer configuration, e.g., containing the updated CSI report configuration of the source cell.

9.	The source gNB-CU sends an LTM CONFIGURATION UPDATE message to the candidate gNB-CU(s) to update the LTM configurations of candidate cell(s) for subsequent LTM.

10.	The candidate gNB-CU sends a UE CONTEXT MODIFICATION REQUEST message to the candidate gNB-DU(s) containing the information of candidate cells for subsequent LTM.

11.	The candidate gNB-DU responds with a UE CONTEXT MODIFICATION RESPONSE message.

12.	The candidate gNB-CU sends the LTM CONFIGURATION UPDATE ACKNOWLEDGE message to the source gNB-CU.

Editor’s Note: FFS on whether step 9 and 12 are mandatory or optional.

13.	The source gNB-CU sends a DL RRC MESSAGE TRANSFER message to the source gNB-DU, which includes the generated RRCReconfiguration message with the LTM configuration.

14.	The source gNB-DU forwards the received RRCReconfiguration message to the UE.

15.	The UE responds to the source gNB-DU with an RRCReconfigurationComplete message.

16.	The source gNB-DU forwards the RRCReconfigurationComplete message to the source gNB-CU via an UL RRC MESSAGE TRANSFER message.

17. If early data forwarding is applied, the source gNB sends the EARLY STATUS TRANSFER message to other candidate gNB(s).

NOTE 1: Early data forwarding can be triggered before the source gNB triggers a MAC CE Cell Switch Command to the UE to change cell, timing is left to implementation.

18.	Early synchronization to the target candidate cell(s) may be performed.

19. The candidate gNB-DU sends a DU-CU TA INFORMATION TRANSFER message to the candidate gNB-CU, which includes the TA value, the associated PRACH resource information, and etc.

20.	The candidate gNB-CU forwards the TA value and the associated CFRA resource information to source gNB-CU via the TA INFORMATION TRANSFER message.

Editor’s Note: Details are FFS on step 20.

21. The source gNB-CU forwards the TA value, and the associated CFRA resource information to the source gNB-DU in the CU-DU TA INFORMATION TRANSFER message.

22.	The UE performs L1 measurements on the configured LTM candidate cell(s) and transmits L1 measurement reports to the source gNB-DU.

23.	The source gNB-DU determines to initiate inter-gNB-CU LTM.

24.	The source gNB-DU decides to execute cell switch to a target cell and transmits an LTM cell switch command MAC CE triggering cell switch by including a target configuration ID which indicates the index of the candidate configuration of the target cell, a beam indicated with a TCI state or beams indicated with DL and UL TCI states, and a timing advance command for the target cell, if available. The UE switches to the target cell and applies the candidate configuration indicated by the target configuration ID.

25.	The source gNB-DU sends the DU-CU CELL SWITCH NOTIFICATION message to source gNB-CU to indicate the initiation of the Cell Switch Command to the UE including the target cell ID, the TCI state ID(s), and the TA values for subsequent LTM.

26.	The source gNB-CU sends the CELL SWITCH NOTIFICATION message to the target gNB-CU to indicate the initiation of Cell Switch command to the UE.

Editor’s Note: Details are FFS on step 26.

27.	The target gNB-CU forwards the target cell ID, the TCI state ID, and the TA values for subsequent LTM to the target gNB-DU in the CU-DU CELL SWITCH NOTIFICATION message.

28.	The target gNB-DU detects the UE access.

29.	The target gNB-DU sends the ACCESS SUCCESS message to the target gNB-CU with the target cell ID.

30.	The UE sends an RRCReconfigurationComplete message to the target gNB-DU.

31.	The target gNB-DU forwards the RRCReconfigurationComplete message to the target gNB-CU via an UL RRC MESSAGE TRANSFER message.

32/33. The target gNB-CU sends the HANDOVER SUCCESS message to the source gNB-CU to inform that the UE has successfully accessed the target cell. In return, the source gNB-CU sends the SN STATUS TRANSFER message following the principles described in step 7 of Intra-AMF/UPF Handover in clause 9.2.3.2.1.

NOTE2:	Late data forwarding may be initiated as soon as the source gNB receives the HANDOVER SUCCESS message.

34. The target gNB-CU sends a PATH SWITCH REQUEST message to 5GC to trigger 5GC to switch the DL data path towards the target gNB-CU and to establish an NG-C interface instance towards the target gNB-CU.

35.	The 5GC confirms the PATH SWITCH REQUEST message with the PATH SWITCH REQUEST ACKNOWLEDGE message to target gNB-CU.

36.	The target gNB-CU may send UE CONTEXT RELEASE to inform the source gNB-CU about the success of the handover if no LTM candidate cell(s) exist in the source gNB. The source gNB can then release radio and C-plane related resources associated to the UE context. Any ongoing data forwarding may continue.

Editor’s Note: Working Assumption: Reuse the existing XnAP UE CONTEXT RELEASE message at the source gNB if no LTM candidate cell(s) exist in the source gNB.

37.	The source gNB-CU may send the UE CONTEXT RELEASE COMMAND message to the source gNB-DU to release the resources of source cell.

38.	The source gNB-DU responds with a UE CONTEXT RELEASE COMPLETE message.

NOTE 3: The steps 17-38 can be performed multiple times for subsequent LTM cell switch executions.

From the above overview of inter-gNB-CU LTM procedure, the impacts on Xn interface are mainly reflected in the following aspects:

## Issue 1: HO REQ Single vs. Parallel messages

In the last RAN3 meeting, there was a discussion for this issue and Working Assumption is reached as below.

WA: For inter-CU LTM mobility, a separate LTM request message (i.e. HANDOVER REQUEST message) is used for each candidate cell. The error handling of multiple UE associations need to be considered.

Considering inter-CU LTM has similar procedure to CHO, so we suggest to turn this WA into agreement, which has less impact on current spec and it is also the majority view.

Proposal 1. Turn WA into Agreement: For inter-CU LTM mobility, a separate LTM request message (i.e. HANDOVER REQUEST message) is used for each candidate cell.

## Issue 2: FFS the detail in HO REQ/ACK message

Regarding the initiation of LTM, we already reached the agreement Reuse existing Xn Handover Request and Handover Request ACK for Inter-CU LTM initial preparation. However, need FFS on the detail information in HO REQ/ACK.

The below agreements on Stage-2 signaling flows and procedures are reached in the RAN2#126 meeting:

Stage-2 signaling flows and procedures - The preparation of inter-CU LTM configuration is initiated by the source gNB-CU. - For each candidate cell, the preparation of lower layer configuration is initiated by the candidate gNB-CU, based on the LTM request from the source gNB-CU. RAN2 assumes the interaction between the candidate gNB-CU and candidate gNB-DU follows the same signaling procedure for intra-CU LTM. - The source gNB-CU is responsible to collect the configurations and information of candidate cells from multiple candidate gNB-CUs and generates the common CSI resource configuration for L1 measurement on candidate cells. - In order to support subsequent LTM, the source gNB-CU needs to inform the candidate gNB-CU(s) about the common CSI resource configuration and the collected information of candidate cells from multiple candidate gNB-CUs. The candidate gNB-CU(s) responds with the candidate configuration to the source gNB-CU accordingly (if needed).

Therefore, to follow the above conclusions and similar enhancements for F1AP UE Context Setup Request/Response message in Rel-18 Intra-CU LTM procedure, we propose the following.

Proposal 2: Enhance Handover Request and Handover Request Acknowledge message with below related LTM information included for the LTM preparations. FFS to include these LTM configuration information in the inter-node RRC message or in the explicit IEs in the HO REQ/ACK.

- HO REQ message to include:

  - LTM Information Request IE

    - LTM indicator = initiation

    - LTM reference configuration (if available), or its request.

    - SSB / CSI resource configuration of candidate cells (if available)

  - LTM configuration ID mapping list (for all the LTM candidate cells)

  - Early Sync Information Request

- HO REQ ACK message to include:

  - LTM Information Acknowledge IE

    - LTM reference configuration if requested

    - Complete configuration indicator

    - SSB / CSI resource configuration allocated by the requested candidate cell

  - Early Sync Information which contains DL/UL early sync info allocated by the requested candidate cell.

  - Data Forwarding proposal for admitted PDU session and DRB which to be used for subsequent LTM

## Issue 3: FFS the LTM configuration update procedure for subsequent LTM

For supporting subsequent inter-CU LTM, RAN3 have reached the following agreement last meeting:

Introduce a new UE associated Class-1 XnAP procedure to update the LTM configurations for subsequent LTM. Confirm the name of the new procedure as “LTM Configuration Update”.

FFS on whether this procedure is mandatory or optional and FFS on details in message.

To support subsequent inter-CU LTM, the LTM information (e.g., early sync configuration, LTM configuration ID, and data forwarding addresses, etc) may need to sync up among candidate gNBs.

In Rel-18 inter-DU intra-CU LTM, the following note is captured in stage 2:

NOTE 2: Step 7 may also be triggered after step 19, or after step 22 by implementation for subsequent LTM.

Similar as F1AP, there are possible three options to support LTM information transfer to support subsequent LTM:

- Option 1: Send the collected LTM information to the candidate gNB(s) during the initial LTM preparation.

- Option 2: Send the collected LTM information to the target gNB when the source gNB (non-split gNB) sends LTM MAC CE to UE or source gNB-CU (split gNB) receives the DU-CU Cell Switch Notification message.

- Option 3: Send the collected LTM information to the target gNB when the source gNB receives the Handover Success message from the target gNB.

We think all the options can work and therefore this procedure can be optional. However, from the view of data forwarding, when candidate gNBs get the data forwarding addresses and LTM configurations after each execution of LTM, it could delay the subsequent LTM triggering and data forwarding operation in the new serving gNB, especially for the UE with high speed mobility. From this sense, we think the better approach is Option 1 and prefer to make this procedure mandatory.

Proposal 3: RAN3 to agree that LTM Configuration Update procedure for subsequent LTM is sent to candidate gNBs during initial LTM preparation.

Regarding the FFS on the details in message, following similar enhancements for F1AP UE Context Modification Request/Response message for subsequent LTM configuration transfer in Rel-18 Intra-CU LTM procedure, we propose the following.

Proposal 4: The new message LTM CONFIGURATION UPDATE/ACKNOWLEDGE includes below LTM configuration information in the inter-node RRC message or in the explicit IEs in message.

- LTM CONFIGURATION UPDATE message to include:

  - LTM Information Modification IE

    - LTM indicator = modification

    - SSB/CSI resource configuration allocated for all the accepted candidate cells

    - LTM reference configuration

    - LTM configuration ID mapping list (for all the accepted LTM candidate cells)

  - Early Sync Information which contains DL/UL early sync info allocated by each accepted candidate cell.

  - Data Forwarding TNL Address provided by other candidate gNB(s)

- LTM CONFIGURATION UPDATE ACK message to include:

  - LTM Information Acknowledge IE

    - the updated lower layer configuration (e.g., the updated CSI report configuration for each accepted candidate cell)

    - Complete configuration indicator for each accepted candidate cell

## Issue 4: FFS the message type and detail in TA INFORMATION TRANSFER message

Regarding early TA information transfer from candidate gNB-DU to source gNB-DU via candidate gNB-CU and source gNB-CU in step 20, we already agreed to introduce a new TA INFORMATION TRANSFER message to transfer the TA information. But whether it is a UE associated message or non-UE associated message is still FFS. Since R18 F1AP DU-CU/CU-DU TA INFORMATION TRANSFER message is a non-UE associated message, to inherit the legacy principle, we suggest this XnAP message is also specified as a non-UE associated message. Furthermore, following the current detail IEs in F1AP DU-CU/CU-DU TA INFORMATION TRANSFER, we have a proposal below.

Proposal 5: TA INFORMATION TRANSFER message is defined as a non-UE associated XnAP message to transfer early TA information from candidate gNB to source gNB including related LTM information below.

- XnAP TA INFORMATION TRANSFER message to include:

  - Candidate Cell ID

  - TA value

  - Preamble Index

  - RA-RNTI

  - Tag ID Pointer

  - Source gNB-DU ID (indicate the corresponding source gNB-DU to which source gNB-CU forwards TA information)

## Issue 5: FFS the detail in Cell Switch Notification message

Regarding cell switch notification from source gNB-DU to candidate gNB-DU via source gNB-CU and candidate gNB-CU, we already agree to introduce a new XnAP Cell Switch Notification message. But the details are FFS on this message.

Following the current detail IEs in F1AP DU-CU CELL SWITCH NOTIFICATION in R18 intra-CU LTM, since TA values may be also forwarded from source gNB-DU to candidate gNB-DU for subsequent LTM, we have a below proposal.

Proposal 6: TA Information List IE can be included in XnAP Cell Switch Notification message to forward the valid TA values of candidate cells for reuse in subsequent LTM.

## Issue 6: FFS on UE association between candidate CUs

In last meeting, we agreed to allow to setup a UE association between candidate CUs to support direct transmit of message from candidate CUs with a subsequent LTM scenario.

Assume there are three gNBs, i.e., gNB1, gNB2 and gNB3. When gNB1 initiates inter-CU LTM and candidate cells are under gNB2 and gNB3, UE association between gNB1-2 and gNB1-3 will be setup. When a UE moves to gNB2, gNB2 becomes a new source gNB. A new UE association between gNB2 and 3 is needed to exchange message directly between gNB2 and 3 without help of gNB 1 (original source node).

When gNB1 informs gNB2 of XnAP UE ID allocated by gNB3 (between gNB1), gNB2 can reuse the ID or setup a new XnAP UE ID between gNB2 and gNB3. Anyway, key point here is that gNB1 should share its association to all the candidate.

There are two alternative ways as below.

Alt 1. During the LTM preparation phase, the candidate gNB allocate UE XnAP ID and send to other candidate gNBs

One simple way is to put the information in the LTM Configuration Update message. Recall that LTM Configuration Update message will be transmitted to the all candidate CUs after HO Req/ACK messages are exchanged, it is possible to include UE XnAP IDs allocated by other candidate CUs in the LTM Configuration Update message. As we have LTM configuration ID mapping list IE in the LTM CONFIGURATION UPDATE message, XnAP UE ID might be added as below.

LTM Configuration ID Mapping List

This IE indicates the list of LTM cells associated with its configuration IDs.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Configuration ID Mapping Item IEs |  | 1..< maxnoofLTMCells> |  |  | - | - |
| >LTM Cell ID | M |  | Target Cell Global ID IE | This IE is used to globally identify an cell. | - |  |
| >LTM Configuration ID | O |  | INTEGER (1..8) | Corresponds to the LTM-CandidateId IE | - |  |
| >Candidate NG-RAN node UE XnAP ID reference | O |  | NG-RAN node UE XnAP ID IE | The NG-RAN node UE XnAP ID uniquely identifies a UE over the Xn interface within the NG-RAN node. Allocated at the candidate NG-RAN node serving Target Cell |  |  |

As XnAP UE ID could be different for each LTM configuration distinguished by LTM cell ID, mapping between XnAP UE ID and LTM cell ID (or LTM Configuration ID) is needed.

Alt 2. The serving gNB include its UE association information with all candidate CUs when Cell Switch is occurred

Instead of adding information in the preparation phase, the serving gNB could include its UE association information when cell switch is occurred, i.e., in the CELL SWITCH NOTIFICATION message.

In this option, the information is only transferred to the new serving gNB, unlike Alt 1 where UE association information is transferred to the all candidate CUs.

Proposal 7: UE association information between current serving gNB and each candidate CUs will be transferred to all the candidate CUs during LTM preparation phase (Alt 1) or to the new serving CU when cell switch occurs (Alt 2).

## Issue 7: FFS on LTM release from candidate gNB

Regarding this remaining issue, we reach a WA last meeting as below.

WA: Reuse the existing XnAP UE CONTEXT RELEASE message at the source gNB if no LTM candidate cell(s) exist in the source gNB.

The reason why we kept this as WA is to consider the scenario when gNB should role as anchor node when UE association is not allowed between candidate CUs. The Issue 6 resolve this concern, we can now turn WA into agreement.

Proposal 8: Turn WA into agreement: Reuse the existing XnAP UE CONTEXT RELEASE message at the source gNB if no LTM candidate cell(s) exist in the source gNB.

Conclusion

This contribution firstly analyses the whole message flows of inter-CU LTM and propose some enhancements for Xn interface on below aspects. Therefore, we invite RAN3 to agree below proposals for supporting the basic inter-CU LTM procedure.

## Issue 1: HO REQ Single vs. Parallel messages

Proposal 1. Turn WA into Agreement: For inter-CU LTM mobility, a separate LTM request message (i.e. HANDOVER REQUEST message) is used for each candidate cell.

## Issue 2: FFS the detail in HO REQ/ACK message

Proposal 2: Enhance Handover Request and Handover Request Acknowledge message with below related LTM information included for the LTM preparations. FFS to include these LTM configuration information in the inter-node RRC message or in the explicit IEs in the HO REQ/ACK.

- HO REQ message to include:

  - LTM Information Request IE

    - LTM indicator = initiation

    - LTM reference configuration (if available), or its request.

    - SSB / CSI resource configuration of candidate cells (if available)

  - LTM configuration ID mapping list

  - Early Sync Information Request

- HO REQ ACK message to include:

  - LTM Information Acknowledge IE

    - LTM reference configuration if requested

    - Complete configuration indicator

    - SSB / CSI resource configuration allocated by the requested candidate cell

  - Early Sync Information which contains DL/UL early sync info allocated by the requested candidate cell.

  - Data Forwarding proposal for admitted PDU session and DRB which to be used for subsequent LTM

## Issue 3: FFS the LTM configuration update procedure for subsequent LTM

Proposal 3: RAN3 to agree that LTM Configuration Update procedure for subsequent LTM is sent to candidate gNBs during initial LTM preparation.

Proposal 4: The new message LTM CONFIGURATION UPDATE/ACKNOWLEDGE includes below LTM configuration information in the inter-node RRC message or in the explicit IEs in message.

- LTM CONFIGURATION UPDATE message to include:

  - LTM Information Modification IE

    - LTM indicator = modification

    - SSB/CSI resource configuration allocated for all the accepted candidate cells

    - LTM reference configuration

    - LTM configuration ID mapping list (for all the accepted LTM candidate cells)

  - Early Sync Information which contains DL/UL early sync info allocated by each accepted candidate cell.

  - Data Forwarding TNL Address provided by other candidate gNB(s)

- LTM CONFIGURATION UPDATE ACK message to include:

  - LTM Information Acknowledge IE

    - the updated lower layer configuration (e.g., the updated CSI report configuration for each accepted candidate cell)

    - Complete configuration indicator for each accepted candidate cell

## Issue 4: FFS the message type and detail in TA INFORMATION TRANSFER message

Proposal 5: TA INFORMATION TRANSFER message is defined as a non-UE associated XnAP message to transfer early TA information from candidate gNB to source gNB including related LTM information below.

- XnAP TA INFORMATION TRANSFER message to include:

  - Candidate Cell ID

  - TA value

  - Preamble Index

  - RA-RNTI

  - Tag ID Pointer

  - Source gNB-DU ID (indicate the corresponding source gNB-DU to which source gNB-CU forwards TA information)

## Issue 5: FFS the detail in Cell Switch Notification message

Proposal 6: TA Information List IE can be included in XnAP Cell Switch Notification message to forward the valid TA values of candidate cells for reuse in subsequent LTM.

## Issue 6: FFS on UE association between candidate CUs

Proposal 7: UE association information between current serving gNB and each candidate CUs will be transferred to all the candidate CUs during LTM preparation phase (Alt 1) or to the new serving CU when cell switch occurs (Alt 2).

## Issue 7: FFS on LTM release from candidate gNB

Proposal 8: Turn WA into agreement: Reuse the existing XnAP UE CONTEXT RELEASE message at the source gNB if no LTM candidate cell(s) exist in the source gNB.

Reference

[1] R3-245032 (BL CR to 38.300) Inter-CU LTM basic procedure

[2] R3-245841 (TP for LTM BLCR for TS38.300): Update on inter-CU LTM basic procedure

[3] R3-245842 (TP for TS 38.423) Xn support for inter-CU LTM