3GPP TSG-RAN WG3 Meeting #126	R3-247361

Orlando, USA, 18– 22 November, 2024

Agenda item:	8.1

Source: 	Huawei

Title: 	Discussion on the PDU set information marking support without PDU set QoS parameters

Document for:	Discussion and Decision

# 1. Introduction

SA2 has discussed supporting PDU Set marking on the GTP-U header when the PDU Set QoS parameters are absent. To confirm the approach, SA2 asked RAN3 to provide feedback on what information to be sent from NG-RAN to SMF in S2-2411253 [1]. In this contribution, we will analyse the alternatives and give our view.

# 2. Discussion

In S2-2411253, SA2 introduced a new solution for PDU Set information provision. CN may inform NG-RAN that PDU Set information marking is supported via a dedicated PDU Set Information Marking Support Indicator. Upon receiving the indicator, NG-RAN may send some information to CN to activate the PDU Set Information Marking. The main content of the LS is excerpted below.

At the CN side, the PDU Set information marking on the GTP-U header of the downlink packets can be supported, even when the PDU Set QoS parameters are absent. Related to this, SA2 has agreed the following: - If the PDU Set QoS parameters are absent, but the PDU Set Information marking is supported, the SMF sends NG-RAN a new PDU Set Information Marking Support Indicator to indicate that DL PDU Set Information Marking is supported. - Based on the received PDU Set Information Marking Support Indicator from the CN, the NG-RAN sends information to the SMF to activate the PDU Set Information Marking for the PDU Set based handling (e.g., PSI based discarding due to congestion). Based on the NG-RAN provided information, the SMF instructs the PSA UPF to perform PDU Set information marking in the GTP-U header. Q1:SA2 would like RAN2 to provide feedback (e.g. the usefulness) on the PDU Set based handling without PDU set QoS parameters. SA2 would like to understand what information is sent by the NG-RAN to the SMF. Some companies think the NG-RAN can respond with a PDU Set Based Handling Support Indication, to indicate that PDU Set based handling is supported and to activate the PDU Set information marking. Some other companies think the NG-RAN could dynamically request the SMF to activate or deactivate the PDU Set Information marking at any time with an NG-RAN initiated procedure. Q2:SA2 would like RAN3 feedback which approach to support in Rel-19.

Regarding the information sent to CN, there are two alternatives considered by SA2.

- Alternative 1: Reuse the R18 PDU Set Based Handling Indicator.

In R18, the NG-RAN nodes which support PDU Set based handling can send the PDU Set Based Handling Indicator to the SMF if the PDU Set QoS parameters are received. With the indicator, the SMF can indicate the UPF to perform PDU Set Information Marking in the GTP-U header.

The R18 PDU Set Based Handling Indicator can be easily extended to support the R19 scenario with minor specification impacts. As mentioned by SA2 in the incoming LS, there will be a specific indicator from the SMF to NG-RAN to indicate that DL PDU Set Information Marking is supported by the CN in case there is no PDU Set QoS parameter. Such indicator from SMF can be used as a new condition to trigger the NG-RAN node to send the PDU Set Based Handling Indicator. In this case, there is no impact to the RAN-to-CN messages from the ASN.1 perspective. Some simple modification to the procedure text would be enough. As an example, a potential way to implement the NGAP specification can be found as following.

### 8.2.1 PDU Session Resource Setup If the PDU Set QoS Parameters IE or the PDU Set Information Marking Support Indicator IE is included in the PDU SESSION RESOURCE SETUP REQUEST message, the NG-RAN node shall, if supported, report in the PDU SESSION RESOURCE SETUP RESPONSE message the PDU Set based Handling Indicator IE in the PDU Session Resource Setup Response Transfer IE. If the PDU Set based Handling Indicator IE is included in the PDU Session Resource Setup Response Transfer IE in the PDU SESSION RESOURCE SETUP RESPONSE message, the SMF shall, if supported, handle this information as specified in TS 23.501 [9].

### 8.2.3 PDU Session Resource Modify If the PDU Set QoS Parameters IE or the PDU Set Information Marking Support Indicator IE is included in the PDU SESSION RESOURCE MODIFY REQUEST message, the NG-RAN node shall, if supported, report in the PDU SESSION RESOURCE MODIFY RESPONSE message the PDU Set based Handling Indicator IE in the PDU Session Resource Modify Response Transfer IE. If the PDU Set based Handling Indicator IE is included in the PDU Session Resource Modify Response Transfer IE in the PDU SESSION RESOURCE MODIFY RESPONSE message, the SMF shall, if supported, handle this information as specified in TS 23.501 [9].

- Alternative 2: Introduce a new indication in the NG-RAN initiated procedures.

Another method from SA2’s LS is to introduce a new indication in the NG-RAN initiated procedures so that the NG-RAN node can proactively request for PDU Set Information Marking at any time and RAN can ask CN to deactivate the PDU Set Information Marking. However, the motivation of such solution is unclear. According to the incoming LS, the NG-RAN node can only request for PDU Set information after knowing that the CN has the capability to perform PDU Set Information Marking. With this principle, the interaction should be triggered by the CN with the PDU Set Information Marking Support Indicator, so that NG-RAN can response whether PDU Set information is needed. The indication shall not be triggered by NG-RAN directly.

Besides, alternative 2 also has much more impacts to the specification. A new IE needs to be designed while it actually has the same effect as the existing PDU Set Based Handling Indicator IE. And this may need more description about procedure to avoid confliction of the existing IE and the new introduced IE. So, we think such redundant design should be avoided.

Based on the above analysis, we think alternative 1 should be adopted for R19 from the perspective of technical reasonability and specification conciseness. The corresponding draft reply LS is provided in [3].

Proposal 1: RAN3 reply to SA2 the existing PDU Set Based Handling Indicator IE can be reused for PDU Set Information Marking Support without PDU Set QoS parameters in Rel-19.

# 3. Conclusion

In this contribution, we analyzed the question asked by SA2 in the incoming LS. The following proposal was made:

Proposal 1: RAN3 reply to SA2 the existing PDU Set Based Handling Indicator IE can be reused for PDU Set Information Marking Support without PDU Set QoS parameters in Rel-19.

# 4. Reference

[1]. S2-2411253, TSG SA WG2, LS for PDU Set Information Marking Support without QoS parameters.

[2]. 3GPP TS 38.413: "NG-RAN, NG Application Protocol (NGAP)".

[3]. R3-247362, Huawei, [draft] Reply LS on PDU set information marking support without PDU set QoS parameters.