3GPP TSG-RAN WG3 Meeting #126	R3-247363

Orlando, USA, 18 - 22 November, 2024

Agenda Item:	12.2

Source:	Huawei

Title:	(TP for TS 38.413) Architecture and Access control for WAB

Document for:	Discussion

1	Introduction

In the last meeting, the following agreements were achieved for the architecture design of Rel-19 WAB [1]. How to prevent multi-hop topology is still open and some candidate solutions are listed below.

The scenario where a WAB-gNB serves a WAB-MT(s) should be preventable by means of different standard based solutions

RAN3 to specify solutions to prevent the multi-hop WAB topology, where multi-hop WAB means that a WAB MT connects to a WAB gNB. Discussions on multi hop WAB topology are out of scope.

For multi-hop prevention, RAN3 to down select to following three solutions:

Solution 1: The WAB-gNB uses dedicated frequencies and/or PCIs. FFS on any other legacy OTA parameters.

Solution 2: Use the slice dedicated for backhauling, i.e. use a list of S-NSSAIs in RRCsetupcomplete to do access control and/or use a list of S-NSSAIs in handover signalling. No CN upgrade is needed.

Solution 3: WAB-gNB-cells broadcast a new indicator in SIB to bar WAB-MT, and the WAB-MT avoids (re)selection of cells broadcasting this indicator.

Solution5: In case of handover for a WAB-node, the new WAB-node indication is included in the HO request, then the target BH-RAN node can perform access control for this WAB-node.

In this paper, we further analyse the scenario, architecture, access control and the user location information report for WAB.

2	Discussion

2.1	Scenario and architecture of WAB

2.1.1	Architecture: Management of Xn interface for WAB

The WID objective includes “Support of Xn interface(s) by the WAB-gNB with the WAB-MTs serving BH RAN node and with other surrounding gNBs, including how to avoid setting up Xn between WAB-gNBs” [2]. So, there are two issues to be discussed.

2.1.1.1 How to manage Xn interface efficiently for WAB-gNB

The mobility of the WAB-node results in dynamic changes to the Xn interface of the WAB-gNB. The WAB-gNB may need to establish new Xn interface to the newly appeared neighbour gNBs frequently. One issue here is how can the WAB-gNB acquire the TNL address of the neighbour gNBs. In legacy scenario, the TNL address of the neighbour gNBs is either configured by OAM or obtained via the Xn-C TNL address discovery procedure. The frequent update to the Xn interface will result in large overhead for these interactions. Moreover, the delay of such interactions will be another challenging factor especially considering all the transmission should be via MT’s PDU session.

So, we need to think about a more efficient way for the WAB-gNB to establish Xn interface towards neighboring gNBs (including BH-gNB). As an alternative, the BH-gNB can provide assistant information about its own neighboring gNBs (e.g., including its neighbour gNBs' TNL information) to the WAB-node.

Proposal 1: The BH-gNB can provides the TNL information of neighbour gNBs to the WAB node.

2.1.1.2 How to avoid setting up Xn between WAB-gNBs

Another issue is how to avoid establishing Xn between two WAB-nodes. The essential thing is the WAB-node should be aware of the node type of another WAB-gNB, for either to determine not initializing Xn setup towards a WAB-gNB, or to reject the Xn setup in case the Xn setup request is received from a WAB-gNB. It is proposed:

Proposal 2: To avoid establishing Xn between two WAB-nodes, the WAB-node should be aware of the node type of another WAB-gNB.

2.1.2	Scenario: in-band or out-band deployment

During SI phase, both in-band deployment and out-band deployment were considered for WAB and the in-band scenario was not excluded by the WI. Thus, one issue here is how to determine the in-band/out-band operational mode for the WAB node. According to TR 38.799 [3], after the WAB-MT setup, during the WAB-gNB initialization phase OAM provides the WAB-gNB the initial configuration including the operating frequency. On this stage the OAM may determine the in-band/out-band mode and configure the WAB-gNB frequency accordingly. Alternatively, the OAM may preconfigure the in-band/out-band mode and the WAB-gNB frequency before the WAB-MT setup. It is proposed:

- Proposal 3: OAM configures the inband/outband mode to WAB-node.

The in-band mode and out-band mode bring different requirements for the BH-RAN.

For out-band, the collocated WAB-gNB and WAB-MT use different frequency band, so there is no mutal interference and no resource coordination is needed between the collocated WAB-gNB and WAB-MT.

But for in-band, on the contrary, the resources used by the collocated WAB-gNB and WAB-MT must be coordinated. The BH-RAN node must be an upgraded gNB to be aware of which WAB-gNB and which WAB-MT are collocated, and implement the resource multiplexing. Therefore, for the in-band deployment, the WAB node needs to select a proper BH-gNB. It is proposed:

Proposal 4: To support the in-band WAB deployment, the BH-RAN must be upgraded to aware of WAB and can support the resource multiplexing. RAN3 to discuss how to ensure the in-band WAB selects a proper BH-gNB.

2.1.3	Scenario: NTN backhaul

According to the NOTE 5 in the WID, NTN backhaul is considered between the WAB-MT and the BH-gNB. According to TS 23.501 [5] the UE’s 5QI is set to a fixed value (5QI=10) as long as the UE is served by NTN, with the PDB directly set to a really large value (1100ms).

Fig 1. Special QoS parameters for NTN UE in TS 23.501

In the legacy NTN scenario, according to the TS 38.413 [4], the gNB will provide the RAT Information which indicates the NR satellite RAT Type to AMF during the NG Setup procedure or the RAN Configuration Update procedure as follows.

-------------Excerpted from TS 38.413-------------

9.3.1.125	RAT Information

This IE provides RAT related information associated with a TAC, used as described in TS 23.501 [5].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| RAT Information | M |  | ENUMERATED (unlicensed, nb-IoT, ..., nR-LEO, nR-MEO, nR-GEO, nR-OTHERSAT) |  |

In the WAB scenario, if the backhaul link uses NTN, the UE’s CN is naturally unaware of that according to the current RAT information report as shown above, because the WAB-gNB itself is not an NTN gNB. But if the WAB-MT adopts NTN backhaul, the served UE’s QoS parameters cannot be normally configured but should be configured considering the bottleneck of NTN backhaul. The UE’s CN needs to know the backhaul is via NTN and configures the NTN related QoS parameters for UE’s QoS flow. Therefore, it is proposed:

Proposal 5:	If the backhaul is NTN link, the UE’s CN should know the BH link type is NTN.

2.2	Access control of WAB

The Objective of the WID includes the handling of mechanisms to avoid multi-hop WAB topology. In addition, in the previous LS S2-2407345 from SA2 [6], they also ask RAN3 to discuss the access control regarding how to prevent the multi-hop access and how to prevent the self-access for the WAB-MT.

2.2.1	Prevent multi-hop access

According to the last meeting agreements, 4 solutions were still on the table for preventing the multi-hop access in case of the initial access and the handover, and to be further down-selected. We provide our views regarding each solution (Solution 1, 2, 3, 5) as follows.

- Solution 1: The WAB-gNB uses dedicated frequencies and/or PCIs.

Solution 1 is up to operator implementation, and can guarantee the WAB-MT will never camp on a WAB-gNB cell.  However, the usage of solution 1 is quite limited, because it depends on whether the operator has spare frequency bands/reserved PCI space. If not, this solution will impact the existing network because if the WAB dedicated frequency band/PCI has been used by existing gNBs. So, Solution 1 is unfriendly for the operators with resource constraint.

Observation 1:  The usage of Solution 1 is quite limited, because it depends on whether the operator has spare frequency bands/reserved PCI space.

- Solution 2: Use the slice dedicated for backhauling, i.e. use a list of S-NSSAIs in RRCsetupcomplete to do access control and/or use a list of S-NSSAIs in handover signalling. No CN upgrade is needed.

There are several issues regarding this solution 2. Firstly, the requested S-NSSAI is only used by RAN for AMF selection, and this IE is optionally included in the RRCsetupcomplete message. If RAN perform access control based on WAB specific slice ID in MSG 5, this will cause new behaviour for RAN and change the WAB-MT’s behaviour (the requested S-NSSAI in MSG5 should be mandatory at least for WAB-MT). So, the claimed benefit of this solution (proponents of this solution want to avoid Uu impact via solution 2) does not exist.

Secondly, if the access control is implemented by AMF through checking the S-NSSAI, this is not RAN-based solution anymore, it may only work at very late stage (when AMF reject the registration request from WAB-MT if access via another WAB-gNB) and cannot prevent the WAB-MT to access another WAB-gNB again. SA2 should be involved to assess the feasibility and potential impact.

Thirdly, if the WAB-MT supports more slices other than the WAB specific slice, it can still be served by the WAB-gNB using other slices. However, in this case the best choice for WAB-MT is to select another proper BH-gNB, but not to be served by the WAB-gNB without the right for backhauling.

Fourthly, solution 2 requires that all BH-gNBs should be upgraded to support the WAB specific slice, and it is unclear how to prevent the WAB specific slice being supported by the cell served by the WAB-gNB. One candidate method is the NS-AoS mechanism which allows the NS-AoS includes TAs where the network slice is not available in some cells of the TA. But in this condition, each time any WAB-gNB enters or leaves the TA, NAS signalling is needed for updating slice location availability information to all the WAB-MTs in the TA, causing huge signalling overhead.

In summary, it is observed:

Observation 2:  Solution 2 has the following problems:

- Slice-based access control at RAN will cause impact to both RAN and the WAB-MT, thus it againsts the claimed benefits of this solution.

- If the slice-based access control is performed by AMF, such solution works at very late stage and cannot prevent the WAB-MT to access another WAB-gNB again, and in addition, SA2 should be involved to check the feasibility and impact.

- If the allowed slice list of WAB-MT contains not only the WAB specific slice, but also other slices, the multi-hop access cannot be avoided, and it delays the WAB-MT find a proper BH-gNB.

- This solution requires that all BH-gNBs to be upgraded to support the WAB specific slice, and it is unclear how to prevent the WAB specific slice being supported by the cell served by the WAB-gNB. If using the NS-AoS method, it will cause frequent NAS singaling for updating slice location availability information to all the WAB-MTs in the TA.

•	Solution 3: WAB-gNB-cells broadcast a new indicator in SIB to bar WAB-MT, and the WAB-MT avoids (re)selection of cells broadcasting this indicator.

For solution 3, WAB-gNB broadcasts a WAB-barred or WAB-cell indication in SIB1. The current 38.331 defines variety of barring IEs for the access control, e.g., cellBarredNTN, cellBarredNES, etc. It is conventional to introduce a new barring bit for the AS access control. This solution can avoid multi-hop topology at early stage (i.e., when the WAB-MT try to access the network), and it allows the BH-gNB which is lack of enough resources for backhaul to bar the WAB-MT.

- Solution 5: In case of handover for a WAB-node, the new WAB-node indication is included in the HO request, then the target BH-RAN node can perform access control for this WAB-node.

For the case of handover, the target BH-RAN needs to know the WAB-node indication. If the target BH-RAN is a WAB-gNB, it can reject the WAB-MT’s handover request. How to design the WAB-node indication (e.g., introducing a new IE or use the WAB specific slice ID if defined) in handover request can be further discussed.

It is proposed:

Proposal 6: For multi-hop avoidance, RAN3 agree the following solutions:

- Solution 3: WAB-gNB-cells broadcast a new indicator in SIB to bar WAB-MT, and the WAB-MT avoids (re)selection of cells broadcasting this indicator.

- Solution5: In case of handover for a WAB-node, the WAB-node indication is included in the HO request, then the target BH-RAN node can perform access control for this WAB-node.

2.2.2	Prevent Self-access

- The second case of accessing restriction listed in the SA2 LS [6] is to avoid WAB-MT access the co-located WAB-gNB.

- First, since the WAB-gNB and WAB-MT should be coordinated for the transmission and receiving operation (e.g., TDM, FDM, SDM, etc), it is not possible for the WAB-MT to consider the cell served by its co-located WAB-gNB as the suitable cell when performing cell selection even if full-duplex is supported since some isolation/interference cancellation needs to be conducted. Otherwise, the transmitting of WAB-gNB and WAB-MT will cause severe interference to each other.

- Second, even if the WAB-MT incidently detects the signal of its co-located WAB-gNB, it is easy to prevent this WAB-MT from accessing this cell based on simple implementation, e.g., the implementation can set restriction that a WAB-MT never access the cell configured to this WAB-node by OAM. On the other hand, the RAN-based broadcasting “WAB-support/WAB-barred” can also avoid such issue.

- Observation 3: With proper multiplexing coordination/isolation, the WAB-MT will not detect signal of cell served by its co-located WAB-gNB. Even if the WAB-MT incidently detects the signalling, it is easy to prevent this WAB-MT from accessing this cell based on simple implementation.

Proposal 7: RAN3 to agree that “MWAB-UE accessing MWAB-gNB belonging to same MWAB” is not an issue to be solved, because this can be avoided by proper setting/implementation.

2.3	User location information

The WID objective also includes “UE’s ULI that reflect the WAB node’s location”, regarding the issue 2 in SA2’s LS S2-2407345 [6]. The issue 2 was stated in the LS as “SA2 has discussed and concluded that Additional ULI format will be determined by RAN WG and SA2 will align based on RAN feedback. Please refer to related conclusion in clause 8.5 of TR 23.700-06”.

9.3.1.260	Mobile IAB-MT User Location Information

This IE contains the NR CGI and the TAI for the co-located mobile IAB-MT of the mobile IAB-DU which serves the UE.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| NR CGI | M |  | 9.3.1.7 | The NR CGI of the cell, which is the serving cell of the mobile IAB-MT co-located with the mobile IAB-DU that serves the UE. |
| TAI | M |  | 9.3.3.11 | The TAI supported by the cell, which is the serving cell of the mobile IAB-MT co-located with the mobile IAB-DU which serves the UE. |

The SA2’s conclusion is clear that the additional ULI to be provided to the UE’s AMF is the TAC/Cell ID of the cell serving the MWAB-UE, if the WAB-MT and WAB-gNB belongs to same PLMN, then it is same as the additional ULI for mobile IAB, which is excerpted from TS 38.413 as below.

--------------------Excerpted from TR 23700-60-j00------------------------

8.5	KI#5: Support of location services for UEs when MWAB(s) is involved Conclusion

The following aspects are taken as way forward during normative work:

-	Existing principles (e.g. the two options for LMF to retrieve the MWAB location either via NRPPa procedure or via GMLC) are re-used for UE location service when MWAB is involved as described in sol#11.

The MWAB-gNB provides to the AMF an Additional ULI. The Additional ULI is:

-	TAC/Cell ID of the cell serving the MWAB-UE if the PLMN serving the MWAB-UE and the PLMN broadcasted by the MWAB-gNB are the same PLMN.

-	If the PLMN serving the MWAB-UE and the PLMN broadcasted by the MWAB-gNB are not the same PLMN, which below option used for the Additional ULI will be determined in normative phase.

-	option 1: mapped TAC/Cell ID based on geo-location of the MWAB based on input from OAM.

-	option 2: geo-location of the MWAB.

-	option 3: TAC/Cell ID of the cell serving the MWAB-UE in other PLMN.

-	The AMF may provide the received Additional ULI to LMF in LCS request.

NOTE:	If option 2 is selected in normative phase, support of passing Additional ULI to LMF needs to be reviewed.

For the inter-PLMN case, i.e., the WAB-MT and WAB-gNB belongs to different PLMN, there are three options provided by SA2’s conclusion. The contents from SA2’s TR are copied as below.

- Among the three options, the signalling impact to NGAP looks same for option 1 and option 3. However, to support option 1, OAM should provide the mapping relationship between geo-location and the TAC/cell ID to the WAB-gNB, whether and how can OAM support such functionality needs to check with SA5. Moreover, in some location, there maybe no coverage for the UE’s PLMN, then it is unclear how can the “mapped TAC/cell ID” being provided.

- In option 2, the geo-location of the WAB will be transmitted in NG interface. This is technical feasible, but the stage-3 signaling design will be different from legacy location information used in NG interface (i.e., TAI and cell ID are normally used to indicate UE’s location information in NG interface), then the design for intra-PLMN and inter-PLMN scenario will be different.

- For option 3, the signalling design is same as the “Mobile IAB-MT User Location Information”, and how can the AMF use the TAC/cell ID of the WAB-MT’s serving cell should be up to CN’s implementation, e.g., AMF may use the additional ULI to determine the location information based on the mapping information provided from OAM. Using such option, the impact to NG interface will be same as the same PLMN scenario.

- Observation 4: For additional ULI provision of inter-PLMN scenario of WAB:

- Option 1 requires additional functionality of OAM and this solution will be useless in the area without any coverage of WAB-gNB’s PLMN.

- Option 2 is technical feasible but the signalling design for location information is different from legacy and intra-PLMN case.

- Option 3 is same as the legacy design, then the intra-PLMN and inter-PLMN can have unified solution.

- Based on the above analysis, we think the option 3 is the best solution and it can ensure a unified solution for both inter-PLMN case and intra-PLMN case. It is proposed:

- Proposal 8: RAN3 to provide unified signalling design for additional ULI report in both intra-PLMN case and inter-PLMN case, then option 3 is recommended for the inter-PLMN case.

- After RAN3 has clear conclusion on the solutions, RAN3 should send reply LS to SA2 to provide the answer for the access control issue and ULI issue. A draft reply LS is provided in Annex B based on the analysis in this paper.

3	Conclusion

In this contribution we discuss the architecture and access control issues in the WAB scenario.

Scenario and architecture

Proposal 1: The BH-gNB can provides the TNL information of neighbour gNBs to the WAB node.

Proposal 2: To avoid establishing Xn between two WAB-nodes, the WAB-node should be aware of the node type of another WAB-gNB.

- Proposal 3: OAM configures the inband/outband mode to WAB-node.

Proposal 4: To support the in-band WAB deployment, the BH-RAN must be upgraded to aware of WAB and can support the resource multiplexing. RAN3 to discuss how to ensure the in-band WAB selects a proper BH-gNB.

Proposal 5:	If the backhaul is NTN link, the UE’s CN should know the BH link type is NTN.

Access control

Observation 1:  The usage of Solution 1 is quite limited, because it depends on whether the operator has spare frequency bands/reserved PCI space.

Observation 2:  Solution 2 has the following problems:

- Slice-based access control at RAN will cause impact to both RAN and the WAB-MT, thus it againsts the claimed benefits of this solution.

- If the slice-based access control is performed by AMF, such solution works at very late stage and cannot prevent the WAB-MT to access another WAB-gNB again, and in addition, SA2 should be involved to check the feasibility and impact.

- If the allowed slice list of WAB-MT contains not only the WAB specific slice, but also other slices, the multi-hop access cannot be avoided, and it delays the WAB-MT find a proper BH-gNB.

- This solution requires that all BH-gNBs to be upgraded to support the WAB specific slice, and it is unclear how to prevent the WAB specific slice being supported by the cell served by the WAB-gNB. If using the NS-AoS method, it will cause frequent NAS singaling for updating slice location availability information to all the WAB-MTs in the TA.

Proposal 6: For multi-hop avoidance, RAN3 agree the following solutions:

- Solution 3: WAB-gNB-cells broadcast a new indicator in SIB to bar WAB-MT, and the WAB-MT avoids (re)selection of cells broadcasting this indicator.

- Solution5: In case of handover for a WAB-node, the WAB-node indication is included in the HO request, then the target BH-RAN node can perform access control for this WAB-node.

- Observation 3: With proper multiplexing coordination/isolation, the WAB-MT will not detect signal of cell served by its co-located WAB-gNB. Even if the WAB-MT incidently detects the signalling, it is easy to prevent this WAB-MT from accessing this cell based on simple implementation.

Proposal 7: RAN3 to agree that “MWAB-UE accessing MWAB-gNB belonging to same MWAB” is not an issue to be solved, because this can be avoided by proper setting/implementation.

User location information

- Observation 4: For additional ULI provision of inter-PLMN scenario of WAB:

- Option 1 requires additional functionality of OAM and this solution will be useless in the area without any coverage of WAB-gNB’s PLMN.

- Option 2 is technical feasible but the signalling design for location information is different from legacy and intra-PLMN case.

- Option 3 is same as the legacy design, then the intra-PLMN and inter-PLMN can have unified solution.

- Proposal 8: RAN3 to provide unified signalling design for additional ULI report in both intra-PLMN case and inter-PLMN case, then option 3 is recommended for the inter-PLMN case.

4	References

- RAN3 #125b Chairman notes.

- RP-242359, New WID on additional topological enhancements for NR

- TR 38.799, Study on additional topological enhancements for NR, V19.0.0

- TS 38.413, NG Application Protocol (NGAP), Stage2, V18.3.0

- TS 23.501, System architecture for the 5G System (5GS), Stage 2, V18.5.0

- S2-2407345, LS on questions regarding FS_VMR_Ph2, SA2

Annex A:	TP for TS 38.413

Start of Change

9.3.1.16	User Location Information

This IE is used to provide location information of the UE.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| CHOICE User Location Information | M |  |  |  | - |  |
| >E-UTRA user location information |  |  |  |  |  |  |
| >>E-UTRA CGI | M |  | 9.3.1.9 |  | - |  |
| >>TAI | M |  | 9.3.3.11 |  | - |  |
| >>Age of Location | O |  | Time Stamp 9.3.1.75 | Indicates the UTC time when the location information was generated. | - |  |
| >>PSCell Information | O |  | NG-RAN CGI 9.3.1.73 |  | YES | ignore |
| >NR user location information |  |  |  |  |  |  |
| >>NR CGI | M |  | 9.3.1.7 |  | - |  |
| >>TAI | M |  | 9.3.3.11 | This IE is ignored if the NR NTN TAI Information IE is present. | - |  |
| >>Age of Location | O |  | Time Stamp 9.3.1.75 | Indicates the UTC time when the location information was generated. | - |  |
| >>PSCell Information | O |  | NG-RAN CGI 9.3.1.73 |  | YES | ignore |
| >>NID | O |  | 9.3.3.42 |  | YES | reject |
| >>NR NTN TAI Information | O |  | 9.3.3.53 |  | YES | ignore |
| >>Mobile IAB-MT User Location Information | O |  | 9.3.1.260 | Indicates the user location information of a mobile IAB-MT, which is co-located with the mobile IAB-DU which serves the UE. | YES | ignore |
| >>WAB-MT User Location Information | O |  | 9.3.1.X | Indicates the user location information of a WAB-MT, which is co-located with the WAB-gNB which serves the UE. | YES | ignore |
| >N3IWF user location information |  |  |  |  |  |  |
| >>IP Address | M |  | Transport Layer Address 9.3.2.4 | UE's local IP address used to reach the N3IWF | - |  |
| >>Port Number | O |  | OCTET STRING (SIZE(2)) | UDP or TCP source port number if NAT is detected. | - |  |
| >>TAI | O |  | 9.3.3.11 |  | YES | ignore |
| >TNGF user location information |  |  |  |  | YES | ignore |
| >>TNAP ID | M |  | OCTET STRING | TNAP Identifier used to identify the TNAP. Details in TS 29.571 [35]. | - |  |
| >>IP Address | M |  | Transport Layer Address 9.3.2.4 | UE's local IP address used to reach the TNGF. | - |  |
| >>Port Number | O |  | OCTET STRING (SIZE(2)) | UDP or TCP source port number if NAT is detected. | - |  |
| >>TAI | O |  | 9.3.3.11 |  | YES | ignore |
| >TWIF user location information |  |  |  |  | YES | ignore |
| >>TWAP ID | M |  | OCTET STRING | TWAP Identifier used to identify the TWAP. Details in TS 29.571 [35]. | - |  |
| >>IP Address | M |  | Transport Layer Address 9.3.2.4 | Non-5G-Capable over WLAN device's local IP address used to reach the TWIF. | - |  |
| >>Port Number | O |  | OCTET STRING (SIZE(2)) | UDP or TCP source port number if NAT is detected. | - |  |
| >>TAI | O |  | 9.3.3.11 |  | YES | ignore |
| >W-AGF user location information |  |  |  | Indicates the location information via wireline access as specified in TS 23.316 [34]. | YES | ignore |
| >>W-AGF user location information | M |  | 9.3.1.164 |  | - |  |

Next Change

9.3.1.X	WAB-MT User Location Information

This IE contains the NR CGI and the TAI for the co-located WAB-MT of the WAB-gNB which serves the UE.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| NR CGI | M |  | 9.3.1.7 | The NR CGI of the cell, which is the serving cell of the WAB-MT co-located with the WAB-gNB that serves the UE. |
| TAI | M |  | 9.3.3.11 | The TAI supported by the cell, which is the serving cell of the WAB-MT co-located with the WAB-gNB which serves the UE. |

End of Change

Annex B:	Reply LS to SA2

3GPP TSG-RAN3 Meeting #126	R3-xxxxxx

Orlando, USA, 18-22 November, 2024

Title:	[DRAFT] Reply LS on access restriction and additional ULI of WAB

Response to:	S2-2407345/ R3-244019

Release:	Rel-19

Work Item:	FS_VMR_Ph2,NR_WAB_5GFemto-Core

Source:	Huawei [to be: RAN3]

To:	SA2

Cc:	RAN2

Contact Person:

Name:	Yuanping Zhu

Tel. Number:

E-mail Address:	zhuyuanping@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	 -

1. Overall Description:

RAN3 would like to thank SA2 for their LS on the RAN impact considerations for WAB, and provide the following replies to SA2’s question:

- Issue 1:

SA2 has discussed the access control (to restrict MWAB-UE accessing MWAB-gNB cell) for below two cases:

a)	MWAB-UE accessing any MWAB-gNB in IDLE and CONNECTED mode (documented as one candidate solution in clause 6.2.3.6 of TS 23.700-06); and

b)	MWAB-UE accessing MWAB-gNB belonging to same MWAB.

SA2 assume that those scenarios will be discussed by RAN WGs, and ask for RAN feedback.

RAN3’s Answer:

For multi-hop prevention, RAN3 agree the following RAN-based solution: WAB-gNB-cells broadcast a new indicator in SIB to bar WAB-MT, and the WAB-MT avoids (re)selection of cells broadcasting this indicator.

RAN3 do not think the “MWAB-UE accessing MWAB-gNB belonging to same MWAB” is an issue to be solved, because this can be avoided by proper seting/implementation.

- Issue-2:

SA2 has discussed and concluded that Additional ULI format will be determined by RAN WG and SA2 will align based on RAN feedback. Please refer to related conclusion in clause 8.5 of TR 23.700-06.

RAN3’s Answer:

RAN3 will provide unified NGAP signaling design which contains TAC/cell ID of the WAB-MT’s serving cell for the additional ULI report in both intra-PLMN case and inter-PLMN case.

2. Actions:

To SA2:

ACTION: 	RAN3 asks SA2 to take the above feedback into account.

3. Date of Next RAN3 Meetings:

Updated meeting schedule can be found at: https://portal.3gpp.org/?tbid=373&SubTB=381#/

RAN3#127	     2025-02-17 - 2025-02-21		Athens, GR

RAN3#127bis	2025-04-07 – 2025-04-11		China, CN