3GPP TSG RAN WG3 Meeting #126										R3-247389

Orlando, USA, 18 - 22 November, 2024

Agenda Item:	13.2

Source:	CATT

Title:	(draft LS)Discussion on support of semi-persistent CSI-RS resource

Document for:	Approval

- Introduction

RAN3 has received an LS R3-247006 / R1-2409283 for inter-CU LTM from RAN1.

Working Assumption In addition to periodic CSI-RS, semi-persistent CSI-RS is supported for candidate cell L1-RSRP measurement for gNB scheduled reporting from RAN1 perspective The following working assumption was made in RAN1#118bis for candidate cell L1-RSRP measurement based on CSI-RS: RAN1 foresees the RAN3 spec impact to enable this functionality for inter-DU/inter-CU case, and hence RAN3’s view is needed to confirm the working assumption. Therefore, RAN1 would like to ask RAN3 for the feasibility of specifying the signalling for coordination between serving cell and candidate cell(s) on the transmission of semi-persistent CSI-RS(s) and any other potential issues (e.g. RAN3 workload).

This paper tries to answer the question indicated in the incoming LS and provide corresponding reply LS.

- Discussion

In the LS, RAN1 confirmed the periodic CSI-RS is supported, and asking RAN3 for the feasibility of supporting semi-persistent CSI-RS, including specifying the signalling for coordination between serving cell and candidate cell(s) on the transmission of semi-persistent CSI-RS(s) and any other potential issues (e.g. RAN3 workload).

- Periodic CSI-RS: CSI-RS is activated after configured, i.e., UE can measure the CSI-RS.

- Semi-persistent CSI-RS: after configured, Semi-persistent CSI-RS need an additional MAC CE for activation, and then UE can measure the semi-persistent CSI-RS.

From the perspective of configuration, there is no different between supporting Periodic CSI-RS and Semi-persistent CSI-RS. The source shall request candidate to provide CSI-RS information by UE association message, and candidate cells provided CSI-RS configuration to source gNB. After collected from all accepted candidate cells, source generating the common CSI-RESOURCE to each candidate cell or generate CSI-REPORT. After the interaction, source CU can configure the entire candidate configuration to the UE.

Proposal 1: From the perspective of configuration, supporting Periodic CSI-RS and Semi-persistent CSI-RS is no difference, and shall use the same procedure.

The key issue to support Semi-persistent CSI-RS is that the source node and the target node shall inform each other when the candidate cell active the semi-persistent CSI-RS. There are two possible options from RAN3 perspective to supporting Semi-persistent CSI-RS:

- Option 1: Source node trigger:

After configuration the RS to UE, source node decides to trigger the Semi-persistent CSI-RS of candidate cell, and sending XnAP message to candidate node for request candidate node active Semi-persistent CSI-RS.

- Option 2:Candidate node trigger:

After configuration the RS to UE, candidate node decides to trigger the Semi-persistent CSI-RS, and sending XnAP message to source node for informing source node the Semi-persistent CSI-RS is active.

From our perspective, the option 1 is more reasonable, as source node has the fully knowledge of measurement results and UE location information. When the source node aware UE is near to the candidate cell, it will consider triggering the candidate cell active its Semi-persistent CSI-RS. Accordingly, the source node also needs to indicate UE measurement for activated CSI-RS, which should be discussed in RAN2.

Proposal 2: RAN3 to discuss the coordination between serving cell and candidate cell(s) on support of semi-persistent CSI-RS(s), two options can take as baseline.

- Option 1: Source node decides to trigger activation of Semi-persistent CSI-RS, and request candidate node to active it. The message for such requesting can be FFS.

- Option 2: Candidate node decides to trigger activation of Semi-persistent CSI-RS, and inform source node of such activation it. The message for such informing can be FFS.

In the LS, RAN1 also asks RAN3 whether existing other potential issue, e.g., work load. Based the above analyses, the only enhancement RAN3 need to specify is to introduce an interaction between source node and candidate node for request/inform candidate cell active the pre-configured Semi-persistent CSI-RS. Considering activation/deactivation can reduce the energy cost effective, the work load is acceptable form RAN3 perspective.

Proposal 3: Based on above analyses, the workload of supporting Semi-persistent CSI-RS is acceptable.

- Conclusion

Based on the above analysis, we provide the following proposal.

Proposal 1: From the perspective of configuration, supporting Periodic CSI-RS and Semi-persistent CSI-RS is no difference, and shall use the same procedure.

Proposal 2: RAN3 to discuss the coordination between serving cell and candidate cell(s) on support of semi-persistent CSI-RS(s), two options can take as baseline.

- Option 1: Source node decides to trigger activation of Semi-persistent CSI-RS, and request candidate node to active it. The message for such requesting can be FFS.

- Option 2: Candidate node decides to trigger activation of Semi-persistent CSI-RS, and inform source node of such activation it. The message for such informing can be FFS.

Proposal 3: Based on above analyses, the workload of supporting Semi-persistent CSI-RS is acceptable.

- Draft LS

3GPP TSG RAN WG3 Meeting ##126										R3-24xxxx

Orlando, U.S.A., 18 - 22 November, 2024

Title:	[Draft] Reply LS for LS on the support of semi-persistent CSI-RS resource for LTM candidate cells

Response to:	R3-247006/ R1-2409283 LS on the support of semi-persistent CSI-RS resource for LTM candidate cells

Release:	Release 19

Work Item:	NR_Mob_Ph4-Core

Source:	CATT [TSG RAN WG3]

To:	RAN1

Cc:	RAN2

Contact Person:

Name:	Jiaying Sun

E-mail Address:	sunjiaying@catt.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

RAN3 would like to thank RAN1 for their LS about supporting semi-persistent CSI-RS resource for LTM candidate cells.

RAN3 confirmed it is feasible to specify the signalling for coordination between serving cell and candidate cell(s) on the transmission of semi-persistent CSI-RS(s), and the workload is acceptable.

2. Actions:

To RAN1 and RAN2:

ACTION: 	RAN3 respectfully asks RAN1 to take the above into account.

3. Date of Next TSG-RAN WG3 Meetings:

TSG RAN WG3 Meeting #127	17th– 21st February 2025	Athens, GR