# 3GPP TSG-RAN WG3 Meeting #126										R3-247392

# Orlando, USA, 18 – 22 November 2024

Agenda Item:	15.2

Source:	CATT

Title:	Discussion about IoT NTN

Document for:	Decision

- Introduction

- Support of Store&Forward (S&F) satellite operation with full eNB as regenerative payload, therefore: - Define the necessary enhancements into E-UTRAN (network & UE) to support S&F operation for delay-tolerant services [RAN3, RAN2] - At least specify necessary enhancements e.g. related to S1 protocol, especially to address the feeder link switch over as needed [RAN3]

In this paper, we provide our initial discussion about S&F scenario, and start with the attachment procedure.

- Discussion

S&F operation is designed for delay-tolerant, non-real-time IoT NTN services in the area where can be visited by satellite but without gateway infrastructure. The most typical case is intelligent sensor allocated in the mid-sea or the remote area. Usually, the common requirement for such UEs is uploading measurement results to the network regularly for monitoring.

In last meeting, we discussed the network architecture of support S&F, including eNB only, Split MME and full CN on board. To align among all WG and avoid duplication work, we suggest following SA2’s assumption, and only support Split MME and full CN on board for S&F as basic architecture.

Proposal 1：Only support split MME and Full CN on board architecture during S&F satellite operation.

Considering the service characteristic, an effective method is pre-register the UE before dropping them to remote area. Pre-attachment is benefit especially for Control Plane CIoT. With pre-attachment, UE can trigger the INITIAL UE MESSAGE to the satellite regardless the implementation on the satellite, and finish the measurement results reporting.

Proposal 2: For S&F scenario, UE can be pre-registered by TN/NTN network by implementation.

2.1 Regenerative payload for S&F operation

In the S&F operation, the precondition is the regenerative payload is supported by the eNB on board, and considering whether exist any S1AP impact.

In split MME case, SA2 introduced a new metric, list of satellite ID, which is generate by MME for indicating the UE can re-attempt attach to which satellites. Similar, in Full CN case, S&F monitoring list is also introduced for the same purpose. One open issue is whether needs to send the satellite list information to the eNB explicitly. We give a further clarification on this issue.

Usually, the UE obtains the ephemeris information from the broadcasting SIB information, including SIB31, SIB32, and SIB33.

- SIB 31 contains satellite assistance information for the serving cell

- SIB 32 contains satellite assistance information for prediction of discontinuous coverage.

- SIB 33 contains satellite assistance information for neighbour cells.

stemInformationBlockType32 information element

-- ASN1START

SystemInformationBlockType32-r17 ::= SEQUENCE {

satelliteInfoList-r17				SatelliteInfoList-r17	OPTIONAL,	-- Need OR

lateNonCriticalExtension			OCTET STRING					OPTIONAL,

...,

[[	satelliteInfoList-v1800			SatelliteInfoList-v1800	OPTIONAL	-- Need OR

]],

[[	satelliteInfoList-v1830			SatelliteInfoList-v1830	OPTIONAL	-- Need OR

]]

}

SatelliteInfoList-r17 ::=	SEQUENCE (SIZE (1..maxSat-r17)) OF SatelliteInfo-r17

maxSat-r17					INTEGER ::= 4	-- Maximum number of satellites

According to RAN2 definition above, limited by the size of SIB, the max number of ephemeris set as 4. If this satellite information is transfer to UE by NAS, the eNB have no idea of which emphases shall be broadcast to the UE. As eNB only can broadcast 4 ephemeris, the eNB only can do best effort broadcasting. By this way, from the network side it will increase the load for broadcasting unnecessary information, form the UE side, it may only get the satellite list from MME, but don’t have the ephemeris of the incoming satellite, and cannot attach successful.

Proposal 3: MME transfers the list of satellite and the S&F monitoring list of satellites IDs to the eNB explicitly for assisting ephemeris broadcast. Detail message FFS.

  - Regenerative payload for non-S&F operation

In lasting meeting, SA2 identify the misalignment of WI scope between RAN and SA, and sending the LS S2-2410918 for further check RAN3 view.

SA2 would like to inform RAN3 that the conclusions for FS_5GSAT_Ph3 for “KI#1: Support of Regenerative-based satellite access” without Store and Forward in clause 8.1 of TR 23.700-29 apply to both E-UTRAN/EPS and NG-RAN/5GS and therefore the related RAN3 impacts can affect both S1 and NG procedures. On the contrary, conclusions for FS_5GSAT_Ph3 for “KI#2: Support of Store and Forward Satellite operation” in clause 8.2 of TR 23.700-29 apply only to E-UTRAN/EPS.

In SA2 WI, the regenerative including both E-UTRAN and NG-RAN, however, in RAN3 scope, only support of S&F with full Regenerative is list clearly in the WI, whether support Regenerative when satellite in non-S&F operation is not been mentioned.

From our perspective, it is naturally to also support Regenerative payload when satellite in non-S&F operation. Otherwise satellite only can support transparent mode or provide service by the MME on board. If only supported transparent mode, it shall lead to some kind of waste as already have regenerative payload on the satellite. If provide service by the MME on board, it may face the limitation of store capability, e.g., MME-on board need to store the register information of all possible UEs, which lead to huge challenge especial for full CN on board.

We take split MME as an example to further explanation, the principle also suitable for full CN onboard.

- When the feeder link is not available, satellite shall work on S&F mode, UE can send the user data to the satellite when the service link available and stored in MME-onboard.

- When the feeder link is available, for previous stored user data during S&F operation mode, the MME-onboard shall transfer the user data to the MME-ground by the Feeder link 1, which is a link between MME-onborad and MME-ground, and defined by SA2.

- When the feeder link is available, for normal UE, satellite can provide service by the regenerative payload on board, and transfer the user data to the on ground CN by Feeder link 2, which is S1 interface and shall be enhance in RAN3.

One thing needs to be high light is the intention of supporting regenerative payload is for enable satellite provide service for non-S&F mode UE when feeder link is available. Instead of enable S&F UE switch to non-S&F mode. For the UE mode switch, we think it is a corner case as IoT UE usually not maintain connection for a long time, and can be avoided by network implementation, e.g. eNB allows connected S&F mode UE transfer data for a while after it switch mode to non-S&F, or eNB rejects some long connection UE access before it switch mode.

Proposal 4: It is proposed RAN3 to support regenerative payload when feeder link available for UE in non-S&F mode.

Proposal 5: Clarify the WI in RP#106 for supporting regenerative payload for non-S&F operate.

  - Support PWS for NB-IoT

A revised WID was approved in RAN#105 (Sept 2024) to further enhance the support of NB-IoT and eMTC for NTN in RP-242397. Based this, RAN2 sends LS to RAN3 about their progress and request RAN3 to check whether any alignment is needed. E.g., check RAN3 spec., whether any limitation to block supporting PWS IoT-NTN.  .

After check RAN3 specification, we found in RAN3, we never have such limitation. And once the PWS for IoT-NTN is supported by RAN2, it can be supported in RAN3 naturally.

Proposal 6: Send reply LS to RAN2, confirming there is no misalignment identified in RAN3 for supporting PWS for NB-IoT.

3. Conclusion

Based on the discussion in this paper, we have the following observations and proposals:

Proposal 1：Only support split MME and Full CN on board architecture during S&F satellite operation.

Proposal 2: For S&F scenario, UE can be pre-registered by TN/NTN network by implementation.

Proposal 3: MME transfers the list of satellite and the S&F monitoring list of satellites IDs to the eNB explicitly for assisting ephemeris broadcast. Detail message FFS.

Proposal 4: It is proposed RAN3 to support regenerative payload when feeder link available for UE in non-S&F mode.

Proposal 5: Clarify the WI in RP#106 for supporting regenerative payload for non-S&F operate.

Proposal 6: Send reply LS to RAN2, confirming there is no misalignment identified in RAN3 for supporting PWS for NB-IoT.

4. Reference

[1]	TR 23.700 Study on integration of satellite components in the 5G architecture.

[2]	S2-2410918 Reply LS on Support of Regenerative-based Satellite Access

5. Draft LS

Title:	[DRAFT] Reply LS PWS support for NB-IoT NTN

Response to:

Release:	Rel-19

Work Item:	IoT_NTN_Ph3-Core

Source:	CATT [to become RAN3]

To:	SA5,

Cc:

Contact Person:	Jiaying Sun

sunjiaying@catt.cn

Send any reply LS to:	3GPP Liaisons Coordinator, 3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

RAN3 thanks RAN2 to informing the progress about supporting PWS for IoT-NTN, and after checking RAN3 specification, RAN3 found there is no limitation to block supporting PWS IoT-NTN, and no further specification impact identified in RAN3.

2. Actions:

To SA2 group:

ACTION: 	RAN3 kindly asks RAN2 to take the above feedback into account.

3. Date of Next TSG-RAN3 Meetings:

TSG RAN WG3 Meeting #127	17 – 21 February 2025	Athens, Greece