3GPP TSG-RAN3 Meeting #126	R3-247393

# Orlando, US, 18th – 22th Nov 2024

Agenda Item:	11.4

Source:	CATT

Title:	Discussion on measurement correlation for continuous MDT

Document for:	Decision

- Introduction

In last meeting, the issue list in SI further expounded to FFSs shown as below.

Further discussions are needed on how to define a temporary identifier for Continuous MDT

Further discussions are needed on the solution for measurement correlation in Continuous MDT

In this paper, we main focus on the correlation issues.

2. Discussion

In legacy management based MDT, the management system sends a Trace Session activation request to the gNB, which including MDT configuration and the corresponding the Trace Reference (TR). After that, gNB shall select the suitable UEs for MDT data collection and assign Trace Recording Session Reference (TRSR) to the selected UE. The selection is based on the area received from the management system, the area where UE is located, user consent information and UE capability.

Observation 1: In legacy MDT, TR is assigned by the management system for specific trace task(MDT configuration), while the TRSR is assigned by gNB corresponding to the selected UE.

To satisfy the requirement of AI training, OAM shall be able to identify multiple MDT reports is provided by the same UE in a continuous time duration. The key issue block the correction is the anonymization requirement of management-based MDT data, in the MDT Record from gNB to the TCE, only including the TRSR and TR information instead of UE persistent ID.

There exist several kinds of solutions on the table:

- Solution #1: Create an identifier from CN/OAM and maintained in the Core Network

- Solution #2: Introduce a new “job type”, e.g., immediate MDT and logged MDT.

- Solution #3: Reuse TR/TRSR for correlation.

Solution#1 requires CN/OAM providing an identifier for UE, and stores such identifier at core network side. Once UE back to connected state, the AMF shall resend this information to the gNB, and the MDT report provide by this UE shall always include the allocated identifier for realize the correlation at OAM side. However, this solution required including UE specific identifier in MDT report, which is contradict with anonymization requirement. If go this way, we should check the feasibility with SA5 and SA3 for the security issue.

Observation 2: Introduce a UE identifier with MDT recorder may contradict with anonymization requirement.

Solution#2 requires OAM define a new job type including both immediate MDT and logged MDT, which is not align with the current definition of job type, thus will have impact on the current MDT mechanism.

Observation 3: Introduce a job type including two kind of MDT is not aligning with the current definition of job type.

Solution#1 try to reuse current exist metric for realized such correlation, one possibility method could be extending current MDT Record, and including both current TR/TRSR and the subsequent TR/TRSR in the MDT Record.

To simplify the discussion, it is assumed that both gNBs (gNB1 and gNB2) belong to the same area scope and all satisfy the user consent requirement of UE. The management system configures both gNBs (gNB 1 and gNB2) with same immediate MDT configuration (TR1) and management-based logged MDT configuration(TR2). The gNB1 choose UE to execute continuous MDT and configured the two MDT configurations to UE separately.

- UE across RRC states

In the Fig.1, we illustrate an example of MDT measurement correlation when UE experience RRC_connected, RRC_idle and back to RRC_connected.

Fig.1 UE across different RRC states

Step1-2: The OAM decides to configure Management Based MDT for continuous MDT towards the NG-RAN. And configure immediate MDT configuration (TR1) and logged MDT configuration (TR2) to the NG-RAN.

Step3-5: NG-RAN selects suitable UE to execute MDT measurement. NG-RAN sends immediate MDT configuration and logged MDT configuration to the UE, separately.

Step 6-7: UE sends the MDT report of immediate MDT to the gNB, and gNB save MDT measurement in the MDT Record.

Step 8: gNB shall send received MDT Records to TCE, including the TR1, TRSR1 (current MDT Record, legacy). In addition, for MDT report correlation, it shall also include the TR, TRSR for the subsequent MDT assigned by the gNB, it indicate the TR, TRSR will be used when UE in RRC_idle/inactive, e.g., TR2, TRSR2.

Step 9-10: UE transits to RRC_idle, and execute logged MDT based on the previous configuration received at step 5.

Step 11-12: UE backs to RRC_connected, and based on network request, UE sends logged MDT report to the gNB.

Step 13: UE sends the measurement report of logged MDT to the gNB, and gNB save UE measurement in the MDT Record, and sending this MDT Record to TCE. In the MDT Record head, it shall include the TR2, TRSR2 (current MDT Record, legacy). In addition, for MDT report correlation, it shall also include the TR, TRSR for the subsequent MDT assigned by the gNB, it indicate the TR,TRSR will be used when UE in RRC_connected, e.g., TR1, TRSR3.

Step 14-16: when UE transits to RRC_connected, gNB shall re-configure the Management based MDT configuration the UE, at least the immediate MDT configuration, as soon as possible. The UE shall execute immediate MDT after configured.

Step 17: Similar with step8, when send the MDT report, the gNB shall also include assigned TR, TRSR for the subsequent MDT.

Based on the above, OAM can correlate the MDT report received at step 8, 13 and 17 by the current TR, TRSR (Trace ID) and subsequent TR, TRSR (Trace ID).

- UE across RRC states in different gNB

Solution#1 is also applicable when UE across different RRC state in different gNB. In the fig.2, we illustrate an example of MDT measuremennt correlation when UE experience RRC_connected, RRC_idle and back to RRC_connected at the different gNB.

Fig.2 UE across different RRC states in different gNB

Step 1-10: Similar with step 1-8 in figure 1, OAM configure both immediate MDT and logged MDT configuration to the NG-RAN, and gNB choose suitable UE and sending the configuration. UE send the measurement report of immediate MDT to the gNB, and gNB save UE measurement in the MDT Record. gNB send the MDT record to TCE, including the TR1, TRSR1 for the current MDT Record and the assign TR, TRSR for the subsequent MDT.

Step 11-13: UE transits to RRC_idle in gNB1, and execute logged MDT based on the previous configuration received at step 5. UE transits to RRC_connected in gNB2, and based on network request, UE sends logged MDT report to the gNB2.

Step 14-15: UE sends the measurement report of logged MDT to the gNB2, and gNB2 save UE measurement in the MDT Record, and sending this MDT Record to TCE. including the TR2, TRSR2 (current MDT Record, legacy) and the assigned TR, TRSR for the subsequent MDT by gNB2.

Proposal 1: RAN3 to discuss introducing the assigned TR, TRSR for the subsequent MDT in the MDT Record for measurement correlation.

For the above solution, the only enhancement is including the TR, TRSR for subsequent MDT in the MDT Record, and there is no impact on RAN3. As the Reporting format is configured by SA5, we can further check with SA5 whether such enhancement is feasible and request corresponding action.

Proposal 2: Check with SA5 whether it is feasible to including the TR, TRSR for subsequent MDT in the MDT Record.

3. Conclusion

Based on the discussion in this paper, we have the following observations and proposals:

Proposal 1: RAN3 to discuss introducing the assigned TR, TRSR for the subsequent MDT in the MDT Record for measurement correlation.

Proposal 2: Check with SA5 whether it is feasible to including the TR, TRSR for subsequent MDT in the MDT Record.

4. Reference

[1] TR38.743