3GPP TSG-RAN WG3 Meeting #126	R3-247399

Orlando, US, 18 - 22 Nov, 2024

Title:	(TP for TS 38.300 and TS 38.413) Support MBS broadcast service

Agenda Item:	14.2

Source:	Huawei

Document type:	other

# 1	Introduction

In order to indicate the intended MBS service area via NG interface, the last RAN3#125bis meeting made the following agreement, and agreed the LS to other groups.

RAN3 takes the assumption that geographical service area information can be transmitted from CN with the same format as defined in Uu.

So far RAN3 has agreed that the intended MBS service area provided from the AMF to the gNB can be mapped cell ID(s), TA(s), or geographical service area information, where the geographical service area is aligned with RAN2 format.

Taking the above into account, this paper provides our further considerations and proposals.

# 2	Discussion

## 2.1 gNB behaviour for the mapped cell ID

The NR CGI IE representing the mapped cell ID is included in the MBS Service Area Information, and can be used to indicate the intended MSB service area. Clearly, the mapping between the mapped cell ID and the exact geographical area over the Uu is configured in the RAN side.

Upon the gNB receives the mapped cell ID, the gNB needs to translate or convert it to the detailed geographical area information, and broadcast it over the Uu interface. It is suggested to describe it in stage 2 to indicate this gNB translation behaviour.

- RAN3 to agree that the gNB translates the received mapped cell ID(s) into the geographical area(s), and broadcasts over the Uu interface, which can be captured in the stage 2.

## 2.2 Overlapping issue of multiple geographical areas

In the BLCR [1], the MBS service area information could include NR CGI, TAI or the new small area as the intended MBS service area.

#### 9.3.1.209 MBS Service Area Information This IE contains MBS service area information. IE/Group Name; Presence; Range; IE type and reference; Semantics description. MBS Service Area Cell List; 0..<maxnoofCellsforMBS>. >NR CGI; M; 9.3.1.7. MBS Service Area TAI List; 0..<maxnoofTAIforMBS>. >TAI; M; 9.3.3.11. MBS Service Area Small Area List; 0..<maxnoofSmallAreasforMBS>. >Small Area; M; 9.3.1.x. Editor’s Note: The IE name and encoding are pending RAN2 discussion.

When supporting all the possible representation formats of MBS service area, an issue may arise that the indicated small service area may overlap with each other, or the geographical area may overlap with the mapped cell ID or TAI.

From our point of view, it would be easy that one area should complement to another area info, i.e., there is no overlapping area. For example, the small area could be separated from another small area or another mapped cell ID. But on the other hand, it should be also possible that these indicated areas could be partially overlapped, especially to indicate the MBS service for those neighbouring locations. It is suggested to discuss if there is need to restrict the indicated MBS service area no overlapping, partially overlap or not.

- RAN3 to discuss if there is need to restrict the indicated MBS service areas no overlapping, or it allows at least partial overlap or not.

## 2.3 MBS broadcast procedures

With the satellite movement, the MBS broadcast session resources need to be setup or removed in time. Typically, when the satellite covers the intended service area, the MBS broadcast session can be setup, otherwise, it can be released. Currently, the related MBS Broadcast Session Management Messages over the NGAP can at least include:

- Broadcast Session Setup procedure: To request the NG-RAN node to setup MBS session resources for a broadcast MBS session

- Broadcast Session Release procedure: To release the MBS session resources related to a previously established broadcast MBS session

- Broadcast Session Release Required procedure: To trigger the AMF to release the MBS session resources related to a previously established broadcast MBS session.

It seems to us that the existing broadcast session procedures can be used for NTN to setup or release the broadcast session, without any further changes.

- RAN3 to agree that no further update of MBS broadcast procedures is needed to enable the intended MBS service area for NTN.

## 2.4 stage 3 TP

In the BLCR [1], the small area is marked as FFS and pending to RAN2 as follows.

#### 9.3.1.x Small Area This IE provides information on the Small Area for MBS Broadcast Service. IE/Group Name; Presence; Range; IE type and reference; Semantics description. Small Area; M; [FFS]; Pending RAN2 discussion. Editor’s Note: The IE name and encoding are pending RAN2 discussion.

The previous RAN2-127 bis meeting made the following agreements, but the stage3 details are to be further specified, which does not provide much help to the RAN3 stage3.

Agreements:

1.	For each MBS service we include one or more intended service area IDs into MCCH. FFS whether the list of the intended service areas (and related IDs) is also included in MCCH or if it is provided in a new or existing SIB. We will consider possible enhancements (including enhancements left up to UE implementation) to allow UE skipping MCCH re-acquisition when UE is not within intended service area of any interested broadcast service.

In our understanding, the small area in the MBS Service Area Information should be defined as an Octet String type, and refer to TS 38.331. Regarding the size of the maxnoofSmallAreasforMBS, currently it is set as 65536 in [1]. This can be agreed in RAN3, and can be further revisited when the RAN2 stage 3 is steady.

- Update the stage3 TP by defining the IE type as Octet String, and remove the FFS of  maxnoofSmallAreasforMBS.

# 3	Conclusion

In this paper, we provide the following observations and proposals:

- RAN3 to agree that the gNB translates the received mapped cell ID(s) into the geographical area(s), and broadcasts over the Uu interface, which can be captured in the stage 2.

- RAN3 to discuss if there is need to restrict the indicated MBS service areas no overlapping, or it allows at least partial overlap or not.

- RAN3 to agree that no further update of MBS broadcast procedures is needed to enable the intended MBS service area for NTN.

- Update the stage3 TP by defining the IE type as Octet String, and remove the FFS of  maxnoofSmallAreasforMBS.

# 4	Reference

- R3-245863 Support for Regenerative Payload and MBS broadcast in NR NTN, CATT et al.

# Annex1	Text Proposal for TS 38.300 (on top of R3-245861)

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

### 16.14.5	NG-RAN signalling

The Cell Identity, as defined in TS 38.413 [26] and TS 38.423 [50], used in following cases corresponds to a Mapped Cell ID, irrespective of the orbit of the NTN payload or the types of service links supported:

-	The Cell Identity indicated by the gNB to the Core Network as part of the User Location Information;

-	The Cell Identity used for Paging Optimization in NG interface;

-	The Cell Identity used for Area of Interest;

-	The Cell Identity used for PWS;

-	The Cell Identity used for MBS broadcast service.

The Cell Identity included within the target identification of the handover messages allows identifying the correct target cell. The cell identity used in the NG and Xn handover messages, Xn Setup and Xn NG-RAN Node Configuration Update procedures is expected to be Uu Cell ID.

The Cell Identities used in the RAN Paging Area during Xn RAN paging allow the identification of the correct target cells for RAN paging.

NOTE 1:	The Cell Identity used for RAN Paging is assumed to typically represent a Uu Cell ID.

The mapping between Mapped Cell IDs and geographical areas is configured in the RAN and Core Network.

NOTE 2:	A specific geographical location may be mapped to multiple Mapped Cell ID(s), and such Mapped Cell IDs may be configured to indicate differerent geographical areas (e.g. overlapping and/or with different dimensions).

The gNB is responsible for constructing the Mapped Cell ID based on the UE location information received from the UE, if available. The mapping may be pre-configured (e.g., up to operator's policy) or up to implementation.

NOTE 3:	As described in TS 23.501 [3], the User Location Information may enable the AMF to determine whether the UE is allowed to operate at its present location. Special Mapped Cell IDs or TACs may be used to indicate areas outside the serving PLMN's country.

The gNB reports the broadcasted TAC(s) of the selected PLMN to the AMF as part of ULI. In case the gNB knows the UE's location information, the gNB may determine the TAI the UE is currently located in and provide that TAI to the AMF as part of ULI.

To support the intended service area of a broadcast service (e.g. MBS broadcast) in case the satellite footprint covers a larger area, the intended service area may be in a form of cell ID list, TAI list, geographical area or any combination as defined in TS 38.413 [26]. When the gNB receives the cell ID list, it translates the cell ID to the correspdonding geographical area(s), and broadcasts over the Uu interface according to TS 38.331 [12].

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>

# Annex2	Text Proposal for TS 38.413 (on top of R3-245863)

-----------------Next Change-------------------

#### 9.3.1.209	MBS Service Area Information

This IE contains MBS service area information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MBS Service Area Cell List |  | 0..<maxnoofCellsforMBS> |  |  |
| >NR CGI | M |  | 9.3.1.7 |  |
| MBS Service Area TAI List |  | 0..<maxnoofTAIforMBS> |  |  |
| >TAI | M |  | 9.3.3.11 |  |
| MBS Service Area Small Area List |  | 0..<maxnoofSmallAreasforMBS> |  |  |
| >Small Area | M |  | 9.3.1.x |  |

Editor’s Note: The IE name and encoding are pending RAN2 discussion.

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsforMBS | Maximum no. of cells allowed within one MBS Service Area. Value is 8192. |
| maxnoofTAIforMBS | Maximum no. of TAs allowed within one MBS Service Area. Value is 1024. |
| maxnoofSmallAreasIforMBS | Maximum no. of small areas allowed within one MBS Service Area. Value is 65536. |

-----------------Next Change-------------------

#### 9.3.1.x	Small Area

This IE provides information on the Small Area for MBS Broadcast Service.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Small Area | M |  | OCTET STRING | Pending RAN2 discussion |

Editor’s Note: The IE name is pending RAN2 discussion.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>