3GPP TSG-RAN WG3 Meeting #126										R3-247421

Orlando, US, November 16th – 22nd, 2024

Agenda Item:	20.2

Source:	Huawei

Title:	Discussion on RAN3 impacts for AI/ML positioning

Document for:	Discussion and decision

# Introduction

In the last meeting RAN3 started discussing the objectives of the Rel-19 WI “Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface” – latest WID in [1]. To be specific, the discussion was on the specification support for AI/ML-based positioning accuracy enhancements.

This contribution focuses on AI/ML assisted positioning Case 3a and Direct AI/ML positioning Case 3b – being the only cases of AI/ML positioning that have RAN3 impacts – and provides proposals based on the latest progress in RAN1 and in RAN3.

# Discussion

In the following we further discuss aspects related to model training, model inference and model monitoring for AI/ML assisted positioning Case 3a and Direct AI/ML positioning Case 3b, that is, NG-RAN node-assisted positioning with a gNB-side model (Case 3a) and NG-RAN node assisted positioning with LMF-side model (Case 3b), respectively. In the last RAN3 meeting the following agreements and open issues were captured in the meeting minutes [2]:

Consider aggregated gNB case first, then split architecture. The existing LCS framework is re-used for AI/ML positioning. Case 3a: The AI/ML model (inference and training functions) is at the gNB. Case 3a agreements: - LMF provides ground truth label and related data (i.e., part B information as called by RAN1) to NG-RAN. FFS on the content of the ground truth label and related data pending on RAN1 progress. FFS on the signalling design. - How the LMF obtains the ground truth label and related data information is outside of RAN3 scope. - NG-RAN obtains measurements from TRP (i.e., part A information as called by RAN1). The details of the measurements are still pending and subject to further discussion in RAN1. - WA: RAN3 will proceed with Option A: NG-RAN node performs monitoring metric calculation for its own model. Case 3b: Re-using the Measurement procedure defined in NRPPa to support requesting and reporting the related measurements to LMF for Case 3b. The details of the related measurements are still pending and subject to further discussion in RAN1. For case 3b: - NG-RAN gets measurements (i.e., part A information as called by RAN1) from TRP and provides them to LMF. The details of the measurements are still pending and subject to further discussion in RAN1.

For AI/ML positioning, the preliminary Rel-18 study phase – whose outcome is summarized in TR 38.843 [3] – identified that, for Case 3a, the output of the AI/ML model hosted in the gNB is an intermediate measurement to be used for successive determination of the UE location at the Location Management Function (LMF). While, for Case 3b, the output of the AI/ML model hosted in the LMF is an estimate (prediction) of the UE location which is inferred by means of measurements provided by the NG-RAN.

In order to assess the RAN3 impacts for supporting AI/ML positioning Case 3a and Case 3b we will discuss model training, model inference and model monitoring aspects for both cases.

## 2.1 Model training considerations

Based on the agreements from last meeting, RAN3 already agreed that for Case 3a inference and training functions are located in the gNB. However, in our understanding, the AI/ML model training function can also be located in the OAM. This is based on what has been reported in TR 38.843 clause 7.2.4, where the training data generation and termination points are identified for the gNB-side model as follows [3]:

### 7.2.4 Positioning accuracy enhancements **** Skip non-relevant text **** For data collection, model transfer/delivery, and function-to-entity mapping analysis, various scenarios unfold when the data generation and termination entities differ. For instance, for: - Model Training: **** Skip non-relevant text **** - For gNB-side model, training data can be generated by the gNB, while the termination point for training data may include the gNB, or OAM.  Note: RAN2 identified the case in which LMF may be used for gNB-side model training. However, no conclusion was reached, as this depends on the RAN1 progress. **** Skip non-relevant text ****

Both possibilities concerning the termination point for training data – gNB or OAM – are aligned with the deployment scenarios also specified by RAN3 as part of the “AI/ML for NG-RAN” WI in Rel-18, as excerpted below from TS 38.300 clause 16.20.2 (for the non-split architecture) [4]:

### 16.20.2 Principles **** Skip non-relevant text **** For the deployment of AI/ML for NG-RAN the following scenarios may be supported: - AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the NG-RAN node; - AI/ML Model Training and AI/ML Model Inference are both located in the NG-RAN node. **** Skip non-relevant text ****

Therefore, we have the following proposal:

Proposal 1: For AI/ML assisted positioning Case 3a the AI/ML model training can also be located in the OAM other than in the gNB.

For Case 3b, instead, the AI/ML model training is located at the LMF, which is the termination point of training data, as reflected in TR 38.843 clause 7.2.4 [3]:

### 7.2.4 Positioning accuracy enhancements **** Skip non-relevant text **** For data collection, model transfer/delivery, and function-to-entity mapping analysis, various scenarios unfold when the data generation and termination entities differ. For instance, for: - Model Training: **** Skip non-relevant text **** o For LMF-side model, the LMF is the termination point for training data.

As per previous RAN1 agreements, a training data sample is composed of two parts, a “measurement” (Part A in RAN1 discussions) and a “ground-truth label” (Part B in RAN1 discussions). For Part A RAN1 agreed that it is generated by the TRP/gNB (for both Case 3a and Case 3b), that is, NG-RAN; while Part B can be generated at least by the LMF for Case 3a, while for Case 3b Part B can be generated by PRU, non-PRU UE with estimated location or LMF. RAN1 also agreed on the components that could be included in Part A and Part B of the training data sample.

******** RAN1#116bis ******** Agreement For training data generation of AI/ML based positioning Case 3a and 3b, the measurement and its related data (e.g., timestamp) are generated by TRP/gNB. Agreement For training data collection of AI/ML based positioning, the collected data sample can include the following components: Part A: - channel measurement - quality indicator of channel measurement - time stamp of channel measurement Part B: - ground truth label (or its approximation) - quality indicator of label - time stamp of label Note: "Part A" and "Part B" terminologies are only for RAN1 discussion purpose, and may not be used in specification. Note: contents in Part A and Part B may or may not be generated by different entities. Note: Part A and/or Part B, and their contents may or may not apply for each case FFS: detailed definition of channel measurement Working Assumption For training data generation of AI/ML based positioning Case 3b, the label and its related data (e.g., time stamp) can be generated by: - PRU - FFS: Non-PRU UE with estimated location - LMF Note: transfer of label and its related data is out of RAN1 scope. Agreement For training data generation of AI/ML based positioning Case 3a, the label and its related data (e.g., time stamp) can be generated by at least: - LMF Note: transfer of label and its related data is out of RAN1 scope. Note: whether other network entities can generate label for Case 3a is out of RAN1 scope. ******** RAN1#117 ******** Working Assumption For training data generation of AI/ML based positioning Case 3b, the label and its related data (e.g., time stamp) can be generated by: - PRU - Non-PRU UE with estimated location - LMF Note: transfer of label and its related data is out of RAN1 scope. Note: It is assumed that user data privacy of non-PRU UE is preserved. Note: Previous related working assumption made in RAN1#116bis for training data generation of AI/ML based positioning Case 3b will not need to be confirmed.

In the last RAN1 meeting (RAN1#118bis) RAN1 discussed further details of Part A’s content of the training data sample as follows:

******** RAN1#118bis ******** Agreement For training data collection of AI/ML based positioning, the quality indicator of timing information in Part A when reported is: - When applicable, the existing IE for timing quality, i.e., NR-TimingQuality in 37.355 and IE “Timing Measurement Quality” in 38.455; - FFS: details on how to associate quality indicator to timing information

For Case 3a, for both Part A and Part B, RAN3 also reached the following agreements in the last meeting [2]:

Case 3a agreements: - LMF provides ground truth label and related data (i.e., part B information as called by RAN1) to NG-RAN. FFS on the content of the ground truth label and related data pending on RAN1 progress. FFS on the signalling design. - How the LMF obtains the ground truth label and related data information is outside of RAN3 scope. - NG-RAN obtains measurements from TRP (i.e., part A information as called by RAN1). The details of the measurements are still pending and subject to further discussion in RAN1.

Based on the above, RAN3 expects to receive further inputs from RAN1 in order to discuss the signalling details to enable, for Case 3a, the LMF to provide Part B of the training data sample to NG-RAN, but details of Part B were not discussed in the last RAN1 meeting. For Part A, instead, since it is generated by the TRP/gNB for Case 3a, there is no RAN3 impact in making this information available in the entity performing AI/ML model training, i.e., the NG-RAN (if the AI/ML model training is located in the NG-RAN). Part A’s content for Case 3a is still under discussion in RAN1 and our understanding is that it reflects the content of the measurement report provided by the gNB to the LMF which includes the inference results (discussed in Section 2.2).

Even though RAN1 didn’t agree on a specific indication concerning the user data privacy requirements for the provisioning of Part B from the LMF to the NG-RAN for Case 3a (discussed in this Section for AI/ML model training), we think that a similar note from RAN1 about the user data privacy as included in the description of Option A for the AI/ML model monitoring options for Case 3a should also be considered for the model training as well – note that the model monitoring considerations for Case 3a can be found in Section 2.3 of this contribution.

In addition, RAN1 agreed that the purpose for which a certain information is provided does not need to be indicated in the specifications. In our view, this is in line with the principle which drove the specification of “AI/ML for NG-RAN” since Rel-18, for which the following was specified in TS 38.300 clause 16.20.2 [4]:

### 16.20.2 Principles **** Skip non-relevant text **** Signalling procedures used for the exchange of information to support AI/ML for NG-RAN, are use case and data type agnostic, which means that the intended usage (e.g., input, output, feedback) of the data exchanged via these procedures is not indicated.

Therefore, it is suggested that the same principle is also considered during the discussions in RAN3 about the impacts for supporting Case 3a. Based on the above, we have the following proposal:

Proposal 2: For Case 3a, signalling from LMF to gNB for transferring Part B information of the training data sample can be discussed in RAN3 only after further RAN1 input concerning Part B’s content is received.

- RAN3 assumes that the provisioning of Part B ensures and preserves the corresponding user data privacy requirements.

- Purpose, such as “training data collection”, will not necessarily be specified in RAN3 specification.

Proposal 3: For Case 3a, there is no RAN3 impact for Part A of the training data sample to be made available in the NG-RAN (if AI/ML model training is located in the NG-RAN). Part A’s content reflects the content of the measurement report provided by the gNB to the LMF which includes the inference results.

On the other hand, for Case 3b, there is RAN3 impact in providing Part A of the training data sample from NG-RAN to LMF, and this is also reflected in the agreement that RAN3 reached in the last meeting [2]:

For case 3b: - NG-RAN gets measurements (i.e., part A information as called by RAN1) from TRP and provides them to LMF. The details of the measurements are still pending and subject to further discussion in RAN1.

For Part B of the training data sample, instead, if for Case 3b it is generated by PRU/Non-PRU UE with estimated location, there is no RAN3 impact since this information can be provided via LPP protocol to the LMF. Still for Case 3b, if Part B is generated by the LMF, then there is as well no RAN3 impact since this information is already available in the entity performing AI/ML model training, i.e., LMF.

In the last RAN3 meeting the open point on the details of the measurements (Part A) being pending and subject to further discussion in RAN1 was captured in the meeting minutes for both Case 3a and Case 3b [2]. Therefore, RAN3 expects to receive further inputs from RAN1 in order to discuss the details of Part A’s content which, for Case 3b, needs to be made available at the LMF to perform AI/ML model training. Part A’s content for Case 3b is still under discussion in RAN1 and our understanding is that it reflects the content of the measurement report provided by the gNB to the LMF for successive AI/ML model inference as per the agreement from RAN1#118bis meeting (discussed in Section 2.2).

Still based on RAN1 agreements, for Case 3b, Part A of the training data sample includes a timestamp of the channel measurement. For this piece of information RAN1 also agreed that the existing IE “Time Stamp” in TS 38.455 (NRPPa protocol) can be reused from RAN1 perspective:

******** RAN1#118 ******** Agreement For training data collection of AI/ML based positioning case 3b, for time stamp of channel measurement, - For channel measurement generated by TRP/gNB, existing IE “Time Stamp” in TS 38.455 can be reused from RAN1 perspective. Note: Purpose, such as above “training data collection", will not necessarily be specified in RAN 1 specifications.

Moreover, within Part A of a training data sample for Case 3b, the quality indicator of the channel measurement (where the channel measurement could be either a timing information or a paired timing information and power information for Case 3b, as per RAN1#116 meeting) is also included. And in the last RAN1#118bis meeting RAN1 agreed that the quality indicator in Part A reuses the existing IE “Timing Measurement Quality” in TS 38.455. Therefore, for Case 3b, there is no additional RAN3 impacts on how to transfer the time stamp and quality indicator both associated to the channel measurement to the LMF.

Proposal 4: For Case 3b, there is no RAN3 impact in providing the LMF with the time stamp and quality indicator of the channel measurement included in Part A of the training data sample.

The above Proposal 4 is also in line with the agreement that RAN3 reached in the last meeting concerning the actual reporting of measurements to the LMF for Case 3b, which will be performed by re-using the existing measurement procedure (and hence messages) already specified in NRPPa [5]:

Case 3b: Re-using the Measurement procedure defined in NRPPa to support requesting and reporting the related measurements to LMF for Case 3b. The details of the related measurements are still pending and subject to further discussion in RAN1.

In addition, RAN1 agreed in RAN1#118 meeting a Note that the purpose for which a certain information is provided does not need to be indicated in the specifications. In our view, this is in line with the principle which drove the specification of “AI/ML for NG-RAN” since Rel-18 as captured in TS 38.300 clause 16.20.2 [4], therefore, it is suggested that the same principle is also considered during the discussions in RAN3 about the impacts for supporting Case 3b. Based on the above, we have the following proposal:

Proposal 5: For the discussion of RAN3 impacts of Case 3b, RAN3 to agree that the purpose, such as “training data collection”, or “monitoring”, or “inference” will not necessarily be specified in RAN3 specification when providing Part A of the training data sample to the LMF.

## 2.2 Model inference considerations

For Case 3a, to enable the AI/ML model in the gNB to generate measurements for successive determination at LMF side of the UE location, some input data needs to be provided to the AI/ML model itself which will then generate a proper output. In previous RAN1 meetings the following agreements were achieved concerning the AI/ML model input information:

******** RAN1#116 ******** Agreement For Rel-19 AI/ML based positioning, the measurements for determining model input are based on the DL PRS and UL SRS defined in TS38.211. Note: The use of SRS for MIMO resource is transparent to UE. ******** RAN1#118 ******** Conclusion From RAN1 perspective, for Case 3a measurements, - The existing procedures can be reused in terms of SRS configuration. - Note: parameter values for SRS configuration can be further discussed - These measurements can be used for multiple aspects related to case 3a, e.g. training data collection, monitoring, or inference procedures. - Note: Purpose, such as the training data collection, monitoring, or inference procedures mentioned above, will not necessarily be specified in RAN 1 specifications

Based on the above agreements, our understanding is that the AI/ML model in the gNB for Case 3a uses UL SRS-based measurements as model input for inference purposes (other than for other procedures such as training and monitoring). And, as per Proposal 2 above and based on RAN1 agreements, the purpose for which information are provided and used shall not be indicated in RAN3 specifications.  RAN1 made a conclusion in RAN1#118 meeting that the existing procedures can be reused for the SRS configuration, even though the details of the existing parameters (e.g., values) can be further discussed.

Proposal 6: The AI/ML model in the gNB for Case 3a uses SRS-based measurements as model input also for inference purposes, by reusing the existing procedures for the SRS configuration.

With regards to the AI/ML model output for Case 3a, i.e., information that could be reported from the NG-RAN to the LMF, RAN1 agreed that at least LOS/NLOS indicator and/or timing information are supported for reporting.

******** RAN1#116 ******** Agreement For AI/ML assisted positioning Case 3a, at least LOS/NLOS indicator and/or timing information are supported for reporting. - If LOS/NLOS indicator is reported, the indicator can be reported as soft indicator or hard indicator as defined in 38.214. - If timing information is reported, the timing information at least can be reported via UL RTOA or gNB Rx-Tx time difference as defined in 38.215. - Note: details of the report are pending further discussion.

For both pieces of information RAN1 also considered the existing methods already specified, that is, soft/hard indicator for the LOS/NLOS indicator, and UL RTOA (Uplink Relative Time of Arrival) or gNB Rx-Tx time difference for the timing information.

In the last meeting RAN1 agreed further aspects related to the content of the measurement report from gNB to LMF for Case 3a, as follows:

******** RAN1#118bis ******** Agreement From RAN1 perspective, when timing information is reported for Rel-19 AI/ML positioning Case 3a, at least the following are mandatorily or optionally supported in a measurement report from gNB to LMF: - (Mandatory) timing information; - (Optional) Quality of the timing information; - Existing IE “Timing Measurement Quality” can be reused. - (Mandatory) Time stamp. FFS: LOS/NLOS indicator. Note: The final decision of “mandatory” or “optional” presence of each field is up to RAN3.

The above agreement from RAN1 is related to the presence (mandatory/optional) of components (Information Elements, IEs) that constitute the measurement report from gNB to LMF in case timing information (either UL RTOA or gNB Rx-Tx difference) is reported as part of the AI/ML model output for Case 3a. As part of the above RAN1 agreement there is also a Note stating that the final decision of “mandatory” or “optional” presence of each field is up to RAN3.

By checking the latest NRPPa specification [5], the following can be excerpted:

#### 9.1.4.4	MEASUREMENT REPORT

This message is sent by the NG-RAN node to report positioning measurements for the target UE.

Direction: NG-RAN node  LMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | ignore |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| LMF Measurement ID | M |  | INTEGER (1..65536, …) |  | YES | reject |
| RAN Measurement ID | M |  | INTEGER (1..65536, …) |  | YES | reject |
| TRP Measurement Response List |  | 1 |  |  | YES | reject |
| >TRP Measurement Response Item |  | 1..<maxnoofMeasTRPs> |  |  | - |  |
| >>TRP ID | M |  | 9.2.24 |  | - |  |
| >>TRP Measurement Result | M |  | 9.2.37 |  | - |  |
| >>Cell ID | O |  | NR CGI 9.2.9 | The Cell ID of the TRP identified by the TRP ID IE. | YES | ignore |

#### 9.2.37	TRP Measurement Result

This information element contains the measurement result.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Measured Result Item |  | 1 .. <maxnoPosMeas> |  |  | - |  |
| >CHOICE Measured Results Value | M |  |  |  | - |  |
| >>UL Angle of Arrival |  |  | 9.2.38 |  |  |  |
| >>UL SRS-RSRP |  |  | INTEGER (0..126) |  |  |  |
| >>UL RTOA Measurement |  |  | 9.2.39 |  |  |  |
| >>gNB Rx-Tx Time Difference |  |  | 9.2.40 |  |  |  |
| >>Z-AoA |  |  | 9.2.67 |  | YES | reject |
| >>Multiple UL-AoA |  |  | 9.2.71 |  | YES | reject |
| >>UL SRS-RSRPP |  |  | 9.2.72 |  | YES | reject |
| >>UL RSCP |  |  | 9.2.92 |  | YES | reject |
| >Time Stamp | M |  | 9.2.42 |  | - |  |
| >Measurement Quality | O |  | 9.2.43 |  | - |  |
| >Measurement Beam Information | O |  | 9.2.57 |  | - |  |
| >SRS Resource type | O |  | 9.2.73 |  | YES | ignore |
| >ARP ID | O |  | 9.2.75 |  | YES | ignore |
| >LoS/NLoS Information | O |  | 9.2.77 |  | YES | ignore |
| >Mobile TRP Location Information | O |  | 9.2.88 |  | YES | ignore |
| >Measured Frequency Hops | O |  | ENUMERATED (singleHop, multiHop, …) |  | YES | ignore |
| >Aggregated Positioning SRS Resource ID List |  | 0..1 |  | Indicates the used Positioning SRS resources across aggregated carriers. | YES | ignore |
| >>Aggregated Positioning SRS Resource ID Item |  | 2..< maxnoaggregatedPosSRS-Resources > |  |  | - |  |
| >>>Positioning SRS Resource ID | M |  | INTEGER (0..63) |  |  |  |
| >>>Point A | M |  | INTEGER (0..3279165) |  | YES | ignore |
| >>>SCS Specific Carrier |  | 1 |  |  | YES | ignore |
| >>>>Offset To Carrier | M |  | INTEGER (0..2199, …) |  |  |  |
| >>>>Subcarrier Spacing | M |  | ENUMERATED(kHz15, kHz30, kHz60, kHz120, …, kHz480, kHz960) |  |  |  |
| >>>>Carrier Bandwidth | M |  | INTERGER (1..275, …) |  |  |  |
| >>>PCI | O |  | INTEGER (0..1007) |  | YES | ignore |
| >Measurement Based On Aggregated Resources | O |  | ENUMERATED(true, …) |  | YES | ignore |

Based on the current NRPPa specification, in our understanding the presence of IEs (timing information, quality of the timing information, time stamp) in the measurement report from gNB to LMF in case timing information (either UL RTOA or gNB Rx-Tx difference) is reported for Case 3a is already aligned with the presence as suggested in the RAN1 agreement. Therefore, we think RAN3 should confirm what RAN1 already indicated for the presence of IEs in the measurement report for Case 3a, which has no additional standard impact on NRPPa.

Proposal 7: RAN3 to confirm the RAN1 agreement that, when timing information is reported for Case 3a, the presence of IEs in the measurement report from gNB to LMF is as in legacy NRPPa, that is:

- Timing information: mandatory IE

- Quality of the timing information: optional IE

- Time stamp: mandatory IE.

On the other hand, for Case 3b, to enable the AI/ML model in the LMF to directly infer the UE location some input data needs to be provided to the AI/ML model itself. In previous RAN1 meetings the following agreements were achieved concerning the AI/ML model input information:

******** RAN1#116 ******** Agreement For Rel-19 AI/ML based positioning, the measurements for determining model input are based on the DL PRS and UL SRS defined in TS38.211. Note: The use of SRS for MIMO resource is transparent to UE. Agreement For AI/ML based positioning case 3b, at least the following types of time domain channel measurements are supported for reporting: - timing information; - paired timing information and power information. Agreement In Rel-19 AI/ML based positioning, regarding the time domain channel measurements, RAN1 investigate the following alternatives: - Alternative (a). Sample-based measurements, where the timing information is an integer multiple of sampling periods. - Alternative (b). Path-based measurements, where the timing information is according to the detected path timing and may not be an integer multiple of sampling periods. The issues to be studied include, but not limited to, the following: - Tradeoff of positioning accuracy and signaling overhead - Impact and necessary details of gNB/UE implementation to obtain the channel measurement values. - Whether the same Alternative(s) applies to all cases or not - Applicability and necessity of specifying the Alternative(s) to different cases - Note: different sub-cases may have different issues. Note: In addition to timing information, the components for the channel measurement for model input may also include power and potentially phase. To provide the type of the channel measurement in their investigation. ******** RAN1#116bis ******** Agreement For AI/ML based positioning Case 3b, for gNB channel measurements reported to LMF, the timing information is represented relative to the existing UL RTOA reference time T0+tSRS as defined in TS 38.215. FFS: whether it is applicable when Case 3b is used to support multi-RTT. ******** RAN1#117 ******** Agreement Sample-based measurement is defined as: - The measurement is composed of Nt' samples of the estimated channel response in time domain. The timing information for the Nt' samples are reported with a timing granularity T, where T=2kxTc. k represents the timing reporting granularity factor. Tc is the basic time unit for NR. - The corresponding measurement (e.g., power if reported) corresponds to the measurement for the reported Nt' samples. - Nt' and k can be signalled - FFS: the value range of Nt'; the value range of integer k for the timing granularity T. - The timing information is defined relative to a reference time Further discussion is expected on the determination of Nt' and k (including signaling) , and a rule to be introduced for selecting Nt' samples. Note: It doesn’t imply the definition of Sample-based measurement will be captured into the spec. Agreement Path-based measurement refers to the measurement in the existing specifications (up to Rel-18) including measurement reporting, with potential enhancements on the number of reported paths (if needed). ******** RAN1#118 ******** Agreement For the definition of sample-based measurement, select Nt’ samples out of a list of Nt consecutive samples, - The Nt samples have timing granularity T. - FFS: the starting time of the list of Nt samples. - FFS: the value range of Nt For the sample-based measurement (if accepted in Rel-19), - For measurement by TRP/gNB, the Nt’ selected samples are expected to be those with the highest power. - Note: Choice of the maximum value of Nt, Nt’ should take into account the need to preserve proprietary implementation. Agreement For AI/ML positioning Case 2b and 3b, regarding the power information for determining the model input, - For downlink power measurement, use DL PRS-RSRPP defined in TS 38.215 as a starting point. - For measurement report of DL PRS-RSRPP, use the existing measurement report mapping table for PRS-RSRPP in 38.133 as a starting point. - For uplink power measurement, use UL SRS-RSRPP defined in TS 38.215 as a starting point. - For measurement report of UL SRS-RSRPP, use the existing measurement report mapping table for SRS-RSRPP in 38.133 as a starting point. Agreement For Rel-19 AI/ML based positioning Case 3b, regarding sample-based measurement (if supported), from RAN1 perspective, - LMF can signal parameter values of Nt, Nt', k to gNB via NRPPa. ******** RAN1#118bis ******** Agreement From RAN1 perspective, for model inference of AI/ML positioning Case 3b, at least the following are mandatorily or optionally supported in a measurement report from gNB to LMF: - (Mandatory) Channel measurement; - (Optional) Quality of the channel measurement; - FFS: details of the quality - (Mandatory) Time stamp of the channel measurement.

By specifically referring to the RAN1 agreement from the last meeting (RAN1#118bis) concerning the content of the measurement report from gNB to LMF for Case 3b, we noticed that also for Case 3b RAN1 discussed about the presence (mandatory/optional) of components (IEs) that constitute the measurement report. As previously analyzed, similar discussion occurred in the last RAN1 meeting also for Case 3a, where a Note stating that the final decision of “mandatory” or “optional” presence of each field is up to RAN3 was captured in RAN1 meeting minutes. In our understanding this Note also applies for the agreement from RAN1#118bis above reported for Case 3b, even if this has not been explicitly captured by RAN1 – the motivation of that is because the agreements about the presence of components that constitute the measurement report for Case 3b and Case 3a have been taken in different times during the meeting week, which led RAN1 in including the Note only for one of the two AI/ML positioning cases having RAN3 impacts. By still referring to the latest NRPPa specification [5], in our understanding and same as for Case 3a, the presence of IEs (channel measurement, quality of the channel measurement, time stamp) in the measurement report from gNB to LMF for Case 3b is already aligned with the presence as suggested in the RAN1 agreement. Therefore, we think RAN3 should confirm what RAN1 already indicated for the presence of IEs in the measurement report for Case 3b as well, which has no additional standard impact on NRPPa.

Proposal 8: RAN3 to confirm the RAN1 agreement that the presence of IEs in the measurement report from gNB to LMF for Case 3b is as in legacy NRPPa, that is:

- Channel measurement: mandatory IE

- Quality of the channel measurement: optional IE

- Time stamp of the channel measurement: mandatory IE.

For Case 3b, RAN1 agreed that channel measurements supported for reporting can be either timing information or paired timing information and power information (RAN1#116).

In our understanding the power information associated to channel measurements that are provided from NG-RAN to the LMF are based on the existing UL SRS (Uplink Sounding Reference Signal), that is, UL SRS Reference Signal Received Path Power (UL SRS-RSRPP) measurement. Therefore, we think there is no standard impact in reporting such information to from NG-RAN to LMF for Case 3b.

Proposal 9: RAN3 to confirm that for Case 3b the power information associated to channel measurements to be provided from NG-RAN to LMF which can be used as model input are based on the existing UL SRS. There is no standard impact for reporting the power information (UL SRS-RSRPP) from NG-RAN to LMF.

- The purpose of the report such as “model input” will not necessarily be specified in RAN3 specification.

Moreover, still for Case 3b, for the timing information of channel measurements, RAN1 agreed that the timing information is represented relative to a reference time which is the existing UL RTOA (Uplink Relative Time of Arrival) reference time T0+tSRS as defined in TS 38.215 (RAN1#116bis). Hence in our view there is no RAN3 impact in reporting timing information of channel measurements from NG-RAN to LMF for Case 3b.

Proposal 10: RAN3 to confirm that for Case 3b there is no standard impact for reporting the timing information associated to channel measurements from NG-RAN to LMF.

Regarding the time domain channel measurements for Case 3b, RAN1 is also investigating two alternatives:

- Sample-based measurements, where the timing information is an integer multiple of sampling periods.

- Path-based measurements, where the timing information is according to the detected path timing and may not be an integer multiple of sampling periods, and for which RAN1 also agreed that it refers to the measurement in the existing specifications (up to Rel-18) including measurement reporting, with potential enhancements on the number of reported paths (if needed).

For the sample-based measurements RAN1 agreed some aspects such as its definition, the number of samples of the estimated channel response in time domain and the time granularity for the reporting. It is still under discussion whether sample-based measurements will be supported in Rel-19. It is then suggested waiting for further RAN1 progress before starting discussions in RAN3 on how to enable signalling from LMF to gNB of sample-based measurements configuration parameters (Nt, Nt', k as agreed by RAN1).

Proposal 11: RAN3 to wait RAN1 confirmation on whether sample-based channel measurements for Case 3b will be supported in Rel-19 before starting discussions in RAN3 on how to enable signalling from LMF to gNB of configuration parameters (Nt, Nt', k as agreed by RAN1).

For the path-based measurements, instead, they follow the existing specifications as per RAN1 agreement, and the only potential enhancements could be on the number of reported paths (RAN1#117). Also in this case, it is suggested waiting for further RAN1 progress before RAN3 makes changes in NRPPa to support an increased number of paths to be reported to the LMF for Case 3b.

Proposal 12: RAN3 to wait further RAN1 progress on whether enhancements on the number of reported paths in path-based channel measurements for Case 3b are needed before RAN3 makes changes in NRPPa to support an increased number of paths to be reported to the LMF.

## 2.3 Model monitoring considerations

Another important aspect for a proper AI/ML model’s handling is the model monitoring. In the last meeting, for model monitoring in Case 3a, RAN3 set a Working Assumption (WA) based on what RAN1 identified as Option A for the model monitoring metric calculation [2]:

Case 3a agreements: - WA: RAN3 will proceed with Option A: NG-RAN node performs monitoring metric calculation for its own model.

For model monitoring in Case 3a, RAN1 identified two options:

- Option A, where the NG-RAN (hosting the AI/ML model) performs the monitoring metric calculation – this is the option covered in the WA from the last RAN3 meeting;

- Option B, where the LMF performs the monitoring metric calculation for the model hosted in NG-RAN.

Before discussing the peculiarities of Option A and Option B, and then decide which option to specify, we think RAN3 should first agree on a general principle (hence applicable for both Case 3a and Case 3b) concerning the location of the model monitoring function with respect to the entity which is in charge of the AI/ML model inference or, in general, of taking a proper “monitoring decision” for the model based on inference results (e.g., (de-)activation, fallback to legacy positioning methods, etc.). In our view, it would be better that the entity hosting the AI/ML model is also the entity that performs the model monitoring metric calculation, which is also the entity which has knowledge of the model being used. This because in our understanding the “monitoring decision” should be based on the model-specific monitoring metric calculated by considering two main components, that is, the inference results (i.e., model output) and the ground-truth labels.

Proposal 13: RAN3 to agree the general principle (applicable for Case 3a and Case 3b) that the entity hosting the AI/ML model is also the entity that performs the model monitoring metric calculation.

For the specific case of AI/ML assisted positioning Case 3a, the inference results are generated by the AI/ML model hosted in the gNB and sent to LMF for successive determination of the UE location, while ground-truth labels are provided by the LMF and transferred to the gNB. It should be noted that the gNB could have the ground-truth labels already available as they might have been previously provided by the LMF for training the model in the gNB (i.e. Part B of the training data sample), therefore in principle there could not be any further need to transfer ground-truth labels for model monitoring purposes. If this is not the case, ground-truth labels can anyway be made available at the gNB without any additional standard impact on top of the signalling to be specified by RAN3 to enable model’s training.

Based on the above, in Option A, the NG-RAN node may calculate the monitoring metric based on the inference results and the ground-truth labels already available in the gNB side, and take a proper “monitoring decision” accordingly.

For Option B, where the monitoring metric is calculated at the LMF, further specification impact would be required. For example, some model-specific information could be needed to be provided to the LMF for metric calculation as well as signalling back from LMF to gNB, e.g., to inform the gNB about the monitoring outcome would be needed to be specified.

In Option A, instead, the monitoring metric calculation is performed in the gNB directly, which has knowledge of the AI/ML model used for inference and, in principle, the gNB already has the needed set of information (i.e., inference results, ground-truth labels and knowledge of the model) to then calculate the monitoring metric and determine the most appropriate “monitoring decision” accordingly. Hence, in our understanding, Option A has less standardization impact with respect to Option B.

In the last RAN3 meeting there were some companies which expressed concerns on Option A due to the fact that, in principle, the model monitoring metric calculation is a function of inference results (from the gNB) and ground-truth labels (from the LMF). However, these two pieces of information are not comparable, because the ground-truth label is the UE location, while the inference result is an intermediate measurement (timing information and/or LOS/NLOS indication) that will be used by LMF to determine the UE location. Therefore, in order for Option A to work, the LMF should provide as ground-truth label not the UE location but an information derived from it and that can be compared with the intermediate measurement inferred by the AI/ML model in the gNB. And, as a consequence, this poses a requirement in the LMF to derive such kind of information from the actual ground-truth label (i.e., the UE location) that could be easily comparable with the inferred intermediate measurement available at the gNB.

It is worth to note that the intermediate measurement available at the gNB is based on the propagation delay of a direct path between the TRP/gNB and the UE, and the propagation delay is based on the relative distance between the TRP/gNB and the UE. In this regard, the intermediate result can be viewed as a part of the UE location, i.e., ranging information, without any angular information. Hence, we do not see the need for the LMF to provide information different from the ground-truth label (i.e., the UE location) to the gNB to perform the model monitoring metric calculation as per Option A.

Based on the above, we think that the WA from the last RAN3 meeting should be turned into an agreement.

Proposal 14: For Case 3a, RAN3 to agree that the “WA: RAN3 will proceed with Option A: NG-RAN node performs monitoring metric calculation for its own model.” from the last RAN3 meeting is turned into an agreement.

For Case 3b, instead, in previous RAN1 meetings the following agreement was reached:

******** RAN1#116 ******** Agreement For LMF-side model, RAN1 studies whether/what assistance information and/or measurement report may be sent from UE/PRU, and/or gNB to LMF to assist at least for the performance monitoring. - RAN1 understands that it is out of RAN1 scope to define monitoring metric calculation and related model management decisions for LMF-side model.

Based on the above, our understanding is that RAN1 is still studying whether and what information needs to be provided from the NG-RAN to the LMF for model monitoring in Case 3b. Therefore, it is suggested waiting for further RAN1 progress before starting the discussion in RAN3.

Proposal 15: RAN3 to wait further RAN1 progress on whether and what information needs to be provided from the NG-RAN to the LMF for model monitoring in Case 3b.

# Conclusions

Model training considerations for Case 3a

Proposal 1: For AI/ML assisted positioning Case 3a the AI/ML model training can also be located in the OAM other than in the gNB.

Proposal 2: For Case 3a, signalling from LMF to gNB for transferring Part B information of the training data sample can be discussed in RAN3 only after further RAN1 input concerning Part B’s content is received.

- RAN3 assumes that the provisioning of Part B ensures and preserves the corresponding user data privacy requirements.

- Purpose, such as “training data collection”, will not necessarily be specified in RAN3 specification.

Proposal 3: For Case 3a, there is no RAN3 impact for Part A of the training data sample to be made available in the NG-RAN (if AI/ML model training is located in the NG-RAN). Part A’s content reflects the content of the measurement report provided by the gNB to the LMF which includes the inference results.

Model training considerations for Case 3b

Proposal 4: For Case 3b, there is no RAN3 impact in providing the LMF with the time stamp and quality indicator of the channel measurement included in Part A of the training data sample.

Proposal 5: For the discussion of RAN3 impacts of Case 3b, RAN3 to agree that the purpose, such as “training data collection”, or “monitoring”, or “inference” will not necessarily be specified in RAN3 specification when providing Part A of the training data sample to the LMF.

Model inference considerations for Case 3a

Proposal 6: The AI/ML model in the gNB for Case 3a uses SRS-based measurements as model input also for inference purposes, by reusing the existing procedures for the SRS configuration.

Proposal 7: RAN3 to confirm the RAN1 agreement that, when timing information is reported for Case 3a, the presence of IEs in the measurement report from gNB to LMF is as in legacy NRPPa, that is:

- Timing information: mandatory IE

- Quality of the timing information: optional IE

- Time stamp: mandatory IE.

Model inference considerations for Case 3b

Proposal 8: RAN3 to confirm the RAN1 agreement that the presence of IEs in the measurement report from gNB to LMF for Case 3b is as in legacy NRPPa, that is:

- Channel measurement: mandatory IE

- Quality of the channel measurement: optional IE

- Time stamp of the channel measurement: mandatory IE.

Proposal 9: RAN3 to confirm that for Case 3b the power information associated to channel measurements to be provided from NG-RAN to LMF which can be used as model input are based on the existing UL SRS. There is no standard impact for reporting the power information (UL SRS-RSRPP) from NG-RAN to LMF.

- The purpose of the report such as “model input” will not necessarily be specified in RAN3 specification.

Proposal 10: RAN3 to confirm that for Case 3b there is no standard impact for reporting the timing information associated to channel measurements from NG-RAN to LMF.

Proposal 11: RAN3 to wait RAN1 confirmation on whether sample-based channel measurements for Case 3b will be supported in Rel-19 before starting discussions in RAN3 on how to enable signalling from LMF to gNB of configuration parameters (Nt, Nt', k as agreed by RAN1).

Proposal 12: RAN3 to wait further RAN1 progress on whether enhancements on the number of reported paths in path-based channel measurements for Case 3b are needed before RAN3 makes changes in NRPPa to support an increased number of paths to be reported to the LMF.

Model monitoring considerations for Case 3a and Case 3b

Proposal 13: RAN3 to agree the general principle (applicable for Case 3a and Case 3b) that the entity hosting the AI/ML model is also the entity that performs the model monitoring metric calculation.

Model monitoring considerations for Case 3a

Proposal 14: For Case 3a, RAN3 to agree that the “WA: RAN3 will proceed with Option A: NG-RAN node performs monitoring metric calculation for its own model.” from the last RAN3 meeting is turned into an agreement.

Model monitoring considerations for Case 3b

Proposal 15: RAN3 to wait further RAN1 progress on whether and what information needs to be provided from the NG-RAN to the LMF for model monitoring in Case 3b.

# References

- RP-242399, Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Qualcomm, RAN#105

- 3GPP RAN3#125b Chairman’s notes (RAN3_125bis_agenda_20241018_1430_EOM.doc)

- 3GPP TR 38.843, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (v18.0.0), Rel-18

- 3GPP TS 38.300, NR and NG-RAN Overall Description; Stage 2, Rel-18

- 3GPP TS 38.455, NG-RAN; NR Positioning Protocol A (NRPPa), Rel-18