3GPP TSG-RAN WG3 Meeting #126	R3-247422

Orlando, USA, 18 – 22 Nov 2024

Agenda Item:	20.1

Source:	ZTE Corporation

Title:	[Draft LS] Discussion on reply LS on AIML data collection

Document for:	Other

- Introduction

RAN sent the LS[1] to SA groups, especially SA2, about the requirements for data collection for UE sided model training for potential solution (i.e. Option 1b, 2, 3).

Following is the requirements agreed:

1.	The data collected is secured and data integrity and confidentiality for that data is ensured.

2.	User data privacy, anonymity and user consent is respected.

3.	The MNO has full control of the standardized data collection transfer process and can manage data transfer to the server for UE-side data collection, without the need of SLA for this purpose. This includes initiating, terminating, and fully managing data transfer.

4.	MNO has full visibility for standardized data.

5.	The design is futureproof and extendable.

FFS/study if and how to handle non-standardized data (i.e. partial visibility).

FFS controllability on data collection

SA2 is now assessing the feasibility of UE-side data collection and requires further clarification from RAN2 on the above requirements [2]. One of the questions also requires RAN3’s feedback. In this contribution, we provide our understanding of this question from the RAN3 perspective.

- Discussion

Following question raised by SA2 requires RAN3’s feedback:

Q1: Are there any aspects of the UE-data collection controllability, that required NG-RAN involvement? If so, what is the involvement of NG-RAN in UE-data collection controllability, e.g., what aspects of MNO controllability would require NG-RAN involvement and what would such involvement be?

As an example of the kind of feedback that is requested, some companies in SA2 understand that initiating (e.g., triggering), terminating collection of UE-side data and controlling data transfer may require NG-RAN involvement, and it is currently not clear what this involvement may be.

This question requires RAN3 to address whether UE data collection controllability needs to include the NG-RAN node.

Before addressing this question, RAN3, in the "AI/ML for NG-RAN" topic, focuses data collection solely on inter NG-RAN nodes. When model training/inference is performed on the NG-RAN side, the NG-RAN node initiates a DATA COLLECTION INITIATION procedure to neighboring NG-RAN nodes, requesting specific data (e.g., predicted resource stauts, energy cost, etc) and reporting configuration. The receiving NG-RAN nodes then report prediction data or measurement accordingly. Since it is the NG-RAN node that utilizes this information, the data request needs to be initiated by the NG-RAN node.

- In the RAN3-led “AI/ML for NG-RAN” topic, NG-RAN node initiates the data collection procedure to request the report of AI/ML related data from neighbour NG-RAN nodes.

For RAN-led “AI/ML for Air” topic, it involves AI/ML assisted positioning, AI/ML assisted beam management, and AI/ML assisted CSI prediction. In RAN2#127bis meeting, RAN2 only agreed that “Data Collection initation and configuration for data collection is under network control”. However, depending on the specific use case, network control can be managed by different network entities. For AI/ML assisted positioning, LMF may be the network entity to control data collection process, while for AI/ML asissted beam management and AI/ML assisted CSI prediction, it may be CN or NG-RAN, which depends on the details of collected data.

From the RAN3 perspective, we cannot directly respond to whether NG-RAN involvement is necessary for data collection control. The specific data required for each case needs to be determined by RAN1 and RAN2, after which RAN3 can make an informed judgment based on that information.

- The network entity responsible for controlling the data collection process may vary based on the requirements of AI/ML-assisted use cases.

- RAN3 cannot directly respond to whether NG-RAN involvement is necessary for UE-side data collection without knowing the specific type and requirement of UE-side collection involved.

Currently, RAN3 supports UE-side data collection through MDT for data gathering, as well as QoE mechanisms. For MDT mechanism, OAM can configure the MDT configuration based on the requirement to NG-RAN through management-based MDT or signaling-based MDT. For QoE data collection, OAM can also deliver QoE configurations to the RAN side and specify which QoE measurements are RAN-visible. For these RAN-visible parameters, NG-RAN can be involved by initiating the corresponding RAN-visible configurations to the UE for use.

- RAN3 already supports MDT data collection as well as QoE data collection including RAN visible QoE measurements for UE data collection.

RAN2 is also discussing the topic of UE-data collection controllability, RAN2 will provide SA2 with a clearer explanation. Further inputs from RAN1 and RAN2 on UE data collection of the corresponding use cases are needed.

- CN/OAM/OTT collection of UE-sided model training data [RAN2/RAN1]:

  - For the FS_NR_AIML_Air study use cases, identify the corresponding contents of UE data collection

  - Analyse the UE data collection mechanisms identified during the FS_NR_AIML_Air (TR 38.843 section 7.2.1.3.2) study along with the implications and limitations of each of the methods

Observation 4   RAN2 will also respond to SA2’s question based on the current discussions and an analysis of all physical layer cases.

If majority companies in RAN3 are really required to respond, we suggest providing a more general answer to Q1, describe what we specified in “AI/ML for NG-RAN” topic, and states that RAN3 cannot give clear clarification to whether NG-RAN involvement is necessary for UE-side data collection without knowing the specific type and requirement of UE-side collection involved.

- If a reply from RAN3 is indeed necessary, RAN3 can send a LS providing a more general answer to Q1. This should include a description of what has been specified for the data collection process under the “AI/ML for NG-RAN” topic and a statement that RAN3 cannot provide a clear clarification at this time.

The reply LS is provided in the Annex below.

- Conclusion

Following is our observations and proposals:

- In the RAN3-led “AI/ML for NG-RAN” topic, NG-RAN node initiates the data collection procedure to request the report of AI/ML related data from neighbour NG-RAN nodes.

- The network entity responsible for controlling the data collection process may vary based on the requirements of AI/ML-assisted use cases.

- RAN3 cannot directly respond to whether NG-RAN involvement is necessary for UE-side data collection without knowing the specific type and requirement of UE-side collection involved.

Observation 3 	RAN3 already supports MDT data collection as well as QoE data collection including RAN visible QoE measurements for UE data collection.

Observation 4   RAN2 will also respond to SA2’s question based on the current discussions and an analysis of all physical layer cases.

- If a reply from RAN3 is indeed necessary, RAN3 can send a LS providing a more general answer to Q1. This should include a description of what has been specified for the data collection process under the “AI/ML for NG-RAN” topic and a statement that RAN3 cannot provide a clear clarification at this time.

4	Reference

[1] RP-242389, LS on AIML data collection, RAN

[2]	R3-247017, Reply LS on AML data collection, SA2

5	Annex – Reply LS to SA2

3GPP TSG-RAN WG3 Meeting #126	R3-24xxxx

Orlando, USA, 18 – 22 Nov 2024

Title:	[Draft] Reply LS on AIML data collection

Response to:	R3-247017(S2-2411191)

Release:	Release 19

Work Item:	NR_AIML_Air, NR_AIML_air-Core

Source:	ZTE Corporation[to be RAN3]

To:	SA2

Cc:	RAN2, TSG SA, TSG RAN, SA3, SA5

Contact person:	Chen Jiajun

Chen.jiajun1@zte.com.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

# 1	Overall description

RAN3 thanks SA2 for the LS in R3-247017(S2-2411191) on AI/ML data collection.

In R18 AI/ML for NG-RAN WI, RAN3 focuses data collection solely on inter NG-RAN nodes. The NG-RAN node initiates a Data Collection Initiation procedure to neighboring NG-RAN nodes via Xn, requesting specific data (e.g., predicted radio resource stauts, energy cost, etc).

And the following mechanisms related to UE data collection are supported in RAN3:

- MDT data collection which is triggered from OAM

- QoE data collection including RAN visible QoE measurements.

For Question 1, RAN3 cannot directly respond to whether NG-RAN involvement is necessary for UE-side data collection without knowing the specific UE-side data to be collected.

# 2	Actions

To SA2

ACTION: 	RAN3 kindly ask SA2 to take information above into account.

# 3	Dates of next RAN3 meetings

Updated meeting schedule can be found at: https://portal.3gpp.org/?tbid=373&SubTB=381#/

RAN3#127				2025-02-17 – 2025-02-21				Athens, GR

RAN3#127bis			2025-04-07 – 2025-04-11				China, CN