3GPP TSG-RAN WG3 Meeting #126	R3-247428

Orlando, USA, 18th – 22th November, 2024

Agenda Item:	12.2

Source: 	Lenovo

Title: 	Architecture and configuration for WAB-node

Document for:	Discussion and decision

- Introduction

A new WID for additional topological enhancements [1] has been approved to be discussed in Rel-19, and following detail objectives are identified for Wireless Access Backhaul (WAB) focus on RAN3.

The objectives of the Wireless Access Backhaul (WAB) work are as follows: - Specifications for the support of WAB including [RAN3]: - Support of a WAB-node including a WAB-gNB and a WAB-MT. - Support of backhauling of the WAB-gNB’s NG, Xn and OAM traffic over the WAB-MT’s PDU session(s). - Support of Xn interface(s) by the WAB-gNB with the WAB-MTs serving BH RAN node and with other surrounding gNBs, including how to avoid setting up Xn between WAB-gNBs. - Defining the behaviour of WAB-node in case the authorization status of WAB-MT and/or WAB-gNB changes. - Network integration procedures for WAB nodes. - Handling of WAB-gNB’s traffic (including Xn, NG and OAM traffic) during WAB-node mobility, including the case where the WAB-MT’s BH PDU session changes. - Support the UE’s AMF change for UEs connected to, or camped on, a WAB-gNB. - UE’s ULI that reflect the WAB node’s location. - The handling of: - PCI collision avoidance. - Reconfiguration of TAC and RANAC on WAB-gNBs. - Mechanisms to avoid multi-hop WAB topology. - Radio-resource coordination between access and backhaul links. - NG connection management.

In this contribution, the architecture and configuration for WAB are discussed.

- Discussion

Xn interface in WAB architecture

- Support of Xn interface(s) by the WAB-gNB with the WAB-MTs serving BH RAN node and with other surrounding gNBs, including how to avoid setting up Xn between WAB-gNBs.

Different to IAB-node, WAB-node has the gNB functionality and the WAB-gNB may establish Xn connection for the following scenarios.

Scenario 1: Xn connection between WAB-gNBs of two WAB-nodes

Scenario 2: Xn connection between a WAB-gNB and its BH RAN node

Scenario 3: Xn connection between a WAB-gNB and a surrounding RAN node

Currently, in the WAB-node integration procedure, the WAB-gNB may establish Xn connection(s) towards the BH RAN node and/or other surrounding NG RAN node(s), if needed.

For the Xn connection between the WAB-gNB and its BH RAN node, the WAB-gNB can trigger the Xn connection setup to the BH RAN node after WAB-gNB setup procedure. And it’s not optimal for the BH RAN node to trigger the Xn setup procedure because the BH RAN node is transparent for the gNB setup of the WAB-node. As for the TNL address for the BH RAN node, the WAB-gNB can obtain it via legacy Xn-C TNL address discovery procedure or it can be indicated from the BH RAN node directly.

Proposal 1: For Xn connection between WAB-gNB and its BH RAN node, it's the WAB-gNB to trigger the Xn setup procedure.

For the Xn connection between the WAB-gNB and the surrounding RAN node, both the WAB-gNB and the surrounding RAN node need to discovery each other before Xn connection setup. As legacy, a gNB can find another gNB for Xn connection via measurement report or OAM configuration. However, legacy procedure may be not timely enough due to the mobility of the WAB-node. Alternatively, the WAB-gNB and the surrounding RAN node can discovery each other with the help of the BH RAN node, e.g., the WAB-gNB and the surrounding RAN node can discovery each other upon receiving the neighbour cell information from the BH RAN node. Then each of the two nodes can trigger Xn connection setup procedure.

Proposal 2: For Xn connection between WAB-gNB and surrounding RAN node, both the WAB-gNB and the surrounding RAN node can trigger the Xn setup procedure.

The main motivation to setup Xn connection for a WAB-node is to accelerate the Xn based handover of the served UEs of the WAB-node. However, due to mobility of the WAB-node, it’s unexpected to perform UE handover between two mobile WAB-nodes. Therefore, Xn connection between two WAB-nodes needs to be avoided.

The WAB-gNB can be aware of the type of another WAB-gNB (whether the gNB is WAB-gNB or not) via 1) served UE’s measurement report, 2) WAB-MT’s measurement report or 3) neighbour information from BH RAN node. And then the WAB-gNB avoid to setup Xn interface to another WAB-gNB. Alternatively, without type information of another WAB-gNB, the type of node is included in Xn setup request message, based on the type information in the Xn setup request message, the target RAN node can determine whether to accept Xn interface setup or not.

Proposal 3: WAB-gNB can obtain the type of another WAB-gNB and then avoid to setup Xn interface to another WAB-gNB. Or the WAB-gNB can include the type information in the Xn setup request message.

Multi-hop WAB topology

SA2 would like to inform RAN3 that SA2 had progressed the Rel-19 FS_VMR_Ph2 study and documented the conclusions in clause 8 in TR 23.700-06. Issue-1: SA2 has discussed the access control (to restrict MWAB-UE accessing MWAB-gNB cell) for below two cases: a) MWAB-UE accessing any MWAB-gNB in IDLE and CONNECTED mode (documented as one candidate solution in clause 6.2.3.6 of TS 23.700-06); and b) MWAB-UE accessing MWAB-gNB belonging to same MWAB. SA2 assume that those scenarios will be discussed by RAN WGs, and ask for RAN feedback.

For case a), it can be solved by the implementation of WAB-node. And for case b), based on the discussion on last meeting, there are many candidate solutions below can be used to avoid a WAB-MT to access another WAB-gNB.

For multi-hop prevention, RAN3 to down select to following three solutions:

Solution 1: The WAB-gNB uses dedicated frequencies and/or PCIs. FFS on any other legacy OTA parameters.

Solution 2: Use the slice dedicated for backhauling, i.e. use a list of S-NSSAIs in RRCsetupcomplete to do access control and/or use a list of S-NSSAIs in handover signalling. No CN upgrade is needed.

Solution 3: WAB-gNB-cells broadcast a new indicator in SIB to bar WAB-MT, and the WAB-MT avoids (re)selection of cells broadcasting this indicator.

Solution5: In case of handover for a WAB-node, the new WAB-node indication is included in the HO request, then the target BH-RAN node can perform access control for this WAB-node.

For Solution 1, it can be left to implementation of network deployment, at the same time, this solution may have some limitations on the deployment to allocate separate frequencies/PCIs for WAB. And for Solution 2, this can be also based on the implementation of operator, and once the WAB use the default slice, the solution may be workable. While for Solution 3 and Solution 5, they are feasible and will introduce some stage-3 impacts, however, the impacts are obvious and minor, and we don’t need to spend much time on the normative work.

Proposal 4: WAB-gNB broadcasts a WAB indication in SIB1 to avoid other WAB-MT to camp on the cell of WAB-gNB. And in case of handover, a new WAB-node indication is included in the handover request for access control at the target BH RAN node.

Authorization of WAB-MT and/or WAB-gNB

As agreed in the SI phase, the authorization of the WAB-MT is different from the service authorization of the WAB-gNB. Authorization of the WAB-MT provides the WAB-MT with the right to support backhauling the traffic of the co-located WAB-gNB via BH PDU session(s), while Authorization of the WAB-gNB provides the service authorization, i.e., the right to serve UEs. And the authorization of the WAB-MT is controlled by the WAB-MT’s 5GC while OAM of WAB-gNB controls the authorization of the WAB-gNB.

In case WAB-gNB becomes not-authorized, the UEs served by the WAB-gNB can either be handed over to other RAN nodes or they can be released, after which the NG and Xn connection(s) of the WAB-gNB can be removed. And In case WAB-MT becomes not-authorized, based on the conclusion below from SA2, the WAB-gNB will be turned into not-authorized by OAM accordingly, and the same behaviours are taken for served UEs and NG/Xn interfaces.

-	MWAB-gNB's OAM is assumed to be aware of the MWAB-UE authorization limitations (pre-determined authorizations), and may (pre-)configure/trigger MWAB-gNB to turn on/shut down accordingly.

Proposal 5: In case WAB-MT becomes not-authorized, WAB-gNB will be turned into not-authorized by OAM accordingly, and the UEs served by the WAB-gNB can either be handed over to other RAN nodes or they can be released, after which the NG and Xn connection(s) of the WAB-gNB can be removed.

PCI collision avoidance

For mobile IAB deployments, the legacy mechanisms can be reused for PCI collision detection. The PCI space can be partitioned between mobile IAB cells and stationary cells by implementation. This can be also reused for WAB deployments.

Proposal 6: For WAB, the legacy mechanisms can be reused for PCI collision detection. The PCI space can be partitioned between WAB cells and stationary cells by implementation.

In addition, in case CU-DU split, if the gNB-CU detects PCI conflict, the gNB-CU may select a new PCI value from the preconfigured PCI list for the NR cell and send it to the gNB-DU by either F1 Setup procedure or gNB-CU configuration update procedure. However, CU-DU split architecture is out-of-scope of R19 WAB, and there is no F1 interface between BH RAN node and the WAB-node. Then PCI reconfiguration via F1AP for mobile IAB-node is not appliable for WAB node.

Proposal 7: Due to CU-DU split is out-of-scope of R19 WAB, PCI reconfiguration via F1AP for mobile IAB-node is not appliable for WAB node.

Radio resource coordination

WAB network in R19 only supports one-hop backhauling, where the WAB-node is directly connected to the BH RAN node. In this WAB topology, the downlink transmission from WAB-gNB to the served UEs is scheduled by the WAB-gNB itself, and the uplink transmission from WAB-MT to the BH RAN node is scheduled by the BH RAN node.

In a centralized resource allocation to avoid inter-cell or inter-node resource interference, the BH RAN node should provide some semi-static reserved DL/UL resources for each served WAB-nodes, and how to schedule on these semi-static resources is left to the implementation of WAB-node’s scheduling algorithm. However, there is no F1 interface between the BH RAN node and the WAB-node, the legacy F1AP message used for resource configuration on the IAB-node cannot be reused. Then the resource configuration may need to be delivered via the XnAP sigalling between the BH RAN node and the WAB-gNB.

According to the IAB specifications, interpreting the resources available to the IAB-DU depends on the D/U/F configuration at the IAB-DU because the L1 availability indication indicates availability of soft DL symbols, soft UL symbols, and soft flexible symbols in a slot instead of indicating availability of each symbol separately. That, in part, reduces the overhead of the L1 availability indication signaling. A similar principle can be adopted for the L1/L2 resource usage indication for WAB. As a result, the BH RAN node may require knowledge of the D/U/F configuration at the WAB-gNB. For this purpose, the WAB-gNB can use Xn signaling to inform the BH RAN node of its own D/U/F resource configurations.

Proposal 8: The WAB-gNB can signal its D/U/F configuration to the BH RAN node over Xn.

As for the resource configuration on DU of the BH RAN node, legacy F1AP GNB-DU RESOURCE CONFIGURATION can be reused, which includes the resource configuration (D/U/F/NA/H/S) on the gNB-DU of the BH RAN node, and it further includes the resource configuration and the multiplexing info of the WAB-gNB, which is helpful for the gNB-DU of the BH RAN node to avoid scheduling the WAB-MT in the slot/symbol with hard resources in the collocated WAB-gNB.

Proposal 9: Legacy F1AP GNB-DU RESOURCE CONFIGURATION can be reused for resource configuration on gNB-DU of the BH RAN node.

Backhaul traffic mapping

After the WAB-MT setup with 5GC, at least one PDU session needs to be established for transporting the signalling related to NG setup. While for the NG-U traffic for the served UEs, all the NG-U traffic may be mapped to the same PDU session for the NG-C signaling. Alternatively, additional PDU sessions can be established to transport the NG-U and different PDU sessions are used to the NG-U traffic with different QoS requirements. In addition, different types of control plane signalling (e.g., non-UE associated signalling and UE associated signalling) may be mapped to different PDU sessions.

Proposal 10: NG-C signalling and NG-U traffic can be mapped to the same PDU session between WAB-MT and MT’s UPF, alternatively, different types of NG-C signalling and NG-U traffic with different QoS requirements can be mapped to different PDU sessions.

# Conclusion

This contribution aims to discuss WAB architecture and configuration. And following observations and proposals are concluded.

Proposal 1: For Xn connection between WAB-gNB and its BH RAN node, it's the WAB-gNB to trigger the Xn setup procedure.

Proposal 2: For Xn connection between WAB-gNB and surrounding RAN node, both the WAB-gNB and the surrounding RAN node can trigger the Xn setup procedure.

Proposal 3: WAB-gNB can obtain the type of another WAB-gNB and then avoid to setup Xn interface to another WAB-gNB. Or the WAB-gNB can include the type information in the Xn setup request message.

Proposal 4: WAB-gNB broadcasts a WAB indication in SIB1 to avoid other WAB-MT to camp on the cell of WAB-gNB. And in case of handover, a new WAB-node indication is included in the handover request for access control at the target BH RAN node.

Proposal 5: In case WAB-MT becomes not-authorized, WAB-gNB will be turned into not-authorized by OAM accordingly, and the UEs served by the WAB-gNB can either be handed over to other RAN nodes or they can be released, after which the NG and Xn connection(s) of the WAB-gNB can be removed.

Proposal 6: For WAB, the legacy mechanisms can be reused for PCI collision detection. The PCI space can be partitioned between WAB cells and stationary cells by implementation.

Proposal 7: Due to CU-DU split is out-of-scope of R19 WAB, PCI reconfiguration via F1AP for mobile IAB-node is not appliable for WAB node.

Proposal 8: The WAB-gNB can signal its D/U/F configuration to the BH RAN node over Xn.

Proposal 9: Legacy F1AP GNB-DU RESOURCE CONFIGURATION can be reused for resource configuration on gNB-DU of the BH RAN node.

Proposal 10: NG-C signalling and NG-U traffic can be mapped to the same PDU session between WAB-MT and MT’s UPF, alternatively, different types of NG-C signalling and NG-U traffic with different QoS requirements can be mapped to different PDU sessions.

# Reference

- RP-242395. New WID on additional topological enhancements for NR.

- R3-244019. LS on questions regarding FS_VMR_Ph2. SA2.