3GPP TSG-RAN WG3 Meeting #126	R3-247430

Orlando, USA, 18th – 22th November, 2024

Agenda Item:	12.3

Source: 	Lenovo

Title: 	Architecture and access control for NR Femto

Document for:	Discussion and decision

- Introduction

A new WID for additional topological enhancements [1] has been approved to be discussed in Rel-19, and following detail objectives are identified for 5G Femto fucus on RAN3.

The objectives of the 5G Femto work are as follows: - Specification to support NR Femto architecture with optional NR Femto GW for NG interface [RAN3]. - Specification to support access control for NR Femtos operating in open, hybrid and closed modes reusing existing CAG functionality [RAN3]. NOTE 10: For NR Femto access control, only stage 2 impact is expected on this objective. NOTE 11: Coordination with other WGs (e.g. SA2, SA3) when needed.

In this contribution, the architecture for NR Femto with GW for NG interface, access control and handover mechanisms for NR Femto are discussed.

- Discussion

In last meeting, following WA had been achieved for supported cell number of NR Femto.

WA: an NR Femto may have more than one cell.

For LTE, the ECGI only has 28 bits to globally identify a 4G cell, and it also need to use 28 bits to identify a Home eNB, then a Home eNB only support one cell, otherwise, the length of ECGI needs to be extended to support multi-cell of Home eNB.

9.2.1.37 Global eNB ID This information element is used to globally identify an eNB (see TS 36.401 [2]). IE/Group Name; Presence; Range; IE type and reference; Semantics description. PLMN Identity; M; 9.2.3.8. CHOICE eNB ID; M. >Macro eNB ID. >>Macro eNB ID; M; BIT STRING (SIZE(20)); Equal to the 20 leftmost bits of the Cell Identity IE contained in the E-UTRAN CGI IE (see subclause 9.2.1.38) of each cell served by the eNB.. >Home eNB ID. >>Home eNB ID; M; BIT STRING (SIZE(28)); Equal to the Cell Identity IE contained in the E-UTRAN CGI IE (see subclause 9.2.1.38) of the cell served by the eNB.. >Short Macro eNB ID. >> Short Macro eNB ID; M; BIT STRING (SIZE(18)); Equal to the 18 leftmost bits of the Cell Identity IE (see subclause 9.2.1.38) of each cell served by the eNB.. >Long Macro eNB ID. >> Long Macro eNB ID; M; BIT STRING (SIZE(21)); Equal to the 21 leftmost bits of the Cell Identity IE (see subclause 9.2.1.38) of each cell served by the eNB.. 9.2.1.38 E-UTRAN CGI This information element is used to globally identify a cell (see TS 36.401 [2]). IE/Group Name; Presence; Range; IE type and reference; Semantics description. PLMN Identity; M; 9.2.3.8. Cell Identity; M; BIT STRING (SIZE(28)); The leftmost bits of the Cell Identity correspond to the eNB ID (defined in subclause 9.2.1.37)..

However, in NR, NCGI has 36 bits. Then an NR Femto node can support more than one cell in case we still use 28 bits to identify an NR Femto node.

9.3.1.6 Global gNB ID This IE is used to globally identify a gNB (see TS 38.300 [8]). IE/Group Name; Presence; Range; IE type and reference; Semantics description. PLMN Identity; M; 9.3.3.5. CHOICE gNB ID; M. >gNB ID. >>gNB ID; M; BIT STRING (SIZE(22..32)); Equal to the leftmost bits of the NR Cell Identity IE contained in the NR CGI IE of each cell served by the gNB.. 9.3.1.7 NR CGI This IE is used to globally identify an NR cell (see TS 38.300 [8]). IE/Group Name; Presence; Range; IE type and reference; Semantics description. PLMN Identity; M; 9.3.3.5. NR Cell Identity; M; BIT STRING (SIZE(36)); The leftmost bits of the NR Cell Identity IE correspond to the gNB ID (defined in subclause 9.3.1.6)..

In addition, based on the operator’s requirements raised in last meeting, NR Femto needs to support multiple cells for certain frequency bands. In this meeting, we can turn the WA into agreement, and then discuss how to identify the cell of NR Femto.

Proposal 1: An NR Femto node may have more than one cell.

In LTE, because the Macro eNB ID only support at most 21 bits, and it needs 28 bits to identify a Home eNB, then separate eNB ID IEs were used for Macro eNB and Home eNB. As in NR, the gNB ID has been extended to at most 32 bits, if NR Femto node also uses 28 bits as the Home eNB, the NR Femto at most occupies 1/16 of the space of gNB ID. Then the global gNB ID can be reused directly to identify the NR Femto node.

Proposal 2: The global gNB ID can be reused directly to identify the NR Femto node.

The NR Femto GW appears to the NR Femto node as an AMF, and in legacy, NG-RAN node supports connection to multiple AMFs in the 5GS architecture. However, to simplify the complexity of the NR Femto node and reuse LTE HeNB as much as possible, a NR Femto node shall only connect to a single NR Femto GW at one time, and will not simultaneously connect to another NR Femto GW or AMF, and Xn interface shall not be established between the NR Femto GW and other nodes.

Proposal 3: A NR Femto node shall only connect to a single NR Femto GW at one time, and will not simultaneously connect to another NR Femto GW or AMF.

Proposal 4: Not support Xn interface between NR Femto GW and other nodes.

As specified in TS 38.410, a NAS node selection function is located in the NG-RAN node to determine the AMF association of the UE. However, as for LTE HeNB, selection of an MME at UE attachment is hosted by the HeNB GW instead of the HeNB. In case of NR Femto node, because NR Femto node is a simplified node and may only connect to a single NR Femto GW at one time, then the NNSF function can be located at the NR Femto GW.

Proposal 5: NNSF function is located at the NR Femto GW.

As concluded in TR 38.799, NR Femto node can support operating in open, hybrid and closed modes reusing existing CAG functionality.

For the access control, the open, hybrid and closed access mode can be supported as specified in the TR 38.799.

- With the existing CAG mechanism, the open, hybrid and closed access mode, can be supported as follows: - To support the open access mode: The NR Femto node activates a PLMN cell, which can be accessed by legacy UE without access control of CAG. - To support the hybrid access mode: The NR Femto activates a cell shared by both PLMN and CAG, through broadcasting both the plmn-IdentityInfoList and the npn-IdentityInfoList-r16 in the SIB1, but without the cellReservedForOtherUse. Then, this cell is accessible to UEs which have the allowed CAG list including a CAG-ID broadcasted by the cell. For the legacy UE not supporting CAG, this cell is viewed as a normal PLMN cell. - To support the closed access mode: The NR Femto node activates an NPN-only cell by broadcasting the cellReservedForOtherUse IE with value “true”, then this cell can only be accessed by the UEs whose allowed CAG list includes a CAG-ID broadcasted by the cell.

While for mobility of NR Femto, following Xn-based handover scenarios can be supported as LTE HeNB.

-  gNB or any NR Femto node -> open access NR Femto node

-  gNB or any NR Femto node -> hybrid access NR Femto node

-  Hybrid access NR Femto node or closed access NR Femto node -> closed access NR Femto node

-  Any NR Femto node -> gNB

Proposal 6: Support following Xn-based handover scenarios for NR Femto.

-  gNB or any NR Femto node -> open access NR Femto node

-  gNB or any NR Femto node -> hybrid access NR Femto node

-  Hybrid access NR Femto node or closed access NR Femto node -> closed access NR Femto node

-  Any NR Femto node -> gNB

However, for the UE using CAG, it can be configured an Allowed CAG list and optional a CAG-only indication, which indicates whether the UE is only allowed to access 5GS via CAG cells. Then for the UE handover of NR Femto, the handover cannot be supported if the target cell is a non-CAG cell.

Proposal 7: For the UE handover of NR Femto, the handover cannot be supported if the target cell is a non-CAG cell.

In addition, as in LTE HeNB, a proximity indication was introduced for mobility of hybrid cell. However, only stage 2 impact is expected on this objective. Then, we suggest to not introduce the proximity indication for NR Femto in this release.

Proposal 8: Not introduce the proximity indication for NR Femto.

# Conclusion

This contribution aims to discuss architecture, access control and handover mechanisms for NR Femto. And following observations and proposals are concluded.

Proposal 1: An NR Femto node may have more than one cell.

Proposal 2: The global gNB ID can be reused directly to identify the NR Femto node.

Proposal 3: A NR Femto node shall only connect to a single NR Femto GW at one time, and will not simultaneously connect to another NR Femto GW or AMF.

Proposal 4: Not support Xn interface between NR Femto GW and other nodes.

Proposal 5: NNSF function is located at the NR Femto GW.

Proposal 6: Support following Xn-based handover scenarios for NR Femto.

-  gNB or any NR Femto node -> open access NR Femto node

-  gNB or any NR Femto node -> hybrid access NR Femto node

-  Hybrid access NR Femto node or closed access NR Femto node -> closed access NR Femto node

-  Any NR Femto node -> gNB

Proposal 7: For the UE handover of NR Femto, the handover cannot be supported if the target cell is a non-CAG cell.

Proposal 8: Not introduce the proximity indication for NR Femto.

# Reference

- RP-242395. New WID on additional topological enhancements for NR.

- TR 38.799. Study on additional topological enhancements for NR (Release 19).