3GPP TSG-RAN WG3 #126	R3-247447

Orlando, USA, 18th – 22nd Nov 2024

Agenda Item:	20.2

Source:	Lenovo

Title: 		AIML for gNB assisted positioning

Document for:	Discussion and Approval

# Introduction

In the last RAN3 meeting, the following agreements were made. In this paper, we further discuss the rest issues.

Case 3b:

Re-using the Measurement procedure defined in NRPPa to support requesting and reporting the related measurements to LMF for Case 3b. The details of the related measurements are still pending and subject to further discussion in RAN1.

Case 3a:

The AI/ML model (inference and training functions) is at the gNB.

Model Training: Label information as training data from LMF to gNB? Data collection from UE or TRP or from LMF?

Model performance monitoring: OptionA or OptionB?

Option A. gNB performs monitoring metric calculation for its own model.

Option B. LMF performs monitoring metric calculation for the model located at the gNB.

The existing LCS framework is re-used for AI/ML positioning.

Case 3a agreements:

- LMF provides ground truth label and related data (i.e., part B information as called by RAN1) to NG-RAN.

FFS on the content of the ground truth label and related data pending on RAN1 progress. FFS on the signalling design.

- How the LMF obtains the ground truth label and related data information is outside of RAN3 scope.

- NG-RAN obtains measurements from TRP (i.e., part A information as called by RAN1).

The details of the measurements are still pending and subject to further discussion in RAN1.

- WA: RAN3 will proceed with Option A: NG-RAN node performs monitoring metric calculation for its own model.

For case 3b:

- NG-RAN gets measurements (i.e., part A information as called by RAN1) from TRP and provides them to LMF.

The details of the measurements are still pending and subject to further discussion in RAN1.

# Discussion

## Case 3a: AIML for NG-RAN node assisted positioning with gNB-side model

RAN1 agreements relevant to Case 3a: Agreement For AI/ML assisted positioning Case 3a, at least LOS/NLOS indicator and/or timing information are supported for reporting. - If LOS/NLOS indicator is reported, the indicator can be reported as soft indicator or hard indicator as defined in 38.214. - If timing information is reported, the timing information at least can be reported via UL RTOA or gNB Rx-Tx time difference as defined in 38.215. - Note: details of the report are pending further discussion. Agreement For training data generation of AI/ML based positioning Case 3a and 3b, the measurement and its related data (e.g., timestamp) are generated by TRP/gNB. Agreement For training data generation of AI/ML based positioning Case 3a, the label and its related data (e.g., time stamp) can be generated by at least: - LMF Note: transfer of label and its related data is out of RAN1 scope. Note: whether other network entities can generate label for Case 3a is out of RAN1 scope. Agreement For AI/ML positioning Case 3a, for model performance monitoring metric calculation in label-based model monitoring, study the feasibility of the following options. To provide information on how to generate information on ground truth label for each option. - Option A. NG-RAN node performs monitoring metric calculation for its own model. - Option B. LMF performs monitoring metric calculation for the model located at the NG-RAN node. Note: Final selection of Option A and Option B is out of RAN1 scope, but RAN1 can make recommendation about the option(s), and potential support of Option A and/or Option B is pending RAN3 confirmation. Note: Exact method to perform the monitoring metric calculation is up to implementation Agreement For AI/ML positioning Case 3a, for performance monitoring metric calculation in label-based monitoring, from RAN1 perspective, Option A and Option B are feasible, - Option A. NG-RAN node performs monitoring metric calculation for its own model. - Option B. LMF performs monitoring metric calculation for the model located at the NG-RAN node. Note: Final selection of Option A and Option B is out of RAN1 scope. Potential support of Option A and/or Option B is pending RAN3 confirmation. Note: Exact method to perform monitoring metric calculation is up to implementation. Note: For Option A, RAN1 assumes that user data privacy needs to be preserved. Conclusion From RAN1 perspective, for Case 3a measurements, - The existing procedures can be reused in terms of SRS configuration. - Note: parameter values for SRS configuration can be further discussed - These measurements can be used for multiple aspects related to case 3a, e.g. training data collection, monitoring, or inference procedures. - Note: Purpose, such as the training data collection, monitoring, or inference procedures mentioned above, will not necessarily be specified in RAN 1 specifications Agreement From RAN1 perspective, when timing information is reported for Rel-19 AI/ML positioning Case 3a, at least the following are mandatorily or optionally supported in a measurement report from gNB to LMF: - (Mandatory) timing information; - (Optional) Quality of the timing information; - Existing IE “Timing Measurement Quality” can be reused. - (Mandatory) Time stamp. FFS: LOS/NLOS indicator. Note: The final decision of “mandatory” or “optional” presence of each field is up to RAN3.

Based on the RAN1 agreements above, we have identified the following aspects that would require RAN3 discussion and has possible specification impact on NRPPa:

- Training:

  - Where is gNB-sided model trained?

  - What is the required data for training?

  - The procedure of training data collection

- Inference:

  - What is the inference output?

- Monitoring:

  - Which entity performs the monitoring?

- Activation/Deactivation decision:

  - Which entity makes the decision to activate the gNB-sided model?

In the following we try to analyse each aspect.

### Training Data Collection

LMF provides ground truth label and related data (i.e., part B information as called by RAN1) to NG-RAN.

FFS on the content of the ground truth label and related data pending on RAN1 progress. FFS on the signalling design.

In the last RAN3 meeting, it was agreed that LMF provides ground truth label and related data to NG-RAN node, while it is unclear how it can be done over NRPPa interface. In our view, considering there is no existing NRPPa procedure that can be reused for such purpose, the easiest way would be introduce a new NRPPa procedure for gNB to request the ground truth label and related data to NG-RAN node.

In addition, the message requesting ground truth data sent from gNB to LMF should be a non-UE associated message, as it doesn’t make sense for gNB to requesting training data in a per UE manner.

For Case 3a, RAN3 introduces new non-UE associated NRPPa procedure for gNB to request and receive the training data from LMF.

### Inference

What is the inference output?

According to the RAN1 agreements, at least LOS/NLOS indicator and/or timing information are supported for reporting. RAN3 can simply confirm.

For Case 3a, RAN3 assumes the gNB-sided model will output at least LOS/NLOS indicator and/or timing information until RAN1’s further update.

### 2.1.3	Monitoring

Which entity performs the monitoring?

Based on RAN1 discussion, there are two options to perform monitoring:

- Option A.	NG-RAN node performs monitoring metric calculation for its own model.

- Option B.	LMF performs monitoring metric calculation for the model located at the NG-RAN node.

WA: RAN3 will proceed with Option A: NG-RAN node performs monitoring metric calculation for its own model.

In our view, Option A and Option B are not mutual exclusive thus both can be supported. For example, when a UE is moving, the measured SRS by gNB may or may not be suitable to produce an accurate enough NLOS/LOS determination due to the applicability of the AI model, gNB may stop using the AI based method. In another example, LMF may be aware of the ground truth of the PRU/UE location or NLOS/LOS status, LMF can simply compare the inference output (NLOS/LOS indicator) or the estimated UE location based on the inference output with the ground truth to understand if the AIML model of the gNB is performing well.

For Case 3a, RAN3 is suggested to support both gNB based monitoring and LMF based monitoring.

### 2.1.4	Activation/Deactivation decision

Which entity makes the decision to activate the gNB-sided model?

For activating a gNB-sided model, one needs to know in which scenario the trained AI model will be applicable. This is by nature known by gNB itself, but difficult to transfer the meta data about the gNB-sided AI model to LMF. Therefore, it should be upon gNB’s own decision if an AIML model will be activated and used to improve the positioning accuracy for case 3a.

For Case 3a, it is upon gNB’s decision if an AIML based method will be used to improve the positioning accuracy.

For deactivating a gNB-sided model, since LMF is the final consumer of the AIML based estimation of LOS/NLOS or timing information, and it is LMF that determines the UE location based on the AIML generated result provided by the gNB, we believe LMF should be able to at least telling the gNB to stop using AIML based method if the accuracy of UE location estimation degrades, based on LMF’s monitoring.

For Case 3a, LMF can instruct the gNB to stop using AIML based method, e.g., if the location accuracy determined by LMF degrades.

## Case 3b: AIML for NG-RAN node assisted positioning with LMF-side model

RAN1 agreements relevant to Case 3b: Agreement For AI/ML based positioning case 3b, at least the following types of time domain channel measurements are supported for reporting: - timing information; - paired timing information and power information. Agreement For AI/ML based positioning Case 3b, for gNB channel measurements reported to LMF, the timing information is represented relative to a reference time. - FFS: Whether any specification impact of the reference time used to represent the timing information. Details of the reference time Agreement For AI/ML based positioning Case 3b, for gNB channel measurements reported to LMF, the timing information is represented relative to the existing UL RTOA reference time T0+tSRS as defined in TS 38.215. FFS: whether it is applicable when Case 3b is used to support multi-RTT Working Assumption For training data generation of AI/ML based positioning Case 3b, the label and its related data (e.g., time stamp) can be generated by: - PRU - FFS: Non-PRU UE with estimated location - LMF Note: transfer of label and its related data is out of RAN1 scope. Working Assumption For training data generation of AI/ML based positioning Case 3b, the label and its related data (e.g., time stamp) can be generated by: - PRU - Non-PRU UE with estimated location - LMF Note: transfer of label and its related data is out of RAN1 scope. Note: It is assumed that user data privacy of non-PRU UE is preserved. Note: Previous related working assumption made in RAN1#116bis for training data generation of AI/ML based positioning Case 3b will not need to be confirmed. Agreement For training data collection of AI/ML based positioning case 3b, for time stamp of channel measurement, - For channel measurement generated by TRP/gNB, existing IE “Time Stamp” in TS 38.455 can be reused from RAN1 perspective - Note: Purpose, such as above “training data collection", will not necessarily be specified in RAN 1 specifications. Agreement For AI/ML positioning Case 2b and 3b, regarding the power information for determining the model input, - For downlink power measurement, use DL PRS-RSRPP defined in TS 38.215 as a starting point. - For measurement report of DL PRS-RSRPP, use the existing measurement report mapping table for PRS-RSRPP in 38.133 as a starting point. - For uplink power measurement, use UL SRS-RSRPP defined in TS 38.215 as a starting point. - For measurement report of UL SRS-RSRPP, use the existing measurement report mapping table for SRS-RSRPP in 38.133 as a starting point. Agreement For Rel-19 AI/ML based positioning Case 3b, regarding sample-based measurement (if supported), from RAN1 perspective, - LMF can signal parameter values of Nt, Nt', k to gNB via NRPPa. Agreement From RAN1 perspective, for model inference of AI/ML positioning Case 3b, at least the following are mandatorily or optionally supported in a measurement report from gNB to LMF: - (Mandatory) Channel measurement; - (Optional) Quality of the channel measurement; - FFS: details of the quality - (Mandatory) Time stamp of the channel measurement.

According to RAN1 progress so far, for case 3b, the only RAN3 impact would be any enhancement to support new measurement exchange between gNB and LMF. For example,

For sample-based measurement (if supported), LMF can signal parameter values of Nt, Nt', k to gNB via NRPPa.

When gNB reports the channel measurement result to LMF, gNB may also report timing information and paired timing information and power information.

For Case 3b, based on RAN1 discussion, RAN3 supports exchanging the following information between gNB and LMF over NRPPa:

  - From LMF to gNB, if sample-based measurement is supported, LMF can signal parameter values of Nt, Nt', k to gNB

  - From gNB to LMF, gNB will report (mandatory) channel measurement, (optional) quality of the channel measurement, and (mandatory) time stamp of the channel measurement.

# Conclusion

Based on the discussion above, we observe:

Based on the discussion above, we propose:

Proposal 1	For Case 3a, RAN3 introduces new non-UE associated NRPPa procedure for gNB to request and receive the training data from LMF.

Proposal 2	For Case 3a, RAN3 assumes the gNB-sided model will output at least LOS/NLOS indicator and/or timing information until RAN1’s further update.

Proposal 3	For Case 3a, RAN3 is suggested to support both gNB based monitoring and LMF based monitoring.

Proposal 4	For Case 3a, it is upon gNB’s decision if an AIML based method will be used to improve the positioning accuracy.

Proposal 5	For Case 3a, LMF can instruct the gNB to stop using AIML based method, e.g., if the location accuracy determined by LMF degrades.

Proposal 6	For Case 3b, based on RAN1 discussion, RAN3 supports exchanging the following information between gNB and LMF over NRPPa:

a.	From LMF to gNB, if sample-based measurement is supported, LMF can signal parameter values of Nt, Nt', k to gNB

b.	From gNB to LMF, gNB will report (mandatory) channel measurement, (optional) quality of the channel measurement, and (mandatory) time stamp of the channel measurement.