3GPP TSG-RAN WG3 #126		                                       R3-247467

Orlando, US, 18th – 22nd Nov, 2024

Source:	CATT

Title:	(TP on 38.423)Support of AI/ML enabled Slicing

Agenda Item:	11.2

Document for:	Discussion and Decision

Introduction

In last RAN3 meeting, the following agreements have been achieved:

Enhance the Data Collection Reporting Initiation procedure and Data Collection Reporting procedure to enable requesting and reporting of the predicted radio resource status per slice and predicted slice available capacity over the Xn interface.

WA: UE performance per each PDU session associated to a requested S-NSSAI is reported from target node to source node.

Introduce Slice To Report List for Data Collection IE including PLMN ID and S-NSSAI in DATA COLLECTION REQUEST message.

The FFS is captured below:

FFS if other finer granularity of UE performance is feasible.

The failure of the predicted radio resource status per slice and predicted slice available capacity should be indicated to the sending node? Per cell per slice?

In the document, we will further provide our analysis for AI/ML enabled Slicing.

Discussion

The situation for Predicted Slice Available Capacity and Predicted radio resource status per slice differs in terms of support in Rel-18. Specifically, predicted cell capacity is not supported in Rel-18 and is not defined in the Report Characteristics for Data Collection IE. Therefore, it is logical to define the tenth bit as an indication for Predicted Slice Available Capacity request. The source node can indicate the slices it is interested in via the Slice To Report List IE in the DATA COLLECTION REQUEST message. Additionally, in the Data Collection Update message, the Predicted Slice Available Capacity IE should be introduced to support the reporting of these measurements.

Proposal 1: It is proposed to define the tenth bit of Characteristics for Data Collection IE within Data Collection Request message as the indication of Predicted Slice Available Capacity request and introduce Predicted Slice Available Capacity IE in DATA COLLECTION UPDATE message.

As to collecting Predicted radio resource status per slice between gNBs, the measurement was already supported in Rel-18 whereas the granularity is per SSB level without slicing level supported. Looking at the structure of Predicted Radio Resource Status IE, per slice level is already supported as an optional IE. So, for the report of slicing level predicted radio resource status, Predicted Radio Resource Status IE could be reused.

Proposal 2: It is proposed to reuse Predicted Radio Resource Status IE in DATA COLLECTION UPDATE message to support Collecting Predicted radio resource status per slice between gNBs.

For the measurement request, since the metric Predicted Radio Resource Status is already defined in Characteristics for Data Collection IE within Data Collection Request message, a feasible solution is using the presence of aforementioned new introduced Slice To Report List IE alongside the presence of Predicted Radio Resource Status to request slicing level predicted Resource Status. However, this option presents two scenarios as below that cannot be distinguished at the target node.

Scenario 1: The source node request both per slicing level predicted area resource status and Predicted Slice Available Capacity. In this scenario, following the option above, the source node sets both the first bit and the tenth bit to 1, and includes the Slice To Report List IE in Data Collection Request message.

Scenario 2: The source node request SSB level predicted area resource status without slicing information considered but also requests Predicted Slice Available Capacity. If this scenario exists, the source node includes the same information as scenario 1.

If it is an unreasonable scenario to request per slicing level capacity while only request per SSB level resource status without Slicing information considered, then it suffices to use the combination of the first bit in Characteristics for Data Collection IE and Slice To Report List IE to indicate the request for per slicing Predicted Radio Resource Status. Otherwise, another bit needs to be defined in Characteristics for Data Collection IE to request per slicing level Radio Resource Status.

Proposal 3: It is proposed for RAN3 to discuss whether it is needed to introduce one more bit to indicate the requirement of per slice predicted Area Radio Resource Status besides current “Predicted Radio Resource Status" bit in Report Characteristics for Data Collection IE.

For the following FFS captured in last meeting:

The failure of the predicted radio resource status per slice and predicted slice available capacity should be indicated to the sending node? Per cell per slice?

Currently, only per cell failure case is supported in DATA COLLECTION RESPONSE message. For the scenario that slice level failure is involved, there may arise a situation where upon receiving request message, insufficient training data is not available for a specific slice, rendering the target node unable to make prediction. To address this, Measurement Temporarily not Available cause value which has been defined could be reused for above case to indicate per cell per slice level failure.

Proposal 4: It is proposed to support the indication of per slice level failure in DATA COLLECTION RESPONSE message.

To support AI/ML based slicing optimization, there are two slice related measurements introduced: Predicted Slice Available Capacity and Predicted radio resource status per slice. Accordingly, another issue on per cell per slice failure is whether it is needed to introduce slice measurement failed report characteristics to indicate the specific failed measurement. From our point of view, it is necessary to also indicate the failed metric on top of the slicing information to support the scenario that only part of the prediction failed in one slice.

Here is an example:

In the DATA COLLECTION REQUEST message, the following information is included:

| Cell ID | Slice ID | Report Characteristics |
| --- | --- | --- |
| A | 1,2 | Predicted Slice Available Capacity Predicted radio resource status per slice |
| B | 1,3 | Predicted radio resource status per slice |

In the DATA COLLECTION RESPONSE message, to indicate the capability of prediction of different cells and different slices, the following information could be included with the introduction of Slice Measurement Failed Report Characteristics IE:

| Cell ID | Cell Measurement Failed Report Characteristics | Slice ID | Slice Measurement Failed Report Characteristics | description |
| --- | --- | --- | --- | --- |
| Cell A | Not present | Slice 1 | Predicted Slice Available Capacity | For slice 1, Predicted Slice Available Capacity is not initiated. For slice 1, Predicted radio resource status per slice is initiated. |
|  |  | Slice 2 | Not present | For slice 2, both Predicted Slice Available Capacity and Predicted radio resource status per slice are initiated. |
| Cell B | Predicted radio resource status per slice | X |  | For cell B, Predicted radio resource status per slice (including both slice 1and 3) is not initiated. |

Proposal 5: It is proposed to introduce Slice Measurement Failed Report Characteristics IE in DATA COLLECTION RESPONSE message to indicate the metrics for which the per slice level prediction failed.

Conclusions

Based on the discussion in section 2 the followings are proposed:

Proposal 1: It is proposed to define the tenth bit of Characteristics for Data Collection IE within Data Collection Request message as the indication of Predicted Slice Available Capacity request and introduce Predicted Slice Available Capacity IE in DATA COLLECTION UPDATE message.

Proposal 2: It is proposed to reuse Predicted Radio Resource Status IE in DATA COLLECTION UPDATE message to support Collecting Predicted radio resource status per slice between gNBs.

Proposal 3: It is proposed for RAN3 to discuss whether it is needed to introduce one more bit to indicate the requirement of per slice predicted Area Radio Resource Status besides current “Predicted Radio Resource Status" bit in Report Characteristics for Data Collection IE.

Proposal 4: It is proposed to support the indication of per slice level failure in DATA COLLECTION RESPONSE message.

Proposal 5: It is proposed to introduce Slice Measurement Failed Report Characteristics IE in DATA COLLECTION RESPONSE message to indicate the metrics for which the per slice level prediction failed.

TP for TS38.423

- 8.4.13	Data Collection Reporting Initiation

- 8.4.13.1	General

This procedure is used by an NG-RAN node to request from another NG-RAN node the reporting of information to support, e.g., AI/ML in NG-RAN.

The procedure uses non UE-associated signalling.

- 8.4.13.2	Successful Operation

Figure 8.4.13.2-1: Data Collection Reporting Initiation, successful operation

NG-RAN node1 initiates the procedure by sending the DATA COLLECTION REQUEST message to NG-RAN node2 to start information reporting or to stop information reporting. Upon receipt, NG-RAN node2:

-	shall initiate the requested information reporting according to the parameters given in the request in case the Registration Request for Data Collection IE is set to "start"; or

-	shall stop all measurements and predictions and terminate the reporting in case the Registration Request for Data Collection IE is set to "stop".

If the Registration Request for Data Collection IE is set to "start" in the DATA COLLECTION REQUEST message and the Report Characteristics for Data Collection IE indicates cell-specific information reporting, the Cell To Report List for Data Collection IE shall be included.

If NG-RAN node2 is capable of providing all of the requested information, it shall initiate the information reporting as requested by NG-RAN node1 and respond with the DATA COLLECTION RESPONSE message.

If NG-RAN node2 is capable of providing some but not all of the requested information, it shall initiate the information reporting for the admitted requested information and include the Node Measurement Initiation Result List IE or the Cell Measurement Initiation Result List IE or both in the DATA COLLECTION RESPONSE message.

If the Reporting Periodicity for Data Collection IE in the DATA COLLECTION REQUEST message is present, this indicates the periodicity for the reporting of configured measurement objects. The NG-RAN node2 shall report only once, unless otherwise requested within the Reporting Periodicity for Data Collection IE.

If the Requested Prediction Time IE in the DATA COLLECTION REQUEST message is present, it indicates the specific point in time to which the prediction of the requested information applies. The NG-RAN node2 shall take it into account when generating the requested predicted information.

If the UE Trajectory Collection Configuration IE is present in the DATA COLLECTION REQUEST message, the NG-RAN node2 shall take it into account for the configuration of UE trajectory collection and reporting. NG-RAN node2 shall report the UE trajectory only once. NG-RAN node2 shall terminate the collection when at least one of the following conditions is fulfilled:

-	the time since UE was successfully handed over to NG-RAN node2 is equal to the value of the Collection Time Duration for UE Trajectory IE;

-	the number of visited cells within NG-RAN node2 is equal to the value of the Number of Visited Cells IE, if included;

-	UE moves to RRC_INACTIVE or RRC_IDLE state;

-	UE is handed over to a cell belonging to an NG-RAN node different from NG-RAN node2.

The result of the UE trajectory collection is reported at the next available DATA COLLECTION UPDATE message.

If the UE Performance Collection Configuration IE is present in the DATA COLLECTION REQUEST message, the NG-RAN node2 shall take it into account for the configuration of UE performance collection and reporting. NG-RAN node2 shall terminate the collection when at least one of the following conditions is fulfilled:

-	the time since UE was successfully handed over to NG-RAN node2 is equal to the value of the Collection Time Duration for UE Performance IE;

-	UE moves to RRC_INACTIVE or RRC_IDLE state;

-	UE is handed over to another cell.

The result of the UE performance collection is reported at the next available DATA COLLECTION UPDATE message.

Interaction with the Data Collection Reporting procedure

When starting a measurement, the Report Characteristics for Data Collection IE in the DATA COLLECTION REQUEST message indicates the type of objects NG-RAN node2 performs measurements or predictions on. NG-RAN node2 shall include in the DATA COLLECTION UPDATE message:

-	the SSB Area Radio Resource Status List IE, excluding the DL scheduling PDCCH CCE usage IE and UL scheduling PDCCH CCE usage IE, included in the Predicted Radio Resource Status IE, if the first bit, "Predicted Radio Resource Status" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by NG-RAN node2.

-	the Slice Radio Resource Status List IE, included in the Predicted Radio Resource Status IE, if the first bit, "Predicted Radio Resource Status" of the Report Characteristics for Data Collection IE is set to "1" and Slice To Report List IE is present in the DATA COLLECTION REQUEST message, and if the measurement object is admitted by NG-RAN node2.

-	the Predicted Number of Active UEs IE, if the second bit, "Predicted Number of Active UEs" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by NG-RAN node2.

-	the Predicted RRC Connections IE, if the third bit, "Predicted RRC Connections" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by NG-RAN node2.

-	the Average UE Throughput DL IE, if the fourth bit, "Average UE Throughput DL" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by NG-RAN node2.

-	the Average UE Throughput UL IE, if the fifth bit, "Average UE Throughput UL" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by NG-RAN node2.

-	the Average Packet Delay IE, if the sixth bit, "Average Packet Delay" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by NG-RAN node2.

-	the Average Packet Loss DL IE, if the seventh bit, "Average Packet Loss DL" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by NG-RAN node2.

-	the Energy Cost IE, if the eighth bit, "Energy Cost" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by NG-RAN node2.

-	the Measured UE Trajectory IE, if the ninth bit, "Measured UE Trajectory" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by NG-RAN node2.

-	the Predicted Slice Available Capacity IE, if the tenth bit, " Predicted Slice Available Capacity" of the Report Characteristics for Data Collection IE is set to "1" and Slice To Report List IE is present in the DATA COLLECTION REQUEST message and if the measurement object is admitted by NG-RAN node2.

- 8.4.13.3	Unsuccessful Operation

Figure 8.4.13.3-1: Data Collection Reporting Initiation, unsuccessful operation

If none of the requested information can be initiated, NG-RAN node2 shall send the DATA COLLECTION FAILURE message with an appropriate cause value.

- 8.4.13.4	Abnormal Conditions

For the same Measurement ID, if the initiating NG-RAN node1 does not receive either the DATA COLLECTION RESPONSE message or the DATA COLLECTION FAILURE message, the NG-RAN node1 may reinitiate the Data Collection Reporting Initiation procedure towards the same NG-RAN node, provided that the content of the new DATA COLLECTION REQUEST message is identical to the content of the previously unacknowledged DATA COLLECTION REQUEST message.

If the NG-RAN node2 receives a DATA COLLECTION REQUEST message which includes the Registration Request for Data Collection IE set to "stop" and if the NG-RAN node2 Measurement ID value received in the DATA COLLECTION REQUEST message is not used, the NG-RAN node2 shall initiate DATA COLLECTION FAILURE message with an appropriate cause value.

If in the Report Characteristics for Data Collection IE bitmap all bits are set to "0" in the DATA COLLECTION REQUEST message, then NG-RAN node2 shall initiate a DATA COLLECTION FAILURE message with an appropriate cause value.

If the NG-RAN node2 receives a DATA COLLECTION REQUEST message which includes the Registration Request for Data Collection IE set to "start" and the NG-RAN node1 Measurement ID IE corresponding to an existing on-going Data Collection reporting, then NG-RAN node2 shall initiate a DATA COLLECTION FAILURE message with an appropriate cause value.

- 9.1.3.26	DATA COLLECTION REQUEST

This message is sent by NG-RAN node1 to NG-RAN node2 to initiate the requested information reporting according to the parameters given in the message.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node1 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID | C-ifRegistrationRequestForDataCollectionStop |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | ignore |
| Registration Request for Data Collection | M |  | ENUMERATED(start, stop, …) | Type of request for which the information is required. | YES | reject |
| Report Characteristics for Data Collection | C-ifRegistrationRequestForDataCollectionStart |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates the object the NG-RAN node2 is requested to report. First Bit = Predicted Radio Resource Status, Second Bit = Predicted Number of Active UEs, Third Bit = Predicted RRC Connections Fourth Bit = Average UE Throughput DL, Fifth Bit = Average UE Throughput UL, Sixth Bit = Average Packet Delay, Seventh Bit = Average Packet Loss DL Eighth Bit = Energy Cost Ninth Bit = Measured UE Trajectory Tenth Bit = Predicted Slice Available Capacity Other bits are ignored by the NG-RAN node2. | YES | reject |
| Cell To Report List for Data Collection |  | 0..1 |  | Cell ID list to which the request applies. | YES | ignore |
| >Cell To Report Item for Data Collection |  | 1 .. <maxnoofCellsinNG-RANnode> |  |  | – |  |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 | Indicates an NR Cell Identity. | – |  |
| >>Slice to Report List for Data Collection |  | 0..1 |  | S-NSSAI list to which the request applies. | YES | ignore |
| >>>Slice to Report Item for Data Collection |  | 1 .. < maxnoofBPLMNs > |  |  | - |  |
| >>>>PLMN Identity | M |  | 9.2.2.4 | Broadcast PLMN | - |  |
| >>>>S-NSSA List |  |  | 1 |  | - |  |
| >>>>>S-NSSA Item |  | 1 .. < maxnoofSliceItems> |  |  | - |  |
| >>>>>>S-NSSAI | M |  | 9.2.3.21 |  | - |  |
| Reporting Periodicity for Data Collection | O |  | ENUMERATED(500ms, 1000ms, 2000ms, 5000ms, 10000ms, …) | Periodicity that can be used for reporting of requested objects. Also used as the averaging window length for all objects if supported. | YES | ignore |
| Requested Prediction Time | O |  | INTEGER (1..60, ...) | For one time reporting, it indicates the point in time, measured from reception of the DATA COLLECTION REQUEST message, for which predictions are provided. In periodic reporting, for each subsequent DATA COLLECTION UPDATE message, the point in time is shifted by the reporting periodicity. (unit: second) | YES | ignore |
| UE Trajectory Collection Configuration | O |  | 9.2.3.185 |  | YES | ignore |
| UE Performance Collection Configuration | O |  | 9.2.3.186 |  | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifRegistrationRequestForDataCollectionStop | This IE shall be present if the Registration Request for Data Collection IE is set to the value “stop”. |
| ifRegistrationRequestForDataCollectionStart | This IE shall be present if the Registration Request for Data Collection IE is set to the value “start”. |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |
| maxnoofBPLMNs | Maximum no. of broadcast PLMNs by a cell. Value is 12. |
| maxnoofSliceItems | Maximum no. of signalled slice support items. Value is 1024. |

- 9.1.3.27	DATA COLLECTION RESPONSE

This message is sent by NG-RAN node2 to NG-RAN node1 to indicate that the requested information, for all or part of the measurement objects included in the reporting, is successfully initiated.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node1 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | reject |
| Node Measurement Initiation Result List |  | 0..1 |  | List of measurement objects that failed to be initiated in the node. | YES | reject |
| >Node Measurement Initiation Result Item |  | 1 .. <maxFailedMeasPerNode> |  |  | – |  |
| >>Node Measurement Failed Report Characteristics | M |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates measurement objects that failed to be initiated in the NG-RAN node2. First Bit = Energy Cost, Second Bit = Average UE Throughput DL, Third Bit = Average UE Throughput UL, Fourth Bit = Average Packet Delay, Fifth Bit = Average Packet Loss DL, Sixth Bit = Measured UE Trajectory. Other bits are ignored by the NG-RAN node1. | – |  |
| >>Cause | M |  | 9.2.3.2 | Failure cause for measurement objects for which the measurement cannot be initiated. | – |  |
| Cell Measurement Initiation Result List |  | 0..1 |  | List of measurement objects that failed to be initiated per cell. | YES | reject |
| >Cell Measurement Initiation Result Item |  | 1 .. <maxnoofCellsinNG-RANnode> |  |  | – |  |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 | Indicates an NR Cell Identity. | – |  |
| >>Cell Measurement Failure Cause List |  | 0..1 |  | Indicates that NG-RAN node2 could not initiate the measurement for at least one of the requested measurement objects in the cell. | – |  |
| >>>Cell Measurement Failure Cause Item |  | 1 .. <maxFailedCellMeasObjects> |  |  | – |  |
| >>>>Cell Measurement Failed Report Characteristics | M |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates measurement objects that failed to be initiated in the NG-RAN node2. First Bit = Predicted Radio Resource Status, Second Bit = Predicted Number of Active UEs, Third Bit = Predicted RRC Connections. Fourth Bit = Predicted Slice Available Capacity Other bits are ignored by the NG-RAN node1. | – |  |
| >>>>Cause | M |  | 9.2.3.2 | Failure cause for measurement objects for which the measurement cannot be initiated. | – |  |
| >>Slice Measurement Initiation Result List |  | 0..1 |  | List of measurement objects that failed to be initiated per cell per slice. |  |  |
| >>>Slice Measurement Initiation Result Item |  | 1 .. < maxnoofBPLMNs > |  |  |  |  |
| >>>>PLMN Identity | M |  | 9.2.2.4 | Broadcast PLMN |  |  |
| >>>>S-NSSAI List |  | 1 |  |  |  |  |
| >>>>>S-NSSAI Item |  | 1 .. < maxnoofSliceItems> |  |  |  |  |
| >>>>>>S-NSSAI | M |  | 9.2.3.21 |  |  |  |
| >>>>>>Slice Measurement Failure Cause Item |  | 1 .. <maxFailedSliceMeasObjects> |  |  |  |  |
| >>>>>>>Slice Measurement Failed Report Characteristics | M |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates measurement objects that failed to be initiated in the NG-RAN node2. First Bit = Predicted Radio Resource Status per slice, Second Bit = Predicted Slice Available Capacity, Other bits are ignored by the NG-RAN node1. |  |  |
| >>>>>>>Cause | M |  | 9.2.3.2 | Failure cause for certain slice for which the measurement cannot be initiated. |  |  |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |
| maxFailedCellMeasObjects | Maximum number of measurement objects that can fail per cell. Value is 124. |
| maxFailedMeasPerNode | Maximum number of measurement objects that can fail per node. Value is 124. |
| maxnoofBPLMNs | Maximum no. of broadcast PLMNs by a cell. Value is 12. |
| maxnoofSliceItems | Maximum no. of signalled slice support items. Value is 1024. |
| maxFailedSliceMeasObjects | Maximum number of slice objects that can fail per slice. Value is 32. |

- 9.1.3.29	DATA COLLECTION UPDATE

This message is sent by NG-RAN node2 to NG-RAN node1 to report the requested information.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| NG-RAN node1 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | reject |
| Cell Measurement Result for Data Collection List |  | 0..1 |  |  | YES | ignore |
| >Cell Info Result for Data Collection Item |  | 1 .. < maxnoofCellsinNG-RANnode > |  |  | – |  |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 | Indicates an NR Cell Identity. | – |  |
| >>Predicted Radio Resource Status | O |  | Radio Resource Status 9.2.2.50 | The IE only includes the SSB Area Radio Resource Status List IE, excluding the DL scheduling PDCCH CCE usage IE and UL scheduling PDCCH CCE usage IE. | – |  |
| >>Predicted Number of Active UEs | O |  | Number of Active UEs 9.2.2.62 |  | – |  |
| >>Predicted RRC Connections | O |  | RRC Connections 9.2.2.56 |  | – |  |
| >>Predicted Slice Available Capacity | O |  | Slice Available Capacity 9.2.2.55 |  | – |  |
| UE Associated Info Result List |  | 0..1 |  |  | YES | ignore |
| >UE Associated Info Result Item |  | 1 .. < maxnoofUEReports > |  |  | – |  |
| >>UE Assistant Identifier | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | NG-RAN node UE XnAP ID allocated by NG-RAN node1. | – |  |
| >>UE Performance | O |  | 9.2.3.179 |  | – |  |
| >>Measured UE Trajectory | O |  | 9.2.3.182 | It contains information about cells that a UE has connected to. | – |  |
| Node Associated Info Result |  | 0..1 |  |  | YES | ignore |
| >Energy Cost | O |  | INTEGER (0..10000,…) | The node level measured Energy Consumption index. Value 0 indicates the minimum measured Energy Consumption and 10000 indicates the maximum measured Energy Consumption. | - | - |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |
| maxnoofUEReports | Maximum no. UE s for which information can be reported by a NG-RAN node. Value is 16. |