3GPP TSG-RAN WG3 #126																		       R3-247485

Orlando, USA, 18th – 22nd November 2024

Agenda Item:	13.2

Source:	Ericsson

Title:	Network coordination for transmission of semi-persistent CSI-RS

Document for:	Discussion, Agreement

# Introduction

At this meeting, RAN3 received an incoming LS from RAN1 in R3-247006, with a request to provide feedback on the feasibility for a Working Assumption to support semi-persistent CSI-RS for LTM candidate cells. The text of the LS is reported below:

Working Assumption In addition to periodic CSI-RS, semi-persistent CSI-RS is supported for candidate cell L1-RSRP measurement for gNB scheduled reporting from RAN1 perspective The following working assumption was made in RAN1#118bis for candidate cell L1-RSRP measurement based on CSI-RS: RAN1 foresees the RAN3 spec impact to enable this functionality for inter-DU/inter-CU case, and hence RAN3’s view is needed to confirm the working assumption. Therefore, RAN1 would like to ask RAN3 for the feasibility of specifying the signalling for coordination between serving cell and candidate cell(s) on the transmission of semi-persistent CSI-RS(s) and any other potential issues (e.g. RAN3 workload).

# Discussion

In our view, achieving a coordination between serving cell and candidate cell on the transmission of semi-persistent CSI-RS means that the source DU receives updated information on the status of the transmission of semi-persistent CSI-RS in cell of a candidate DU, e.g., whether such transmission has started or stopped. Overall, we think this is feasible, and we describe below the foreseen impacts for intra-CU LTM and inter-CU LTM.

## Intra-CU LTM

For intra-CU LTM, the coordination can be initiated by the CU, for example during the LTM preparation phase. In this case, the CU could request to the candidate DU to provide the status of the transmission of semi-persistent CSI-RS (e.g., in UE Context Setup Request), and the candidate DU returns the information (e.g., in UE Context Setup Response). Then the CU sends the information to the source DU (e.g., via the DL RRC Message Transfer).

If, over time, there is a change in the status of the transmission of semi-persistent CSI-RS for a certain cell of a candidate DU, the coordination can be initiated by the candidate DU to send updated information to the source DU via the CU. In this case, a Class 2 procedure appears more suitable than the existing Class 1 procedures, such as the UE Context Setup or UE Context Modification procedures. A possible way can be to use an UL RRC Message Transfer (DU to CU) to signal the information from candidate DU to CU, and then use a DL RRC Message Transfer (CU to DU) to send the information to the source DU. There does not seem to be any IE in the current messages used for intra-CU LTM to achieve the coordination just described. On the other hand, it is feasible to extend existing messages used for intra-CU LTM, i.e., UE Context Setup Request/Response, UE Context Modification Request/Response, DL RRC Message Transfer, UL RRC Message Transfer.

Considering that the information to be transferred does not concern RRC parameters, reuse of DL/UL RRC Message Transfer, even if doable, it’s not the cleanest way, and an alternative option can be to introduce a new procedure for this purpose. In this case, a class 2 procedure that can be initiated either by the CU or the DU seems sufficient. If this approach is preferrable, RAN3 will need of course to discuss the details.

In summary, for intra-CU LTM, it is feasible to achieve a coordination initiated by CU feasible, at least as follows:

- CU-initiated coordination, Option 1: extend UE Context Setup and DL RRC Message Transfer procedures;

- CU-initiated coordination, Option 2: extend UE Context Setup and UE Context Modification procedures.

- CU-initiated coordination, Option 3: extend UE Context Setup and introduce a new class 2 procedure (FFS)

Also, for intra-CU LTM, it is feasible to achieve a coordination initiated by a candidate DU, at least as follows:

- C-DU-initiated coordination, Option 1: extend UL RRC Message Transfer and DL RRC Message Transfer;

- C-DU-initiated coordination, Option 2: extend DL RRC Message Transfer and UE Context Modification;

- C-DU-initiated coordination, Option 3: introduce a new class 2 procedure (FFS).

For example, if the coordination is initiated by a candidate DU:

- in option 1) the candidate DU can send to the CU an UL RRC MESSAGE TRANSFER message carrying a new IE (e.g., CSI-RS transmission status) to inform the CU about the status of the transmission of semi-persistent CSI-RS in candidate cell. The CU forwards the information to the source DU in a DL RRC MESSAGE TRANSFER.

- in option 2) the candidate DU can initiate the coordination sending an UL RRC MESSAGE TRANSFER message to the CU providing the (updated) status of the transmission of semi-persistent CSI-RS in candidate cell. The CU then triggers a UE Context Modification towards the source DU that receives the information.

- in option 3) the candidate DU can initiate the coordination sending a new message (FFS, e.g., CSI-RS Transmission) to the DU providing the (updated) status of the transmission of semi-persistent CSI-RS in candidate cell. The CU then forwards the information to the source DU in a new message (FFS, e.g., CSI-RS Transmission).

An overview of the options described to achieve the coordination in the intra-CU LTM case is shown in Figure 1.

Figure 1. Coordination on the status of the transmission of semi-persistent CSI-RS for intra-CU LTM

- RAN3 to discuss solutions to achieve intra-CU coordination on the status of the transmission of semi-persistent CSI-RS(s), for example:

  - For CU initiated coordination, extend UE Context Setup and UE Context Modification or introduce a new procedure for sending the information from CU to DU

  - For candidate-DU initiated coordination:

    - extend UL RRC Message Transfer and DL RRC Message Transfer

    - extend UE Context Setup Request/Response and UL RRC Message Transfer

    - introduce a new procedure for sending the information from CU to DU and vice versa

## Inter-CU LTM

For inter-CU LTM, Xn impact needs to be considered and we can leverage on some of the agreements already achieved in Rel-19. For example, the coordination can be initiated by the source CU in the Handover Request at initial LTM preparation (see Source-CU initiated coordination, Option 1 in the figure below). The source CU requests the candidate CU to provide the status of the transmission of semi-persistent CSI-RS in the candidate cell. Reusing the solution for intra-CU coordination, the candidate CU sends the request to the candidate DU and the candidate DU returns the information to the candidate CU. The candidate CU then sends the information to the source CU via Xn in the Handover Request Acknowledge. Finally, at the source gNB, the source CU informs the source DU using one of options described for the intra-CU LTM case. Another possibility to achieve the coordination when initiated by the source CU (see Source-CU initiated coordination, Option 2) is to use the LTM Configuration Update procedure, where the candidate CU sends to the source CU the transmission status in the LTM Configuration Update Acknowledge.

If the status of the transmission of semi-persistent CSI-RS for a cell of a candidate CU changes over time, the coordination can be initiated by the candidate DU under the candidate CU (see Candidate-CU initiated coordination), in the same way as described for the intra-CU case. In terms of Xn impact, the candidate CU can trigger an LTM Configuration Update to inform the source CU about the updated information. In this case the sending of the transmissions status is done in the LTM Configuration Update. At source gNB, the same procedures as for the intra-CU case can be reused.

An overview of the options described to achieve the coordination in the inter-CU LTM case is shown in Figure 2.

Figure 2. Coordination on the status of the transmission of semi-persistent CSI-RS for inter-CU LTM

- RAN3 to discuss the solution to achieve inter-CU coordination between serving cell and candidate cells on the transmission of semi-persistent CSI-RS(s), including:

  - For source-CU initiated coordination, extend the Handover Request and the LTM Configuration Update

  - For candidate-CU initiated coordination, extend the LTM Configuration Update

Based on the presented analysis, we think RAN3 can indicate a positive answer to the RAN1 request on feasibility, and we provide a draft LS reply in the Appendix.

- Send a reply LS to RAN1 to confirm the feasibility of coordination between serving cell and candidate cell(s) on the transmission of semi-persistent CSI-RS(s).

# Conclusions and Proposals

In this paper we propose:

Proposal 1	RAN3 to discuss solutions to achieve intra-CU coordination on the status of the transmission of semi-persistent CSI-RS(s), for example:

o	For CU initiated coordination, extend UE Context Setup and UE Context Modification or introduce a new procedure for sending the information from CU to DU

o	For candidate-DU initiated coordination:

	extend UL RRC Message Transfer and DL RRC Message Transfer

	extend UE Context Setup Request/Response and UL RRC Message Transfer

	introduce a new procedure for sending the information from CU to DU and vice versa

Proposal 2	RAN3 to discuss the solution to achieve inter-CU coordination between serving cell and candidate cells on the transmission of semi-persistent CSI-RS(s), including:

o	For source-CU initiated coordination, extend the Handover Request and the LTM Configuration Update

o	For candidate-CU initiated coordination, extend the LTM Configuration Update

Proposal 3	Send a reply LS to RAN1 to confirm the feasibility of coordination between serving cell and candidate cell(s) on the transmission of semi-persistent CSI-RS(s).

# Appendix – Reply LS to RAN1

3GPP TSG-RAN WG3 #126																		       R3-24xxxx

Orlando, USA, 18th – 22nd November 2024

Title:	[Draft] LS Reply on the support of semi-persistent CSI-RS resource for LTM candidate cells

Response to:	R3-247006

Release:	Rel-19

Work Item:	NR_Mob_Ph4-Core

Source:	Ericsson [To be RAN3]

To:	RAN1

Cc:	RAN2

Contact Person:

Name: 		Liwei Qiu

E-mail Address:	liwei.qiu@ericsson.com

Send any reply LS to:      3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall description:

RAN3 has discussed RAN1 LS in R3-247006, indicating RAN1 Working Assumption to support semi-persistent CSI-RS for candidate cell L1-RSRP measurement, and the related request to RAN3 to indicate the feasibility of specifying the signaling for coordination between serving cell and candidate cell(s) on the transmission of semi-persistent CSI-RS(s). It is RAN3 understanding that the purpose of such coordination is to let the serving cell be aware of the current status of the transmission of semi-persistent CSI-RS (e.g., started, stopped).

Based on the above understanding, RAN3 agreed that specifying the signaling for such coordination is achievable.

RAN3 has also agreed to specify a solution when RAN1 confirms that the requested coordination is needed.

2. Actions:

To RAN1 group.

ACTION: 	RAN3 respectfully asks RAN1 to take the above reply into account and requests RAN1 to inform RAN3 when the need for the requested coordination is agreed in RAN1.

3. Date of Next TSG RAN WG3 Meetings:

TSG RAN WG3 Meeting #127	17th– 21st February 2025	Athens, GR