3GPP TSG-RAN WG3 Meeting #126	R3-247486

Orlando, USA, 18 – 22 November 2024

Agenda item:	12.3

Source:	Nokia, TMO US, AT&T, Verizon Wireless, BT, Charter, ZTE

Title:	[TP for BL CR NR Femto 38.300] Introduction of Functional aspects of NR Femto architecture

WID/SID:	FS_WAB_5GFemto_NR - Release 19

Document for:	Discussion and Decision

# 1	Introduction

This is the TP for functional aspect to support NR Femto in release 19.

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

### 4.X.2	Functional Split

An NR Femto hosts the same functions as a gNB as described in clause 4.1, with the following additional specifics in case of connection to the NR Femto GW:

-	Discovery of a suitable Serving NR Femto GW;

-	An NR Femto shall only connect to a single NR Femto GW at one time, namely no NG-C Flex function shall be used at the NR Femto:

-	The NR Femto will not simultaneously connect to another NR Femto GW, or another AMF.

-	The TAC and PLMN ID used by the NR Femto shall also be supported by the NR Femto GW;

-	Selection of an AMF at UE attachment is hosted by the NR Femto GW instead of the NR Femto. Upon reception of the GUAMI from a UE, the NR Femto shall include it in the INITIAL UE MESSAGE message.

-	NR Femtos may be deployed without network planning. An NR Femto may be moved from one geographical area to another and therefore it may need to connect to different NR Femto GWs depending on its location;

-	Signalling the GUAMI of the Source AMF and the Source AMF UE NGAP ID to the NR Femto GW in the NGAP PATH SWITCH REQUEST message.

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

The NR Femto GW hosts the following functions:

-	Relaying UE-associated NG application part messages between the AMF serving the UE and the NR Femto serving the UE, except the UE CONTEXT RELEASE REQUEST message received from the NR Femto with an explicit GW Context Release Indication. In that case, the NR Femto GW terminates the NGAP UE Context Release Request procedure and releases the UE context if it determines that the UE identified by the received UE NGAP IDs is no longer served by an NR Femto attached to it. Otherwise it ignores the message.

-	In case of NGAP INITIAL CONTEXT SETUP REQUEST message and NGAP HANDOVER REQUEST message, informing the NR Femto about any GUAMI corresponding to the serving AMF, the AMF UE NGAP ID assigned by the AMF and the AMF UE NGAP ID assigned by the NR Femto GW for the UE. In case of NGAP PATH SWITCH REQUEST ACKNOWLEDGE message, informing the NR Femto about the AMF UE NGAP ID assigned by the AMF and the AMF UE NGAP ID assigned by the NR Femto GW for the UE.

NOTE:	In case of NGAP INITIAL UE MESSAGE message, whether to verify, for a closed NR Femto, that the indicated cell access mode and CAG ID are valid for that NR Femto is pending SA3 study.

-	Terminating non-UE associated NG application part procedures towards the NR Femto and towards the AMF. In case of NG PWS RESTART INDICATION message and PWS FAILURE INDICATION message, replacing the gNB ID of the NR Femto by the NR Femto GW ID before sending the PWS RESTART INDICATION message (respectively the PWS FAILURE INDICATION message) to the AMF.

NOTE:	In case of NG SETUP REQUEST message, whether to verify that the identity used by the NR Femto is valid and determining whether the access mode of the NR Femto is closed or not is pending SA3 study.

NOTE:	In case of NG PWS RESTART INDICATION message and PWS FAILURE INDICATION message, whether to verify that the indicated cell identity is valid is pending SA3 study.

-	Upon receiving an OVERLOAD START/STOP message, the NR Femto GW should send the OVERLOAD START/STOP message towards the NR Femto(s) including in the message the identities of the affected AMF node(s). The NR Femto uses this information received from the OVERLOAD START message to identify to which traffic the above defined rejections shall be applied. The NR Femto shall apply the defined rejections until reception of an OVERLOAD STOP message applicable to this traffic, or until the NR Femto receives a further OVERLOAD START message applicable to the same traffic, in which case it shall replace the ongoing overload action with the newly requested one.

NOTE:	If an NR Femto GW is deployed, non-UE associated procedures shall be run between NR Femtos and the NR Femto GW and between the NR Femto GW and the AMF.

-	Supporting TAC and PLMN ID used by the NR Femto.

-	Routing the NGAP PATH SWITCH REQUEST message towards the AMF based on the GUAMI of the source AMF received from the NR Femto.

A list of CAG IDs may be included in the NGAP PAGING message. If included, the NR Femto GW may use the list of CAG IDs for paging optimisation.

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

In addition to functions specified in clause 4.1, the AMF hosts the following functions:

-	Access control for initial access of UEs that are members of Closed Access Groups (CAG):

NOTE:	In case of an NR Femto directly connected, whether to verify that the identity used by the NR Femto is valid when receiving the NGAP SETUP REQUEST message and determining whether the access mode of the NR Femto is closed or not is pending SA3 study.

NOTE:	In case of an NR Femto directly connected, whether to verify, for a closed NR Femto, that the indicated cell access mode and CAG ID are valid when receiving the NGAP INITIAL UE MESSAGE message is pending SA3 study.

NOTE:	In case of an NR Femto directly connected, whether to verify that the indicated gNB identity of the NR Femto is valid when receiving the NGAP PWS RESTART INDICATION message and the NG PWS FAILURE INDICATION message is pending SA3 study.

-	Routing of handover messages, Uplink RAN Configuration Transfer and Downlink RAN Configuration Transfer messages towards NR Femto GWs based on the Selected TAI contained in these messages.

NOTE:	If routing ambiguities are to be avoided, a TAI used in an NR Femto GW should not be reused in another NR Femto GW.

NOTE:	The AMF or NR Femto GW should not include the list of CAG IDs for paging when sending the paging message directly to an un-trusted NR Femto or gNB (to be checked by SA3).

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>