3GPP TSG-RAN WG3 #126	R3-247495

# Orlando, USA, Nov 18 – 22, 2024

Agenda Item:	10.3.1

Source:	Ericsson

Title: 	SON/MDT enhancements for NTN

Document for:	Discussion

# 1	Introduction

RAN plenary decided during RAN#103 to support further data collection for Self-organizing Networks (SON). One of the topics that was endorsed was the SON enhancements for non-terrestrial networks.

This paper builds on the previous submissions by discussing SON for NTN on a range of different topics such as RLF enhancements, SHR enhancements and MDT area scope related enhancements.

# 2	Discussion

## 2.1 MDT related enhancements for NTN

Using the existing MDT framework in an NTN network requires further considerations and enhancements to cope with the specificities of NTN. One of the key differences between the NTN and the TN networks is the fact that, compared to the TN, in NTN the mapping between the coverage of an NTN cell and the geographical area covered by that cell changes over the time. Indeed, as a satellite follows its orbit the coverage of the satellite cells swipes the surface of the earth meaning that, at two different points in time the same geographical area may be covered by two different NTN cells.  In this context it may be difficult to define an Area Scope for MDT that enables measuring the NTN service coverage in a given geographical area. It is therefore proposed to extend the definition of the MDT Area Scope to enable collection of immediate and logged MDT measurements for the NTN coverage in a specific geographical area.

Proposal 1: RAN3 to extend the definition of the MDT Area Scope to enable collection immediate and logged MDT measurements for the NTN coverage in a specific geographical area.

In the previous meetings, some companies proposed that we should add mapped cell ID to the Area Scope of MDT to support NTN networks. This is not very optimal as the mapped cell ID still translates to large geographical areas. On the other hand, enabling collection of MDT in smaller geographical areas and/or enabling finer resolution when defining the geographical areas is more useful as described below.

Proposal 2: Don’t include mapped cell ID for logged MDT in Area Scope of MDT.

Instead of mapped cell IDs in the Area Scope, it is better to include geographical areas with smaller granularity (e.g. GNSS coordinates). These will allow an operator to collect MDT more precisely, for example around a lake, mountain, instead of the whole NTN cell which are big. The exact structure and formulation of these areas of smaller granularity can be discussed in the upcoming meetings.

Proposal 3: Support geographical areas with smaller granularity in the Area Scope of MDT for both logged and immediate MDT.

The Area Scope of MDT needs specific enhancements related to indicate to the UE, that the MDT has to be collected in NTN networks. To do so, we need to add an NTN wide indicator in the Area Scope of MDT. This will allow the network to indicate to the UE to collect MDT only in NTN. There is currently no method in the specifications to indicate that MDT must be collected in a specific type of network, NTN in this case. This will allow an operator to build a coverage map of NTN network and detect problems specific to that type of RAT.

Other options such as indicating the CGI of the cell can be used, however they have an upper limit on the number of identifiers and as such is unsuitable for this scenario.

Proposal 4: Include NTN wide indication in the MDT Area Scope to enable MDT collection.

Another aspect of NTN that requires further consideration with respect to MDT is the reporting of UE MDT measurements. Indeed, compared to TN, reporting of UE measurements over the NTN uplink comes with a much higher energy cost on the UE side. This cost is the natural consequence of the fact that for the NTN uplink the distance between the transmitter (the UE on the surface of the earth) and the receiver (the satellite high in orbit) is an order of magnitude higher compared to the distance between the transmitter and the receiver in case of TN uplink (e,g.,  hundreds/thousands of km for NTN vs hundreds/ thousands of meters for TN). In this context and whenever possible, it is preferable to avoid wasting UE’s battery on sending the MDT reports while the UE is under the service coverage of a NTN and to differ these reports for the times when the UE moves back under the coverage of a TN.

Proposal 5: RAN3 to extend the MDT framework to enable instructing the UE and/or the network to avoid triggering UE reporting of MDT measurements while the UE is connected to NTN.

## 2.2 RLF related enhancements for NTN

Time-based CHO

During the previous meeting, there were discussions on distinguish if the RLF happens before or after T1. Three alternatives are agreed:

WA: down-select among below three alternatives. Alt 1: the RLF report includes for single time-based trigger condition: - Absolute time (measured UTC time) when RLF occurred Alt 2: a flag to indicate whether the RLF happens before T1 or after T1 Alt 3: based on existing timers in RLF report. Further check the granularity of the current timers.

Regarding Alt 3, the existing timers in RLF report include timeSinceFailure, timeConnFailure and timeSinceCHO-Reconfig. One issue with reusing these timers for NTN especially for too late CHO execution, is that the t1-Threshold defined in time-based CHO is in 10ms units whereas the granularity of the existing timers in RLF report is in seconds or 100ms. Let’s take timeSinceCHO-Reconfig as an example to show a case when it cannot be used to distinguish if the RLF happens before or after T1 in the figure below. For simplicity, we assume that CHO is configured at the absolute time of 0ms, and t1-Threshold is defined as the absolute time of 90ms, duration is 100ms. In the figure below, if the reported timeSinceCHO-Reconfig is 100ms, then gNB may deduce that the RLF happened after T1, but maybe the truth is that the RLF happened at the absolute time of 80ms, depending on UE implementation, UE may report timeSinceCHO-Reconfig as 100ms due to the granularity problem, which would result in a wrong conclusion in the NW side.

CHO configuration

0ms

t1-Threshold

90ms

t1-Threshold + duration

190ms

timeSinceCHO-Reconfig = 100ms

RLF

There were some proposals during last meeting of not reporting the absolute timestamp to the network as some UE may not have the UTC time. We think that as the time-based CHO triggering condition is configured by t1-Threshold in UTC time, thus, UEs that are capable of time-based CHO triggering condition should also have the ability to report the absolute timestamp with no extra cost. Therefore, it is beneficial to have the absolute time in the RLF report with not much extra cost for the UE.

If the above seems too stringent, it would also be possible to have a relative time instead, which would be the time between T1 and the failure (could be negative or positive).

Proposal 6: RAN3 to suggest to RAN2 to include absolute time when the RLF occurs in the RLF report, or relative time between failure and T1.

Joint trigger condition with time-based/location-based CHO and measurement based CHO

Last RAN3 meeting had an agreement regarding on joint trigger conditions as:

UE reports the fulfilled CHO trigger conditions before RLF occurs in case of “time and measurement based trigger condition” or “location and measurement based trigger condition”, it is up to RAN2 to decide to reuse the existing IE or not.

In current RLF-Report, the fulfilled execution condition(s) information at the moment of handover failure or radio link failure is logged in firstTriggeredEvent and timeBetweenEvents. These IEs in RLF-Report can accommodate the time and measurement based trigger condition or location and measurement based trigger condition, without requiring additional modifications.

Observation 1: The fulfilled CHO trigger conditions before RLF occurs in case of “time and measurement based trigger condition” or “location and measurement based trigger condition” can be logged in RLF-Report naturally. No action required from RAN2.

Satellite switch with re-sync

One of these additions would be related to the satellite switch with re-sync procedure, as defined in rel-18. It would be beneficial for the network to understand that this procedure was performed and failed. This could be done by e.g. reusing the “hof” codepoint as connection failure type in the RLF Report, and by adding a new indication (explicit or implicit) that the mobility procedure as failed. A new a new value for the lastHO-Type field (e.g. satSwitchWithResync) could also be added.

Proposal 7: RAN3 to suggest to RAN2 to enhance the RLF Report with indication that a satellite switch with re-sync procedure was performed and failed.

## 2.3 RACH-less HO related enhancements for NTN

During the RAN3 #125, there was an FFS agreed related to the enhancements for RACH-less HO:

Discuss MRO for RACH-less HO of NTN, only for those different issues identified compared with MRO for LTM.

RACH-less handover was standardized in Rel-18 for NTN that was further applicable to other WIs such as mobile IAB among others. RACH-less HO for NTN needs to be optimized as SON enhancements for RACH-less HO were not considered previously. RACH-less HO optimizations are also being discussed in other Work Items, therefore we propose the following.

Proposal 8: RAN3 to discuss SON related enhancements for the RACH-less HO procedure for NTN that are different than the ideas identified for LTM.

Proposal 9: Any stage 2 and stage 3 implementations should be taken in the LTM work item.

One of the differences between NTN and LTM RACH-less access is the fallback mechanism defined for NTN. This can be described as followed:

- UE is configured with RACH-less, including a TA information, and an RSRP threshold in case of configured grant RACH-less.

- At the time of HO, if RSRP of the SSBs in the target cell is lower than the configured threshold, UE will initiate Random-Access procedure.

It is useful for the network (i.e. source node) to understand that RACH-less was not performed while configured, in order to optimize HO failure, time, resources, etc…

Moreover, when considering the fallback from RACH-less to RACH-based HO, the discussions and agreements in LTM work item should also be considered.

Observation 2: In case of mobility failure or near-failure, it is beneficial for the source node to understand that RACH-less was configured but that fallback to Random-Access procedure was performed instead.

Proposal 10: SON reports (RLF Report and SHR) need to contain information allowing the network to know that fallback from RACH-less to RACH-based was performed.

Looking at the fallback mechanism in more details, when uplink grant for RACH-less handover is configured, the UE will initiate RACH-less HO or Random-Access procedure for HO by checking if there is SSB corresponding to the configured uplink grant with SS-RSRP above a RSRP threshold is available. Thus, the information of the measured RSRP for the SSB beam(s) for the configured grant can be used for the NW to optimize the configured uplink grant for RACH-less handover configuration. For example, if the failure happens for a fall-backed RACH-based HO, it may be because of too many RACH collisions in NTN cell. Similarly, in the RACH optimization for 2-Step RA in Rel-17, dlPathlossRSRP IE was introduced which can be used by NW to tune the RSRP threshold for selection between 2-step RA type and 4-step RA type. It is therefore proposed to include the measured RSRP information in SON reports.

Proposal 11: UE logs the information of the measured RSRP for the SSB beam(s) corresponding to the configured grant for RACH-less HO if there is a fallback from RACH-less to RACH-based HO.

# 3	Summary

In this discussion paper, NTN SON/MDT enhancements were discussed, and the following observations and proposals were made:

Proposal 1: RAN3 to extend the definition of the MDT Area Scope to enable collection immediate and logged MDT measurements for the NTN coverage in a specific geographical area.

Proposal 2: Don’t include mapped cell ID for logged MDT in Area Scope of MDT.

Proposal 3: Support geographical areas with smaller granularity in the Area Scope of MDT for both logged and immediate MDT.

Proposal 4: Include NTN wide indication in the MDT Area Scope to enable MDT collection.

Proposal 5: RAN3 to extend the MDT framework to enable instructing the UE and/or the network to avoid triggering UE reporting of MDT measurements while the UE is connected to NTN.

Proposal 6: RAN3 to suggest to RAN2 to include absolute time when the RLF occurs in the RLF report, or relative time between failure and T1.

Observation 1: The fulfilled CHO trigger conditions before RLF occurs in case of “time and measurement based trigger condition” or “location and measurement based trigger condition” can be logged in RLF-Report naturally. No action required from RAN2.

Proposal 7: RAN3 to suggest to RAN2 to enhance the RLF Report with indication that a satellite switch with re-sync procedure was performed and failed.

Proposal 8: RAN3 to discuss SON related enhancements for the RACH-less HO procedure for NTN that are different than the ideas identified for LTM.

Proposal 9: Any stage 2 and stage 3 implementations should be taken in the LTM work item.

Observation 2: In case of mobility failure or near-failure, it is beneficial for the source node to understand that RACH-less was configured but that fallback to Random-Access procedure was performed instead.

Proposal 10: SON reports (RLF Report and SHR) need to contain information allowing the network to know that fallback from RACH-less to RACH-based was performed.

Proposal 11: UE logs the information of the measured RSRP for the SSB beam(s) corresponding to the configured grant for RACH-less HO if there is a fallback from RACH-less to RACH-based HO.

# 4	References

- RP-234038, “New WID: Data collection for SON (Self-Organising Networks)/MDT (Minimization of Drive Tests) in NR standalone and MR-DC (Multi-Radio Dual Connectivity) Phase 4”, RAN#102, Edinburgh, Scotland, December 2023.