3GPP TSG-RAN WG3 Meeting #126	R3-247497

# Orlando, U.S.A, 18th – 22nd November, 2024

Source:	Ericsson, InterDigital, Jio

Title:	On SON enhancements for Network Slicing

Agenda item:     10.3.2

Document for:	Discussion, Decision

# 1	Introduction

The release 19 work item for Data Collection for SON/MDT in NR standalone and MR-DC focuses on enhancements for Rel-17/Rel-18 SON/MDT features such as MRO enhancement for LTM, CHO with candidate SCGs and subsequent CPAC, as well as on SON/MDT enhancements for Intra NTN mobility and Network Slicing [1].

During RAN3#125-bis meeting, RAN2 sent an LS to RAN3 (R3-245762 [2]) describing agreements reached due to workload issues in RAN2. As part of this LS RAN2 indicated that it has deprioritized discussions on SON/MDT for Slicing, and asked RAN3 to prioritize solutions that are RAN3-based.

In the context of discussions in RAN3, Ericsson in our paper to RAN3#125-bis [3] proposed SON enhancements, where Successful Handover Reports are enhanced with user plane interruption times, and a threshold place on user plane interruption time to be used as an additional trigger for reporting SHR reports to the network. Proposals from other companies [4, 5, 6] also propose other SHR-related enhancements.

It was however agreed that such enhancements to SHR reports will not be pursued by RAN3 as part of Release 19 enhancements, both due to a lack of consensus in the group and due to the received LS from RAN2 which asked RAN3 to avoid any RAN2 impacts in the pursued solutions.

# 2	Discussion

The Release 19 discussions on SON/MDT for Network Slicing have so far agreed to introduce Network Slicing related MDT enhancements (in the form of a Network Slice Area Scope) and other logged MDT discussions are underway. It is however important to note that no SON related enhancement has been agreed upon for further normative discussions in RAN3. The RAN2 decision to deprioritise SON/MDT for Slicing is another limiting factor in the solution space that RAN3 can explore to bring SON enhancements in the context of Network Slicing.

Given the interest from different companies in RAN3 to include SON related enhancements in this release, and the mandate from RAN plenary to introduce SON enhancements for Network Slicing, it is therefore proposed that RAN3 should discuss network-based solutions for SON enhancements.

Proposal 1: RAN3 to discuss network-based solutions for improving observability into network slicing

In our contributions to earlier meetings, we had proposed , with the support of other companies, for the  introduction of per slice user-plane interruption times during handovers as  an insightful metric in the context of better network slicing observability. The user-plane interruption time can be used to identify both situations where there is a slice discontinuity during a handover event, and also obtain observability into the different slice-specific policies that may be active in neighbouring cells where certain slices may be prioritized over another. Therefore, Network-based observability of per slice user-plane interruption times can still provide valuable insights in the context of Network Slicing.

Proposal 2: RAN3 to further discuss network-based approaches to collect per slice user-plane interruption times during handovers.

# Conclusion

The following proposals are made in this paper:

Proposal 1: RAN3 to discuss network-based solutions for improving observability into network slicing

Proposal 2: RAN3 to further discuss network-based approaches to collect per slice user-plane interruption times during handovers.

# References

- RP-234038: WID: Data collection for SON/MDT in NR standalone and MR-DC

- LS to RAN3 on way forward on Rel-19 SONMDT

- R3-245396 Discussion on SON enhancements for network slicing (Ericsson)

- R3-245146 Discussion on SON enhancements for network slicing (China Telecom)

- R3-245312 [TP to BL CR to 38.300, MDT] MDT solution for slice support and slice-related mobility enhancements (Nokia)

- R3-245345 Further consideration on SON/MDT for Slicing (ZTE Corporation)