3GPP TSG-RAN WG3 Meeting #126	R3-247509

# Orlando, USA, 18-22 November 2024

Title: 	On the special case of Nrb 12 and Nrb 20 Transmission Bandwidth configurations

Source: 	Ericsson, Strict, Qualcomm

Agenda item:	9.2

Document Type:	Discussion and agreement

# 1. Introduction

In recent RAN3 discussions proposals have been brought forward to exchange information over the Xn interface concerning the following cell configurations:

- Cells using the band n100 with 3MHz channels and the special configuration of Nrb 12 at Synch Raster frequency of 920.73,

- Cells using the band n100 with 5MHz channels and the special configuration of Nrb 20 at Synch Raster frequency of 921.45.

This paper discusses these cases and explains how it is already possible to derive, from legacy information exchanged over the Xn and F1 interfaces, that a cell follows the configurations above.

# 2. Discussion

The first point to clarify concerns the CRs submitted at the last RAN3 meeting in R3-245047 and R3-245048.

These CRs proposed to add new code points to the IE that specifies the transmission bandwidth of a cell, for example the NR Transmission Bandwidth IE signalled as part of the Served Cell Information NR IE over the Xn interface. The proposed change is provided below:

#### 9.2.2.20	NR Transmission Bandwidth

The NR Transmission Bandwidth IE is used to indicate either the UL or the DL transmission bandwidth.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| NR SCS | M |  | ENUMERATED (scs15, scs30, scs60, scs120, …, scs480, scs960) | The values scs15, scs30, scs60 and scs120 corresponds to the sub carrier spacing in TS 38.104 [24]. |
| NR NRB | M |  | ENUMERATED (nrb11, nrb18, nrb24, nrb25, nrb31, nrb32, nrb38, nrb51, nrb52, nrb65, nrb66, nrb78, nrb79, nrb93, nrb106, nrb107, nrb121, nrb132, nrb133, nrb135, nrb160, nrb162, nrb189, nrb216, nrb217, nrb245, nrb264, nrb270, nrb273, ..., nrb33, nrb62, nrb124, nrb148, nrb248, nrb44, nrb58, nrb92, nrb119, nrb188, nrb242, nrb15, nrb12, nrb20) | This IE is used to indicate the UL or DL transmission bandwidth expressed in units of resource blocks "NRB" (TS 38.104 [24]). The values nrb11, nrb18, etc. correspond to the number of resource blocks "NRB" 11, 18, etc. The values nrb12 and nrb20 are used to indicate the 12PRB transmission bandwidth within 3MHz channel and 20PRB transmission bandwidth within 5MHz channel (Table 5.4.3.1-3 of TS 38.104[24]). |

It should be noted that the transmission bandwidth signalled for a cell over the Xn and F1 interfaces consists of the Maximum Transmission Bandwidth configuration specified in TS 38.101-1 and in TS38.101-2 and reported below:

From TS38.101-1

### 5.3.2 Maximum transmission bandwidth configuration The maximum transmission bandwidth configuration NRB for each UE channel bandwidth and subcarrier spacing is specified in Table 5.3.2-1. Table 5.3.2-1: Maximum transmission bandwidth configuration NRB SCS (kHz); 3 MHz; 5 MHz; 10 MHz; 15 MHz; 20 MHz; 25 MHz; 30 MHz; 35 MHz; 40 MHz; 45 MHz; 50 MHz; 60 MHz; 70 MHz; 80 MHz; 90 MHz; 100 MHz. NRB; NRB; NRB; NRB; NRB; NRB; NRB; NRB; NRB; NRB; NRB; NRB; NRB; NRB; NRB; NRB. 15; 15; 25; 52; 79; 106; 133; 160; 188; 216; 242; 270; N/A; N/A; N/A; N/A; N/A. 30; N/A; 11; 24; 38; 51; 65; 78; 92; 106; 119; 133; 162; 189; 217; 245; 273. 60; N/A; N/A; 11; 18; 24; 31; 38; 44; 51; 58; 65; 79; 93; 107; 121; 135. ### 5.3.3 Minimum guardband and transmission bandwidth configuration The minimum guardband for each UE channel bandwidth and SCS is specified in Table 5.3.3-1, Table 5.3.3-1: Minimum guardband for each UE channel bandwidth and SCS (kHz) SCS (kHz); 3 MHz; 5 MHz; 10 MHz; 15 MHz; 20 MHz; 25 MHz; 30 MHz; 35 MHz; 40 MHz; 45 MHz; 50 MHz; 60 MHz; 70 MHz; 80 MHz; 90 MHz; 100 MHz. 15; 142.5; 242.5; 312.5; 382.5; 452.5; 522.5; 592.5; 572.5; 552.5; 712.5; 692.5; N/A; N/A; N/A; N/A; N/A. 30; N/A; 505; 665; 645; 805; 785; 945; 925; 905; 1065; 1045; 825; 965; 925; 885; 845. 60; N/A; N/A; 1010; 990; 1330; 1310; 1290; 1630; 1610; 1590; 1570; 1530; 1490; 1450; 1410; 1370. NOTE: The minimum guardbands have been calculated using the following equation: GBChannel = (BWChannel x 1000 (kHz) - NRB x SCS x 12) / 2 - SCS/2, where NRB are from Table 5.3.2-1 and GBChannel expressed in kHz.

From TS38.101-2:

### 5.3.2 Maximum transmission bandwidth configuration The maximum transmission bandwidth configuration NRB for each UE channel bandwidth and subcarrier spacing is specified in Table 5.3.2-1 Table 5.3.2-1: Maximum transmission bandwidth configuration NRB SCS (kHz); 50 MHz; 100 MHz; 200 MHz; 400 MHz; 800 MHz; 1600 MHz; 2000 MHz. NRB; NRB; NRB; NRB; NRB; NRB; NRB. 60; 66; 132; 264; N/A; N/A; N/A; N/A. 120; 32; 66; 132; 264; N/A; N/A; N/A. 4801; N/A; N/A; N/A; 66; 124; 248; N/A. 9601; N/A; N/A; N/A; 33; 62; 124; 148. Note 1: This SCS is optional in this release of the specification.. ### 5.3.3 Minimum guardband and transmission bandwidth configuration The minimum guardband for each UE channel bandwidth and SCS is specified in Table 5.3.3-1. Table 5.3.3-1: Minimum guardband for each UE channel bandwidth and SCS (kHz) SCS (kHz); 50 MHz; 100 MHz; 200 MHz; 400 MHz; 800 MHz; 1600 MHz; 2000 MHz. 60; 1210; 2450; 4930; N/A; N/A; N/A; N/A. 120; 1900; 2420; 4900; 9860; N/A; N/A; N/A. 480; N/A; N/A; N/A; 9680; 42640; 85520; N/A. 960; N/A; N/A; N/A; 9440; 42400; 85280; 147040. NOTE: The minimum guardbands have been calculated using the following equation: GBChannel = (BWChannel x 1000 (kHz) - NRB x SCS x 12) / 2 - SCS/2, where NRB are from Table 5.3.2-1 and GBChannel expressed in kHz. The minimum guardband of receiving BS SCS 240 kHz SS/PBCH block for each UE channel bandwidth is specified in table 5.3.3-2 for FR2.

As it can be seen, the values included in the tables above are those included e.g. in the NR Transmission Bandwidth IE signalled as part of the Served Cell Information NR IE over the Xn interface.

It should be noted that the meaning of the Maximum Transmission Bandwidth is that of specifying the number of PRBs a cell can use for each usable frequency point in the cell Channel Bandwidth.

On the contrary, the two exception cases of cell configurations for band n100 described in the Introduction do not imply that the transmission bandwidth of the cell is of 12 PRBs or 20 PRBs for each usable frequency point in the cell Channel Bandwidth.  Those cases only imply that, for a specific frequency point coinciding with the Synch Raster frequency, the use of PRBs is restricted as follows:

- For cells using the NR band n100 with 3MHz channels, usable PRBs are restricted from 15 to 12 only at frequency point 920.73 MHz. For all other frequency points in NR bands n100 ( as well as in n106, n26, n28 and n85), 15 PRBs are used.

- For cells using the NR band n100 with 5MHz channels, usable PRBs are restricted from 25 to 20 only at frequency point 921.45 MHz. For all other frequency points in NR band n100 (and in bands supporting a 5MHz channel bandwidth as per legacy prior to Rel-18), 25 PRBs are used.

This is proven by TS38.101, which reports the following:

#### 5.4.3.1 Synchronization raster and numbering The synchronization raster indicates the frequency positions of the synchronization block that can be used by the UE for system acquisition when explicit signalling of the synchronization block position is not present. A global synchronization raster is defined for all frequencies. The frequency position of the SS block is defined as SSREF with corresponding number GSCN. The parameters defining the SSREF and GSCN for all the frequency ranges are in Table 5.4.3.1-1 for above 3 MHz channel bandwidth and in Table 5.4.3.1-2 for 3 MHz channel bandwidth. For band n100, additional parameters defining the SSREF and GSCN are specified in Table 5.4.3.1-3. The resource element corresponding to the SS block reference frequency SSREF is given in clause 5.4.3.2. The synchronization raster and the subcarrier spacing of the synchronization block is defined separately for each band. The synchronization raster and the corresponding SS block do not cover all possible RF channel bandwidths and locations on Enhanced channel raster. Table 5.4.3.1-1: GSCN parameters for the global frequency raster for above 3 MHz channel bandwidth Frequency range; SS Block frequency position SSREF; GSCN; Range of GSCN. 0 – 3000 MHz; N * 1200kHz + M * 50 kHz, N=1:2499, M ϵ {1,3,5} 1; 3N + (M-3)/2; 22 – 7498. 3000 – 24250 MHz; 3000 MHz + N * 1.44 MHz N = 0:14756; 7499 + N; 7499 – 22255. NOTE 1: The default value for operating bands with which only support SCS spaced channel raster(s) is M=3. NOTE 2: GSCN=2 (corresponding to ARFCN-ValueNR = 250) is a reserved value paired with reserved operating band n200.. Table 5.4.3.1-2: GSCN parameters for the global frequency for 3 MHz channel bandwidth Range of frequencies (MHz); SS block frequency position SSREF; GSCN; Range of GSCN. 0 – 1000; N * 600 kHz + M * 50 kHz + 300 kHz, N = 1:1665, M ϵ {1,3,5} (Note 1); 26638+3N + (M-3)/2; 26640 – 31634. NOTE 1: Only applicable for 15 PRB transmission bandwidth configuration within 3 MHz channel bandwidth with punctured PBCH defined in TS 38.211 [6] clause 7.4.3.1.. Table 5.4.3.1-3: Additional GSCN parameters for band n100 SS Block frequency position SSREF (MHz); GSCN; Note. 920.73; 41637; Only applicable for 12 PRB transmission bandwidth configuration within 3 MHz channel with punctured PBCH defined in TS 38.211 [6] clause 7.4.3.1.. 921.45; 41638; Only applicable for 20 PRB transmission bandwidth configuration within 5 MHz channel with unpunctured PBCH defined in TS 38.211 [6] clause 7.4.3.1..

It can be seen from the above that in a predetermined manner, the 12 PRB configuration can only be used at SS REF (i.e. Synch Raster frequency) = 920.73 MHz, while the 20 PRB configuration can only be used at SS REF (i.e. Synch Raster frequency) = 921.45 MHz.

It is therefore immediately evident that it would be incorrect to add Nrb12 and Nrb 20 to, e.g. the NR Transmission Bandwidth IE signalled over the Xn, because the signalling does not reflect that the “exceptional cases” are only for NR band n100 and that are only for specific sync raster frequencies. Without such information the signalling would imply that the cell can operate using an “exceptional case” on a channel bandwidth located anywhere within an NR band which is not correct since the “exceptional case” are only valid at two specific frequency points in NR n100..

Indeed, not by chance, Nrb 12 and Nrb 20 are not included in tables 5.3.2-1 and 5.3.3-1 of TS38.101-1 and TS38.101-2. Please note that Table 5.3.2-1 defines NRB = 15 for 3 MHz Channel Bbandwidth (CBW) and NRB = 25 for 5 MHz CBW, which are used for both the “exceptional cases” and the “regular cases” since they rely on the same RF framework (i.e., the confinement up to 12 PRB and up to 20 PRB is achieved via network’s scheduling restrictions).

Conclusion 1: It is incorrect to add Nrb 12 and Nrb 20 to the transmission bandwidth IEs signalled per cell over Xn and F1

The question therefore is whether there is any other way to signal to a neighbour NG-RAN node that a cell is subject to such special configuration.

For this reason, we quote the following relation captured in TS38.101 between the RF Reference Frequency (FREF) and the ARFCN (NREF):

### 5.4.2 Channel raster #### 5.4.2.1 NR-ARFCN and channel raster The global frequency channel raster defines a set of RF reference frequencies FREF. The RF reference frequency is used in signalling to identify the position of RF channels, SS blocks and other elements. The global frequency raster is defined for all frequencies from 0 to 100 GHz. The granularity of the global frequency raster is ΔFGlobal. RF reference frequencies are designated by an NR Absolute Radio Frequency Channel Number (NR-ARFCN) in the range (0…2016666) on the global frequency raster. The relation between the NR-ARFCN and the RF reference frequency FREF in MHz is given by the following equation, where FREF-Offs and NRef-Offs are given in Table 5.4.2.1-1 and NREF is the NR-ARFCN. FREF = FREF-Offs + ΔFGlobal (NREF – NREF-Offs) Table 5.4.2.1-1: NR-ARFCN parameters for the global frequency raster Frequency range (MHz); ΔFGlobal (kHz); FREF-Offs (MHz); NREF-Offs; Range of NREF. 0 – 3000; 5; 0; 0; 0 – 599999. 3000 – 24250; 15; 3000; 600000; 600000 – 2016666.

It can be deduced that, for cells using band n100 the equation above becomes:

FREF = ΔFGlobal (NREF),

as these cells have a frequency range of less than 3000MHz.

Therefore, assuming that cells using the Nrb 12 and Nrb 20 configuration have a dedicated RF Reference Frequency, the ARFCN for these cells is also dedicated, namely by knowing the ARFCN it is possible to know that the cell is using the 12 PRB or 20 PRB restriction.

Observation1: assuming that cells using the Nrb 12 and Nrb 20 configuration have a dedicated RF Reference Frequency, the ARFCN for these cells is also dedicated, namely by knowing the ARFCN it is possible to know that the cell is using the 12 PRB or 20 PRB restriction.

In order to prove that the ARFCN of such special cells is unique for these cells we need to embark on a discussion that is very much within RAN4´s terms of reference.

The simplest way of representing this is by showing the following diagrams.

Figure 1: Lowest channel synchronization raster for 12 RBs SSB (3MHz) in band n100

Figure 2: First channel synchronization raster for 20 RBs (5MHz) in lowest edge band n100

Figure 1 and Figure 2 are based on a more complete contribution available in R4-2308539. They show the location in frequency of a 3MHz channel cell (Figure 1) and of a 5MHz channel cell (Figure 2) within band n100 and in case the Synchronisation Raster is fixed at 920.73 and 921.45, respectively.

The important point to note is that such Synchronisation Raster frequencies have been allocated by RAN4 only to the special cell configuration cases discussed in this paper (as shown in Table 5.4.3.1-3 of TS38.101). Namely:

- Synchronisation Raster frequency 920.73 MHz is only used by cells using the band n100 with 3MHz channels and the special configuration of Nrb 12 (at Synch Raster frequency of 920.73 MHz)

- Synchronisation Raster frequency 921.45 MHz is only used by cells using the band n100 with 5MHz channels and the special configuration of Nrb 20 (at Synch Raster frequency of 921.45 MHz)

From Figure 1 and Figure 2 it can also be deduced the following:

- For Synchronisation Raster frequency 920.73 MHz, the RF Reference frequency (Channel Raster) is 921

- For Synchronisation Raster frequency 921.45 MHz, the RF Reference frequency (Channel Raster) is 921.9

An additional piece of information to interpret Figure 1 and Figure 2 comes from the LS in RP-230189, sourced by UIC, where UIC requested to consider the 12 and 20RBs cases and where they also shared their plan for the transition from GSM-R to FRMCS. According to this plan, the 12 and 20RBs FRMCS signal shall always be as close as possible to the lower band edge (as shown in Figure 1 and Figure 2), while GSM-R will remain deployed in the upper edge of the band. Based on this information, and considering as well the European Regulation (ECC Decision(20)02) for that band, the closest possible NR carrier for 3 MHz channel BW shall be at frequency position 921 MHz, and for 5 MHz channel BW the NR carrier shall be at frequency position 921.9 MHz, as shown in Figure 1 and Figure 2. Knowing those carrier frequencies, based on RAN1 design of the 12/20RBs and putctured SSB, the corresponding sync rasters shall be at 920.73 MHz for 12 RBs and at 921.45 MHz for 20RBs,

As the Synchronisation Raster Frequencies above are dedicated to the special cell configurations under discussion it can be concluded that the RF Reference Frequencies for these cells are also unique. Namely, cells using the Nrb 12 and Nrb 20 configuration have a dedicated RF Reference Frequency.

Based on the above, and using the formula

FREF = ΔFGlobal (NREF),

We conclude that cells using the Nrb 12 and Nrb 20 configuration have a dedicated ARFCN.

Conclusion 2: By knowing the ARFCN of a cell it is possible to derive whether such cell is one of such cases

- Cell using the band n100 with 3MHz channels and the special configuration of Nrb 12 at Synch Raster frequency of 920.73 MHz,

- Cell using the band n100 with 5MHz channels and the special configuration of Nrb 20 at Synch Raster frequency of 921.45 MHz.

Conclusion 3: There is no need to signal new information over the Xn and F1 interfaces flagging that a cell uses the special Nrb12 or Nrb 20 configuration as this can be deduced from the NR ARFCN of the cell

# 3. Conclusions

Based on the discussion in this paper, we derived the following observations and proposals:

Conclusion 1: It is incorrect to add Nrb 12 and Nrb 20 to the transmission bandwidth IEs signalled per cell over Xn and F1

Observation1: assuming that cells using the Nrb 12 and Nrb 20 configuration have a dedicated RF Reference Frequency, the ARFCN for these cells is also dedicated, namely by knowing the ARFCN it is possible to know that the cell is using the 12 PRB or 20 PRB restriction.

Conclusion 2: By knowing the ARFCN of a cell it is possible to derive whether such cell is one of such cases

Cell using the band n100 with 3MHz channels and the special configuration of Nrb 12 at Synch Raster frequency of 920.73 MHz,

Cell using the band n100 with 5MHz channels and the special configuration of Nrb 20 at Synch Raster frequency of 921.45 MHz.

Conclusion 3: There is no need to signal new information over the Xn and F1 interfaces flagging that a cell uses the special Nrb12 or Nrb 20 configuration as this can be deduced from the NR ARFCN of the cell