3GPP TSG-RAN WG3 Meeting #126	R3-247519

# Orlando, USA, 18-22 November 2024

Agenda Item:	19.2

Source:	Ericsson

Title:	Discussion on RAN3 impacts to support enhancements for CLI handling for SBFD

Document for:	Discussion, Approval

# 1	Introduction

RAN1 has started work on “Evolution of NR duplex operation: Sub-band full duplex (SBFD)”, as per WID in RP-241614.

As part of this WID, RAN3 has been identified as one of the WGs that needs to contribute to solutions on enhancements for CLI handling, as per the excerpt below (from RP-241614):

- Specify enhancements for CLI handling [RAN1, RAN2, RAN3, RAN4] - Information exchange among gNBs, including [RAN3] - Semi-static cell-specific SBFD time and frequency location configuration - Measurement resource configuration, i.e., SSB and/or periodic NZP CSI-RS - Strongest DL beam information - CLI-mitigation request - … - Note: Without dedicated optimization for dynamic/flexible TDD. - … - Note: RAN3 will not specify enhancements to network signalling to support inter-operator coordination for CLI handling - Notes: Following are assumed for information exchange among gNBs - No normative behavior is implied for the gNB receiving the CLI-mitigation request. No need to further send a stop message for CLI mitigation. Detailed signaling can be discussed in RAN3 - The information exchanges are subject to RAN3’s assessment of feasibility and specification impact

Additionally, RAN1 sent an LS to RAN3 in R3-245006, where the objectives quoted above were clarified as follows:

While the WID objective lists only RAN3 as responsible working group, RAN1 has discussed the sub-bullets of the objective and would like to clarify several PHY/L1-related aspects that are not typically discussed in RAN3. RAN1 would like to inform RAN3 (RAN2 is cc’d) of the following RAN1 agreement. Agreement Send an LS to RAN3 with the following information (cc: RAN2) From RAN1 perspective, exchange among gNBs of the configuration info for a set of one or more periodic NZP CSI-RS resources (relevant IE in 38.331 are NZP-CSI-RS-Resource, NZP-CSI-RS-ResourceSet) is supported. Strongest DL beam information in the context of NZP-CSI-RS is defined as a CSI-RS Resource Indicator (CRI) value. CRI is a relative index in the range [1 .. N], where N is the number of CSI-RS resources within a set of resources for which the configuration is provided to a gNB. Strongest DL beam information in the context of SSBs is defined as an SSB index. SSB index is an absolute index among the SSB(s) for which the configuration is provided to a gNB. Note: RAN1 will not discuss information exchange among gNBs for gNB-gNB CLI handling unless there is an LS from RAN3. # 2 Actions To TSG RAN3 ACTION: RAN1 respectfully asks RAN3 to take the above information into account in their further work.

At the last RAN3 meeting a very preliminary discussion was carried out, where the following agreements and to be continued topics were captured:

RAN3 agrees to Exchange the following information over Xn interface and F1 interface:

- Semi-static cell-specific SBFD time and frequency location configuration with cell granularity (FFS on container or explicit IEs over XnAP and F1AP)

- Measurement resource configuration

- Strongest DL beam information (FFS on the details)

- CLI-mitigation request (FFS on the details)

In this contribution an analysis of the work RAN3 needs to carry out to support enhancements for CLI handling is made and conclusions on a way forward are derived. A TP for TS 38.423 is provided in R3-247520.

# 2	Discussion

## 2.1	Semi-static cell-specific SBFD time and frequency location configuration

The first objective to analyse concerns the exchange of following information between gNBs:

–	Semi-static cell-specific SBFD time and frequency location configuration

According to the discussions in RAN1#118 concerning this point, the following was agreed:

Agreement For configuration of SBFD symbols within a TDD-UL-DL pattern period, the following parameters are supported - A starting slot index - A starting symbol index within the starting slot - An ending slot index - An ending symbol index within the ending slot Working Assumption For cell-specific configuration of frequency locations of SBFD UL subband, for each SCS configuration in SCS-SpecificCarrierList for UL, starting RB and bandwidth of SBFD UL subband are indicated by a RIV-based indication as defined in 38.214 setting NBWPsize=275. For cell-specific configuration of frequency locations of SBFD DL subband(s), for each SCS configuration in SCS-SpecificCarrierList for DL, starting RB and bandwidth of each SBFD DL subband are indicated by a RIV-based indication as defined in 38.214 setting NBWPsize=275. - One or two SBFD DL subbands can be configured

With respect to the information agreed by RAN1, it can be noted that similar information is, already today, exchanged over the F1AP and XnAP via the TDD UL-DL Configuration Common NR IE which contains the tdd-UL-DL-ConfigurationCommon IE contained in the ServingCellConfigCommon IE defined in TS 38.331

Indeed, the RRC IE in question includes the following:

#### – TDD-UL-DL-ConfigCommon The IE TDD-UL-DL-ConfigCommon determines the cell specific Uplink/Downlink TDD configuration. TDD-UL-DL-ConfigCommon information element -- ASN1START -- TAG-TDD-UL-DL-CONFIGCOMMON-START TDD-UL-DL-ConfigCommon ::= SEQUENCE { referenceSubcarrierSpacing SubcarrierSpacing, pattern1 TDD-UL-DL-Pattern, pattern2 TDD-UL-DL-Pattern OPTIONAL, -- Need R ... } TDD-UL-DL-Pattern ::= SEQUENCE { dl-UL-TransmissionPeriodicity ENUMERATED {ms0p5, ms0p625, ms1, ms1p25, ms2, ms2p5, ms5, ms10}, nrofDownlinkSlots INTEGER (0..maxNrofSlots), nrofDownlinkSymbols INTEGER (0..maxNrofSymbols-1), nrofUplinkSlots INTEGER (0..maxNrofSlots), nrofUplinkSymbols INTEGER (0..maxNrofSymbols-1), ..., [[ dl-UL-TransmissionPeriodicity-v1530 ENUMERATED {ms3, ms4} OPTIONAL -- Need R ]] } -- TAG-TDD-UL-DL-CONFIGCOMMON-STOP -- ASN1STOP TDD-UL-DL-ConfigCommon field descriptions. referenceSubcarrierSpacing Reference SCS used to determine the time domain boundaries in the UL-DL pattern which must be common across all subcarrier specific carriers, i.e., independent of the actual subcarrier spacing using for data transmission. Only the following values are applicable depending on the used frequency: FR1: 15, 30, or 60 kHz FR2-1: 60 or 120 kHz FR2-2: 120, 480, or 960 kHz The network configures a not larger than any SCS of configured BWPs for the serving cell. The network or SL-PreconfigGeneral configures a not larger than the SCS of (pre-)configured SL BWP.See TS 38.213 [13], clause 11.1.. TDD-UL-DL-Pattern field descriptions. dl-UL-TransmissionPeriodicity Periodicity of the DL-UL pattern, see TS 38.213 [13], clause 11.1. If the dl-UL-TransmissionPeriodicity-v1530 is signalled, UE shall ignore the dl-UL-TransmissionPeriodicity (without suffix).. nrofDownlinkSlots Number of consecutive full DL slots at the beginning of each DL-UL pattern, see TS 38.213 [13], clause 11.1. In this release, the maximum value for this field is 320.. nrofDownlinkSymbols Number of consecutive DL symbols in the beginning of the slot following the last full DL slot (as derived from nrofDownlinkSlots). The value 0 indicates that there is no partial-downlink slot. (see TS 38.213 [13], clause 11.1).. nrofUplinkSlots Number of consecutive full UL slots at the end of each DL-UL pattern, see TS 38.213 [13], clause 11.1. In this release, the maximum value for this field is 320.. nrofUplinkSymbols Number of consecutive UL symbols in the end of the slot preceding the first full UL slot (as derived from nrofUplinkSlots). The value 0 indicates that there is no partial-uplink slot. (see TS 38.213 [13], clause 11.1)..

Therefore, the most logical direction with respect to this objective would be that RAN2 defined an IE similar to the tdd-UL-DL-ConfigurationCommon IE, for SBFD, and that this IE could be transparently signalled over the F1 and Xn interfaces as part of the Served Cell Information NR IE.(Note: the F1AP IE is called Served Cell Information. However, we use the name “Served Cell Information NR” to refer to both the F1 and Xn IEs).

It is worth noting that RAN2 is already discussing this topic, as per agreements taken at RAN2#127, quoted below:

Cell-specific SBFD time/frequency configuration is provided by SIB1 (or via dedicated signalling to covey cell specific configuration). FFS on UE specific dedicated RRC configuration if needed, pending on RAN1 progress.

RAN3 has also taken the following agreement and FFS statements:

- Semi-static cell-specific SBFD time and frequency location configuration with cell granularity (FFS on container or explicit IEs over XnAP and F1AP)

The option of defining explicit IEs over the Xn interface seems inconsistent with how RAN3 has proceeded so far for similar matters. Defining explicit IEs would imply the risk of diverging from what RAN2 will define. It is worth noting that it is essential that the RAN and the UE share the same understsnding of the SBFD time/frequency configuration details, therefore the most robust and safe option would be to reuse what RAN2 will define and to exchange it over Xn in a transparent container.

Therefore, it is proposed that RAN3 agrees to leave to RAN2 the definition of “Semi-static cell-specific SBFD time and frequency location configuration” as part of RRC information and that RAN3 re-uses the newly defined RRC IEs transparently over the F1 and Xn interfaces for inter node communication.

In light of the above, the following is proposed.

Proposal 1: Capture the following agreement:

- RAN3 agrees to include in the Served Cell Information NR IE the RRC IEs to be defined by RAN2, specifying “Semi-static cell-specific SBFD time and frequency location configuration”, and to signal them transparently over the F1 and Xn interfaces for inter node communication.

## 2.2	Measurement resource configuration, i.e., SSB and/or periodic NZP CSI-RS

The second objective to analyse concerns the exchange of following information between gNBs:

–	Measurement resource configuration, i.e., SSB and/or periodic NZP CSI-RS

With respect to this objective, RAN1 specifies in their LS that:

“From RAN1 perspective, exchange among gNBs of the configuration info for a set of one or more periodic NZP CSI-RS resources (relevant IE in 38.331 are NZP-CSI-RS-Resource, NZP-CSI-RS-ResourceSet) is supported.”

With the above, RAN1 states that configuration for the NZP-CSI-RS signals is already specified in TS 38.331 as part of the NZP-CSI-RS-Resource IE and NZP-CSI-RS-ResourceSet IE. One point to note is that the NZP-CSI-RS-ResourceSet IE points to multiple instances of the NZP-CSI-RS-Resource IE, each identified by means of a NZP-CSI-RS-ResourceId IE, as it can be seen below:

#### NZP-CSI-RS-ResourceSet The IE NZP-CSI-RS-ResourceSet is a set of Non-Zero-Power (NZP) CSI-RS resources (their IDs) and set-specific parameters. NZP-CSI-RS-ResourceSet information element -- ASN1START -- TAG-NZP-CSI-RS-RESOURCESET-START NZP-CSI-RS-ResourceSet ::= SEQUENCE { nzp-CSI-ResourceSetId NZP-CSI-RS-ResourceSetId, nzp-CSI-RS-Resources SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourcesPerSet)) OF NZP-CSI-RS-ResourceId, repetition ENUMERATED { on, off } OPTIONAL, -- Need S aperiodicTriggeringOffset INTEGER(0..6) OPTIONAL, -- Need S trs-Info ENUMERATED {true} OPTIONAL, -- Need R ..., [[ aperiodicTriggeringOffset-r16 INTEGER(0..31) OPTIONAL -- Need S ]], [[ pdc-Info-r17 ENUMERATED {true} OPTIONAL, -- Need R cmrGroupingAndPairing-r17 CMRGroupingAndPairing-r17 OPTIONAL, -- Need R aperiodicTriggeringOffset-r17 INTEGER (0..124) OPTIONAL, -- Need S aperiodicTriggeringOffsetL2-r17 INTEGER(0..31) OPTIONAL -- Need R ]], [[ resourceType-r18 ENUMERATED {periodic} OPTIONAL -- Cond LTM ]] } CMRGroupingAndPairing-r17 ::= SEQUENCE { nrofResourcesGroup1-r17 INTEGER (1..7), pair1OfNZP-CSI-RS-r17 NZP-CSI-RS-Pairing-r17 OPTIONAL, -- Need R pair2OfNZP-CSI-RS-r17 NZP-CSI-RS-Pairing-r17 OPTIONAL -- Need R } NZP-CSI-RS-Pairing-r17 ::= SEQUENCE { nzp-CSI-RS-ResourceId1-r17 INTEGER (1..7), nzp-CSI-RS-ResourceId2-r17 INTEGER (1..7) } -- TAG-NZP-CSI-RS-RESOURCESET-STOP -- ASN1STOP

It would therefore suffice to signal the NZP-CSI-RS-ResourceSet IE and a list of NZP-CSI-RS-Resource IE as specified in TS38.331 over the F1 and Xn information as part of the Served Cell Information NR IE to achieve inter node exchange of such configurations.

Proposal 2: RAN3 to agree that, for inter gNB signalling of “Measurement resource configuration, i.e., SSB and/or periodic NZP CSI-RS”, the NZP-CSI-RS-ResourceSet IE defined in TS38.331 and a list of NZP-CSI-RS-Resource IE defined in TS38.331 will be included in the Served Cell Information NR IE and signalled transparently over the F1AP and XnAP.

The objective also mentions inter node signalling of SSB resource configuration. We note that this information is already contained in the Measurement Timing Configuration IE signalled as part of the Served Cell Information NR IE over F1 and Xn, where the IE contains the MeasurementTimingConfiguration IE defined in TS 38.331. The excerpt below shows the content of the MeasurementTimingConfiguration IE in TS38.331, which contains SSB resource configuration information:–	MeasurementTimingConfiguration

The MeasurementTimingConfiguration message is used to convey assistance information for measurement timing. Direction: en-gNB to eNB, eNB to en-gNB, gNB to gNB, ng-eNB to gNB, gNB to ng-eNB, ng-eNB to ng-eNB, gNB DU to gNB CU, and gNB CU to gNB DU. MeasurementTimingConfiguration message -- ASN1START -- TAG-MEASUREMENT-TIMING-CONFIGURATION-START MeasurementTimingConfiguration ::= SEQUENCE { criticalExtensions CHOICE { c1 CHOICE{ measTimingConf MeasurementTimingConfiguration-IEs, spare3 NULL, spare2 NULL, spare1 NULL }, criticalExtensionsFuture SEQUENCE {} } } MeasurementTimingConfiguration-IEs ::= SEQUENCE { measTiming MeasTimingList OPTIONAL, nonCriticalExtension MeasurementTimingConfiguration-v1550-IEs OPTIONAL } MeasurementTimingConfiguration-v1550-IEs ::= SEQUENCE { campOnFirstSSB BOOLEAN, psCellOnlyOnFirstSSB BOOLEAN, nonCriticalExtension MeasurementTimingConfiguration-v1610-IEs OPTIONAL } MeasurementTimingConfiguration-v1610-IEs ::= SEQUENCE { csi-RS-Config-r16 SEQUENCE { csi-RS-SubcarrierSpacing-r16 SubcarrierSpacing, csi-RS-CellMobility-r16 CSI-RS-CellMobility, refSSBFreq-r16 ARFCN-ValueNR }, nonCriticalExtension SEQUENCE {} OPTIONAL } MeasTimingList ::= SEQUENCE (SIZE (1..maxMeasFreqsMN)) OF MeasTiming MeasTiming ::= SEQUENCE { frequencyAndTiming SEQUENCE { carrierFreq ARFCN-ValueNR, ssbSubcarrierSpacing SubcarrierSpacing, ssb-MeasurementTimingConfiguration SSB-MTC, ss-RSSI-Measurement SS-RSSI-Measurement OPTIONAL } OPTIONAL, ..., [[ ssb-ToMeasure SSB-ToMeasure OPTIONAL, physCellId PhysCellId OPTIONAL ]] } -- TAG-MEASUREMENT-TIMING-CONFIGURATION-STOP -- ASN1STOP

Proposal 3: RAN3 to confirm that SSB resource configuration is already signalled as part of the Measurement Timing Configuration IE contained in the Served Cell Information NR IE over F1 and Xn.

During the last RAN3 meeting the following was captured:

RAN3 agrees to Exchange the following information over Xn interface and F1 interface:

- Measurement resource configuration

Therefore, RAN3 is ready to make further agreements as per proposals 2 and Proposal 3 in this respect.

## 2.3	Strongest DL beam information and CLI mitigation request

The third objective to analyse concerns the exchange of following information between gNBs:

–	Strongest DL beam information

With respect to this objective, RAN1 specifies in their LS that:

“Strongest DL beam information in the context of NZP-CSI-RS is defined as a CSI-RS Resource Indicator (CRI) value. CRI is a relative index in the range [1 .. N], where N is the number of CSI-RS resources within a set of resources for which the configuration is provided to a gNB. Strongest DL beam information in the context of SSBs is defined as an SSB index. SSB index is an absolute index among the SSB(s) for which the configuration is provided to a gNB.”

The strongest DL beam information is an indication sent from a victim gNB to an aggressor gNB of the strongest DL beam detected by the victim gNB. Such indication from victim to aggressor gNB helps taking actions towards CLI mitigation. However, it should be pointed out that the WID is very clear in stating that:

“No normative behavior is implied for the gNB receiving the CLI-mitigation request. No need to further send a stop message for CLI mitigation. Detailed signaling can be discussed in RAN3”.

The above agreement is valid for the “CLI-mitigation request”, but the concept is the same for reception of Strongest DL Beam information, namely a node that receives this information may use it for CLI mitigation although the specific CLI actions (and node behaviour) shall not be specified. Therefore, even though a Strongest DL Beam information may be signalled from one gNB to another, there should be no description in the standard of how the receiving gNB will use this information to mitigate CLI.

Conclusion 1: While the strongest DL beam information is signalled from a gNB to another gNB to indicate the strongest beam detected and to possibly help in taking CLI mitigation actions, RAN3 shall not specify any receiver behaviour concerning CLI mitigation actions.

The strongest DL beam information can be provided with respect to a detected CSI-RS resource or with respect to an SSB. As expressed in the LS from RAN1, a CRI is used in case the strongest DL beam information is provided for a CSI-RS resource. In this case the CRI is an Index (INTEGER) in the range from 1 to N, where “N is the number of CSI-RS resources within a set of resources for which the configuration is provided to a gNB”.

The NZP-CSI-RS resources that can be exchanged between gNBs follow the definitions in the NZP-CSI-RS-Resource IE and NZP-CSI-RS-ResourceSet IE in TS 38.331. When looking at these IEs in TS 38.331 it can be easily deduced that the maximum number of resources per set is defined as follows:

maxNrofNZP-CSI-RS-ResourcesPerSet = 64

Therefore, in case the Strongest DL Beam identifies NZP-CSI-RS resources, the CRI can be an INTEGER (1..64).

On the other hand, in case the Strongest DL Beam identifies SSB resources, the information to exchange should be an SSB index, which is already defined over Xn as SSB Index: INTEGER (0..63). It is of course the case that if the NZP-CSI-RS-Resources in the set is less than 64, the CRI shall not be selected to a number higher than the number of resources in the set.

Proposal 4: It is proposed to represent the CRI as an INTEGER (1..64), where the CRI shall never exceed the number of NZP-CSI-RS-Resources in the set, and to represent an SSB Index by means of the legacy Xn SSB-Index IE

Given that both the CRI and the SSB Index that represent the strongest DL beam information are very small pieces of information, their signalling between gNBs could be achieved by adding this information into existing procedures. Indeed, defining a completely new procedure would incur very high impact when considering the very small amount of information to transfer. It is therefore proposed to signal the strongest DL Beam information, consisting of CRI or SSB Index, by means of the XnAP NG-RAN NODE CONFIGURATION UPDATE message.

Proposal 5: RAN3 to agree to signal the strongest DL Beam information per cell, consisting of CRI or SSB Index, by means of the XnAP NG-RAN NODE CONFIGURATION UPDATE message.

The fourth objective to analyse concerns the exchange of following information between gNBs:

–	CLI-mitigation request

We have already discussed how a CLI mitigation request does not require any normative behaviour at the gNB receiving it. Therefore, a CLI mitigation request is simply an indication from a victim gNB to an aggressor gNB that CLI mitigation is needed for a given cell.

Another important piece of guidance from RAN1 is that there is “No need to further send a stop message for CLI mitigation”.

It is therefore proposed that the CLI mitigation request is encoded as an ENUMERATED per cell in the NG-RAN Configuration Update message.

Proposal 6: RAN3 to agree to signal the CLI Mitigation Request as an ENUMERATED per cell included in the XnAP: NG-RAN Node Configuration Update message

At the last RAN3 meeting the following was captured:

RAN3 agrees to Exchange the following information over Xn interface and F1 interface:

- Strongest DL beam information (FFS on the details)

- CLI-mitigation request (FFS on the details)

Our understanding for adding an FFS for these two items is that some companies wanted to discuss whether a Strongest DL beam Information and CLI itigation request could be also “implicit” and for that not explicitly signalled over the F1 and Xn.

Such approach is presented in R3-245562, there the two following options are presented:

- Option 1 (explicit CLI Mitigation Request): Victim gNB first exchanges the CLI-RS configuration (either for SSB or NZP-CSI-RS or both) with the aggressor gNB. The aggressor gNB measures the channel on the corresponding resources. Once inter-cell CLI is detected, the victim gNB sends a CLI Mitigation Request to the aggressor gNB containing cell id. Upon the reception, the aggressor gNB starts applying the CLI mitigation scheme (i.e., beam nulling). - Option 2 (implicit CLI Mitigation Request): There is no initial CLI measurement resource exchange. When the victim gNB detects increased CLI level based on UL performance degradation, it triggers CLI measurement resource exchange and activation of CLI-RS transmission. Upon the reception of the CLI-RS configuration, the aggressor gNB is expected to perform measurements and also apply the beam nulling scheme. In this way the signalling of CLI measurement resource information is an implicit trigger of CLI mitigation.

We understand that, while the options above refer to the CLI Mitigation Request, they were discussed at the last RAN3 meeting to also cover the case of Strongest DL Beam Information.

We note that Option 1 is in line with the guidance sent by RAN1 in the LS in R3-245006, as such LS describes the CLI Mitigation Requests and Strongest DL Beam Informaiton as explicit pieces of information signalled between gNBs.

Observation 1: Signalling of the CLI Mitigation Request and Strongest DL Beam Information as explicit IEs over the F1 and Xn interfaces is in line with the guidance from RAN1 in their LS in R3-245006

We also note that Option 2 above implies that the reference signals used to evaluate whether CLI interference is present or not seem to be considered as dedicated to CLI mitigation. Namely, they are only activated when CLI detection is needed. 
We believe that the standard shall not mandate such approach. It should be possible to reuse reference signals already in use for other purposes also for the detection of CLI, hence such reference signals would not need to be activated “ad hoc”. As a consequence, an explicit signalling of the CLI Mitigation Requests and Strongest DL Beam Informaiton would be needed.
Further, Option 2 seems to be based on an initial detection of CLI based on UL performance degradation, but how would it be possible to know from which gNB/cell/beam such CLI is generated? Without knowing this information it is not possible to trigger signalling of the CLI measurement resource exchange and activation of CLI-RS transmission toward the aggressor node, as Option 2 describes.

In conclusion, it is proposed that the FFSs are removed from the following agreements and that an explicit signalling of CLI Mitigation Requests and Strongest DL Beam Informaiton is supported over F1 and Xn.

Proposal 7: RAN3 agrees to explicitly exchange the following information over Xn interface and F1 interface as separate IEs:

-	Strongest DL beam information

-	CLI-mitigation request

## 2.5	Overall Solution for CLI mitigation handling

In light of the above analysis, the overall solution proposed to be agreed in RAN3 for CLI mitigation handling consists of the following:

- Include in the Served Cell Information NR over F1AP and XnAP the “Semi-static cell-specific SBFD time and frequency location configuration” to be defined by RAN2 in TS 38.331. This information is to be passed transparently.

- Include in the Served Cell Information NR over F1AP and XnAP a ñlist of NZP-CSI-RS-Resource IEs and NZP-CSI-RS-ResourceSet IE defined in TS 38.331. This information is to be passed transparently.

- Include in the XnAP NG-RAN NODE CONFIGURATION UPDATE message, on a per-cell basis, the Strongest DL Information IE, defined as a CHOICE between the following IEs:

  - CRI: INTEGER (1..64).

  - SSB Index: INTEGER (0..63).

- Include in the XnAP NG-RAN NODE CONFIGURATION UPDATE message, on a per-cell basis, the CLI Mitigation Request defined as an ENUMERATED with value “True”.

We note that at the last RAN3 meeting a number of companies proposed to support the exchange of the CLI Mitigation Request and Strongest DL Beam Information by means of the F1/Xn: Resource Status Request/Response/Update messages.

Such approach is, in our view, not suitable for the CLI mitigation solution defined by RAN1, here are some reasons to justify out position.

The CLI mitigation solution defined by RAN1 is based on a victim gNB signalling to an aggressor gNB an indication that CLI is detected. This happens by signalling the CLI Mitigation Request and/or the Strongest DL Beam Informaiton. Such design implies that the victim gNB shall be free to trigger signlaling towards an aggressor gNB whenever CLI is detected and it is in need of mitigation.

However, if the F1/Xn: Resource Status Request/Response/Update messages are used, a victim gNB will only be able to signal the CLI Mitigation Request and Strongest DL Beam Informaiton to the aggressor gNB if the aggressor has initiated the Resource Status Request/Response procedure. Namely, if the aggressor does not trigger such procedure, no CLI indication can be reported by the victim gNB and no CLI mitigation actions can be triggered at all, deniying the whole purpose of CLI mitigation support.

Observation 2: Using the F1/Xn: Resource Status Request/Response/Update messages for CLI Mitigation Requests and Strongest DL Beam Information prevents a victim gNB to autonomously flag the presence of strong CLI and trigger CLI mitigation actions

Moreover, using the F1/Xn: Resource Status Request/Response/Update messages implies that the CLI Mitigation Request and Strongest DL Beam Informaiton have to be reported periodically. This mechanism does not fit the nature of CLI occurrence, which is volatile, i.e. it might occur for e.g. 1 second, then disappear, then occur again after one minute and so on. Hence the periodic reporting of such information does not fit this process and it may generate the following drawbacks:

- Reporting of short lived CLI events is missed due to course periodicity for the Resource Status Update messages

- Resource Status Reporting messages will be signalled even when there is no detected CLI. This could happen often, causing an unnecessary waste of signalling resources and increase of signalling load

Observation 3: Using the F1/Xn: Resource Status Request/Response/Update messages for CLI Mitigation Requests and Strongest DL Beam Information incurs in missing the occurrence of short lived CLI events and it generates unnecessary signalling when there is no need for CLI mitigation actions

Finally, the WID in [1] is very clear in stating that there is “No need to further send a stop message for CLI mitigation.”. Reusing the F1/Xn: Resource Status Request/Response/Update implies a “start” and a “stop” to the CLI mitigation process, which is not needed.

Observation 4: the WID in RP241614 clearly states that there is “No need to further send a stop message for CLI mitigation.”, while using the F1/Xn: Resource Status Request/Response/Update implies a “start” and a “stop” to the CLI mitigation process. This causes unnecessary complexity without a need for it.

In concludsion, we believe that a better signalling solution for CLI mitigation consists in reusing the Xn: NG-RAN Node Configuraiton Update procedures and the F1: gNB-DU/gNB-CU Configuration Update procedures for the exchange of CLI mitigation repated information.

Proposal 8: RAN3 to agree that CLI mitigation information exchange is supported by means of the Xn: NG-RAN Node Configuraiton Update procedures and the F1: gNB-DU/gNB-CU Configuration Update procedures

# 3	Conclusion

In this paper an analysis of the RAN3´s objectives on Evolution of enhancements for CLI handling, as part of the WI “NR duplex operation: Sub-band full duplex (SBFD)”, has been carried out.

The paper derived the following observations, proposals and conclusions:

Proposal 1: Capture the following agreement:

•	RAN3 agrees to include in the Served Cell Information NR IE the RRC IEs to be defined by RAN2, specifying “Semi-static cell-specific SBFD time and frequency location configuration”, and to signal them transparently over the F1 and Xn interfaces for inter node communication.

Proposal 2: RAN3 to agree that, for inter gNB signalling of “Measurement resource configuration, i.e., SSB and/or periodic NZP CSI-RS”, the NZP-CSI-RS-ResourceSet IE defined in TS38.331 and a list of NZP-CSI-RS-Resource IE defined in TS38.331 will be included in the Served Cell Information NR IE and signalled transparently over the F1AP and XnAP.

Proposal 3: RAN3 to confirm that SSB resource configuration is already signalled as part of the Measurement Timing Configuration IE contained in the Served Cell Information NR IE over F1 and Xn.

Conclusion 1: While the strongest DL beam information is signalled from a gNB to another gNB to indicate the strongest beam detected and to possibly help in taking CLI mitigation actions, RAN3 shall not specify any receiver behaviour concerning CLI mitigation actions.

Proposal 4: It is proposed to represent the CRI as an INTEGER (1..64), where the CRI shall never exceed the number of NZP-CSI-RS-Resources in usethe set, and to represent an SSB Index by means of the legacy Xn SSB- Index IE Proposal 5: RAN3 to agree to signal the strongest DL Beam information per cell, consisting of CRI or SSB Index, by means of the XnAP NG-RAN NODE CONFIGURATION UPDATE message.

Proposal 6: RAN3 to agree to signal the CLI Mitigation Request as an ENUMERATED per cell included in the XnAP: NG-RAN Node Configuration Update message

Observation 1: Signalling of the CLI Mitigation Request and Strongest DL Beam Information as explicit IEs over the F1 and Xn interfaces is in line with the guidance from RAN1 in their LS in R3-245006

Proposal 7: RAN3 agrees to explicitly exchange the following information over Xn interface and F1 interface as separate IEs:

-	Strongest DL beam information

-	CLI-mitigation request Observation 2: Using the F1/Xn: Resource Status Request/Response/Update messages for CLI Mitigation Requests and Strongest DL Beam Information prevents a victim gNB to autonomously flag the presence of strong CLI and trigger CLI mitigation actions

Observation 3: Using the F1/Xn: Resource Status Request/Response/Update messages for CLI Mitigation Requests and Strongest DL Beam Information incurs in missing the occurrence of short lived CLI events and it generates unnecessary signalling when there is no need for CLI mitigation actions

Observation 4: the WID in RP241614 clearly states that there is “No need to further send a stop message for CLI mitigation.”, while using the F1/Xn: Resource Status Request/Response/Update implies a “start” and a “stop” to the CLI mitigation process. This causes unnecessary complexity without a need for it.

Proposal 8: RAN3 to agree that CLI mitigation information exchange is supported by means of the Xn: NG-RAN Node Configuraiton Update procedures and the F1: gNB-DU/gNB-CU Configuration Update procedures

A TP mirroring the above proposals is available for agreement in R3-247520.

# 4	References

- RP-241614, WID on Evolution of NR duplex operation: Sub-band full duplex (SBFD)