3GPP TSG-RAN WG3 Meeting #126	R3-247529

Orlando, USA, 18th- 22nd November, 2024

Agenda item:	20.2

Source: 	Jio

Title: 	AI/ML-Assisted Positioning Focused on Cases 3a and 3b

Document for:     Discussion and Decision

# 1.	Introduction

The objectives for AI/ML positioning within the Work Item on artificial intelligence and machine learning for the NR air interface are as follows [1]:

Agenda:

Positioning accuracy enhancements, encompassing [RAN1/RAN2/RAN3]:

- Direct AI/ML positioning:

(1st priority) Case 1: UE-based positioning with UE-side model, direct AI/ML positioning

(2nd priority) Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning

(1st priority) Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning

- AI/ML assisted positioning

(2nd priority) Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning

(1st priority) Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning

- Specify necessary measurements, signalling/mechanism(s) to facilitate LCM operations specific to the Positioning accuracy enhancements use cases, if any

- Investigate and specify the necessary signalling of necessary measurement enhancements (if any)

- Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE for relevant positioning sub use cases

Note: RAN1 will not discuss any proposals targeting 2nd priority issues in Q4 of 2024.

RAN3 to discuss Cases 3a/3b are NG-RAN node assisted/network-based positioning methods.

# 2.	Discussion

As per RAN3#125bis below items were discussed:

RAN3#125bis:

Consider aggregated gNB case first, then split architecture.

The existing LCS framework is re-used for AI/ML positioning.

Case 3b:

Re-using the Measurement procedure defined in NRPPa to support requesting and reporting the related measurements to LMF for Case 3b. The details of the related measurements are still pending and subject to further discussion in RAN1.

NG-RAN gets measurements (i.e., part A information as called by RAN1) from TRP and provides them to LMF.

The details of the measurements are still pending and subject to further discussion in RAN1.

Case 3a:

The AI/ML model (inference and training functions) is at the gNB.

LMF provides ground truth label and related data (i.e., part B information as called by RAN1) to NG-RAN.

FFS on the content of the ground truth label and related data pending on RAN1 progress. FFS on the signalling design.

How the LMF obtains the ground truth label and related data information is outside of RAN3 scope.

NG-RAN obtains measurements from TRP (i.e., part A information as called by RAN1).

The details of the measurements are still pending and subject to further discussion in RAN1.

WA: RAN3 will proceed with Option A: NG-RAN node performs monitoring metric calculation for its own model.

Background:

In the RAN1 work, two schemes are considered, namely AI/ML direct positioning and AI/ML assisted positioning. We summarize these schemes next. Figure 1 considers AI/ML Direct positioning and Figure 2 considers AI/ML Assisted Positioning.

Case 1: UE-based positioning with UE-side model, direct AI/ML positioning

The UE directly determines its own location using an AI/ML model hosted on the device.

Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning

The UE provides measurements to the LMF, which uses an AI/ML model to compute the UE's location.

Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning

NG-RAN nodes send measurements to the LMF, where an AI/ML model directly infers the UE's position.

Case 2a: UE-assisted/LMF-based positioning with UE-side model

The UE determines its location using an on-device AI/ML model with assistance from the LMF.

Case 3a: NG-RAN node assisted positioning with gNB-side model

The gNB hosts an AI/ML model that assists in positioning the UE using network measurements.

This document aims to enhance support for AI/ML-assisted and direct AI/ML positioning within RAN3's scope by focusing on Cases 3a and 3b. By concisely stating the proposals and providing detailed descriptions, we address critical factors such as data synchronization, privacy, interoperability, and alignment with RAN1's work. This ensures effective decision-making and specification processes.

This approach enables RAN3 to develop robust, efficient, and future-proof specifications that facilitate the integration of AI/ML technologies into network positioning systems, ultimately improving positioning accuracy and performance in diverse environments.

Proposal 1: RAN3 Should Prioritize Enhancing Specifications for AI/ML-Assisted Positioning, Specifically Concentrating on Case 3a (gNB-Side Model) and Case 3b (LMF-Side Model)

RAN3 should focus on enhancing specifications for AI/ML-assisted positioning by targeting:

- Case 3a (gNB-Side Model): In this scenario, the AI/ML model is implemented at the gNB (Next Generation Node B). The gNB collects necessary data for model training and inference, processes input data through the AI/ML model, and performs performance monitoring. This case enhances positioning accuracy using AI/ML techniques hosted directly on the gNB.

- Case 3b (LMF-Side Model): Here, the AI/ML model is hosted at the LMF (Location Management Function). The LMF directly infers the UE's location using measurements provided by NG-RAN nodes. It is responsible for model training, data collection, label generation, and performance monitoring. This centralizes AI/ML processing at the LMF, leveraging network measurements for positioning.

For both cases, RAN3 should concentrate on the following key areas:

- Data Collection: Defining mechanisms for effective data collection necessary for model training and inference, including channel measurements, quality indicators, and associated timestamps. Ensuring efficient, accurate, and secure data collection is essential for the effectiveness of AI/ML models.

- Model Input and Inference: Establishing procedures for processing input data for AI/ML models and reporting positioning results, ensuring compatibility and efficiency. This includes defining data formatting, communication protocols, and processing methods within the network to support AI/ML inference.

- Performance Monitoring: Developing methods for monitoring AI/ML model performance to maintain or improve positioning accuracy over time. This involves defining performance metrics, monitoring processes, and responsibilities for entities involved in performance monitoring.

Alignment with RAN1's agreements on measurement types and data requirements is crucial to ensure consistency and interoperability across the network. Accurate synchronization of data samples (Part A: channel measurements) and labels (Part B: ground-truth data) is essential for effective model training, especially when generated by different entities like the gNB and LMF. Utilizing existing positioning and data exchange protocols will streamline the integration of AI/ML functionalities and reduce the need for extensive specification changes.

Proposal 2: Define Comprehensive Procedures for Data Collection and Label Handling in Cases 3a and 3b, Including Enabling gNB Autonomy in Label Generation and Establishing Protocols for Label Provision from LMF

In Case 3a:

- gNB Autonomy in Label Generation: If the gNB can autonomously generate labels (e.g., determining UE positions without external input), this simplifies data collection and training, reducing dependency on the LMF. This autonomy eliminates the need for additional specification changes to NRPPa (NG-RAN Positioning Protocol A), streamlining the implementation process.

- Procedures for LMF-Provided Labels: When the gNB cannot generate labels autonomously, RAN3 should establish procedures for the LMF to provide ground-truth labels to the gNB. This involves defining secure and efficient transfer protocols for labels and associated data (e.g., timestamps), possibly requiring enhancements to NRPPa or NGAP (NG Application Protocol). Accurate synchronization between labels and measurements is crucial for effective model training.

In Case 3b:

- Measurement Data Transfer: RAN3 should define procedures for NG-RAN nodes to collect measurement data (Part A) and transfer it to the LMF, ensuring data integrity and timely delivery. This includes specifying the handling of measurement data types, inclusion of quality indicators, and synchronization mechanisms to align labels (Part B) with corresponding measurements accurately.

Privacy and Security: These procedures must ensure privacy and security, as transferring labels and measurement data may involve sensitive location information. Implementing stringent privacy measures, such as encryption and authentication, is essential to protect user data and comply with regulatory requirements.

Data Synchronization: Accurate synchronization is critical for effective model training, especially when data is collected by different entities. Clarifying label generation responsibilities prevents duplication of efforts and ensures efficient use of network resources.

Proposal 3: Enhance Measurement Procedures, Including NRPPa, to Accommodate the Specific Needs of AI/ML-Based Positioning in Cases 3a and 3b

RAN3 should adapt measurement procedures to support:

- New Measurement Types: Accommodate sample-based and path-based measurements as investigated by RAN1, including the possible inclusion of phase information to improve positioning accuracy. This requires defining how such data is collected, processed, and communicated within the network.

- Flexible SRS Configurations: Enable the LMF to specify detailed SRS (Sounding Reference Signal) configurations required for training models valid for various settings. This flexibility ensures consistent data collection across different scenarios and allows AI/ML models to be trained and inferred under varying configurations.

- Quality Indicators and Precise Timestamps: Include quality indicators in measurement reports to inform on data reliability, and utilize existing Information Elements (IEs) like "Time Stamp" in TS 38.455 for precise timing information. Accurate timestamps are crucial for synchronizing measurements with labels.

Alignment with RAN1:

Alignment with RAN1's progress is essential, particularly regarding parameter definitions (e.g., Nt, Nt', k). Enhancing data quality and ensuring accurate synchronization between measurements and labels will improve the effectiveness of AI/ML models.

Reusing Existing Procedures:

Reusing existing procedures where possible promotes consistency and reduces the need for extensive specification changes, facilitating smoother integration of AI/ML technologies.

Proposal 4: Clearly Define the Responsibilities of Network Entities for Performance Monitoring, Functionality Management, and Model Placement in Cases 3a and 3b

For Case 3a (gNB-Side Model):

- Performance Monitoring: The gNB should perform monitoring metric calculations for its AI/ML model, leveraging on-node data to reduce latency and improve responsiveness. Privacy concerns must be addressed when sensitive data is processed at the gNB, ensuring compliance with data protection regulations.

- Functionality Management: The gNB is responsible for model selection, activation, deactivation, switching, and fallback operations, ensuring efficient management where the model resides. Interoperability in multi-vendor environments must be considered, as gNB components may come from different vendors.

For Case 3b (LMF-Side Model):

- Performance Monitoring: The LMF should calculate performance metrics, as it handles model training and inference. Centralizing performance monitoring at the LMF simplifies management and ensures consistent monitoring across the network.

- Functionality Management: The LMF manages model operations based on performance monitoring, maintaining consistent positioning performance network-wide.

Balancing Factors: Deciding on node responsibilities involves balancing computational load, network latency, user privacy, and interoperability. Existing procedures can often be utilized for functionality control and performance monitoring, minimizing the need for new specifications.

Proposal 5: Define Procedures for Secure and Efficient Transfer of Measurement Data Between Network Entities in Cases 3a and 3b, and Consider Network Conditions Affecting AI/ML Model Consistency

In Case 3a:

- When the gNB requires ground-truth labels from the LMF, RAN3 should define procedures enabling the gNB to request and receive this information, possibly through new or enhanced NGAP or NRPPa messages.

- Include mechanisms to provide relevant network condition information to AI/ML models, such as changes in synchronization, TRP/ARP locations, and antenna orientations. This ensures models remain accurate despite network changes.

In Case 3b:

- Establish procedures for NG-RAN nodes to collect and transfer measurement data to the LMF, supporting new data types like phase information. Allow flexible data collection channels, such as direct transfer to a server or through OAM systems.

Privacy and Security: Ensuring high-quality measurement data and accurate synchronization is crucial. Secure data transfer procedures are essential to protect sensitive information during measurement data exchange.

Proposal 6: Confirm That Existing Positioning Procedures Are Sufficient to Support AI/ML Model Inference in Cases 3a and 3b, and Plan for Future Enhancements Based on RAN1 Progress

Existing positioning procedures can be reused to support AI/ML-based positioning without significant specification changes. RAN3 should:

- Monitor RAN1 Developments: Closely follow RAN1's progress on new measurement types and be prepared to implement necessary enhancements.

- Plan for Future Enhancements: Adapt to future measurement configurations or data types to ensure specifications can evolve with technological advancements.

Collaboration:

Collaborative progress with RAN1 and other working groups ensures a cohesive approach to standard development, facilitating smoother transitions and interoperability.

Proposal 7: Adopt the Term 'Model Performance Monitoring' to Refer to the Process of Monitoring AI/ML Models, Ensuring Alignment with RAN3 Terminology and Clarity in Discussions

Consistent terminology is crucial for effective communication. Adopting 'Model Performance Monitoring' aligns with RAN3's terminology and clarifies discussions.

Using 'Model Performance Monitoring' ensures:

- Clear Responsibility Definitions: Clarifies which entity is responsible for monitoring and under what circumstances.

- Consistent Documentation: Ensures specifications and discussions use consistent terminology, reducing confusion.

- Enhanced Collaboration: Facilitates better coordination between RAN3 and other groups like RAN1, SA2, and SA5.

This terminology aligns with practices in TR 38.843 and prior RAN3 work, aiding effective communication across 3GPP.

# 3.	Conclusion

Proposal 1: RAN3 Should Prioritize Enhancing Specifications for AI/ML-Assisted Positioning, Specifically Concentrating on Case 3a (gNB-Side Model) and Case 3b (LMF-Side Model)

Proposal 2: Define Comprehensive Procedures for Data Collection and Label Handling in Cases 3a and 3b, Including Enabling gNB Autonomy in Label Generation and Establishing Protocols for Label Provision from LMF

Proposal 3: Enhance Measurement Procedures, Including NRPPa, to Accommodate the Specific Needs of AI/ML-Based Positioning in Cases 3a and 3b

Proposal 4: Clearly Define the Responsibilities of Network Entities for Performance Monitoring, Functionality Management, and Model Placement in Cases 3a and 3b

Proposal 5: Define Procedures for Secure and Efficient Transfer of Measurement Data Between Network Entities in Cases 3a and 3b, and Consider Network Conditions Affecting AI/ML Model Consistency

Proposal 6: Confirm That Existing Positioning Procedures Are Sufficient to Support AI/ML Model Inference in Cases 3a and 3b, and Plan for Future Enhancements Based on RAN1 Progress

Proposal 7: Adopt the Term 'Model Performance Monitoring' to Refer to the Process of Monitoring AI/ML Models, Ensuring Alignment with RAN3 Terminology and Clarity in Discussions