3GPP TSG-RAN WG3 Meeting #126	R3-247533

Orlando, USA, 18th–22nd November, 2024

Title: 	Support for AI/ML enabled Network Slicing

Source: 	Ericsson, InterDigital

Agenda item:	11.2 AI/ML enabled Slicing

Document Type:	Discussion and agreement.

# 1. Introduction

At RAN3#125 the SI on AI/ML in RAN in Rel-19 was completed, and the results of the study are captured in the TR 38.743 [1]. For AI/ML based Network Slicing use case, the TR recommends the exchange of the following input data between neighbouring gNBs:

- Measured/Predicted radio resource status per slice,

- Measured/Predicted slice available capacity.

In addition to the TR conclusions above, the following agreement was captured during the last RAN3 meeting with respect to the AI/ML based Network Slicing use case:

Slice UE performance needs to be introduced, while the granularity to support slice UE performance can be further checked in WI phase.

Furthermore, during RAN3#125bis meeting the following FFS and WA were captured:

The failure of the predicted radio resource status per slice and predicted slice available capacity should be indicated to the sending node? Per cell per slice?

WA: UE performance per each PDU session associated to a requested S-NSSAI is reported from target node to source node.

FFS if other finer granularity of UE performance is feasible.

In this contribution, we assess the implications of the above on the Xn interface and we provide a possible solution implementation in a related Text Proposal for XnAP [2].

# 2. Discussion

## Per slice measurements reporting

For the reporting of per slice radio resource status and slice available capacity between neighbouring gNBs, as presented is sections 9.1.3.21, 9.2.2.50 and 9.2.2.55 in XnAP [2], current standards versions already support the reporting of the abovementioned measurements per cell and per slice. Therefore, no further standards enhancements are needed for the supports of exchanging the measured radio resource status per slice and measured slice available capacity between neighbouring gNBs.

Proposal 1: RAN3 to acknowledge that no further standards enhancements are needed for the support of exchanging the measured radio resource status per slice and measured slice available capacity between neighbouring gNBs.

## Per slice predictions reporting

For the work carried out in the previous releases in the framework of AI/ML for NG-RAN, the Resource Status Reporting Initiation and the Resource Status Reporting procedures were used as models for the design of the Data Collection Reporting Initiation and Data Collection Reporting procedures. The design principle de facto established, is to use Resource Status Reporting procedure to report the measured values for the requested parameters, and to use Data Collection Reporting procedure to report the predicted values for the requested parameters.

In the context of the AI/ML based Network Slicing, as presented in the previous section, the Resource Status Reporting procedure is used to report the measured radio resource status per slice and measured slice available capacity between neighbouring gNBs. For this reason and following the design principle already established in this group, the Resource Status Reporting Initiation and Resource Status Reporting procedures could serve as models for the design of the Data Collection Reporting Initiation and Data Collection Reporting procedures for the support of reporting the predicted radio resource status per slice and predicted slice available capacity between neighbouring gNBs.

Proposal 2: RAN3 to use the Resource Status Reporting Initiation and Resource Status Reporting procedures as models for designing the Data Collection Reporting Initiation and Data Collection Reporting procedures for the support of reporting the predicted radio resource status per slice and predicted slice available capacity between neighbouring gNBs.

Considering now the slice-specific predictions that need to be reported via the Data Collection Reporting procedure, it is proposed that such predictions reuse the definitions of the legacy IEs for the measured versions of these metrics.

The legacy versions of these metrics are shown below:

#### 9.2.2.50	Radio Resource Status

The Radio Resource Status IE indicates the usage of the PRBs per cell for MIMO, per SSB area, and per slice for all traffic in Downlink and Uplink and the usage of PDCCH CCEs for Downlink and Uplink scheduling.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| CHOICE Radio Resource Status Type | M |  |  |  | – |  |
| >ng-eNB |  |  |  |  |  |  |
| >>DL GBR PRB usage | M |  | INTEGER (0..100) | Per cell DL GBR PRB usage | – |  |
| >>UL GBR PRB usage | M |  | INTEGER (0..100) | Per cell UL GBR PRB usage | – |  |
| >>DL non-GBR PRB usage | M |  | INTEGER (0..100) | Per cell DL non-GBR PRB usage | – |  |
| >>UL non-GBR PRB usage | M |  | INTEGER (0..100) | Per cell UL non-GBR PRB usage | – |  |
| >>DL Total PRB usage | M |  | INTEGER (0..100) | Per cell DL Total PRB usage | – |  |
| >>UL Total PRB usage | M |  | INTEGER (0..100) | Per cell UL Total PRB usage | – |  |
| >>DL scheduling PDCCH CCE usage | O |  | INTEGER (0..100) |  | YES | ignore |
| >>UL scheduling PDCCH CCE usage | O |  | INTEGER (0..100) |  | YES | ignore |
| >gNB |  |  |  |  |  |  |
| >>SSB Area Radio Resource Status List |  | 1 |  |  | – |  |
| >>>SSB Area Radio Resource Status Item |  | 1..<maxnoofSSBAreas> |  |  | – |  |
| >>>>SSB Index | M |  | INTEGER (0..63) |  | – |  |
| >>>>SSB Area DL GBR PRB usage | M |  | INTEGER (0..100) | Per SSB area DL GBR PRB usage in percentage of the cell total PRB number. | – |  |
| >>>>SSB Area UL GBR PRB usage | M |  | INTEGER (0..100) | Per SSB area UL GBR PRB usage in percentage of the cell total PRB number. | – |  |
| >>>>SSB Area DL non-GBR PRB usage | M |  | INTEGER (0..100) | Per SSB area DL non-GBR PRB usage in percentage of the cell total PRB number. | – |  |
| >>>>SSB Area UL non-GBR PRB usage | M |  | INTEGER (0..100) | Per SSB area UL non-GBR PRB usage in percentage of the cell total PRB number. | – |  |
| >>>>SSB Area DL Total PRB usage | M |  | INTEGER (0..100) | Per SSB area DL Total PRB usage in percentage of the cell total PRB number. | – |  |
| >>>>SSB Area UL Total PRB usage | M |  | INTEGER (0..100) | Per SSB area UL Total PRB usage in percentage of the cell total PRB number. | – |  |
| >>>>DL scheduling PDCCH CCE usage | O |  | INTEGER (0..100) |  | YES | ignore |
| >>>>UL scheduling PDCCH CCE usage | O |  | INTEGER (0..100) |  | YES | ignore |
| >>Slice Radio Resource Status List |  | 0..1 |  |  | YES | ignore |
| >>>Slice Radio Resource Status Item |  | 1..< maxnoofBPLMNs > |  |  | – |  |
| >>>>PLMN Identity | M |  | 9.2.2.4 |  | – |  |
| >>>>S-NSSAI Radio Resource Status List |  | 1 |  |  | – |  |
| >>>>>S-NSSAI Radio Resource Status Item |  | 1..<maxnoofSliceItems> |  |  | – |  |
| >>>>>>S-NSSAI | M |  | 9.2.3.21 |  | – |  |
| >>>>>>Slice DL GBR PRB usage | M |  | INTEGER (0..100) | Per slice DL GBR PRB usage in percentage of the cell total PRB number. | – |  |
| >>>>>>Slice UL GBR PRB usage | M |  | INTEGER (0..100) | Per slice UL GBR PRB usage in percentage of the cell total PRB number. | – |  |
| >>>>>>Slice DL non-GBR PRB usage | M |  | INTEGER (0..100) | Per slice DL non-GBR PRB usage in percentage of the cell total PRB number. | – |  |
| >>>>>>Slice UL non-GBR PRB usage | M |  | INTEGER (0..100) | Per slice UL non-GBR PRB usage in percentage of the cell total PRB number. | – |  |
| >>>>>>Slice DL Total PRB allocation | M |  | INTEGER (0..100) | Total amount of DL PRBs available per cell for the slice if all the resources the slice could access were usable. | – |  |
| >>>>>>Slice UL Total PRB allocation | M |  | INTEGER (0..100) | Total amount of UL PRBs available per cell for the slice if all the resources the slice could access were usable. | – |  |
| >>MIMO PRB usage Information | O |  |  |  | YES | ignore |
| >>>DL GBR PRB usage for MIMO | M |  | INTEGER (0..100) | Per cell DL GBR PRB usage for MIMO in percentage of the cell total PRB number as defined in TS 38.314 [42]. | – |  |
| >>>UL GBR PRB usage for MIMO | M |  | INTEGER (0..100) | Per cell UL GBR PRB usage for MIMO in percentage of the cell total PRB number as defined in TS 38.314 [42]. | – |  |
| >>>DL non-GBR PRB usage for MIMO | M |  | INTEGER (0..100) | Per cell DL non-GBR PRB usage for MIMO in percentage of the cell total PRB number as defined in TS 38.314 [42]. | – |  |
| >>>UL non-GBR PRB usage for MIMO | M |  | INTEGER (0..100) | Per cell UL non-GBR PRB usage for MIMO in percentage of the cell total PRB number as defined in TS 38.314 [42]. | – |  |
| >>>DL Total PRB usage for MIMO | M |  | INTEGER (0..100) | Per cell DL Total PRB usage for MIMO in percentage of the cell total PRB number as defined in TS 38.314 [42]. | – |  |
| >>>UL Total PRB usage for MIMO | M |  | INTEGER (0..100) | Per cell UL Total PRB usage for MIMO in percentage of the cell total PRB number as defined in TS 38.314 [42]. | – |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofSSBAreas | Maximum no. SSB Areas that can be served by a NG-RAN node cell. Value is 64. |
| maxnoofSliceItems | Maximum no. of signalled slice support items. Value is 1024. |
| maxnoofBPLMNs | Maximum no. of broadcast PLMNs by a cell. Value is 12. |

#### 9.2.2.55	Slice Available Capacity

The Slice Available Capacity IE indicates the amount of resources per network slice that are available per cell relative to the total NG-RAN resources per cell. The Slice Available Capacity Value Downlink IE and the Slice Available Capacity Value Uplink IE can be weighted according to the ratio of the corresponding cell capacity class values contained in the Composite Available Capacity Group IE, if available.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Slice Available Capacity |  | 1..< maxnoofBPLMNs > |  |  |
| >PLMN Identity | M |  | 9.2.2.4 | Broadcast PLMN |
| >S-NSSAI Available Capacity List |  | 1 |  |  |
| >>S-NSSAI Available Capacity Item |  | 1 .. < maxnoofSliceItems> |  |  |
| >>>S-NSSAI | M |  | 9.2.3.21 |  |
| >>>Slice Available Capacity Value Downlink | M |  | INTEGER (0..100) | Value 0 indicates no available capacity, and 100 indicates maximum available capacity . Slice Available Capacity Value Downlink should be measured on a linear scale. |
| >>>Slice Available Capacity Value Uplink | M |  | INTEGER (0..100) | Value 0 indicates no available capacity, and 100 indicates maximum available capacity. Slice Available Capacity Value Uplink should be measured on a linear scale. |

| Range bound | Explanation |
| --- | --- |
| maxnoofSliceItems | Maximum no. of signalled slice support items. Value is 1024. |
| maxnoofBPLMNs | Maximum no. of PLMN Ids.broadcast in a cell. Value is 12. |

In order to derive predicted versions of the measured metrics above, i.e. Predicted Slice Radio Resource List and Predicted Slice Available Capacity, it is proposed that a similar approach to that used to derive the Predicted Radio Resource Status IE is used. Namely, it is proposed that the Xn: DATA CLLECTION UPDATE message includes a Predicted Radio Resource Status IE that contains also the predicted Slice Radio Resource List.

At the same time, it is proposed that the Xn: DATA CLLECTION UPDATE message includes a Predicted Slice Available Capacity IE that is defined as the legacy Slice Available Capacity IE. An example of how to encode this information in the Xn: DATA COLLECTION UPDATE is provided below:

#### 9.1.3.29	DATA COLLECTION UPDATE

This message is sent by NG-RAN node2 to NG-RAN node1 to report the requested information.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| NG-RAN node1 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | reject |
| Cell Measurement Result for Data Collection List |  | 0..1 |  |  | YES | ignore |
| >Cell Info Result for Data Collection Item |  | 1 .. < maxnoofCellsinNG-RANnode > |  |  | – |  |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 | Indicates an NR Cell Identity. | – |  |
| >>Predicted Radio Resource Status | O |  | Radio Resource Status 9.2.2.50 | The IE only includes the SSB Area Radio Resource Status List IE, excluding the DL scheduling PDCCH CCE usage IE and UL scheduling PDCCH CCE usage IE and the Slice Radio Resource Status List IE. | – |  |
| >>Predicted Number of Active UEs | O |  | Number of Active UEs 9.2.2.62 |  | – |  |
| >>Predicted RRC Connections | O |  | RRC Connections 9.2.2.56 |  | – |  |
| >>Predicted Slice Available Capacity | O |  | Slice Available Capacity 9.2.2.55 |  | – |  |

Proposal 3: RAN3 to agree that the Xn Predicted Radio Resource Status IE contains also the Slice Radio Resource List IE and that the Predicted Slice Available Capacity IE is defined by reusing the Slice Available Capacity IE

Another aspect to analyse is that the predicted Slice Radio Resource List, the Predicted Slice Available Capacity and the per slice UE Performance information are cell-level measurements. Indeed, the per slice Radio Resource Status and Available Capacity are expressed as a fraction of the total Radio Resource Status or Composite Available Capacity, respectively, which are defined per cell. As for the UE performance, the measurements are performed in the target cell (and only in the target cell) after a successful handover execution. More precisely, if the UE leaves the target cell before the UE Performance Collection Duration expires, the collection of UE performance measurements is interrupted.

For these reasons, when the source gNB requests slice specific predictions/measurements from the target gNB, it should be able to specify for each cell in the target gNB the list of slices for which the per slice predictions/measurements are requested.

Proposal 4: RAN3 to agree to add, for each cell in the Cell To Report Item for Data Collection IE of the DATA COLLECTION REQUEST message, a new IE defining the list of slices for which slice specific predictions and/or measurements are requested in the cell. This approach is in line with the structure of the legacy Xn: Resource Status Request message

## Per slice UE performance feedback reporting

In the last meeting the following working assumption and FFS were captured for the collection of UE Performance measurements on a per-slice basis .

WA: UE performance per each PDU session associated to a requested S-NSSAI is reported from target node to source node.

FFS if other finer granularity of UE performance is feasible.

In the current standards, for a UE which is using services from slice A and slice B in a cell of a source gNB and which is handed over to a cell in a target gNB while continuing to use the services provided by slice A and slice B, the measurements collected as part of the UE performance consist of, Average UE Throughput DL, Average UE Throughput UL, Average Packet Delay, Average Packet Loss DL. These measurements can be collected on a per DRB level and per S-NSSAI level, namely measurements can be derived for each DRB of a given S-NSSAI.

During RAN3-125bis, some companies proposed to achieve a per DRB per slice UE performance granularity. However, we note that this is not possible because each gNB may have a different mapping of QoS flows per DRB, hence per DRB UE performance metrics would not be comparable between source and target gNB.

Another option provided was to achieve a per QoS Flow UE Performance granularity, where the list of QoS flows for which UE performance metrics are provided would consist of those QoS flows forming a DRB. While this is possible, it could be questioned whether the solution is viable. The source gNB, in fact, may not be able to compare the per QoS Flow list performance with a performance that it could have measured. That is because, again, the target gNB may have adopted a different QoS flow to DRB mapping. The figure below explains this point in more details:

Figure 1: reporting of per QoS flow list performance

As it can be seen in the Figure, collection of per QoS Flow list performance reduces to collection of per DRB performance. However, gNB1 cannot compare the performance collected by gNB2 for the QoS Flow lists corresponding to gNB2´s DRBs with the performance of the DRBs gNB1 was serving. Therefore, this performance metric is not useful as it cannot lead to an evaluation of whether the performance in gNB2 was better or worse than in gNB1.

On the other end, we note that both gNB1 and gNB2 can collect performance for the PDU Session X (associated to a specific S-NSSAI). If gNB2 reports the UE performance per PDU Session, gNB1 can compare it with the performance measured for the same PDU Session. Such comparison is valid and it reveals the performance of the UE for a given PDU Session when served in gNB2 and in gNB1.

Considering the above, the following is proposed:

Proposal 5: RAN3 to turn the working assumption into an agreement, and introduce per PDU Session per-slice UE Performance measurements collection.

Another topic that needs to be discussed is how to request the per slice UE Performance feedback measurements in the DATA COLLECTION REQUEST message.

In the current standards the “per cell” UE performance consists of Average UE Throughput DL, Average UE Throughput UL, Average Packet Delay, Average Packet Loss DL, and the requesting NG-RAN node can independently request each one of these measurements by setting the appropriate bit in the Report Characteristics for Data Collection IE of the DATA COLLECTION REQUEST message.

As for reporting of slice specific performance measurements, we propose to follow the existing standard implementation for the UE Performance reporting in the Data Collection Reporting Initiation and Data Collection Reporting procedures. This implies allowing the requesting NG-RAN node to independently request the reporting of Slice Average UE Throughput DL, Slice Average UE Throughput UL, Slice Average Packet Delay, Slice Average Packet Loss DL by setting the appropriate/dedicated bit of the Report Characteristics for Data Collection IE, of the DATA COLLECTION REQUEST message as presented below.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node1 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID | C-ifRegistrationRequestForDataCollectionStop |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | ignore |
| Registration Request for Data Collection | M |  | ENUMERATED(start, stop, …) | Type of request for which the information is required. | YES | reject |
| Report Characteristics for Data Collection | C-ifRegistrationRequestForDataCollectionStart |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates the object the NG-RAN node2 is requested to report. First Bit = Predicted Radio Resource Status, Second Bit = Predicted Number of Active UEs, Third Bit = Predicted RRC Connections Fourth Bit = Average UE Throughput DL, Fifth Bit = Average UE Throughput UL, Sixth Bit = Average Packet Delay, Seventh Bit = Average Packet Loss DL Eighth Bit = Energy Cost Ninth Bit = Measured UE Trajectory Tenth Bit = Slice Average UE Throughput DL, Eleventh Bit = Slice Average UE Throughput UL, Twelfth Bit = Slice Average Packet Delay, Thirteenth Bit = Slice Average Packet Loss DL Fourteenth Bit = Predicted Slice Available Capacity Other bits are ignored by the NG-RAN node2. | YES | reject |
| Cell To Report List for Data Collection |  | 0..1 |  | Cell ID list to which the request applies. | YES | ignore |
| >Cell To Report Item for Data Collection |  | 1 .. <maxnoofCellsinNG-RANnode> |  |  | – |  |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 | Indicates an NR Cell Identity. | – |  |
| >>Slice to Report List for Data Collection |  | 0..1 |  | S-NSSAI list to which the request applies. | YES | ignore |
| >>>Slice to Report Item for Data Collection |  | 1 .. < maxnoofBPLMNs > |  |  | - |  |
| >>>>PLMN Identity | M |  | 9.2.2.4 | Broadcast PLMN | - |  |
| >>>>S-NSSA List |  |  | 1 |  | - |  |
| >>>>>S-NSSA Item |  | 1 .. < maxnoofSliceItems> |  |  | - |  |
| >>>>>>S-NSSAI | M |  | 9.2.3.21 |  | - |  |
| Reporting Periodicity for Data Collection | O |  | ENUMERATED(500ms, 1000ms, 2000ms, 5000ms, 10000ms, …) | Periodicity that can be used for reporting of requested objects. Also used as the averaging window length for all objects if supported. | YES | ignore |
| Requested Prediction Time | O |  | INTEGER (1..60, ...) | For one time reporting, it indicates the point in time, measured from reception of the DATA COLLECTION REQUEST message, for which predictions are provided. In periodic reporting, for each subsequent DATA COLLECTION UPDATE message, the point in time is shifted by the reporting periodicity. (unit: second) | YES | ignore |
| UE Trajectory Collection Configuration | O |  | 9.2.3.185 |  | YES | ignore |
| UE Performance Collection Configuration | O |  | 9.2.3.186 |  | YES | ignore |

Proposal 6: RAN3 to extend the definition of the bits of the Report Characteristics for Data Collection IE to enable the requesting NG-RAN node to request the reporting of Slice Average UE Throughput DL, Slice Average UE Throughput UL, Slice Average Packet Delay, Slice Average Packet Loss DL as part of per slice UE Performance measurements reporting.

## Per slice predictions reporting initiation failure indication

At the last RAN3 meeting the following FFS was captured with respect to the per slice predictions reporting initiation failure indication in the DATA COLLECTION RESPONSE message.

The failure of the predicted radio resource status per slice and predicted slice available capacity should be indicated to the sending node? Per cell per slice?

For a NG-RAN node that in a cell supports two different slices, e.g., slice A and slice B, the deployed AIML models runs the inference of predicted radio resource status per slice and predicted slice available capacity independently for each slice. As an example, the AIML model may take as input the resources configuration and/or the load per slice which, considering slice local conditions, may be very different from slice A to slice B. Thus, it may so happen that due to slice local conditions, the AIML model implementation is able to infer predicted radio resource status per slice and predicted slice available capacity for slice A but not for slice B, In this case it is very important for the reporting NG-RAN node to be able to indicate the measurements initiation failure independently for slice A and slice B as a direct consequence of the local conditions for each slice in the cell of the reporting NG-RAN node.

Considering the above example we propose the following:

Proposal 7: RAN3 to agree per slice measurements reporting initiation failure indication in the DATA COLLECTION RESPONSE message for the per slice requested measurements.

# 3. Conclusions

In this contribution we present a straightforward plan for implementing the agreements previously made for AI/ML based Network Slicing, plan from which we derive the following proposals:

Proposal 1: RAN3 to acknowledge that no further standards enhancements are needed for the support of exchanging the measured radio resource status per slice and measured slice available capacity between neighbouring gNBs.

Proposal 2: RAN3 to use the Resource Status Reporting Initiation and Resource Status Reporting procedures as models for designing the Data Collection Reporting Initiation and Data Collection Reporting procedures for the support of reporting the predicted radio resource status per slice and predicted slice available capacity between neighbouring gNBs.

Proposal 3: RAN3 to agree that the Xn Predicted Radio Resource Status IE contains also the Slice Radio Resource List IE and that the Predicted Slice Available Capacity IE is defined by reusing the Slice Available Capacity IE

Proposal 4: RAN3 to agree to add, for each cell in the Cell To Report Item for Data Collection IE of the DATA COLLECTION REQUEST message, a new IE defining the list of slices for which slice specific predictions and/or measurements are requested in the cell. This approach is in line with the structure of the legacy Xn: Resource Status Request message

Proposal 5: RAN3 to turn the working assumption into an agreement and introduce per PDU Session per-slice UE Performance measurements collection.

Proposal 6: RAN3 to extend the definition of the bits of the Report Characteristics for Data Collection IE to enable the requesting NG-RAN node to request the reporting of Slice Average UE Throughput DL, Slice Average UE Throughput UL, Slice Average Packet Delay, Slice Average Packet Loss DL as part of per slice UE Performance measurements reporting.

Proposal 7: RAN3 to agree per slice measurements reporting initiation failure indication in the DATA COLLECTION RESPONSE message for the per slice requested measurements.

A CR mirroring the proposals above is available in R3-247534

# 4. References

[1]	3GPP TR 38.743 V19.0.0 (2024-09)

[2]	3GPP TS 38.423 V18.3.0 (2024-09)