3GPP TSG-RAN WG3 Meeting #126	R3-247545

Orlando, USA, 18th - 22th November 2024

Agenda Item:	10.3.1

Source:	ZTE Corporation

Title:	(TP to BL CR for 38.300) Further discussion on SON/MDT enhancements for NTN

Document for:	Discussion

1  Introduction

In the previous meetings, some agreements on SON/MDT enhancements for NTN were reached as below.

Logged MDT enhancement based on scenarios to be worked first, work on immediate MDT if time allows. MRO mechanisms for intra-NTN should be addressed based on failure scenarios first, work on near failure scenarios later. Take the connection failure scenarios in §15.5.2.2.2 of TS 38.300 as starting point with necessary update, if needed, in MRO for intra-NTN mobility. MRO for intra-NTN mobility shall consider the following trigger conditions: - Measurement-based trigger condition; - A time-based trigger condition; - A location-based trigger condition; - A time-based and Measurement-based combined trigger condition; - A location-based and Measurement-based combined trigger condition; Do not consider MRO for feeder link switch. MRO for intra-NTN mobility shall consider both conditional and non-conditional intra-NTN handover. MRO for CHO with a time-based trigger condition: Case 1 Too Late CHO Execution: RLF happens in the source cell before UE executes CHO (before or after T1 threshold starts), and the UE re-connects the cell other than the source cell. - Distinguish if the RLF happens before or after T1 Case 2 Too Early CHO Execution/CHO Execution to Wrong Cell: HOF happens or RLF happens shortly after successful CHO, and the UE re-connects in the source cell or a third cell. - FFS on whether it matters if RLF happens within the duration or after the duration period. WA: down-select among below three alternatives. Alt 1: the RLF report includes for single time-based trigger condition: - Absolute time (measured UTC time) when RLF occurred Alt 2: a flag to indicate whether the RLF happens before T1 or after T1 Alt 3: based on existing timers in RLF report. Further check the granularity of the current timers. MRO for CHO with a location-based trigger condition: For earth fixed case, there is no need to report the measured distance from the UE. FFS on the earth moving case. MRO for CHO with joint trigger conditions: UE reports the fulfilled CHO trigger conditions before RLF occurs in case of “time and measurement based trigger condition” or “location and measurement based trigger condition”, it is up to RAN2 to decide to reuse the existing IE or not. Whether to consider the fallback issue from RACH-less HO to RACH-based HO for NTN? New finer area scope for MDT: Is this for signaling based MDT or for management-based MDT? Introduce a geographical area (a reference location plus a radius) as the area scope for MDT or a mapped cell ID list?

In this contribution, we provide our further views on this topic, including the MRO for intra-NTN failure scenarios and the logged MDT enhancement based on scenarios.

2  Discussion

2.1  MRO for intra-NTN mobility failure

Considering the CHO with a time-based trigger condition, as we agreed before, the NW needs to distinguish if the RLF happens before or after T1. In this case, the time when the failure happens (e.g., absolute time) could be useful for the NW to analyse the failure and adjust the configuration of time-based trigger condition for the UE.

Regarding the time information, some companies had concern that whether the existing time info in the RLF report was enough. While, as given in TS38.331, the T1 should be in the form of UTC time, and the existing time info, such as timeConnFailure-r16 or timeSinceFailure-r16 or timeSinceCHO-Reconfig, is in form of relative time. How to derive the time when the failure happens needs to be clarified.

------------------------------------------------------------TS 38.331----------------------------------------------------------------------

------------------------------------------------------------TS 38.331----------------------------------------------------------------------

In addition, the granularity of the different time info is also different.

As for the flag to indicate whether the RLF happens before or after T1, it could be an compromise way, while we still think the time when failure occurs in form of absolute time is more straightforward.

Proposal 1: Regarding the MRO for the intra-NTN CHO with time-based trigger condition, the absolute time information when the failure happens should be included in the RLF report.

For the CHO with location-based trigger condition, considering the earth-moving scenario, the condition Event D2 should be applied. As given in below, the movingReferenceLocation and the corresponding epoch time and satellite ephemeris of the serving cell are provided in SIB19, and the referenceLocation and the corresponding epoch time and satellite ephemeris of the neighbor cell are provided in MeasObjectNR.

------------------------------------------------------------TS 38.331----------------------------------------------------------------------

------------------------------------------------------------TS 38.331----------------------------------------------------------------------

With above information, the UE is able to decide when to trigger the location-based CHO execution. And the NW is able to derive the specific moving reference location of the serving cell and the reference location of the neighbor cell when the CHO was executed. In this case, based on the existing location info in the RLF report, the NW is able to adjust the configuration of the location-based trigger condition. Therefore, there is no need for the UE to report other location information for earth moving case.

Proposal 2: Regarding the MRO for intra-NTN CHO with location-based trigger condition, there is no need to report the location information where the failure happens in the RLF report.

Considering the conditional intra-NTN handover with multiple trigger conditions, it was agreed that UE shall report the fulfilled trigger conditions before RLF occurs, while it is up to RAN2 to decide whether to reuse the existing IE. And RAN3 just needs to send the LS to RAN2 to indicate this conclusion.

Proposal 3: There is no extra need to enhance the RLF report for the CHO with multiple trigger conditions.

2.1.2  MRO for non-conditional intra-NTN handover

Considering the potential stage 3 impact, since the NG based handover between the NTN base stations should be applied in case of no Xn interface between the NTN base stations, some existing MRO mechanisms only supported over Xn should be also applied over NG, e.g., the missing part over NG, the transfer of assistance information for RRC Re-establishment reporting without RLF report. And the details could be discussed later.

Proposal 4: RAN3 shall apply the existing MRO feature over Xn into NG for intra-NTN mobility, e.g., the transfer of the assistance information for RRC Re-establishment reporting without RLF report.

2.1.3  MRO for RACH-less handover

For intra-NTN mobility, the RACH-less handover could be fallback to RACH based handover, while, after that, there still could be RLF/HOF after the fallback procedure. Therefore, to help the NW analyse the cause of the failure and adjust the handover strategy, an indication on whether the UE has performed the RACH-less HO fallback could be introduced in the RLF report.

Proposal 5: Regarding the MRO for RACH-less handover, an indication on whether the UE has performed the RACH-less HO fallback before the intra-NTN mobility failure should be included in the RLF report.

2.1.4  Stage 2 update

In last meeting, it has been agreed that “Take the connection failure scenarios in §15.5.2.2.2 of TS 38.300 as starting point with necessary update, if needed, in MRO for intra-NTN mobility.”. Therefore, the corresponding stage 2 description could be update to capture the connection failure scenarios for intra-NTN mobility.

Proposal 6: Update the 15.5.2.2.2 of TS 38.300 to capture the connection failure scenarios for intra-NTN mobility.

2.2  Logged MDT enhancement

The NTN cell provides much larger coverage area than the TN cell and it may cover the area of multiple countries. For Logged MDT, the Area Scope/Configuration should be enhanced. As given in TR 38.821, the cell diameter of a NTN cell varies from 50km to 1000km.

------------------------------------------------------------TR 38.821---------------------------------------------------------------------

Table 7.3.2.1.4-1: Time to HO for min/max cell diameter and varying UE speed

| Cell Diameter Size (km) | UE Speed (km/hr) | Satellite Speed (km/s) | Time to HO (s) |
| --- | --- | --- | --- |
| 50 (lower bound) | +500 | 7.56 (NOTE 1) | 6.49 |
|  | -500 |  | 6.74 |
|  | +1200 |  | 6.33 |
|  | - 1200 |  | 6.92 |
|  | Neglected |  | 6.61 |
| 1000 (upper bound) | +500 |  | 129.89 |
|  | -500 |  | 134.75 |
|  | +1200 |  | 126.69 |
|  | - 1200 |  | 138.38 |
|  | Neglected |  | 132.28 |

------------------------------------------------------------TR 38.821---------------------------------------------------------------------

Considering a NTN cell with 500km diameter which covers two different countries (e.g. country A and country B). The Operator in country A may only want to know the NTN specific coverage status in country A instead of the whole NTN cell. In addition, it could be not legal for the Operator in country A to obtain the NTN specific coverage status in country B. Therefore, the existing Area Scope over network interfaces or Area Configuration over Uu is not sufficient, and the new granularity of Area Scope or Area Configuration should be introduced for logged MDT for NTN.

Regarding the MDT type, both the signalling-based MDT and management-based MDT should be considered, there seems to be no need to exclude any MDT type.

Proposal 7: Both the signalling-based MDT and management-based should be supported in MDT for NTN.

Regarding the mapped cell ID, it is deployed by implementation with a fixed geographical area, which is not equal to the TN Uu cell. And the finer granularity for the Area scope can help the MDT to be executed more accurately.

Proposal 8: New granularity of Area Scope/Configuration should be introduced for logged MDT enhancement for NTN.

Since whether to introduce the above enhancement of RLF report and logged MDT for NTN also depends on RAN2 and the corresponding TU in RAN2 is limited, it could be beneficial to inform RAN2 of RAN3’s understanding on the SON/MDT enhancements for NTN.

Proposal 9: An LS on the enhancement of RLF report and logged MDT for NTN should be sent to RAN2.

3  Conclusion

MRO for intra-NTN mobility failure

Proposal 1: Regarding the MRO for intra-NTN CHO with time-based trigger condition, the time information when the failure happens (e.g. absolute time) should be included in the RLF report.

Proposal 2: Regarding the MRO for intra-NTN CHO with location-based trigger condition, there is no need to report the location information where the failure happens in the RLF report.

Proposal 3: There is no extra need to enhance the RLF report for the CHO with multiple trigger conditions.

Proposal 4: RAN3 should apply the existing MRO feature over Xn into NG for intra-NTN mobility, e.g., the transfer of the assistance information for RRC Re-establishment reporting without RLF report.

Proposal 5: Regarding the MRO for RACH-less handover, an indication on whether the UE has performed the RACH-less HO fallback before the intra-NTN mobility failure should be included in the RLF report.

Proposal 6: Update the 15.5.2.2.2 of TS 38.300 to capture the connection failure scenarios for intra-NTN mobility.

Logged MDT enhancement

Proposal 7: Both the signalling-based MDT and management-based should be supported in MDT for NTN.

Proposal 8: New granularity of Area Scope/Configuration should be introduced for logged MDT enhancement for NTN.

Proposal 9: An LS on the enhancement of RLF report and logged MDT for NTN should be sent to RAN2.

The corresponding LS and TP are given in the Annex.

References

- TS 38.331 v18.2.0

- TS 38.300 v18.2.0

- TR 38.821 v16.1.0

Annex

LS to RAN2 on RAN3’s progress on SON/MDT enhancements for NTN

3GPP TSG-RAN WG3 Meeting #126	R3-24xxxx

Orlando, USA, 18th – 22th Nov 2024

Title:	[Draft] LS on SON/MDT enhancements for intra-NTN mobility

Response to:	N/A

Release:	Rel-19

Work Item:	NR_ENDC_SON_MDT_Ph4-Core

Source:	ZTE (to be RAN3)

To:	RAN2

Cc:	N/A

Contact Person:

      - Name:	Jiren Han

      - E-mail Address:	han.jiren@zte.com.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

- Overall Description:

RAN3 has discussed the scope of the work related to the support of SON/MDT enhancements for intra-NTN mobility and identified the following issues RAN2 may need to support:

- - The enhancement of RLF report used for intra-NTN mobility, including the following scenarios.

    - - For the conditional handover with a time-based trigger condition, the time information when the failure happens should be introduced in the RLF report.

    - - For the conditional handover with multiple trigger conditions, the fulfilled CHO trigger conditions before RLF occurs in case of “time and measurement based trigger condition” or “location and measurement based trigger condition should be introduced in the RLF report.

    - - An indication on whether the UE performs the RACH-less HO fallback for the intra-NTN mobility failure

- - The enhancement of logged MDT for NTN, including the new Area Configuration over Uu.

2. Actions:

To RAN2 group.

ACTION: 	RAN3 kindly asks RAN2 to take the above into consideration.

3. Date of Next RAN3 Meetings:

TSG RAN WG3 Meeting #127     17th – 21th, Feb. 2025 		 Athens, Greece

TP for TS 38.300 on connection failure scenarios for intra-NTN mobility

--------------------------------------------------------TS 38.300-------------------------------------------------------------------------

15.5.2.2.2	Connection failure due to intra-system mobility

One of the functions of Mobility Robustness Optimization is to detect connection failures that occur due to Too Early or Too Late Handovers, or Handover to Wrong Cell. These problems are defined as follows:

-	Intra-system Too Late Handover: an RLF occurs after the UE has stayed for a long period of time in the cell; the UE attempts to re-establish the radio link connection in a different cell.

-	Intra-system Too Early Handover: an RLF occurs shortly after a successful handover from a source cell to a target cell or a handover failure occurs during the handover procedure; the UE attempts to re-establish the radio link connection in the source cell.

-	Intra-system Handover to Wrong Cell: an RLF occurs shortly after a successful handover from a source cell to a target cell or a handover failure occurs during the handover procedure; the UE attempts to re-establish the radio link connection in a cell other than the source cell and the target cell.

In the definition above, the "successful handover" refers to the UE state, namely the successful completion of the RA procedure.

In case of CHO, the Too Late Handover, Too Early Handover and Handover to Wrong Cell in the definition above means Too Late CHO Execution, Too Early CHO Execution and CHO Execution to Wrong Cell.

In case of intra-NTN mobility, the Too Late Handover, Too Early Handover, Handover to Wrong Cell, Too Late CHO Execution, Too Early CHO Execution and CHO Execution to Wrong Cell are all applied.

--------------------------------------------------------TS 38.300--------------------------------------------------------------------------