3GPP TSG-RAN WG3 Meeting #126	R3-247549

Orlando, USA, 18th - 22th Nov 2024

Agenda Item:	15.2

Source:	ZTE Corporation

Title:	Further discussion on support of Store and Forward satellite operation for IoT NTN

Document for:	Discussion

1  Introduction

In RAN3#125 meeting[1], the discussion of IoT NTN WI was initiated and the following agreement was reached as below.

- RAN3 decides to work on Split MME architecture and Full CN on board architecture. Whether eNB only on board architecture is feasible or not can be discussed later.

In addition, one incoming LS on PWS support for NB-IoT NTN[2] has been received from RAN2.

In this contribution, we provide our considerations on the Store and Forward (S&F) mechanism, including the potential architectures and the corresponding impact on RAN3.

2  Discussion

2.1 Potential architecture to support S&F mechanism

In RAN3#125 meeting, RAN3 has decided to support the Split MME architecture and Full CN on board architecture in Rel-19 which align with the conclusion in SA2. However, whether eNB only on board  architecture should be supported is still FFS.

Regarding the deployment, the eNB only on board architecture may be the easiest solution to support the S&F mechanism among the three potential options. And some corresponding standard impact over S1 may take the progress of NR NTN into account.

However, there is one issue to be clarified before we support this architecture, i.e., the attach procedure. As given in Section 6.20.2.1 in SA2 TR[3], it will take several rounds to finish the attach procedure. If only one single NGSO satellite serves the UE, it may need a few hours to complete the attach procedure, which is an inefficient solution. Therefore, for this case, the efficient solution of UE attache procedure should be discussed, e.g., whether to support the pre-attachment via the TN or NTN.

Proposal 1: RAN3 shall discuss whether to support the eNB only on board architecture and how to support the corresponding UE attach procedure.

For the other two architectures, there was some concern on whether the eNB and the corresponding MME part should be located on the same satellite or the eNB and the corresponding MME part can be located on the different satellites. Although there is no specific description in the SA2’s TR, based on the different solutions given by SA2, the eNB and the corresponding MME part should be located on the same satellite for both Split MME architecture and Full CN on board architecture.

Proposal 2: RAN3 assumes that the eNB and its corresponding MME part should be located on the same satellite for both Split MME architecture and Full CN on board architecture.

For the Split MME architecture, the corresponding description from SA2 is given as below.

The following option is agreed for supporting Store and Forward operation with a Split MME architecture with the following (informative) principles: 1) In the Split MME architecture, HSS is on the ground. 2) MME functionality is split into two parts: MME-onboard - the MME part which is onboard the satellite and MME-ground - the MME part which is on the ground network with an interface out of scope of 3GPP. 3) The MO data is stored in the MME-onboard when the service link is available and the feeder link is unavailable, and transferred to the ground when the feeder link becomes available. The MT data is stored in the MME-ground or in S-GW when the feeder link is unavailable and transferred to the MME-onboard when the feeder link becomes available. The MT data is stored in the MME-onboard when the feeder link is available and service link is unavailable, and transferred to the UE when service link becomes available. All types of data traffic (e.g. IP, etc.) can be supported and transferred using the existing user plane and control plane procedures defined in EPS. 4) For MO SMS, upon reception of the MO SMS the MME-onboard stores the MO-SMS and may immediately send the delivery report(i.e. RP-ACK) to the UE i.e. as if the MO-SMS has already been successfully delivered to the Service Centre (SC).

In this architecture, the MME functionality is split into MME-onboard and MME-ground. And the MO data is send to MME-onboard and stored in MME-onboard. Then, the MO data is transferred to the MME-ground. Meanwhile, the MT data is located in the MME-ground or the S-GW and transferred to the MME-onboard, then the MT data is stored in MME-onboard. Finally, the MT data is transferred to UE.

Since the interface between MME-on board and MME-on ground is S10, which is out of RAN3 scope, the feeder link switch in this architecture has no impact on RAN3. In addition, the exchange of S&F indication, wait timer and list of satellite IDs between the MME and the UE could be contained in the NAS-PDU via the existing S1AP messages.Therefore, there is no impact on S1.

Proposal 3: For the Split MME architecture, there is no standard impact on RAN3.

For the Full CN on board architecture, the corresponding description from SA2 is given as below.

The following option is agreed for supporting Store and Forward operation with a full CN onboard the satellite with the following (informative) principles: - The whole CN including eNB, MME, SGW, PGW, HSS, E-SMLC, SMSC etc are on board each satellite. Proxies are deployed on the satellite and the ground for application traffic, including support of MT traffic, MO traffic, SMS, etc. - The implementation of the proxies and the interface between them is out of 3GPP scope. - The UE attaches, transfers data (e.g. SMS, MO and MT data, etc.) and detaches from each satellite as required and as determined by the monitoring list. NOTE 5: MT traffic is delivered to the UE after it performs an ATTACH. NOTE 6: After mobility from S&F operation, MT traffic could be stuck in the ground proxy and will only be retrieved once the UE goes back to S&F operation. For MT traffic, the UE attaches to a satellite and to allow delivery of MO traffic from the user or applications on the UE. A UE, based on implementation, could first wait for an indication from the user or from an application on the UE of pending MT traffic or could wait based on knowledge of when MT traffic may arrive, before performing the attach. - Depending on the deployment and implementation (i.e. outside the scope of 3GPP in this release), the HSSs on the satellites may be populated with subscription data either for only the UEs that may access satellite or all UEs that may access the satellite. - Depending on the deployment, the UE may have a USIM enhanced for IOPS, or a USIM dedicated to the satellite network. NOTE 7: The solution does not support the roaming architectures defined in TS 23.501 [2] or TS 23.401 [5].

Since the eNB and full CN are all located on board, the working procedure for this architecture is similar with the legacy IoT NTN with both RAN node and CN on ground, the MT and MO data could be stored in the satellite, and the existing MT/MO procedure should be reused. In addition, the feeder link terminates the proxies of the satellite and the ground, which is our of 3GPP scope. Therefore, there is no impact on RAN3 for this architecture.

Proposal 4: For the Full CN on board architecture, there is no standard impact on RAN3.

2.2 Single-satellite and multi-satellite scenario

All the mentioned three kinds of architecture can support both single-satellite and multi-satellite scenarios. For the single-satellite scenario, the satellite can only connect with the certain UE within one cycle of operation, which means it can only support the delay-tolerant IoT services. While, for the multi-satellite scenario, the satellites can relay the service data or the RRC signalling via ISL and the service delay could be much smaller than the single-satellite scenario.

However, the single-satellite scenario is much easier than the multi-satellite scenario, and it could be taken as the starting point.

Proposal 5: RAN3 shall consider both the single-satellite and multi-satellite scenarios, and the single-satellite scenario can be taken as the starting point.

2.3 Stage 2 description for support of store and forward operation

Since the Split MME architecture and Full CN on board architecture should be supported in Rel-19, the corresponding stage 2 description should be introduced. And the following architecture assumptions and principles should be followed.

The following architecture assumptions are applied to the study: - The 5GC architecture for satellite access for NR as defined in TS 23.501 [2] is used as a baseline. - The EPC architecture for satellite access for IoT as defined in TS 23.401 [5] and the architecture enhancements to facilitate communications with packet data networks and applications as defined in TS 23.682 [7] are used as a baseline. - At least eNB/gNB is assumed to be on board the satellite. - Impacts to UE, network functions and entities are minimised. To the extent possible, existing procedures and functionality is reused. - Inter-Satellite Links (ISL) and Feeder link are assumed to act only as transport layer links and are not specified in 3GPP, and the use of ISL is assumed to have no impact on the reference points between core network entities when these core network entities are onboard. - Store and Forward Satellite Operation assumes that UE-satellite-ground network connectivity can be intermittent as defined in clause 3.1. - Store and Forward Satellite Operation shall work without ISL. - NR UE-Satellite-UE communication assumes a connection to the ground is always available. - NR UE-Satellite-UE communication assumes only IMS services including MMTEL and mission critical communications are considered. - The enhancement for NR UE-Satellite-UE communication assumes a minimum set of core network entities, including at least UPF, is present onboard of the satellites. - NR UE-Satellite-UE communication assumes that the serving satellite will be of MEO or of LEO type. - NR UE-Satellite-UE communication for MMTEL service assumes communication between two UEs under the coverage of the same satellite or different satellites. NOTE: The study of the scenario when two UEs are under the coverage of the same satellite will be prioritized.

Proposal 6: Introduce the stage 2 description of Split MME architecture and Full CN on board architecture following the architecture assumptions and principles.

Based on the LS form SA2, there is some ambiguity on the understanding of RAN3’s previous reply LS. And the reason behind is that SA2 has studied the Rel-19 Regenerative-based satellite access in one SI, including both NR NTN and IoT NTN. While, in RAN3, NR NTN and IoT NTN are discussed separately in different Rel-19 WIs.

The previous reply LS from RAN3[3] was sent in RAN3#124 meeting, and the IoT NTN WI was not started at that time. Since some questions shall be discussed in IoT NTN WI, RAN3 provided some feedback on SA2’s question, especially from NR NTN’s prospective. While, for the mapped cell ID handling, it could be a common understanding of RAN3 that it could be handled as in previous releases for both NR NTN and IoT NTN WI, which could be solved by operator’s implementation.

With respect to NG connection management: RAN3 has just begun to discuss the options, and there is no conclusion yet. RAN3 will continue discussing this issue in Rel-19 and inform SA2 on the conclusion. With respect to IP address changes due to soft feeder link switch: RAN3 confirms that the gNB IP address change due to soft feeder link switch is supported by the existing NGAP procedures. With respect to Mapped Cell ID handling: RAN3 also thinks that the AMF/MME can treat the Mapped Cell ID as in previous releases.

Considering the information from SA2, “the conclusions for FS_5GSAT_Ph3 for “KI#1: Support of Regenerative-based satellite access” without Store and Forward in clause 8.1 of TR 23.700-29 apply to both E-UTRAN/EPS and NG-RAN/5GS”, it is related to the working scope of IoT NTN WI in RAN, i.e., whether RAN3 to support the only eNB as regenerative payload on board without Store and Forward satellite operation in Rel-19 IoT NTN WI. And the details should be discussed in the IoT NTN WI.

2.4 LS from RAN2

In [2], it is mentioned that the broadcast of PWS messages for NB-IoT shall be supporting by re-using the LTE mechanism, and RAN2 thinks there should be specification impact outside of RAN2.

In TS36.300. the mapped cell ID has already applied for PWS. Since, the broadcast of PWS messages is under discussion on RAN2, any other impact on RAN3 may need to wait for more progress in RAN2.

23.21.6 Signalling The Cell Identity, as defined in TS 36.413 [25] and TS 36.423 [42], corresponds to a Mapped Cell ID, irrespective of the orbit of the NTN payload or the types of service links supported in the following cases: - The Cell Identity indicated by the eNB to the Core Network as part of the User Location Information, or as E-UTRAN CGI in the related S1AP messages; - The Cell Identity used for Paging Optimization in S1 interface; - The Cell Identity used for PWS.

Proposal 7: Regarding the broadcast of PWS messages for NB-IoT, RAN3 needs to wait for more progress in RAN2.

2.4 Working scope of Rel-19 WID

As discussed in [4], SA2 has raised the issue in [5] whether RAN3 to support the eNB as regenerative payload on board without Store and Forward satellite operation in Rel-19 IoT NTN WI. By checking the current WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3[6], except the description on Support of Capacity enhancements for uplink, all the contents in the Justification and Objective are related to Store and Forward satellite operation, and there is no evidence to support the eNB as regenerative payload on board without Store and Forward satellite operation in Rel-19 IoT NTN WID. If RAN3 wants to discuss this scenario, it should be clarified in RAN plenary to extend the scope of IoT NTN WID.

Proposal 8: In current IoT NTN WID, there is no evidence to support the eNB as regenerative payload on board without S&F satellite operation.

3  Conclusion

Proposal 1: RAN3 shall discuss whether to support the eNB only on board architecture and how to support the corresponding UE attach procedure.

Proposal 2: RAN3 assumes that the eNB and its corresponding MME part should be located on the same satellite for both Split MME architecture and Full CN on board architecture.

Proposal 3: For the Split MME architecture, there is no standard impact on RAN3.

Proposal 4: For the Full CN on board architecture, there is no standard impact on RAN3.

Proposal 5: RAN3 shall consider both the single-satellite and multi-satellite scenarios, and the single-satellite scenario can be taken as the starting point.

Proposal 6: Introduce the stage 2 description of Split MME architecture and Full CN on board architecture following the architecture assumptions and principles.

Proposal 7: Regarding the broadcast of PWS messages for NB-IoT, RAN3 needs to wait for more progress in RAN2.

Proposal 8: In current IoT NTN WID, there is no evidence to support the eNB as regenerative payload on board without S&F satellite operation.

References

- RAN3#125 Chairman Notes

- R3-247008, LS on PWS support for NB-IoT NTN, RAN2

- TR 23.700-29 v19.0.0

- R3-247547, Further discussion on support of regenerative payload, ZTE

- R3-247016, Reply LS on Support of Regenerative-based Satellite Access, SA2

- RP-241624, Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3, MediaTek Inc. (Rapporteur)

Annex

3GPP TSG-RAN WG3 Meeting #126	R3-24xxxx

# Orlando, USA, 18th - 22th Nov 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.300 | CR | - | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of store and forward operation for regenerative payload |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | IoT_NTN_Ph3-Core |  |  |  |  |  | Date: |  |  | 2024-11-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - In Rel-19 IoT NTN, the Split MME architecture and Full CN on board architecture are supported |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add stage 2 description of the Split MME architecture and Full CN on board architecture for IoT NTN. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - The stage 2 description of the Split MME architecture and Full CN on board architecture for IoT NTN is missing |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1; 4.12; Annex P |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< Start of Changes >>>>>>>>>>>>>>>>>>>>

3	Definitions, symbols and abbreviations

3.1	Definitions

For the purposes of the present document, the following terms and definitions apply.

A2X communication: A communication to support A2X services leveraging PC5 reference points. A2X services are realized by various types of A2X applications, e.g. BRID or DAA.

>>Unchanged part skipped<<

FeMBMS: further enhanced multimedia broadcast multicast service.

FeMBMS/Unicast-mixed cell: cell supporting MBMS transmission and unicast transmission as SCell.

Full CN on board: eNB, MME, SGW, PGW, HSS, E-SMLC, SMSC are on board the satellite.

>>Unchanged part skipped<<

Membership Verification: the process that checks whether a UE is a member or non-member of a hybrid cell.

MME-onboard: Part of MME functionality which is on board the satellite for the Split MME architecture.

MME-ground: Part of MME functionality which is on the ground network for the Split MME architecture.

>>Unchanged part skipped<<

NTN payload: a network node, embarked on board a satellite or high altitude platform station, providing connectivity functions, between the service link and the feeder link. The NTN payload is a TNL node (transparent payload), or embarked with a combination of an eNB and MME-on board (regenerative payload), or embarked with Full CN on board (regenerative payload).

Editor’s Note: The above definitions need further checking.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

4.12	Support of Non-Terrestrial Networks

E-UTRAN supports radio access over non-terrestrial networks for BL UEs, UEs in enhanced coverage and NB-IoT UEs. Support for non-terrestrial networks encompasses platforms that provide radio access through Geosynchronous orbits (GSO), Non-Geosynchronous Orbit (NGSO), which includes Low-Earth Orbit (LEO) and Medium Earth Orbit (MEO) or High Altitude Platform Systems (HAPS).

The Figures 4.12-1,  4.12-X and 4.12-Y below illustrate examples of a Non-Terrestrial Network (NTN) providing non-terrestrial access by means of an NTN payload and an NTN Gateway, depicting a service link between the NTN payload and a UE, and a feeder link between the NTN Gateway and the NTN payload. In Figure 4.12.1 the payload is transparent; in Figure 4.12-X the payload is regenerative and hosts a eNB with MME-on board; in Figure 4.12-Y the payload is regenerative and hosts full CN.

Figure 4.12-1: Overall illustration of an NTN with transparent payload

Figure 4.12-Y: Overall illustration of an NTN with regenerative payload hosting an eNB with MME-on board

Figure 4.12-Y: Overall illustration of an NTN with regenerative payload hosting Full CN

NOTE:	Figures 4.12-1 and Figure 4.12-X illustrate an NTN; RAN4 aspects are out of scope; S10 is out of scope.

The NTN payload transparently forwards the radio protocol received from the UE (via the service link) to the NTN Gateway (via the feeder link) and vice-versa. The following connectivity is supported by the NTN payload:

-	An eNB may serve multiple transparent NTN payloads;

-	An transparent NTN payload may be served by multiple eNBs.

NOTE:	In this release, the transparent NTN-payload may change the carrier frequency, before re-transmitting it on the service link, and vice versa (respectively on the feeder link).

For NTN, the following applies in addition to Network entity related Identities as described in clause 8.2:

-	A Tracking Area corresponds to a fixed geographical area. Any respective mapping is configured in the RAN;

-	A Mapped Cell ID as specified in clause 23.21.5.

Three types of service links are supported:

-	Earth-fixed: provisioned by beam(s) continuously covering the same geographical areas all the time (e.g., the case of GSO satellites);

-	Quasi-Earth-fixed: provisioned by beam(s) covering one geographic area for a limited period of time and a different geographic area during another period of time (e.g., the case of NGSO satellites generating steerable beams);

-	Earth-moving: provisioned by beam(s) whose coverage area slides over the Earth surface (e.g., the case of NGSO satellites generating fixed or non-steerable beams).

With NGSO satellites, the eNB can provide either quasi-Earth-fixed cell coverage or Earth-moving cell coverage, while eNB operating with GSO satellites can provide Earth fixed cell coverage or quasi-Earth-fixed cell coverage.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

23.21.X	Support of Store and Forward satellite operation

To enable the Store and Forward satellite operation, two kinds of architecture on the satellite are supported, i.e., the Split MME architecture and the Full CN on board architecture.

In Split MME architecture, the MME functionality is split into two parts: MME-onboard and MME-ground. The eNB and the MME-on board are located on board, and MME-ground, including the HSS, is located on the ground.

In Full CN on board architecture, the eNB, MME, SCG, PGW, HSS, E-SMLC, SMSC, etc, are located on board.

Editor’s Note: The above description needs further checking.

Annex P (informative):
Example implementation of Non-Terrestrial Networks

The following figure illustrates an example implementation of a Non-Terrestrial Network for transparent NTN payload:

Figure P-1: NTN based E-UTRAN with transparent NTN payload

The eNB depicted in Figure P-1 may be subdivided into non-NTN infrastructure eNB functions and the NTN Service Link provisioning System. The NTN infrastructure may be thought of being subdivided into the NTN Service Link provisioning System and the NTN Control function. The NTN Service Link provisioning System may consist of one or more NTN payloads and NTN Gateways.

The NTN payload is embarked on a spaceborne (or airborne) vehicle, providing a structure, power, commanding, telemetry, attitude control for the satellite (resp. high altitude platform station) and possibly an appropriate thermal environment and radiation shielding.

The NTN Service Link provisioning System maps the Uu radio protocol over radio resources of the NTN infrastructure (e.g. beams, channels, Tx power).

The NTN control function controls the spaceborne vehicles as well as the radio resources of the NTN infrastructure (NTN payload(s) & NTN Gateway(s)). It provides control data, e.g. Ephemeris, to the non-NTN infrastructure eNB functions of the eNB.

Provision of NTN control data to the eNB is out of 3GPP scope.

NOTE:	The transport of Uu protocol between the NTN Service Link provisioning system and the non-NTN infrastructure eNB functions is out of 3GPP scope.

At least the following NTN related parameters are expected to be provided by O&M to the eNB for its operation.

a) Earth fixed beams: for each beam provided by a given NTN-payload:

-	The Cell identifier (S1 and Uu) mapped to the beam;

-	The Cell's reference location (e.g. cell's center and range).

b) Quasi Earth fixed beams: for each beam provided by a given NTN-payload:

-	The Cell identifier (S1 and Uu) and time window mapped to a beam;

-	The Cell's/beam's reference location (e.g. cell's center and range);

-	The time window of the successive switchovers (feeder link, service link);

-	The identifier and time window of all serving NTN payloads and NTN-Gateways.

c) Earth moving beams: for each beam provided by a given NTN-payload:

-	The Uu Cell identifier mapped to a beam and mapping information to fixed geographical areas reported on NG, including information about the beams direction and motion of the beam's foot print on Earth;

-	Its elevation with regard to NTN-payload;

-	Schedule of successive serving NTN-Gateways/eNBs;

-	Schedule of successive switchovers (feeder link, service link).

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>