3GPP TSG-RAN WG3 #126	R3-247552

# Orlando, U.S.A., 18th-22nd November 2024

Agenda Item:	17.3

Source:	Ericsson

Title: 	F1 impact of On-demand SIB1 for UEs in idle/inactive mode

Document for:	Discussion and Approval

# 1	Background

A work item on “Enhancements of network energy savings for NR” was agreed in [1]. The Work Item Description (WID) was revised in [2]. According to the revised WID, the objectives of the work item are the following:

- Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4] - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling) - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum. - Specify support for on-demand SIB1 for UEs in idle/inactive mode [RAN1/2/3] - Specify procedures and signaling method(s) for Case 2 [RAN1/2] - Case 2: UE obtains UL WUS configuration from Cell A, UE transmits UL WUS on NES Cell, UE receives on-demand SIB1 from NES Cell - Triggering method by UL WUS using PRACH - Specify inter NG-RAN node signalling at least for the configuration of UL WUS [RAN3] - Note 1: No modification of SSB will be discussed under this objective - Note 2: - UL WUS: Uplink wake-up signal - Cell A: A cell that is periodically transmitting at least its own SIB1 - NES Cell: A cell that may transmit SIB1 transmission in response to UL WUS from a UE - Note 3: - RAN1 strives to minimize impact to legacy UE - RAN1 specification impact to support this feature should be minimized - Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4] - Adaptation of SSB in time domain, e.g. adapting periodicity - Adaptation of PRACH in time domain - Adaptation of paging occasions including confining the paging occasions in the time domain - Note: there shall be no paging latency increase - Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown - Specify the corresponding core requirements, for the above features [RAN4].

In this contribution, we discuss issues and solutions regarding Objective 2 of the work item (see highlighted text above), focusing on the impact on F1.

A companion contribution discussing inter NG-RAN node signalling and Xn impact is provided in [4].

# 2	Discussion

In the following, we discuss the main principles of the intra NG-RAN node signaling to enable on-demand SIB1 (OD-SIB1) transmission, considering split architecture.

General scenario

Figure 1 illustrates an example deployment scenario for OD-SIB1 transmission, where the “NES Cell” of gNB1 is meant to operate in OD-SIB1 transmission mode and the “Cell A” of gNB2 provides the UL Wake-Up Signal (WUS) configuration (that is needed to request SIB1 transmission in the NES Cell) to UEs. The scenario corresponds to Case 2 as defined by RAN1, assumed for normative work according to the WID.

Figure 1: An example deployment scenario for OD-SIB1 transmission (corresponding to Case 2).

We assume that, before the NES Cell operates in OD-SIB1 transmission mode, gNB1 provides the UL WUS configuration of the NES Cell to gNB2. Then, gNB2 provides the UL WUS configuration of the NES Cell to UEs camping on (or connected to) Cell A. When needed, a UE (in RRC Idle/Inactive mode) sends the UL WUS to the NES Cell and, in response, receives SIB1 from the NES Cell.

We now discuss the different signaling steps within the NES-Cell gNB (gNB1) as well as the Cell-A gNB (gNB2) that are needed to enable OD-SIB1 transmission in a split architecture scenario.

Agreement from the previous meeting, refer to [3]

At the RAN3#125 meeting, the following was agreed:

Case 2 can be supported with the below details:

- UL WUS configuration is decided by NES Cell DU.

- UL WUS configuration transmission from NES Cell DU to NES Cell CU. NES gNB-CU sends UL WUS configuration to Cell A gNB-CU over Xn Interface.

- Upon the reception of the UL WUS configuration, Cell A can provide feedback to NES Cell on whether the WUS configuration Transmission for NES Cell is accepted in Cell A (e.g., confirm, reject). FFS if Cell A broadcasts it right away or not.

Signalling in the NES-Cell gNB

RAN3 has agreed that the UL WUS configuration is decided by the NES-Cell DU and sent to the NES-Cell CU. This is shown in Figure 2. As suggested there, the existing gNB-DU Configuration Update procedure can simply be enhanced to support this functionality, e.g., as part of the Served Cell Information IE. It seems natural to include the UL WUS configuration in the Served Cell Information IE, which already comprises all the configuration information of a cell.

Figure 2: Signalling of the UL WUS configuration from NES-Cell DU to NES-Cell CU.

Proposal 1: Reuse the gNB-DU Configuration Update procedure over F1 for sending the UL WUS configuration of a certain cell from NES-Cell gNB-DU to NES-Cell gNB-CU.

The NES-Cell CU can interpret reception of the UL WUS configuration as interest of the NES-Cell DU to operate the NES Cell in OD-SIB1 transmission mode. Alternatively, an explicit indication could be introduced. The gNB-CU receives the WUS configuration from gNB-DU and starts coordinating the UL WUS configuration provisioning, e.g., by sending a request to start UL WUS configuration provisioning to a neighbouring Cell-A gNB (here gNB2) as discussed in [4].

It has been discussed and reflected in agreements in RAN3 that feedback on UL WUS configuration provisioning is needed over Xn. The same applies to F1 in case of split architecture, i.e., the DU cannot start OD-SIB1 transmission in the NES Cell before its CU indicates that the DU may do so, because the DU does not know if and when the UL WUS configuration provisioning is ensured. Thus, the DU cannot autonomously start OD-SIB1 transmission in a NES Cell without risking making the cell inaccessible for UEs. Hence, the DU requires indication from the CU.

Observation 1: In split architecture, the NES-Cell DU does not know if and when the UL WUS configuration provision for a NES Cell is ensured and thus, cannot autonomously start OD-SIB1 transmission in the NES Cell. Instead, it requires assistance information from the NES-Cell CU.

Proposal 2: In split architecture, the NES-Cell gNB-CU should indicate to the NES-Cell gNB-DU whether a NES Cell operation can be changed to on-demand SIB1 transmission mode.

This is shown in Figure 3. Once the UL WUS configuration provisioning for the NES Cell is ensured, the CU indicates to the DU that OD-SIB1 transmission is allowed in the NES Cell.

Figure 3: Indication from NES-Cell CU to NES-Cell DU that OD-SIB1 transmission is possible/permitted in a NES Cell.

We believe, the gNB-CU Configuration Update procedure can be reused for this purpose, namely the GNB-CU CONFIGURATION UPDATE message may carry an indication, e.g., a list, of cells that can be operated in OD-SIB1 transmission mode. Conceptually, such list closely resembles the Cells Allowed to be Deactivated List IE, introduced in the GNB-CU CONFIGURATION UPDATE message in Rel-18, indicating cells within which the DU is allowed to deactivate SSB beams for network energy saving purpose. Thus, it seems natural to also reuse the gNB-CU Configuration Update procedure for the purpose described above.

Proposal 3: RAN3 to agree to reuse the gNB-CU Configuration Update procedure over F1 for sending from gNB-CU to gNB-DU an indication that certain cell(s) can be operated in on-demand SIB1 transmission mode.

While OD-SIB1 transmission is ongoing, the CU may determine that the UL WUS configuration provisioning for a NES Cell is (temporarily) interrupted, e.g., upon receiving an according indication from a neighbouring Cell-A gNB as discussed in [4]. In that case, the CU can revoke the permission to operate the NES Cell in OD-SIB1 transmission mode, e.g., by sending to the DU a modified (in this case reduced) list of cells for which OD-SIB1 transmission is allowed. In other words, the cells for which OD-SIB1 transmission is no longer allowed will not be on the list anymore. This is shown in Figure 4. Consequently, the DU immediately stops OD-SIB1 transmission, provided that it is ongoing.

Figure 4: Indication from NES-Cell CU to NES-Cell DU that OD-SIB1 transmission is not possible (not permitted) in a NES Cell.

Similarly, if the CU identifies that the current conditions are not suitable for OD-SIB1 transmission, e.g., due to high load of legacy UEs in overlaid cells, it can revoke the permission to operate the NES Cell in OD-SIB1 transmission mode.

Inversely, if the DU identifies that the current conditions are not suitable for OD-SIB1 transmission, e.g., due to large number of SIB1 transmission requests in a NES Cell, resulting in diminishing or even negative NES gains, it can revoke UL WUS configuration of the NES Cell. Alternatively, it may explicit indicate that it has no (longer) interest in operating the NES Cell in OD-SIB1 transmission mode for the time being. This is shown in Figure 6.

Figure 6: Indication from NES-Cell DU to NES-Cell CU that the UL WUS configuration of a NES Cell is invalidated (DU identifies unsuitable conditions for OD-SIB1 transmission).

It is important for the CU to know when the DU has interest to operate a NES Cell in OD-SIB1 transmission mode and when not, because the CU is coordinating the UL WUS configuration provisioning, regardless of whether NES Cell and Cell A(s) are in different gNBs or in the same gNB or even in the same DU, and because unnecessary provisioning of an UL WUS configuration means a waste of radio resources as well as energy.

Signaling in the Cell-A gNB

It was already agreed in RAN2#126 that the UL WUS configuration of a NES Cell can be provided to UEs via a new or existing SIB in Cell A. We assume a new SIB is introduced, here referred to as “SIBx”. However, the proposed solution equally applies in case an existing SIB in Cell A is reused, except SIB1, which is DU owned (and not CU owned).

As shown in Figure 7, when a Cell-A CU identifies a change in the UL WUS configuration provision demand concerning Cell A, i.e., that a new UL WUS configuration needs to be provided, or that an existing UL WUS configuration does not need to be provided anymore, the Cell-A CU sends an updated version of SIBx of Cell A to the Cell-A DU, so that the Cell-A DU provides all (and only) the required UL WUS configuration(s) to UEs in Cell A.

Figure 7: Signalling of the SIBx for Cell A (comprising one or several UL WUS configurations of overlaid NES Cells) from Cell-A CU to Cell-A DU.

The Cell-A CU may identify a change in the UL WUS configuration provision demand based on external indication, e.g., a UL WUS configuration provision start (or stop) request from a neighbouring NES-Cell gNB (here gNB1) as discussed in [4].

If the NES Cell and Cell A are in the same gNB, the identification of a change in the UL WUS configuration provision demand may be based on an indication from a DU of that gNB to the CU.

# 4	Proposals

In this contribution, we discussed issues and high-level solutions for Objective 2 of the work item mentioned above and made the following observations and proposals:

Proposal 1: Reuse the gNB-DU Configuration Update procedure over F1 for sending the UL WUS configuration of a certain cell from NES-Cell gNB-DU to NES-Cell gNB-CU.

Observation 1: In split architecture, the NES-Cell DU does not know if and when the UL WUS configuration provision for a NES Cell is ensured and thus, cannot autonomously start OD-SIB1 transmission in the NES Cell. Instead, it requires assistance from the NES-Cell CU.

Proposal 2: In split architecture, the NES-Cell gNB-CU should indicate to the NES-Cell gNB-DU whether a NES Cell operation can be changed to on-demand SIB1 transmission mode.

Proposal 3: RAN3 to agree to reuse the gNB-CU Configuration Update procedure over F1 for sending from gNB-CU to gNB-DU an indication that certain cell(s) can be operated in on-demand SIB1 transmission mode.

A TP reflecting the proposals above is available below.

# 5	Text Proposal for BL CR for TS 38.473

*************** Start of TP ***************

### 8.2.3	F1 Setup

#### 8.2.3.1	General

The purpose of the F1 Setup procedure is to exchange application level data needed for the gNB-DU and the gNB-CU to correctly interoperate on the F1 interface. This procedure shall be the first F1AP procedure triggered for the F1-C interface instance after a TNL association has become operational.

NOTE:	If F1-C signalling transport is shared among multiple F1-C interface instances, one F1 Setup procedure is issued per F1-C interface instance to be setup, i.e. several F1 Setup procedures may be issued via the same TNL association after that TNL association has become operational.

NOTE:	Exchange of application level configuration data also applies between the gNB-DU and the gNB-CU in case the DU does not broadcast system information other than for radio frame timing and SFN, as specified in the TS 37.340 [7]. How to use this information when this option is used is not explicitly specified.

The procedure uses non-UE associated signalling.

This procedure erases any existing application level configuration data in the two nodes and replaces it by the one received. This procedure also re-initialises the F1AP UE-related contexts (if any) and erases all related signalling connections in the two nodes like a Reset procedure would do.

#### 8.2.3.2	Successful Operation

Figure 8.2.3.2-1: F1 Setup procedure: Successful Operation

The gNB-DU initiates the procedure by sending a F1 SETUP REQUEST message including the appropriate data to the gNB-CU. The gNB-CU responds with a F1 SETUP RESPONSE message including the appropriate data.

The exchanged data shall be stored in respective node and used as long as there is an operational TNL association. When this procedure is finished, the F1 interface is operational and other F1 messages may be exchanged.

If the F1 SETUP REQUEST message contains the gNB-DU Name IE, the gNB-CU may use this IE as a human readable name of the gNB-DU. If the F1 SETUP REQUEST message contains the Extended gNB-DU Name IE, the gNB-CU may use this IE as a human readable name of the gNB-DU and shall ignore the gNB-DU Name IE if included.

If the F1 SETUP RESPONSE message contains the gNB-CU Name IE, the gNB-DU may use this IE as a human readable name of the gNB-CU. If the F1 SETUP RESPONSE message contains the Extended gNB-CU Name IE, the gNB-DU may use this IE as a human readable name of the gNB-CU and shall ignore the gNB-CU Name IE if included.

If the F1 SETUP REQUEST message contains the gNB-DU Served Cells List IE, the gNB-CU shall take into account as specified in TS 38.401 [4].

For NG-RAN, the gNB-DU shall include the gNB-DU System Information IE and the TAI Slice Support List IE in the F1 SETUP REQUEST message.

The gNB-CU may include the Cells to be Activated List IE in the F1 SETUP RESPONSE message. The Cells to be Activated List IE includes a list of cells that the gNB-CU requests the gNB-DU to activate. The gNB-DU shall activate the cells included in the Cells to be Activated List IE and reconfigure the physical cell identity for cells for which the NR PCI IE is included.

If Cells to be Activated List Item IE is included in the F1 SETUP RESPONSE message, and the information for the cell indicated by the NR CGI IE includes the IAB Info IAB-donor-CU IE, the gNB-DU shall, if supported, apply the IAB STC Info IE therein to the indicated cell.

For NG-RAN, the gNB-CU shall include the gNB-CU System Information IE in the F1 SETUP RESPONSE message.

For NG-RAN, the gNB-DU may include the RAN Area Code IE in the F1 SETUP REQUEST message. The gNB-CU may use it according to TS 38.300 [6].

For NG-RAN, the gNB-DU may include Supported MBS FSA ID List IE in the Served Cell Information IE in the F1 SETUP REQUEST message. The gNB-CU may use it according to TS 38.300 [6].

For NG-RAN, the gNB-CU may include Available PLMN List IE, and optionally also Extended Available PLMN List IE in the F1 SETUP RESPONSE message, if the available PLMN(s) are different from what gNB-DU has provided in F1 SETUP REQUEST message, gNB-DU shall take this into account and only broadcast the PLMN(s) included in the received Available PLMN list(s).

For NG-RAN, the gNB-CU may include Available SNPN ID List IE in the F1 SETUP RESPONSE message. If the available SNPN(s) are different from what gNB-DU has provided in F1 SETUP REQUEST message, gNB-DU shall take this into account and only broadcast the SNPN(s) included in the received Available SNPN ID list.

The Latest RRC Version Enhanced IE shall be included in the F1 SETUP REQUEST message and in the F1 SETUP RESPONSE message.

If in F1 SETUP REQUEST message, the Cell Direction IE is present, the gNB-CU should use it to understand whether the cell is for UL or DL only. If in F1 SETUP REQUEST message, the Cell Direction IE is omitted in the Served Cell Information IE it shall be interpreted as that the Cell Direction is Bi-directional.

If the Intended TDD DL-UL Configuration IE is present in the F1 SETUP REQUEST message, the receiving gNB-CU shall use the received information for Cross Link Interference management and/or NR-DC power coordination. The gNB-CU may merge the Intended TDD DL-UL Configuration information received from two or more gNB-DUs. The gNB-CU shall consider the received Intended TDD DL-UL Configuration IE content valid until reception of an update of the IE for the same cell(s).

If the Aggressor gNB Set ID IE is included in the Served Cell Information IE in the F1 SETUP REQUEST message, the gNB-CU shall, if supported, take it into account.

If the Victim gNB Set ID IE is included in the Served Cell Information IE in the F1 SETUP REQUEST message, the gNB-CU shall, if supported, take it into account.

If the F1 SETUP REQUEST message contains the Transport Layer Address Info IE, the gNB-CU shall, if supported, take into account for IPSec tunnel establishment.

If the SFN Offset IE is contained in the Served Cell Information IE in the F1 SETUP REQUEST message, the gNB-CU shall, if supported, use this information to deduce the SFN0 offset of the reported cell.

If the F1 SETUP RESPONSE message contains the Transport Layer Address Info IE, the gNB-DU shall, if supported, take into account for IPSec tunnel establishment.

If the F1 SETUP RESPONSE message contains the Uplink BH Non-UP Traffic Mapping IE, the gNB-DU shall, if supported, consider the information therein for mapping of non-UP uplink traffic.

If the BAP Address IE is included in the F1 SETUP REQUEST message, without an accompanying RRC Terminating IAB-Donor gNB-ID IE, the receiving gNB-CU shall, if supported, consider the information therein for discovering the co-location of an IAB-DU and an IAB-MT.

If the F1 SETUP REQUEST message is received from an IAB-donor-DU, the gNB-CU shall, if supported, include the BAP Address IE in the F1 SETUP RESPONSE message.

NOTE:	How to identify the IAB-donor-DU is up to gNB-CU implementation.

If the F1 SETUP RESPONSE message contains the BAP Address IE, the gNB-DU shall, if supported, store the received BAP address and use it as specified in TS 38.340 [30].

If the NR PRACH Configuration List IE is included in the Served Cell Information IE contained in the F1 SETUP REQUEST message, the gNB-CU may store the information, and forward it to other RAN nodes for RACH optimisation. If the L139 Info IE included in the NR PRACH Configuration List IE is present, it shall contain the Root Sequence Index IE.

If the RedCap Broadcast Information IE is included in the Served Cell Information IE in the F1 SETUP REQUEST message, the gNB-CU may store and use this information to determine a suitable target in case of subsequent outgoing mobility involving RedCap UEs.

If the eRedCap Broadcast Information IE is included in the Served Cell Information IE in the F1 SETUP REQUEST message, the gNB-CU may store and use this information to determine a suitable target in case of subsequent outgoing mobility involving eRedCap UEs.

If the TAI NSAG Support List IE is included in the Served Cell Information IE in the F1 SETUP REQUEST message, the gNB-CU shall, if supported, use this information as specified in TS 23.501 [21].

If both the RRC Terminating IAB-Donor gNB-ID IE and the BAP Address IE are included in the F1 SETUP REQUEST message, the gNB-CU shall, if supported, consider that the BAP address indicated by the BAP Address IE is assigned by the gNB-CU of the RRC-terminating IAB-donor indicated by the RRC Terminating IAB-Donor gNB-ID IE, and use this BAP address and gNB-ID for the subsequent IAB Transport Migration Management procedure towards the RRC-terminating IAB-donor of the mobile IAB-node, as specified in TS 38.423 [28].

If the F1 SETUP REQUEST message contains the Mobile IAB-MT User Location Information IE, the gNB-CU shall, if supported, take it into account when reporting UE location information to the AMF for a UE served by the mobile IAB-node.

If the NCGI to be Updated List IE is included in the F1 SETUP RESPONSE message, the gNB-DU shall, if supported, change the NCGI of the cell indicated by the Old NCGI IE to the NCGI indicated by the New NCGI IE.

If the UL WUS Configuration IE is included in the Served Cell Information IE in the F1 SETUP REQUEST message, the gNB-CU shall, if supported, store the information and initiate the provision of the UL WUS configuration.

Interaction with the gNB-CU Configuration Update procedure:

As specified in TS 38.401 [x], once the UL WUS configuration provisioning is ensured, the gNB-CU shall initiate the gNB-CU Configuration Update procedures.

#### 8.2.3.3	Unsuccessful Operation

Figure 8.2.3.3-1: F1 Setup procedure: Unsuccessful Operation

If the gNB-CU cannot accept the setup, it should respond with a F1 SETUP FAILURE and appropriate cause value.

If the F1 SETUP FAILURE message includes the Time To Wait IE, the gNB-DU shall wait at least for the indicated time before reinitiating the F1 setup towards the same gNB-CU.

#### 8.2.3.4	Abnormal Conditions

Not applicable.

*************** Next change ***************

### 8.2.4	gNB-DU Configuration Update

#### 8.2.4.1	General

The purpose of the gNB-DU Configuration Update procedure is to update application level configuration data needed for the gNB-DU and the gNB-CU to interoperate correctly on the F1 interface. This procedure does not affect existing UE-related contexts, if any. The procedure uses non-UE associated signalling.

NOTE:	Update of application level configuration data also applies between the gNB-DU and the gNB-CU in case the DU does not broadcast system information other than for radio frame timing and SFN, as specified in the TS 37.340 [7]. How to use this information when this option is used is not explicitly specified.

#### 8.2.4.2	Successful Operation

Figure 8.2.4.2-1: gNB-DU Configuration Update procedure: Successful Operation

The gNB-DU initiates the procedure by sending a GNB-DU CONFIGURATION UPDATE message to the gNB-CU including an appropriate set of updated configuration data that it has just taken into operational use. The gNB-CU responds with GNB-DU CONFIGURATION UPDATE ACKNOWLEDGE message to acknowledge that it successfully updated the configuration data. If an information element is not included in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU shall interpret that the corresponding configuration data is not changed and shall continue to operate the F1-C interface with the existing related configuration data.

The updated configuration data shall be stored in both nodes and used as long as there is an operational TNL association or until any further update is performed.

If gNB-DU ID IE is contained in the GNB-DU CONFIGURATION UPDATE message for a newly established SCTP association, the gNB-CU will associate this association with the related gNB-DU.

If Served Cells To Add Item IE is contained in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU shall add cell information according to the information in the Served Cell Information IE. For NG-RAN, the gNB-DU shall include the gNB-DU System Information IE.

If Served Cells To Modify Item IE is contained in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU shall modify information of cell indicated by Old NR CGI IE according to the information in the Served Cell Information IE and overwrite the served cell information for the affected served cell. Further, if the gNB-DU System Information IE is present the gNB-CU shall store and replace any previous information received.

If Served Cells To Delete Item IE is contained in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU shall delete information of cell indicated by Old NR CGI IE.

If Cells Status Item IE is contained in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU shall update the information about the cells, as described in TS 38.401 [4]. If if the Switching Off Ongoing IE is present in the Cells Status Item IE, contained in the GNB-DU CONFIGURATION UPDATE message, and the corresponding Service State IE is set to "Out-of-Service", the gNB-CU shall ignore the Switching Off Ongoing IE.

If Cells to be Activated List Item IE is contained in the GNB-DU CONFIGURATION UPDATE ACKNOWLEDGE message, the gNB-DU shall activate the cell indicated by NR CGI IE and reconfigure the physical cell identity for cells for which the NR PCI IE is included.

If Cells to be Activated List Item IE is contained in the GNB-DU CONFIGURATION UPDATE ACKNOWLEDGE message and the indicated cells are already activated, the gNB-DU shall update the cell information received in Cells to be Activated List Item IE.

If Cells to be Activated List Item IE is included in the GNB-DU CONFIGURATION UPDATE ACKNOWLEDGE message, and the information for the cell indicated by the NR CGI IE includes the IAB Info IAB-donor-CU IE, the gNB-DU shall, if supported, apply the IAB STC Info IE therein to the indicated cell.

If Cells to be Deactivated List Item IE is contained in the GNB-DU CONFIGURATION UPDATE ACKNOWLEDGE message, the gNB-DU shall deactivate all the cells with NR CGI listed in the IE.

If Dedicated SI Delivery Needed UE List IE is contained in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU should take it into account when informing the UE of the updated system information via the dedicated RRC message.

For NG-RAN, the gNB-CU shall include the gNB-CU System Information IE in the GNB-DU CONFIGURATION UPDATE ACKNOWLEDGE message. The SIB type to Be Updated List IE shall contain the full list of SIBs to be broadcast.

For NG-RAN, the gNB-DU may include the RAN Area Code IE in the GNB-DU CONFIGURATION UPDATE message. The gNB-CU shall store and replace any previously provided RAN Area Code IE by the received RAN Area Code IE.

For NG-RAN, the gNB-DU may include the Supported MBS FSA ID List IE in the Served Cell Information IE in the GNB-DU CONFIGURATION UPDATE message. The gNB-CU shall store and replace any previously provided MBS FSA ID list IE by the received MBS FSA ID list IE.

If Available PLMN List IE, and optionally also Extended Available PLMN List IE, is contained in GNB-DU CONFIGURATION UPDATE ACKNOWLEDGE message, the gNB-DU shall overwrite the whole available PLMN list and update the corresponding system information.

If Available SNPN ID List IE is contained in GNB-DU CONFIGURATION UPDATE ACKNOWLEDGE message, the gNB-DU shall overwrite the whole available SNPN ID list and update the corresponding system information.

If in GNB-DU CONFIGURATION UPDATE message, the Cell Direction IE is present, the gNB-CU should use it to understand whether the cell is for UL or DL only. If in GNB-DU CONFIGURATION UPDATE message, the Cell Direction IE is omitted in the Served Cell Information IE it shall be interpreted as that the Cell Direction is Bi-directional.

If the GNB-DU CONFIGURATION UPDATE message includes gNB-DU TNL Association To Remove List IE, the gNB-CU shall, if supported, initiate removal of the TNL association(s) indicated by gNB-DU TNL endpoint(s) and gNB-CU TNL endpoint(s) if the TNL Association Transport Layer Address gNB-CU IE is present, or the TNL association(s) indicated by gNB-DU TNL endpoint(s) if the TNL Association Transport Layer Address gNB-CU IE is absent:

- 	if the received TNL Association Transport Layer Address IE includes the Port Number IE, the gNB-DU TNL endpoint is identified by the Endpoint IP Address IE and the Port Number IE. Otherwise, the gNB-DU TNL endpoints correspond to all gNB-DU TNL endpoints identified by the Endpoint IP Address IE and any port number(s).

- 	if the received TNL Association Transport Layer Address gNB-CU IE includes the Port Number IE, the gNB-CU TNL endpoint is identified by the Endpoint IP Address IE and the Port Number IE. Otherwise, the gNB-CU TNL endpoints correspond to all gNB-CU TNL endpoints identified by the Endpoint IP Address IE and any port number(s).

If the Intended TDD DL-UL Configuration IE is present in the GNB-DU CONFIGURATION UPDATE message, the receiving gNB-CU shall use the received information for Cross Link Interference management and/or NR-DC power coordination. The gNB-CU may merge the Intended TDD DL-UL Configuration information received from two or more gNB-DUs. The gNB-CU shall consider the received Intended TDD DL-UL Configuration IE content valid until reception of an update of the IE for the same cell(s).

If the Aggressor gNB Set ID IE is included in the Served Cell Information IE in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU shall, if supported, take it into account.

If the Victim gNB Set ID IE is included in the Served Cell Information IE in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU shall, if supported, take it into account.

If the GNB-DU CONFIGURATION UPDATE message includes Transport Layer Address Info IE, the gNB-CU shall, if supported, take into account for IPSec tunnel establishment.

If the GNB-DU CONFIGURATION UPDATE ACKNOWLEDGE message includes Transport Layer Address Info IE, the gNB-DU shall, if supported, take into account for IPSec tunnel establishment.

If the GNB-DU CONFIGURATION UPDATE ACKNOWLEDGE message contains the Uplink BH Non-UP Traffic Mapping IE, the gNB-DU shall, if supported, consider the information therein for mapping of non-UP uplink traffic.

If the SFN Offset IE is contained in the Served Cell Information IE in GNB-DU CONFIGURATION UPDATE message, the gNB-CU shall, if supported, use this information to deduce the SFN0 offset of the reported cell.

If the NR PRACH Configuration List IE is included in the Served Cell Information IE contained in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU may store the information, and forward it to other RAN nodes for RACH optimisation. If the L139 Info IE included in the NR PRACH Configuration List IE is present, it shall contain the Root Sequence Index IE.

If the GNB-DU CONFIGURATION UPDATE ACKNOWLEDGE message contains the BAP Address IE, the gNB-DU shall, if supported, store the received BAP address and use it as specified in TS 38.340 [30].

If the Coverage Modification Notification IE is contained in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU shall, if supported, take it into account for Coverage and Capacity Optimization and network energy saving. If the Coverage Modification Cause IE is set to the "network energy saving", gNB-CU may consider those deactivated SSB beams are due to network energy saving.

If the Cells for SON IE is present in the GNB-DU CONFIGURATION UPDATE ACKNOWLEDGE message, the gNB-DU may store or update this information and behaves as follows:

-	For each served cell indicated by the NR CGI IE included within the Cells for SON Item IE, the gNB-DU may adjust the PRACH configuration of this served cell.

-	If the Neighbour NR Cells for SON List IE is present in the Cells for SON Item IE, the gNB-DU may take the PRACH configuration of neighbour cells included in the Neighbour NR Cells for SON List IE into consideration when adjusting the PRACH configuration of the served cell.

If the RedCap Broadcast Information IE is contained in the Served Cell Information IE in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU may store and use this information to determine a suitable target in case of subsequent outgoing mobility involving RedCap UEs.

If the eRedCap Broadcast Information IE is contained in the Served Cell Information IE in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU may store and use this information to determine a suitable target in case of subsequent outgoing mobility involving eRedCap UEs.

If the TAI NSAG Support List IE is included in the Served Cell Information IE in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU shall, if supported, use this information as specified in TS 23.501 [21].

If the gNB-DU Name IE is included in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU may store it or update this IE value if already stored, and use it as a human readable name of the gNB-DU. If the Extended gNB-DU Name IE is included in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU may store it or update this IE value if already stored, and use it as a human readable name of the gNB-DU and shall ignore the gNB-DU Name IE if also included.

If the RRC Terminating IAB-Donor Related Info IE is included in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU shall, if supported, consider that the BAP address indicated by the Mobile IAB-MT BAP Address IE is assigned by the gNB-CU of the RRC-terminating IAB-donor indicated  by the RRC Terminating IAB-Donor gNB-ID IE, and it shall use this  BAP address and gNB ID for the subsequent IAB Transport Migration Management procedure towards the RRC-terminating IAB-donor of the mobile IAB-node as needed, as specified in TS 38.423 [28].

If the GNB-DU CONFIGURATION UPDATE message contains the Mobile IAB-MT User Location Information IE, the gNB-CU shall, if supported, take it into account when reporting UE location information to the AMF for a UE served by the mobile IAB-node.

If the UL WUS Configuration IE is included in the Served Cell Information IE in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU shall, if supported, store the information and initiate the provision of the UL WUS configuration.

Interaction with the gNB-CU Configuration Update procedure:

As specified in TS 38.401 [x], once the UL WUS configuration provisioning is ensured, the gNB-CU shall initiate the gNB-CU Configuration Update procedures.

#### 8.2.4.3	Unsuccessful Operation

Figure 8.2.4.3-1: gNB-DU Configuration Update procedure: Unsuccessful Operation

If the gNB-CU cannot accept the update, it shall respond with a GNB-DU CONFIGURATION UPDATE FAILURE message and appropriate cause value.

If the GNB-DU CONFIGURATION UPDATE FAILURE message includes the Time To Wait IE, the gNB-DU shall wait at least for the indicated time before reinitiating the GNB-DU CONFIGURATION UPDATE message towards the same gNB-CU.

#### 8.2.4.4	Abnormal Conditions

Not applicable.

*************** Next change ***************

### 8.2.5	gNB-CU Configuration Update

#### 8.2.5.1	General

The purpose of the gNB-CU Configuration Update procedure is to update application level configuration data needed for the gNB-DU and gNB-CU to interoperate correctly on the F1 interface. This procedure does not affect existing UE-related contexts, if any. The procedure uses non-UE associated signalling.

#### 8.2.5.2	Successful Operation

Figure 8.2.5.2-1: gNB-CU Configuration Update procedure: Successful Operation

The gNB-CU initiates the procedure by sending a GNB-CU CONFIGURATION UPDATE message including the appropriate updated configuration data to the gNB-DU. The gNB-DU responds with a GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE message to acknowledge that it successfully updated the configuration data. If an information element is not included in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall interpret that the corresponding configuration data is not changed and shall continue to operate the F1-C interface with the existing related configuration data.

The updated configuration data shall be stored in the respective node and used as long as there is an operational TNL association or until any further update is performed.

If Cells to be Activated List Item IE is contained in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall activate the cell indicated by NR CGI IE and reconfigure the physical cell identity for which the NR PCI IE is included.

If the SSBs within the cell to be Activated List IE is included in the Cells to be Activated List Item IE within the gNB-CU CONFIGURATION UPDATE message, the gNB-DU shall, if supported, only activate those SSB beams indicated by the SSB Index IE.

If at least one requested SSB beam in the SSBs within the cell to be Activated List IE is activated, the gNB-DU includes the Cells with SSBs Activated List IE in the GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE message. The gNB-CU shall consider that the SSB beams indicated by the SSBs activated List IE as activated.

If Cells to be Deactivated List Item IE is contained in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall deactivate the cell indicated by NR CGI IE.

If Cells to be Activated List Item IE is contained in the GNB-CU CONFIGURATION UPDATE message and the indicated cells are already activated, the gNB-DU shall update the cell information received in Cells to be Activated List Item IE.

If Cells to be Activated List Item IE is included in the GNB-CU CONFIGURATION UPDATE message, and the information for the cell indicated by the NR CGI IE includes the IAB Info IAB-donor-CU IE, the gNB-DU shall, if supported, apply the IAB STC Info IE therein to the indicated cell.

If the Cells Allowed to be Deactivated List IE is contained in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall, if supported, consider that it is allowed to deactivate the SSB beams within the indicated cells for network energy saving purpose.

If the gNB-CU System Information IE is contained in the gNB-CU CONFIGURATION UPDATE message, the gNB-DU shall include the Dedicated SI Delivery Needed UE List IE in the GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE message for UEs that are unable to receive system information from broadcast.

If Dedicated SI Delivery Needed UE List IE is contained in the GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE message, the gNB-CU should take it into account when informing the UE of the updated system information via the dedicated RRC message.

If the gNB-CU TNL Association To Add List IE is contained in the gNB-CU CONFIGURATION UPDATE message, the gNB-DU shall, if supported, use it to establish the TNL association(s) with the gNB-CU. If the gNB-CU TNL Association To Add List is included in the GNB-CU CONFIGURATION UPDATE message, and if the TNL Association Transport Layer Information IE does not include the Port Number IE, the gNB-DU shall assume that port number value 38472 is used for the endpoint. The gNB-DU shall report to the gNB-CU, in the gNB-CU CONFIGURATION UPDATE ACKNOWLEDGE message, the successful establishment of the TNL association(s) with the gNB-CU as follows:

-	A list of TNL address(es) with which the gNB-DU successfully established the TNL association shall be included in the gNB-CU TNL Association Setup List IE;

-	A list of TNL address(es) with which the gNB-DU failed to establish the TNL association shall be included in the gNB-CU TNL Association Failed To Setup List IE.

If the GNB-CU CONFIGURATION UPDATE message includes gNB-CU TNL Association To Remove List IE, the gNB-DU shall, if supported, initiate removal of the TNL association(s) indicated by gNB-CU TNL endpoint(s) and gNB-DU TNL endpoint(s) if the TNL Association Transport Layer Address gNB-DU IE is present, or the TNL association(s) indicated by gNB-CU TNL endpoint(s) if the TNL Association Transport Layer Address gNB-DU IE is absent:

- 	if the received TNL Association Transport Layer Address IE includes the Port Number IE, the gNB-CU TNL endpoint is identified by the Endpoint IP Address IE and the Port Number IE. Otherwise, the gNB-CU TNL endpoints correspond to all gNB-CU TNL endpoints identified by the Endpoint IP Address IE and any port number(s).

- 	if the received TNL Association Transport Layer Address gNB-DU IE includes the Port Number IE, the gNB-DU TNL endpoint is identified by the Endpoint IP Address IE and the Port Number IE. Otherwise, the gNB-DU TNL endpoints correspond to all gNB-DU node TNL endpoints identified by the Endpoint IP Address IE and any port number(s).

If the gNB-CU TNL Association To Update List IE is contained in the gNB-CU CONFIGURATION UPDATE message the gNB-DU shall, if supported, overwrite the previously stored information for the related TNL Association(s).

- 	if the received TNL Association Transport Layer Address IE includes the Port Number IE, the gNB-CU TNL endpoint is identified by the Endpoint IP Address IE and the Port Number IE. Otherwise, the gNB-CU TNL endpoints correspond to all gNB-CU TNL endpoints identified by the Endpoint IP Address IE and any port number(s).

If in the gNB-CU CONFIGURATION UPDATE message the TNL Association usage IE is included in the gNB-CU TNL Association To Add List IE or the gNB-CU TNL Association To Update List IE, the gNB-DU node shall, if supported, use it as described in TS 38.472 [22].

For NG-RAN, the gNB-CU shall include the gNB-CU System Information IE in the GNB-CU CONFIGURATION UPDATE message. The SIB type to Be Updated List IE shall contain the full list of SIBs to be broadcast.

If Protected E-UTRA Resources List IE is contained in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall protect the corresponding resource of the cells indicated by E-UTRA Cells List IE for spectrum sharing between E-UTRA and NR.

If the GNB-CU CONFIGURATION UPDATE message contains the Protected E-UTRA Resource Indication IE, the receiving gNB-DU should forward it to lower layers and use it for cell-level resource coordination. The gNB-DU shall consider the received Protected E-UTRA Resource Indication IE when expressing its desired resource allocation during gNB-DU Resource Coordination procedure. The gNB-DU shall consider the received Protected E-UTRA Resource Indication IE content valid until reception of a new update of the IE for the same gNB-DU.

If Available PLMN List IE, and optionally also Extended Available PLMN List IE, is contained in GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall overwrite the whole available PLMN list and update the corresponding system information.

If Available SNPN ID List IE is contained in GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall overwrite the whole available SNPN ID list and update the corresponding system information.

If Cells Failed to be Activated Item IE is contained in the GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE message, the gNB-CU shall consider that the indicated cells are out-of-service as defined in TS 38.401 [4].

If the Neighbour Cell Information List IE is present in the GNB-CU CONFIGURATION UPDATE message, the receiving gNB-DU shall use the received information for Cross Link Interference management and/or NR-DC power coordination. The gNB-DU shall consider the received Neighbour Cell Information List IE content valid until reception of an update of the IE for the same cell(s). If the Intended TDD DL-UL Configuration NR IE is absent from the Neighbour Cell Information List IE, whereas the corresponding NR CGI IE is present, the receiving gNB-DU shall remove the previously stored Neighbour Cell Information IE corresponding to the NR CGI.

If the GNB-CU CONFIGURATION UPDATE message includes Transport Layer Address Info IE, the gNB-DU shall, if supported, take into account for IPSec tunnel establishment.

If the GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE message includes Transport Layer Address Info IE, the gNB-CU shall, if supported, take into account for IPSec tunnel establishment.

If the GNB-CU CONFIGURATION UPDATE message contains the Uplink BH Non-UP Traffic Mapping IE, the gNB-DU shall, if supported, consider the information therein for mapping of non-UP uplink traffic.

If the IAB Barred IE is included in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall, if supported, consider it as an indication of whether the cell allows IAB-node access or not.

If the BAP Address IE is included in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall, if supported, store the received BAP address and use it as specified in TS 38.340 [30].

If the CCO Assistance Information IE is contained in the GNB-CU CONFIGURATION UPDATE message, and the NR CGI IE contained in the Affected Cells and Beams IE is served by the gNB-DU, the gNB-DU may use it to determine a new cell and/or beam configuration.

If the CCO Assistance Information IE is contained in the GNB-CU CONFIGURATION UPDATE message and the NR CGI IE contained in the Affected Cells and Beams IE is not served by the gNB-DU, the gNB-DU may use it to adjust coverage of its cells.If the CCO issue detection IE set to "network energy saving" is included in the CCO Assistance Information IE, the gNB-DU may consider the indicated SSB beams by the Affected Cells and Beam IE are deactivated due to network energy saving.

If the Cells for SON IE is present in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU may store or update this information and it behaves as follows:

-	For each served cell indicated by the NR CGI IE included within the Cells for SON Item IE, the gNB-DU may adjust the PRACH configuration of this served cell.

-	If the Neighbour NR Cells for SON List IE is present in the Cells for SON Item IE, the gNB-DU may take the PRACH configuration of neighbour cells included in the Neighbour NR Cells for SON List IE into consideration when adjusting the PRACH configuration of the served cell.

If the gNB-CU Name IE is included in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU may store it or update this IE value if already stored, and use it as a human readable name of the gNB-CU. If the Extended gNB-CU Name IE is included in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU may store it or update this IE value if already stored, and use it as a human readable name of the gNB-CU and shall ignore the gNB-CU Name IE if also included.

If the Mobile IAB Barred IE is included in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall, if supported, consider it as an indication of whether the cell allows mobile IAB-node access.

If the Cells Allowed to Use OD-SIB1 Transmission List IE is included in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU can activate OD-SIB1 transmission in the cells indicated by the Cells Allowed to Use OD-SIB1 Transmission List IE. If OD-SIB1 transmission is activated in a cell not indicated by the Cells Allowed to Use OD-SIB1 Transmission List IE, the gNB-DU shall deactivate OD-SIB1 transmission in the cell.

#### 8.2.5.3	Unsuccessful Operation

Figure 8.2.5.3-1: gNB-CU Configuration Update: Unsuccessful Operation

If the gNB-DU cannot accept the update, it shall respond with a GNB-CU CONFIGURATION UPDATE FAILURE message and appropriate cause value.

If the GNB-CU CONFIGURATION UPDATE FAILURE message includes the Time To Wait IE, the gNB-CU shall wait at least for the indicated time before reinitiating the GNB-CU CONFIGURATION UPDATE message towards the same gNB-DU.

#### 8.2.5.4	Abnormal Conditions

Not applicable.

*************** Next change ***************

#### 9.3.1.10	Served Cell Information

This IE contains cell configuration information of a cell in the gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| NR CGI | M |  | 9.3.1.12 |  | - |  |
| NR PCI | M |  | INTEGER (0..1007) | Physical Cell ID | - |  |
| 5GS TAC | O |  | 9.3.1.29 | 5GS Tracking Area Code | - |  |
| Configured EPS TAC | O |  | 9.3.1.29a |  | - |  |
| Served PLMNs |  | 1..<maxnoofBPLMNs> |  | Broadcast PLMNs in SIB 1 associated to the NR Cell Identity in the NR CGI IE | - |  |
| >PLMN Identity | M |  | 9.3.1.14 |  | - |  |
| >TAI Slice Support List | O |  | Slice Support List 9.3.1.37 | Supported S-NSSAIs per PLMN or per SNPN. | YES | ignore |
| >NPN Support Information | O |  | 9.3.1.156 | Supported NPNs per PLMN. | YES | reject |
| >Extended TAI Slice Support List | O |  | Extended Slice Support List 9.3.1.165 | Additional Supported S-NSSAIs per PLMN or per SNPN. | YES | reject |
| >TAI NSAG Support List | O |  | 9.3.1.273 | NSAG information associated with the slices per TAC, per PLMN or per SNPN. | YES | ignore |
| CHOICE NR-Mode-Info | M |  |  |  | - |  |
| >FDD |  |  |  |  | - |  |
| >>FDD Info |  | 1 |  |  | - |  |
| >>>UL FreqInfo | M |  | NR Frequency Info 9.3.1.17 | This IE is ignored if the Cell Direction IE is included and set to “dl-only”. | - |  |
| >>>DL FreqInfo | M |  | NR Frequency Info 9.3.1.17 | This IE is ignored if the Cell Direction IE is included and set to “ul-only”. | - |  |
| >>>UL Transmission Bandwidth | M |  | Transmission Bandwidth 9.3.1.15 | This IE is ignored if the Cell Direction IE is included and set to “dl-only”. | - |  |
| >>>DL Transmission Bandwidth | M |  | Transmission Bandwidth 9.3.1.15 | This IE is ignored if the Cell Direction IE is included and set to “ul-only”. | - |  |
| >>>UL Carrier List | O |  | NR Carrier List 9.3.1.137 | If included, the UL Transmission Bandwidth IE shall be ignored. | YES | ignore |
| >>>DL Carrier List | O |  | NR Carrier List 9.3.1.137 | If included, the DL Transmission Bandwidth IE shall be ignored. | YES | ignore |
| >TDD |  |  |  |  | - |  |
| >>TDD Info |  | 1 |  |  | - |  |
| >>>NR FreqInfo | M |  | NR Frequency Info 9.3.1.17 |  | - |  |
| >>>Transmission Bandwidth | M |  | 9.3.1.15 |  | - |  |
| >>>Intended TDD DL-UL Configuration | O |  | 9.3.1.89 |  | YES | ignore |
| >>>TDD UL-DL Configuration Common NR | O |  | OCTET STRING | Includes the tdd-UL-DL-ConfigurationCommon contained in the ServingCellConfigCommon IE as defined in TS 38.331 [8] | YES | ignore |
| >>>Carrier List | O |  | NR Carrier List 9.3.1.137 | If included, the Transmission Bandwidth IE shall be ignored. | YES | ignore |
| >NR-U |  |  |  |  | YES | ignore |
| >>NR-U Channel Info List |  | 1..< maxnoofNR-UChannelIDs> |  |  | - |  |
| >>>NR-U Channel Info Item |  |  |  |  | - |  |
| >>>>NR-U Channel ID | M |  | INTEGER (1.. maxnoofNR-UChannelIDs, …) | Index to uniquely identify the part of the NR-U Channel Bandwidth consisting of a contiguous set of resource blocks (RBs) on which a channel access procedure is performed in shared spectrum. Value 1 represents the first part of the NR-U Channel Bandwidth on which a channel access procedure is performed. Value 2 represents the second part of the NR-U Channel Bandwidth on which a channel access procedure is performed, and so on. | - |  |
| >>>>NR-U ARFCN | M |  | INTEGER (0.. maxNRARFCN) | It represents the centre frequency of the NR-U Channel Bandwidth for NR bands restricted to operation with shared spectrum channel access, as defined in TS 37.213 [46]. Allowed values are specified in TS 38.101-1 [26] in Table 5.4.2.3-2, Table 5.4.2.3-3 and Table 5.4.2.3-4. | - |  |
| >>>>NR-U Channel Bandwidth | M |  | ENUMERATED (10MHz, 20MHz, 40MHz, 60 MHz, 80 MHz,. …, 100MHz) |  | - |  |
| Measurement Timing Configuration | M |  | OCTET STRING | Includes the MeasurementTimingConfiguration inter-node message defined in TS 38.331 [8]. | - |  |
| RANAC | O |  | RAN Area Code 9.3.1.57 |  | YES | ignore |
| Extended Served PLMNs List |  | 0..1 |  | This is included if more than 6 Served PLMNs is to be signalled. | YES | ignore |
| >Extended Served PLMNs Item |  | 1 ..<maxnoofExtendedBPLMNs> |  |  | - |  |
| >>PLMN Identity | M |  | 9.3.1.14 |  | - |  |
| >>TAI Slice Support List | O |  | Slice Support List 9.3.1.37 | Supported S-NSSAIs per PLMN or per SNPN. | - |  |
| >>NPN Support Information | O |  | 9.3.1.156 | Supported NPNs per PLMN. | YES | reject |
| >>Extended TAI Slice Support List | O |  | Extended Slice Support List 9.3.1.165 | Additional Supported S-NSSAIs per PLMN or per SNPN. | YES | reject |
| >>TAI NSAG Support List | O |  | 9.3.1.273 | NSAG information associated with the slices per TAC, per PLMN or per SNPN. | YES | ignore |
| Cell Direction | O |  | 9.3.1.78 |  | YES | ignore |
| Broadcast PLMN Identity Info List |  | 0..<maxnoofBPLMNsNR> |  | This IE corresponds to the PLMN-IdentityInfoList IE and the NPN-IdentityInfoList IE (if available) in SIB1 as specified in TS 38.331 [8]. All PLMN Identities and associated information contained in the PLMN-IdentityInfoList IE and NPN identities and associated information contained in the NPN-IdentityInfoList IE (if available) are included and provided in the same order as broadcast in SIB1. NOTE: In case of NPN-only cell, the PLMN Identities and associated information contained in the PLMN-IdentityInfoList IE are not included. | YES | ignore |
| >PLMN Identity List | M |  | Available PLMN List 9.3.1.65 | Broadcast PLMN IDs in SIB1 associated to the NR Cell Identity IE | - |  |
| >Extended PLMN Identity List | O |  | Extended Available PLMN List 9.3.1.76 |  | - |  |
| >5GS-TAC | O |  | OCTET STRING (3) |  | - |  |
| >NR Cell Identity | M |  | BIT STRING (36) |  | - |  |
| >RANAC | O |  | RAN Area Code 9.3.1.57 |  | - |  |
| >Configured TAC Indication | O |  | 9.3.1.87a | NOTE: This IE is associated with the 5GS TAC in the Broadcast PLMN Identity Info List IE | YES | ignore |
| >NPN Broadcast Information | O |  | 9.3.1.157 | If this IE is included the content of the PLMN Identity List IE and Extended PLMN Identity List IE if present in the Broadcast PLMN Identity Info List IE is ignored. | YES | reject |
| Cell Type | O |  | 9.3.1.87 |  | YES | ignore |
| Configured TAC Indication | O |  | 9.3.1.87a | NOTE: This IE is associated with the 5GS TAC on top-level of the Served Cell Information IE | YES | ignore |
| Aggressor gNB Set ID | O |  | gNB Set ID 9.3.1.93 | This IE indicates the associated aggressor gNB Set ID of the cell | YES | ignore |
| Victim gNB Set ID | O |  | gNB Set ID 9.3.1.93 | This IE indicates the associated Victim gNB Set ID of the cell | YES | ignore |
| IAB Info IAB-DU | O |  | 9.3.1.106 |  | YES | ignore |
| SSB Positions In Burst | O |  | 9.3.1.138 |  | YES | ignore |
| NR PRACH Configuration | O |  | 9.3.1.139 |  | YES | ignore |
| SFN Offset | O |  | 9.3.1.208 |  | YES | ignore |
| NPN Broadcast Information | O |  | 9.3.1.157 |  | YES | reject |
| Supported MBS FSA ID List |  | 0..<maxnoofMBSFSAs> |  | Shall contain all MBS Frequency Selection Area Identities associated with the NR CGI. | YES | ignore |
| >MBS Frequency Selection Area Identity | M |  | OCTET STRING(3) |  | – |  |
| RedCap Broadcast Information | O |  | BIT STRING (SIZE(8)) | The presence of this IE indicates that the intraFreqReselectionRedCap IE is broadcast in SIB1 of the corresponding cell, see TS 38.331 [8]. Each position in the bitmap indicates which RedCap UEs are allowed access, according to the setting of RedCap barring indicators in SIB1, see TS 38.331 [8]. First bit = 1Rx, second bit = 2Rx, third bit = halfDuplex, other bits reserved for future use. Value '1' indicates 'access allowed'. Value '0' indicates 'access not allowed”. | YES | ignore |
| eRedCap Broadcast Information | O |  | BIT STRING (SIZE(8)) | The presence of this IE indicates that the intraFreqReselection-eRedCap IE is broadcast in SIB1 of the corresponding cell, see TS 38.331 [8]. Each position in the bitmap indicates which eRedCap UEs are allowed access, according to the setting of the barring indicators in SIB1, see TS 38.331 [8]. First bit = 1Rx, second bit = 2Rx, third bit=half-duplex, other bits reserved for future use. Value '1' indicates 'access allowed'. Value '0' indicates 'access not allowed”. | YES | ignore |
| UL WUS Configuration | O |  | OCTET STRING | Includes the xyz message as defined in subclause a.b.c of TS 38.331. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofBPLMNs | Maximum no. of Broadcast PLMN Ids. Value is 6. |
| maxnoofExtendedBPLMNs | Maximum no. of Extended Broadcast PLMN Ids. Value is 6. |
| maxnoofBPLMNsNR | Maximum no. of PLMN Ids.broadcast in an NR cell. Value is 12. |
| maxnoofNR-UChannelIDs | Maximum no. NR-U Channel IDs in a cell. Value is 16. |
| maxnoofMBSFSAs | Maximum no. of MBS FSAs by a cell. Value is 256. |

*************** Next change ***************

#### 9.2.1.10	GNB-CU CONFIGURATION UPDATE

This message is sent by the gNB-CU to transfer updated information associated to an F1-C interface instance.

NOTE:	If F1-C signalling transport is shared among several F1-C interface instances, this message may transfer updated information associated to several F1-C interface instances.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| Cells to be Activated List |  | 0..1 |  | List of cells to be activated or modified | YES | reject |
| >Cells to be Activated List Item |  | 1.. <maxCellingNBDU> |  |  | EACH | reject |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| >>NR PCI | O |  | INTEGER (0..1007) | Physical Cell ID | - |  |
| >>gNB-CU System Information | O |  | 9.3.1.42 | RRC container with system information owned by gNB-CU | YES | reject |
| >>Available PLMN List | O |  | 9.3.1.65 |  | YES | ignore |
| >>Extended Available PLMN List | O |  | 9.3.1.76 | This is included if Available PLMN List IE is included and if more than 6 Available PLMNs is to be signalled. | YES | ignore |
| >>IAB Info IAB-donor-CU | O |  | 9.3.1.105 | IAB-related configuration sent by the IAB-donor-CU. | YES | ignore |
| >>Available SNPN ID List | O |  | 9.3.1.163 | Indicates the available SNPN ID list. If this IE is included, the content of the Available PLMN List IE and Extended Available PLMN List IE if present in the Cells to be Activated List Item IE is ignored. | YES | ignore |
| >>MBS Broadcast Neighbour Cell List | O |  | 9.3.1.226 |  | YES | ignore |
| >>SSBs within the cell to be Activated List | O |  | 9.3.1.326 | List of SSB beams within the cell requested to be activated. | YES | reject |
| Cells to be Deactivated List |  | 0..1 |  | List of cells to be deactivated | YES | reject |
| >Cells to be Deactivated List Item |  | 1.. <maxCellingNBDU> |  |  | EACH | reject |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| gNB-CU TNL Association To Add List |  | 0..1 |  |  | YES | ignore |
| >gNB-CU TNL Association To Add Item IEs |  | 1..<maxnoofTNLAssociations> |  |  | EACH | ignore |
| >>TNL Association Transport Layer Information | M |  | CP Transport Layer Information 9.3.2.4 | Transport Layer Address of the gNB-CU. | - |  |
| >>TNL Association Usage | M |  | ENUMERATED (ue, non-ue, both, ...) | Indicates whether the TNL association is only used for UE-associated signalling, or non-UE-associated signalling, or both. For usage of this IE, refer to TS 38.472 [22]. | - |  |
| gNB-CU TNL Association To Remove List |  | 0..1 |  |  | YES | ignore |
| >gNB-CU TNL Association To Remove Item IEs |  | 1..<maxnoofTNLAssociation> |  |  | EACH | ignore |
| >>TNL Association Transport Layer Address | M |  | CP Transport Layer Information 9.3.2.4 | Transport Layer Address of the gNB-CU. | - |  |
| >>TNL Association Transport Layer Address gNB-DU | O |  | CP Transport Layer Information 9.3.2.4 | Transport Layer Address of the gNB-DU. | YES | reject |
| gNB-CU TNL Association To Update List |  | 0..1 |  |  | YES | ignore |
| >gNB-CU TNL Association To Update Item IEs |  | 1..<maxnoofTNLAssociations> |  |  | EACH | ignore |
| >>TNL Association Transport Layer Address | M |  | CP Transport Layer Information 9.3.2.4 | Transport Layer Address of the gNB-CU. | - |  |
| >>TNL Association Usage | O |  | ENUMERATED (ue, non-ue, both, ...) | Indicates whether the TNL association is only used for UE-associated signalling, or non-UE-associated signalling, or both. For usage of this IE, refer to TS 38.472 [22]. | - |  |
| Cells to be barred List |  | 0..1 |  | List of cells to be barred. | YES | ignore |
| >Cells to be barred List Item |  | 1.. <maxCellingNBDU> |  |  | EACH | ignore |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| >>Cell Barred | M |  | ENUMERATED (barred, not-barred, ...) |  | - |  |
| >>IAB Barred | O |  | ENUMERATED (barred, not-barred, ...) | Corresponds to information provided in the iab-Support contained in the PLMN-IdentityInfo IE or contained in the NPN-IdentityInfo IE as defined in TS 38.331 [8]. The codepoint value “barred” indicates that the iab-Support is not sent in SIB1, and the codepoint value “not-barred” indicates that the iab-Support is sent in SIB1. | - |  |
| >>Mobile IAB Barred | O |  | ENUMERATED (barred, not-barred, ...) | Corresponds to information provided in the mobileIAB-Support contained in the PLMN-IdentityInfo IE or contained in the NPN-IdentityInfo IE as defined in TS 38.331 [8]. The codepoint value “barred” indicates that the mobileIAB-Support is not sent in SIB1, and the codepoint value “not-barred” indicates that the mobileIAB-Support is sent in SIB1. |  |  |
| Protected E-UTRA Resources List |  | 0..1 |  | List of Protected E-UTRA Resources. | YES | reject |
| >Protected E-UTRA Resources List Item |  | 1.. <maxCellineNB> |  |  | EACH | reject |
| >>Spectrum Sharing Group ID | M |  | INTEGER (1.. maxCellineNB) | Indicates the E-UTRA cells involved in resource coordination with the NR cells affiliated with the same Spectrum Sharing Group ID. | - |  |
| >>E-UTRA Cells List |  | 1 |  | List of applicable E-UTRA cells. | - |  |
| >>>E-UTRA Cells List Item |  | 1 .. <maxCellineNB> |  |  | - |  |
| >>>>EUTRA Cell ID | M |  | BIT STRING (SIZE(28)) | Indicates the E-UTRAN Cell Identifier IE contained in the ECGI as defined in subclause 9.2.14 in TS 36.423 [9]. | - |  |
| >>>>Served E-UTRA Cell Information | M |  | 9.3.1.64 |  | - |  |
| Neighbour Cell Information List |  | 0..1 |  |  | YES | ignore |
| >Neighbour Cell Information List Item |  | 1 .. <maxCellingNBDU> |  |  | EACH | ignore |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| >>Intended TDD DL-UL Configuration | O |  | 9.3.1.89 |  | - |  |
| Transport Layer Address Info | O |  | 9.3.2.5 |  | YES | ignore |
| Uplink BH Non-UP Traffic Mapping | O |  | 9.3.1.103 |  | YES | reject |
| BAP Address | O |  | 9.3.1.111 | Indicates a BAP address assigned to the IAB-donor-DU. | YES | ignore |
| CCO Assistance Information | O |  | 9.3.1.211 | Indicates CCO Assistance Information for cells and beams served by the gNB-DU of the same NG-RAN node or for cells and beams not served by the gNB-DU. | YES | ignore |
| Cells for SON List | O |  | 9.3.1.214 |  | YES | ignore |
| gNB-CU Name | O |  | PrintableString(SIZE(1..150,...)) | Human readable name of the gNB-CU. | YES | ignore |
| Extended gNB-CU Name | O |  | 9.3.1.206 |  | YES | ignore |
| Cells Allowed to be Deactivated List |  | 0..1 |  |  | YES | ignore |
| >Cells Allowed to be Deactivated List Item |  | 1 .. <maxCellingNBDU> |  |  | EACH | ignore |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| Cells Allowed to Use OD-SIB1 Transmission List |  | 0..1 |  |  | YES | ignore |
| > Cells Allowed to Use OD-SIB1 Transmission List Item |  | 1 .. <maxCellingNBDU> |  |  | EACH | ignore |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum numbers of cells that can be served by a gNB-DU. Value is 512. |
| maxnoofTNLAssociations | Maximum numbers of TNL Associations between the gNB-CU and the gNB-DU. Value is 32. |
| maxCellineNB | Maximum no. cells that can be served by an eNB. Value is 256. |
| maxnoofSSBAreas | Maximum no. SSB Areas that can be served by a cell. Value is 64. |

*************** End of TP ***************

# 6	References

[1]	RP-234065	New WID: Enhancements of network energy savings for NR

[2]	RP-242354	Revised WID: Enhancements of network energy savings for NR

[3] 	R3-244708	Summary of offline disc, Ericsson (Moderator)

[4]	R3-245498	Xn impact of On-demand SIB1 for UEs in idle/inactive mode, Ericsson