3GPP TSG-RAN WG3 Meeting #126	R3-247562

Orlando, USA,  18 – 22 November, 2024

Agenda item:	9.2

Source:	Nokia, ZTE, Samsung

Title:	NR-DC QMC Coordination for RRC Segmentation

Document for:	Discussion and Decision

# 1	Introduction

We here provide further analysis on the topic of NR-DC QMC Coordination for RRC Segmentation.

# 2	Discussion

As per TS 38.331 clause 5.3.5.13d, the network performs QMC configuration or reconfiguration by sending the RRC appLayerMeasConfig IE within RRCReconfiguration or RRCResume messages. The starting point for the present discussion that has been running over several RAN3 meetings is that each time the gNB performs such QoE configuration or reconfiguration, the sending node (MN or SN) needs to provide the following information to the UE:

- segmentation allowed information for SRB4; and

- segmentation allowed information for SRB5.

As per current specification, the MN only knows segmentation allowed information for SRB4, and the SN only knows segmentation allowed information for SRB5. The missing information therefore needs to be transferred to the QoE configuring node (MN or SN), which will send it to the UE together with other information for QMC configuration or reconfiguration within the RRC AppLayerMeasConfig IE.

It was common understanding at RAN3#125 that at the network side in case of NR-DC, the node sending the RRC AppLayerMeasConfig IE to the UE will use the QMC Coordination Request/Response mechanism over XnAP to obtain segmentation allowed information from the other node. The corresponding information exchange is illustrated in Figure 1.

Figure 1: Scenarios for QMC configuration and reconfiguration.

As observed in [1], if the segmentation is allowed in SRB4/SRB5, all QoE measurement (subject to all QoE configurations configured to the UE) reported over SRB4/5 will enable RRC segmentation. [1] also observes that the concept of QoE configuring node and non QoE configuring node is subject to a specific QoE configuration. Namely, for different QoE configurations configured to a UE, the QoE configuring node may be different. It is therefore not possible for the network to modify RRC segmentation allowed information for a specific QoE session without also applying the same change for the other QoE sessions that use the same leg (SRB4, SRB5) for QoE reports.

The scenarios for sending AppLayerMeasConfig to the UE are the following:

- Release of a QoE configuration

- Setup of a new QoE configuration

- Modification of an existing QoE configuration:

  - Setup of RAN visible QoE reporting

  - Release of RAN visible QoE reporting

  - Pause reporting

  - Resume a previously paused reporting

We here look at the following basic scenario for NR-DC QMC:

- The MN sets up QoE session A, with QoE reports sent to the MN (SRB4). At this stage there is no QoE session running in the SN, so AppLayerMeasConfig can be sent to the UE while

- including RRC allowed information for SRB4; and

- including or not including the rrc-SegAllowedSRB5 IE without any incidence due to absence of reporting over SRB5.

- The SN sets up QoE session B, with QoE reports sent to the SN (SRB5). For this, the SN requests the Measurement Configuration Application Layer ID from the MN, which, as per CRs under discussion, will also include SRB4 segmentation allowed information in the response to the SN. The SN can therefore set up QoE session B in the UE while also including segmentation allowed information for both SRB4 and SRB5.

- The MN releases QoE session A. As per CRs under discussion, the MN is not able to retrieve segmentation allowed information for SRB5.

Step 2 shows the necessity of including segmentation allowed information for own SRB in QMC Coordination Response as discussed in multiple CRs.

Proposal 1: Include segmentation allowed information for own SRB in QMC Coordination Response.

In order to solve the problem in step 3 above, a possible solution could be that the SN includes segmentation allowed information for SRB5 in the QMC Coordination Request sent in step 2.

Proposal 2: Include segmentation allowed information for own SRB in QMC Coordination Request.

# 3	Conclusion

We have made the following proposals:

Proposal 1: Include segmentation allowed information for own SRB in QMC Coordination Response.

Proposal 2: Include segmentation allowed information for own SRB in QMC Coordination Request.

# References

[1]		R3-245123, Discussion on QMC support in NR-DC for RRC segmentation, Samsung, ZTE, CATT, China Unicom, Lenovo