3GPP TSG-RAN WG3 Meeting #126	R3-247563

Orlando, USA,  18 – 22 November, 2024

Agenda Item:	11.3

Source:	Nokia, Jio, Telecom Italia

Title:	A contribution to energy efficient AI/ML-based CCO issue prediction in split architecture

Document for:	Discussion and Decision

# 1	Introduction

3GPP RAN WGs have worked on mechanisms to reduce the energy consumption in the RAN and in UEs since Rel-10 at least. The effort has essentially focused on energy consumption linked to radio transmission and reception, which corresponds to the major part of RAN energy consumption. The radio will continue to be the main energy consumer also after introduction of AI/ML in the NG-RAN, however we believe that energy efficiency also in other entities than the gNB-DU should be taken into account whenever the standard can make a difference. We therefore in this paper address the topic of energy efficiency in the gNB-CU for AI/ML-based CCO.

# 2	Discussion

Standards support of AI/ML-based NG-RAN use cases are introduced in Rel-18 and Rel-19 in order to enable AI/ML model operation in NG-RAN nodes. This will represent clear enhancements for different aspects of the network operation, including in terms of energy saving. Still, AI/ML-based CCO requires, in order to be operational, an AI/ML model generating predicted CCO issue which is trained and tested for the cell (including cell neighbourhood) to which the CCO issue prediction applies. The inference phase may require real-time measurements from own cell and will possibly require prediction information from other AI/ML models, when such can be provided. TR 38.743 indicates input information to include Measured/Predicted radio resource status from local node and neighbour node. However, it is reasonable to assume that the CCO inference phase additionally requires e.g. predicted UE trajectory information for served UEs.

Of course, the 3GPP standard is AI/ML model agnostic and main responsibility for energy efficient AI/ML algorithms lies on implementions that will make choices relative to Life Cycle Management (e.g. training in OAM vs. training in the gNB) and e.g. dimensioning of the AI/ML algorithms so that energy consumption for AI/ML model trainingand inference is kept at the lowest possible level.

However, in the choice of CCO signalling procedures for split architecture we notice that there is an opportunity also for the standard to contribute to an energy efficient solution for AI/ML-based CCO issue prediction. The standardized legacy CCO signalling framework for split architecture doesn’t provide any mechanism to select the cells for which CCO issue detection is done, and this can be justified by the fact that the gNB-CU will anyway monitor the overall cell performance, cases of failure (e.g. radio link failure due to coverage holes) and cases of reduced UE performance (e.g. cell edge capacity) for the purpose of performance monitoring and KPI reporting to the OAM. Therefore, this is done not necessarily for the purpose of CCO alone.

Observation 1: Monitoring of real-time coverage and capacity issues is done not only for the purpose of legacy CCO but also for performance monitoring and KPI reporting to the OAM.

On the other side, Rel-19 CCO issue prediction by the gNB-CU will not have any other purpose than enabling the gNB-DU to apply a corrective action that prevents the predicted issue to happen. Such prediction is therefore only meaningful if the gNB-DU can apply alternative coverage configurations that can mitigate or solve detected issues.

Observation 2: Prediction of coverage and capacity issues by the gNB-CU is done only for the purpose of CCO.

The gNB-DU should therefore be able to indicate to the gNB-CU the cells for which the gNB-DU would be able to change the coverage state in case a CCO issue is predicted. This could depend on antenna-related capabilities (e.g. whether an Active Antenna System is used for the cell), which would be relatively static information, but could also depend on available coverage configurations from the OAM or on the immediate capability or capacity of the gNB-DU to calculate and enforce a new coverage state for given cells. Making cell information available in the gNB-CU would help to avoid wasted computing efforts and corresponding wasted energy consumption for CCO prediction for cells for which no CCO action can be taken.

Proposal 1: The gNB-CU receives information about the cells for which the gNB-DU is able to change the coverage state, so that it can limit CCO issue prediction to those cells.

A further step in terms of optimization would be that the gNB-DU informs the gNB-CU about its possibilities in terms of coverage change modification. E.g. an extensive choice of alternative coverage configurations may be available for some cell, and a more limited choice of alternative coverage configurations may be available for other cells. The gNB-DU may also have configured information about whether these available coverage configurations are suitable for solving coverage issues or cell edge capacity issues. By receiving such information, the gNB-CU may optimize data collection, model training and model inference in order to ensure that energy and computing capacity is consumed in accordance with the corrective actions available in the gNB-DU.

Proposal 2: The gNB-CU receives information about available alternative coverage configurations available in the gNB-DU, and the type of CCO issue (coverage, cell edge capacity) that can be solved by the gNB-DU.

In terms of signalling procedures, one solution to implement Proposals 1 and 2 would be to introduce Data Collection Reporting Initiation and Data Collection Reporting signalling procedures in F1AP, similarly to what was done in XnAP in Rel-18, as illustrated in Figure 1.

Figure 1: Example of subscription-based mechanism for CCO issue prediction.

However, while the data collection procedures in Figure 1 would be a solution well aligned with signalling for most prediction information standardized in Rel-18, the coexistence between Rel-19 AI/ML based CCO and Rel-17 CCO might turn out to motivate use of different signalling mechanisms. These could be e.g.:

- the gNB indicates cells that can be subject to CCO state changes in Served Cell Information IE (sent in F1 Setup, gNB-DU Configuration Update signalling procedures)

- the gNB-CU indicates the predicted CCO issue information together with the legacy CCO information in the gNB-CU Configuration Update message – this wouldn’t necessarily preclude use of Data Collection Request Initiation procedure to be used to convey the information about eligible cells for CCO issue prediction.

In this context it can be reminded that RAN3 at previous meeting took the following working assumption:

WA: Reusing the CU configuration update procedure to transfer the predicted CCO issue and reusing the DU configuration update procedure to transfer the Future Coverage State over F1.

However, the choice of F1 signalling procedures should be taken based on the energy efficiency aspect discussed in the present paper, as well as the aspect of coexistence with legacy CCO as we discuss in our paper submitted to this meeting in [1]. Confirmation or update of the working assumption should therefore be done after these open points have been concluded.

Proposal 3: Confirmation or update of the working assumption from RAN3#125bis to be done after conclusion reached on energy efficiency and coexistence with legacy CCO.

# 3	Conclusion

We have made the following observations and proposals:

Observation 1: Monitoring of real-time coverage and capacity issues is done not only for the purpose of CCO but also for performance monitoring and KPI reporting to the OAM.

Observation 2: Prediction of coverage and capacity issues by the gNB-CU is done only for the purpose of CCO.

Proposal 1: The gNB-CU receives information about the cells for which the gNB-DU is able to change the coverage state, so that it can limit CCO issue prediction to those cells.

Proposal 2: The gNB-CU receives information about available alternative coverage configurations available in the gNB-DU, and the type of CCO issue (coverage, cell edge capacity) that can be solved by the gNB-DU.

Proposal 3: Confirmation or update of the working assumption from RAN3#125bis to be done after conclusion reached on energy efficiency and coexistence with legacy CCO.

# References

[1]		R3-247564, Discussion on signalling for AI/ML-based CCO, Nokia, Jio