3GPP TSG-RAN WG3 Meeting #126	R3-247565

Orlando, USA,  18 – 22 November, 2024

Agenda item:	17.3

Source:	Nokia

Title:	Xn signalling for On-Demand SIB1

Document for:	Discussion and Decision

# 1	Introduction

We here analyse agreements and open points for Xn signalling to enable network support for on-demand SIB1 from last meeting, and describe a solution for network signalling that may combine the advantages of the different options that were identified.

# 2	Discussion

RAN3#125bis made the following agreements for on-demand SIB1:

The UL WUS configuration will be transferred over Xn via a new class1 defined procedure.

Cell A gNB can decide and signal to NES Cell gNB that it stops the UL WUS configuration broadcast in its SIB.

However, due to lack of time the scenario where the NES cell decides that it reverts to regular SIB1 broadcast, and hence requests cell As to stop broadcasting, was not discussed. Such functionality would help save energy by not broadcasting UL WUS configuration when not needed, and it would also help freeing up broadcast resources in the cell A so that these resources could become available in case another NES cells We therefore propose RAN3 to take the following agreement:

Proposal 1: NES cell gNB can decide and signal to Cell A gNB that the Cell A gNB stops the UL WUS configuration broadcast in its SIB.

Three options were captured at last meeting in the summary of discussion [1]:

- Option 1: After the UL WUS Configuration Provision request and Positive Response (confirm), NES Cell gNB goes to OD-SIB1 Mode. Cell A gNB broadcasts UL WUS configuration via its SIB.

- Option 2: After the UL WUS Configuration Provision request and positive response (confirm), Cell A gNB broadcast received UL WUS configuration via its SIB. NES cell goes to OD-SIB Mode upon reception of the explicit “UL config provided” message from Cell A gNB.

- Option 3: After the UL WUS Configuration Provision request and positive response (confirm), no change in NES Cell or Cell A gNBs.

With option 1, it is assumed that cell A starts the broadcast of UL WUS Configuration once it receives the UL WUS configuration from the NES cell, while option 2 assumes there is some delay before the broadcast becomes effective. Also option 3 assumes that broadcast should not start immediately, in order to take into account the scenario where a NES cell will require UL WUS Configuration broadcast by multiple cell As before it effectively takes the decision to enter OD-SIB1 mode.

The functionality offered by all of these three options, as well as the NES cell decision to stop broadcast reflected by proposal 1, can be supported using the signalling procedures illustrated in Figure 1. These procedures enable the NES cell gNB to request the Cell A gNB to broadcast UL WUS configuration broadcast and to provide event-triggered updates of the broadcast status. The mechanism is similar to the subscription-based mechanism used by resource status reporting for SON and data collection procedures for AI/ML.

This functionality can be obtained based on two signalling procedures:

- UL WUS Broadcast Initiation procedure – which is a class 1 procedure including UL WUS BROADCAST REQUEST and UL WUS BROADCAST RESPONSE messages.

- UL WUS Broadcast Reporting procedure – which is a class 2 procedure based on the UL WUS BROADCAST UPDATE message.

Figure 1: Xn signalling for coordination of UL WUS configuration broadcast between NES Cell gNB and Cell A gNB.

Based on the above discussion and example signalling flow, we make the following proposals:

Proposal 2: Introduce a subscription-based mechanism to enable coordination between a NES cell gNB and a Cell A gNB of UL WUS Broadcast.

Proposal 3: Introduce an UL WUS Broadcast Initiation procedure – which is a class 1 procedure including UL WUS BROADCAST REQUEST and UL WUS BROADCAST RESPONSE messages.

Proposal 4: Introduce an UL WUS Broadcast Reporting procedure – which is a class 2 procedure based on the UL WUS BROADCAST UPDATE message.

# 3	Conclusion

We have made the following proposals:

Proposal 1: NES cell gNB can decide and signal to Cell A gNB that the Cell A gNB stops the UL WUS configuration broadcast in its SIB.

Proposal 2: Introduce a subscription-based mechanism to enable coordination between a NES cell gNB and a Cell A gNB of UL WUS Broadcast.

Proposal 3: Introduce an UL WUS Broadcast Initiation procedure – which is a class 1 procedure including UL WUS BROADCAST REQUEST and UL WUS BROADCAST RESPONSE messages.

Proposal 4: Introduce an UL WUS Broadcast Reporting procedure – which is a class 2 procedure based on the UL WUS BROADCAST UPDATE message.

# References

[1]		R3-245737, Summary of offline discussions: Rel-19 Network Energy Saving, Ericsson (moderator)