3GPP TSG-RAN WG3 Meeting #126	R3-247567

Orlando, USA, 18 – 22 November, 2024

Agenda item:	19.2

Source:	Nokia

Title:	Further discussion on inter-gNB CLI mitigation for SBFD operation

Document for:	Discussion and Decision

# 1	Introduction

We provided at previous meeting an initial discussion on inter-gNB CLI mitigation for SBFD operation in [1], during which the following agreements were taken:

- Semi-static cell-specific SBFD time and frequency location configuration with cell granularity (FFS on container or explicit IEs over XnAP and F1AP)

- Measurement resource configuration

- Strongest DL beam information (FFS on the details)

- CLI-mitigation request (FFS on the details)

We here provide further discussion on RAN3’s part of this work.

# 2	Discussion

Based on discussion at previous meeting [1], we suggest the following signalling flow to be the starting point for more detailed analysis of inter-gNB information exchange for beam nulling.

Figure 1: Flow chart of the inter-gNB signalling used in beam nulling. Both gNB1 and gNB2 have activated SBFD. In this example a cell controlled by the gNB 1 is victim of gNB-to-gNB CLI originated in a cell (aggressor cell) controlled by gNB 2.

Step 1: Exchange of SBFD time and frequency configuration information. This step is covered by the following agreement and open point:

- Semi-static cell-specific SBFD time and frequency location configuration with cell granularity (FFS on container or explicit IEs over XnAP and F1AP)

It seems natural to include this information in the Served Cell Information NR IE, exchanged in the Xn Setup and gNB Configuration Update procedures.

Proposal 1: Include semi-static cell-specific SBFD time and frequency location configuration in the Served Cell Information NR IE.

The content of the Served Cell Information NR IE currently includes the tdd-UL-DL-ConfigurationCommon IE delivered to the UEs via the SIB1 message. Based on RAN1 agreements, it is expected that RRC_IDLE and RRC_CONNECTED UEs are aware of the cell SBFD time and frequency configuration and therefore tdd- UL-DL-ConfigurationCommon should include all the needed information. It is now up to RAN2 to decide on the IE design for the SBFD time and frequency location configuration. This work is already going on in RAN2, and therefore no action in RAN3 is required at this stage.

Step 2: gNB 1 detects CLI based on UL performance degradation. No specific information exchange is needed to support this step.

Proposal 2: gNB1 uses locally available information to detect CLI based on UL performance degradation.

Step 3: Signalling of the CLI mitigation request. Upon the determination of presence of gNB-to-gNB CLI, gNB 1 sends its CLI-RS configuration to gNB 2, which corresponds to the CLI-RS transmitted by the victim cell in gNB 1 at step 4. In other words, we think that the CLI Mitigation Request content should include the measurement resource configuration that should be used by the aggressor gNB to determine/estimate the victim-to-aggressor channel matrix.

This step is covered by the following agreements and open points:

- CLI-mitigation request (FFS on the details)

- Measurement resource configuration

Proposal 3: The CLI Mitigation Request for beam nulling includes the CLI-RS (measurement resource) configuration corresponding to the transmitted CLI-RS.

Step 4: gNB 1 transmits the periodic CLI-RS configured in the prior step.

Step 5: gNB 2 estimates the victim-to-aggressor channel by measuring the CLI-RS of the victim gNB

In this step the gNB 2 uses the CLI-RS measurement configuration provided by gNB1 in step 3.

Step 6: gNB 2 may adjust the DL precoder if DL transmission is scheduled on the identified aggressor beam.

The aggressor beam is identified by gNB2’s own measurement, hence no additional information exchange is required to support this step.

***

For beam pairing, we propose the following signalling flow as starting point for RAN3 discussion (Figure 2).

Figure 2: Flow chart of the inter-gNB signalling used in beam pairing. Example of inter-gNB signalling in support of beam pairing. Both gNB1 and gNB2 have activated SBFD. In this example a cell controlled by the gNB 1 is victim of gNB-to-gNB CLI originating in a cell (aggressor cell) controlled by gNB 2.

Step 1: Exchange of: SBFD time and frequency configuration information; SSB information for CLI measurements. Same as step 1 for beam nulling, exchange of SBFD time and frequency configuration information is covered by the following agreement and open point and proposal 1.

- Semi-static cell-specific SBFD time and frequency location configuration with cell granularity (FFS on container or explicit IEs over XnAP and F1AP)

Exchange of SSB information for CLI measurements is supported by legacy XnAP.

Proposal 4: The gNB1 will use SSB configuration information already included in Served Cell Information NR IE for CLI measurements based on SSB.

Step 2: gNB 1 detects UL performance degradation, and performs SSB-RSRP measurements to identify one or more aggressor cells / SSB beams.

Step 3: Signalling of the CLI mitigation request: gNB 1 indicates the cell ID and SSB indices with highest CLI to gNB 2 (potential aggressor gNB). We believe that the CLI Mitigation Request can be used for this purpose, however the conveyed information will be different for beam nulling and beam pairing.

Proposal 5: The CLI Mitigation Request is structured so that it can convey different information for beam nulling and beam pairing.

Proposal 6: The CLI Mitigation Request for beam pairing can convey one or more SSB indices.

Step 4: gNB 2 sends CSI-RS configuration to gNB1 in response to the CLI Mitigation Request in step 3.

A new procedure might be needed for this step, however also Resource Status Reporting could be considered.

Proposal 7: RAN3 to discuss the message or signalling procedure (new?) to convey the CSI-RS configuration information from aggressor node to victim node for beam pairing.

Step 5: gNB 2 starts transmitting periodic CSI-RS, corresponding to the CSI-RS configuration sent in step 4.

Step 6: gNB1 performs per beam CLI measurements according to the aggressor CSI-RS

Step 7: CLI mitigation request: gNB2 indicates the one or more CRIs with highest measured gNB-to-gNB CLI to the gNB1

Proposal 8: The CLI Mitigation Request for beam pairing can convey one or more CSI Resource Indicators (CRI).

Step 8: gNB2 may avoid DL scheduling in the identified CSI-RS beam during all SBFD slots

# 3	Conclusion

We have made the following proposals:

Proposal 1: Include semi-static cell-specific SBFD time and frequency location configuration in the Served Cell Information NR IE.

Proposal 2: gNB1 uses locally available information to detect CLI based on UL performance degradation.

Proposal 3: The CLI Mitigation Request for beam nulling includes the CLI-RS (measurement resource) configuration corresponding to the transmitted CLI-RS.

Proposal 4: The gNB1 will use SSB configuration information already included in Served Cell Information NR IE for CLI measurements based on SSB.

Proposal 5: The CLI Mitigation Request is structured so that it can convey different information for beam nulling and beam pairing.

Proposal 6: The CLI Mitigation Request for beam pairing can convey one or more SSB indices.

Proposal 7: RAN3 to discuss the message or signalling procedure (new?) to convey the CSI-RS configuration information from aggressor node to victim node for beam pairing.

Proposal 8: The CLI Mitigation Request for beam pairing can convey one or more CSI Resource Indicators (CRI).

# References

[1]		R3-245562, Initial discussion on RAN3 work on Evolution of NR duplex operation: Sub-band full duplex (SBFD), Nokia