3GPP TSG-RAN WG3 Meeting #126	R3-247587

Orlando, Florida, U.S., 18th – 22nd October 2024

Agenda Item:	16.3

Source:	Ericsson

Title:	[TP for TR 38.769] Elements for AIoT RAN – AIoT CN communication and overall signalling

Document for:	Discussions & Approval

# 1	Introduction

The attempt to further investigate details of parameterizing protocol functions discussed in the AIoT architecture part of this study is expected to provide further and more insight in necessary discussions, also with other TSGs/WGs.

This document attempts to introduces common signalling parameters used in in both, AIoT inventory and command AIoT communication, in order to have a common basis to refer to in the message flow description.

Further this document follows partly functional discussions in R3-24xxxx in following not all approach so far discussed in RAN3 (and probably other groups).

Finally, the message flows are provided.

# 2	Discussion

#### 2.1 Information enabling Reader selction in RAN

Reader selection, as discussed in R3-247585, should be performed in a way that AIoT RAN performs the final reader selection while AIoT CN provides respective information.

We suggest introducing an AIoT Area to allow the AIoT CN to indicate a geographical area by means of an identifier, which maps by means of configuration to either a set of readers or an actual geographical information (or both). Respective mapping information is configured both in RAN and CN, with a reader being associated to one or several AIoT area IDs.

NOTE: 	an example for the definition of AIoT areas might be a ware house, where all readers a certain floor (irrespective their topology) are associated with an ID for “floor x”, and on that floor the readers are further associated with an ID for “room y” etc.

Further we suggest introducing an AIoT Transaction Area (in a similar way as an “service area”) to indicate to AIoT RAN in which area AIoT radio resources have to be allocated and coordinated. This AIoT Transaction Area could be composed of either a list of AIoT Area IDs, a list of Reader IDs, in case of topology 2 a list of NR cells or Tracking Areas, for topology 1 it is also possible to indicate virtual/mapped cell/TAs

Further, in order to allow the AIoT CN to select proper AIoT RAN nodes, we suggest introducing in the respective NG/XX interface management procedures (e.g. NG Setup) respective information, which should be either a list of (virtual or “real”) Tracking Areas or a list of AIoT Areas. (maybe this needs more options, FFS)

#### 2.2 Information enabling concurrent and repeated AIoT transactions

We have been discussing whether an AIoT Transaction ID would be need on the RAN/CN interace for AIoT.

We suggest considering supporting the general case, where an AIoT RAN node may receive AIoT requests from several AIoTFs / AFs, and also scenarios where individual concurrent AIoT transactions may be destined for the same AIoT RAN node but e.g. for rather limited (and non-overlapping) AIoT Transaction Areas.

There is also the case where the AIoT CN/AF does not deem the response from AIoT RAN being sufficient and re-issues the AIoT transaction, but does not want the devices that have already responded to respond again. In this case, it is necessary that AIoT RAN gets respective indication (which the AIoT RAN wouldn’t need to recognize as a repetition).

In which way the AIoT RAN would translate this information on actual AIoT radio signaling is out of RAN3 scope.

#### 2.3 Protocol elements for support of security related signalling

How to consider security related protocol elements depends on the authentication/registration schemes developed by SA3. Whether such security related communication can be integrated into currently developed inventory/command signalling schemes cannot be answered to this date.

We need to put this aspect on hold until SA3 finished studies on that aspect.

#### 2.4 Protocol elements for support of command are still missing

While looking at commonalities and the parameterisation of varios XX procedures, it has been noted that respective description would deserve an overhaul, which was tried to be performed by introducing a general section on AIoT specific information elements and refer to them in an expanded section of respective message descriptions.

Further, it has been noted, that Command may also be AIoT device originated and would require respective signalling in the opposite direction (plus receipt confirmation from the AIoT CN).

#### 2.5 Proposals

Proposal 1:	Agree to introduce an AIoT Area, which is an identifier mapping to either a set of readers or an actual geographical information. The mapping is configured in RAN and CN.

Proposal 2;	Agree to indicate an AIoT Transaction Area in each AIoT CN triggered transaction, which either includes AIoT Area IDs or Reader IDs or Tracking Areas or Cell IDs, whereas Tracking Areas and Cell IDs may be virtual/mapped ones.

Proposal 3:	Agree that the AIoTransaction Area is used by the AIoT RAN to finally select the readers necessary to perform the transaction.

Proposal 4:	Agree that the AIoT RAN node is enable to inform the AIoT CN at NG/XX Setup in a way that the AIoT CN can properly select the AIoT RAN nodes at AIoT Transaction intitiation. This information may be based on AIoT Areas or Tracking Areas.

Proposal 5:	Agree to indicate from AIoT CN a uniquely identifiable AIoT Transaction ID that is able to handle concurrently operating AIoT transactions at AIoT RAN and allow the AIoT CN to repeat AIoT transactions w/o AIoT devices to repeat their responses.

Proposal 6:	Add an Editor’s Note that further studies on support of security related protocol functions have to await results from SA3 discussions and are currently FFS.

Proposal 7:	Restructure the section on “Information exchanged between AIoT-CN and AIoT RAN”.

Proposal 8:	Introduce example message flows and protocol elements for Command, also in case of AIoT Device initiated Command.

Final Proposal:	Agree on the text proposal for TR 38.769 below:

# 3	Text Proposal for TR 38.769 v1.1.0 based on R3-247036/R3-245849

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

## 6.5	Impacts on CN-RAN interface

Editor’s Note: Corresponds to the first RAN3 objective in the SID, to identify necessary impacts on signaling and procedures for CN-RAN interface.

### 6.5.1	Information exchanged between A-IoT CN and A-IoT RAN

#### 6.5.1.x	XX Information common to AIoT transactions for all topologies and options

(x.1)	A-IoT Device Identification

-	Dependent on the use case, it may concern a single device or multiple devices.

NOTE 1:	The definition of this identification is out of RAN3 scope.

(x.2)	Reader ID

-	Allows AIoT CN and AIoT RAN to e.g. communicate latest know location of an AIoT device on reader-level granularity or to indicate in which area a certain AIoT transaction shall be performed.

(x.2)	AIoT Area ID

-	Allows the 5GS to associate a reader with one or several AIoT specific areas. The association between an AIoT Area ID and either one or several readers or a geographical area is assumed to be configured in RAN and CN.

(x.3)	AIoT Transaction Area

-	Allows the AIoT CN to select proper AIoT RAN nodes and AIoT RAN to select the readers necessary to perform the AIoT transaction.

-	contains basically geographical (service area) information: either one or several AIoT Area IDs, or one or several reader IDs, Tracking Area(s) (virtual/mapped or actual in case of topology 2), or NR Cell ID(s) (in case of topology 2).

(x.4)	AIoT Transaction ID

-	Allows AIoT RAN and AIoT CN to address different, potentially concurring AIoT transactions stemming from different AIoTFs or different applications.

-	If the AIoT transaction is initiated by the AIoT CN, the AIoT Transaction ID is allocated by the AIoT CN and should be uniquely identifiable by AIoT RAN for a certain amount of time, at least among the readers involved in AIoT transactions initated by the AIoT CN.

-	If the AIoT transaction is initiated by the AIoT RAN, the AIoT Transaction ID is allocated by the AIoT RAN and should be uniquely identifiable for a certain amount of time at least among the readers involved in AIoT transactions initated by readers served by the AIoT RAN.

(x.5)	AIoT upper layer PDU

-	Transparent Container, passed unaltered from the AIoT CN to the AIoT device.

#### 6.5.1.1	Inventory

Inventory can be sent by the A-IoT CN for a single device, or a group of devices, or all devices.

The Inventory Request from the A-IoT CN to the A-IoT RAN, includes the following:

(1)	AIoT Transaction ID (AIoT CN allocated)

(2)	A-IoT Device Identification(s)

-	contains the ID(s) of one or multiple AIoT devices, or one or multiple group IDs or AIoT devices.

Note 1:	The definition of this identification is out of RAN3 scope.

Editor’s Note 1: It is FFS whether A-IoT RAN needs to interpret/store/process it.

(3)	AIoT Transaction Area (area in which the inventory is to be performed)

Editor’s Note 2: It is up to SA2 whether device ID is sent transparent or not.

The Inventory Response from the A-IoT RAN to the A-IoT CN, includes the following:

(1)	AIoT Transaction ID (AIoT CN allocated)

The Inventory Report from the A-IoT RAN to the A-IoT CN, includes the following:

(1)	AIoT Transaction ID (AIoT CN allocated)

(2)	A-IoT Device Identification(s)

(3)	Reader ID(s) involved in the Inventory transaction of the AIoT device identified in (2)

#### 6.5.1.2	Command

##### 6.5.1.2.1	General

Inventory and Command may be performed on XX as follows:

-	Command related AIoT upper layer PDUs contained in subsequent AIoT CN initiated XX communication (Inventory and Command):

-	Command related AIoT upper layer PDU contained in Inventory Request (Command only)

Editor’s Note: Feasibility of “Command-only” is subject to further discussions in other WGs (SA3, RAN2, etc.).

##### 6.5.1.2.2	Inventory and Command

Command in conjunction with Inventory may be AIoT CN or AIoT Device initiated

-	AIoT CN initiated:

Command Request from AIoT CN to AIoT RAN:

(1)	AIoT Transaction ID (AIoT CN allocated)

(2)	A-IoT Device Identification(s)

(3)	AIoT upper layer PDU(s)

Command Response from AIoT RAN to AIoT CN:

(1)	AIoT Transaction ID (AIoT CN allocated)

(2)	A-IoT Device Identification(s)

(3)	AIoT upper layer PDU(s) or Reception Confirmation(s)

-	AIoT device initiated:

Command Required from AIoT RAN to AIoT CN:

(1)	AIoT Transaction ID (AIoT RAN allocated)

(2)	A-IoT Device Identification

(3)	AIoT upper layer PDU

Command Confirm from AIoT CN to AIoT RAN:

(1)	AIoT Transaction ID (AIoT RAN allocated)

(2)	A-IoT Device Identification

(3)	AIoT upper layer PDU or Reception Confirmation

##### 6.5.1.2.3	Command-only

Command-only may be performed on XX as follows:

-	AIoT CN initiated: Command Request and Command Respond as described for Inventory and Command with

Command Response from AIoT RAN to AIoT CN:

(4)	Reader ID(s) involved in the Command transaction of the AIoT device identified in (2)

-	AIoT device initiated: Command Required and Command Confirm as described for Inventory and Command with

Command Confirm from AIoT CN to AIoT RAN:

(4)	Reader ID (s) involved in AIoT device-initiated Command-only transaction

Editor’s Note 1: it is FFS for command on a group of devices, or all devices.

Editor’s Note 2: it is FFS whether A-IoT RAN can remain agnostic of the type of request from the A-IoT CN (need to differentiate command and inventory).

#### 6.5.1.x	XX protocol elements for AIoT session resource control in case of NAS/UP based solutions

NAS/UP based solutions require the possibility for requesting AIoT session resources in advance to the NAS/UP based communication of AIoT CN with the AIoT enabled UE.

XX AIoT session resource control for NAS/UP based solutions does not necessarily carry AIoT device specific information, but only sufficient information in order to configure the AIoT enabled UEs with respective AIoT session resources.

The AIoT Session Resource Request and the AIoT Session Resource Response messages shall carry the same AIoT Transaction ID (CN allocated) as communicated by the AIoT CN directly to the AIoT enabled UE(s).

Editor’s Note 1: Further details are FFS.

#### 6.5.1.y	Termination of AIoT transactions

Any kind of AIoT transaction needs to be terminated on AIoT RAN and AIoT CN side. This may be done either explicitly or implicitly:

-	Both, AIoT RAN and AIoT CN are assumed to terminate the transaction based on a configured timer.

-	Both, AIoT RAN and AIoT CN are allowed to request or command the termination of the transaction by means of explicit XXAP procedures.

-	In case an AIoT transaction consists of several XXAP procedures (e.g. multiple Inventory Report messages) the respective node provides in the last message an “end of transaction” indication.

Editor’s Note 1: Further details are FFS.

#### 6.5.1.z	Protocol elements related security functions on XX

Editor’s Note: How to consider security related protocol elements depends on the authentication/registration schemes developed by SA3. Whether such security related communication can be integrated into currently developed inventory/command signalling schemes is FFS.

#### 6.5.1.a	Protocol elements related XX/NG interface management functions

AIoT CN – AIoT RAN interface management functions concerning the setup and configuration update of an XX/NG interface instance may be built upon existing NGAP protocol functions.

The setup of an NG interface instance requires the AIoT RAN node to provide at least a Global RAN Node ID, a Supported TA List and a Default Paging DRX.

IEs not applicable for e.g. AIoT RAN nodes only supporting topology 1 or connecting directly to the AIoTF (to be further discussed) would need to be either ignored. In case of Supprted TAI List, a virtual/mapped TAI could be configured.

In order to allow the AIoT CN to select proper AIoT RAN nodes, a list of configured AIoT Areas or virtual/mapped TAIs could be provided to the AIoT CN, which could be further used to indicate the AIoT transaction area.

Editor’s Note: Further approaches are FFS.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 6.5.2.2	Candidate procedures for A-IoT Command in Topology 1

###### 6.5.2.2.x1	AIoT CN initiated command – Inventory and Command

Figure 6.5.2.2.x1-1: Message flow for A-IoT CN initated Command in Topology 1

1.	The Inventory procedure is performed as described in section 6.5.2.1.

2a.	The A-IoT CN node sends a Command Request message to the A-IoT RAN.

2b.	The A-IoT RAN node allocates and coordinates the usage of A-IoT radio resources.

3.	The A-IoT RAN node performs the command procedure towards the AIoT device over the A-IoT radio interface.

4.	After receiving the command result from the AIoT device (if any) the AIoT node sends a Command Response message to the A-IoT CN.

NOTE 4: In step 4, the A-IoT RAN node may instead send a Command Failure message to the A-IoT CN indicating that the A-IoT command procedure is failed towards the A-IoT device.

###### 6.5.2.2.x2	AIoT device initiated command – Inventory and Command

Figure 6.5.2.2.x2-1: Message flow for A-IoT device initated Command in Topology 1

1.	The Inventory procedure is performed as described in section 6.5.2.1 with the AIoT indicating that data is available to be provided.

2a.	The A-IoT RAN node allocates and coordinates the usage of A-IoT radio resources.

2b/2c.	The Command Required message is sent by the AIoT device and relayed by the AIoT RAN node ot the AIoT CN

2d/2e	If required, a Command Confirm is send as acknowledgment.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

##### 6.5.3.1.2	NAS/UP solution

Figure 6.5.3.1-2: Message flow for A-IoT Inventory in Topology 2 (if NAS/UP based solution is used)

0a.	Direct communication between the AIoT CN and and the AIoT enabled UE(s) – as of the NAS/UP solution – requires the AIoT CN to first request AIoT radio resources for the transaction via an AIoT Session Resource Request message. 
In the AIoT Session Resource Request message, AIoT CN may provide necessary information to the AIoT enabled gNB (e.g., the AIoT Transaction Area) to assist the AIoT enabled gNB in selection AIoT enabled UEs and allocate/coordinate AIoT radio resources.

0b	The AIoT enabled gNB allocates and coordinates usage of AIoT radio resources.

0c/d.The AIoT enabled UE is configured with AIoT resources it is allowed to use. Respective RRC signalling needs to carry information to allow association with information provided in step 1.

Editor’s Note 1:	RRC based communication is only depicted schematically. RAN2 details FFS.

0e.	The AIoT enabled gNB responds to the resource request to the AIoT CN.

Editor’s Note 2:	In step 0e, the gNB may instead send an AIoT resource failure message to the AIoT CN indicating that no resources are available for the request.

1.	The AIoT CN sends an Inventory request message to the AIoT-enabled UE(s).

Editor’s Note 3:	It is FFS if step 1 is received from the same AIoT CN node than for step 0a.

2.	The AIoT-enabled UE(s) sends an Inventory response message to the AIoT CN.

NOTE 1：In step 2, the AIoT-enabled UE(s) may instead send an Inventory Failure message to the AIoT CN indicating that the inventory procedure could not be initiated towards the AIoT device(s).

3.	The AIoT-enabled UE(s) trigger the inventory procedure towards the AIoT device(s).

4a/4b.	After receiving inventory result reported from the AIoT device(s), the AIoT-enabled UE(s) may send one or multiple Inventory reports towards the AIoT CN including the received inventory result.

NOTE 2: Steps 4a/4b may happen in parallel with Step 3 for different AIoT devices.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>