3GPP TSG-RAN WG3 Meeting #126	R3-247595

Orlando, US, November 18-22, 2024

Agenda Item:	15.2

Source:	THALES, Sateliot, Novamint

Title:	Further discussions on support of Store and Forward operations in NTN

Document for:	Discussion and decision

# 1	Introduction

The approved work item justifications and objectives for NR NTN Evolution in Rel-19 [1] emphasizes the benefit to support Store and Forward operation for NB-IoT NTN.

Benefits specifically associated with the support of Store and Forward (S&F) Satellite operation with regenerative payload: - S&F operation allows for delay-tolerant, non-real-time IoT NTN services to be offered in areas visited by the satellites but with no need to have NTN gateway infrastructure (e.g.mid-sea, remote areas) - Along with support for discontinuous coverage, support for S&F operation is key to facilitate cost-effective deployment of IoT NTN services and enable an immediate operational service with (1) sparse LEO constellations and (2) reduced ground segment infrastructure. - The support of S&F puts the 3GPP IoT NTN solution on par with other non-3GPP solutions intended for massive satellite IoT, which natively already supports S&F services

Support of Store&Forward (S&F) satellite operation with full eNB as regenerative payload, therefore: - Define the necessary enhancements into E-UTRAN (network & UE) to support S&F operation for delay-tolerant services [RAN3, RAN2]. - At least specify necessary enhancements e.g. related to S1 protocol, especially to address the feeder link switch over as needed [RAN3] Note: Strive to minimise UE impact. Note: Coordination with SA2 (Rel-19 SA2 led Sat-Arch ph3 SI) is needed on the detail requirements (e.g. traffic type, or QoS parameters for S&F), network architecture (e.g. whether consider (partial) core network on satellite) etc.; further coordination with CT1 might be required

In the same time, SA2 concluded on the following normative impacts of the store and forward in KI#2 [2] the two architecture considered for the S&F operations, Split MME architecture and full CN onboard:

The following option is agreed for supporting Store and Forward operation with a Split MME architecture with the following normative impacts: - When feeder link is not available and the network supports S&F operation, the network shall be able to inform UE(s) whether S&F Satellite operation is applied, (e.g. eNB broadcast support of S&F operation as part of System Information). - NOTE 1: The trigger for the eNB to broadcast support of S&F operation is based on the decision of RAN WGs. From system perspective the expectation is that if the network does not support S&F operation and the feeder link is not available then eNB switches off and does not broadcast any signal. - When UE initiates Attach or TAU procedure, it indicates support for S&F mode to the MME following existing NAS capability, the MME sends Attach or TAU Reject message to the UE if these procedures cannot be completed due to S&F operation. The Attach or TAU Reject message includes: a) A new information indicating the UE that attach or TAU procedure cannot be completed because of the S&F operation and that the UE can re-attempt the attach or TAU in this PLMN in a next satellite pass. This indicates to the UE that the information contained in the Attach or TAU Request message is stored by the MME and the network will be available to the UE after interaction with ground network. b) Wait timer: Indicates to the UE the time it should wait before re-attempting the Attach/TAU procedure in the current or another satellite of the same PLMN. c) Optionally, The list of Satellite IDs over which the UE may re-attempt the Attach/TAU procedure, after wait timer expires. The Satellite IDs are based on the SIB information broadcasted by eNB. […] The following option is agreed for supporting Store and Forward operation with a full CN onboard the satellite with the following normative impacts: - Store and forward is only supported by EPS. - Optionally the MME provides the UE with a S&F monitoring list of satellites IDs, during attach/TAU. The UE uses the satellites in the S&F monitoring list for MO/MT data/signalling with the CN. The S&F monitoring list can be determination by the CN. How network determines the S&F monitoring list is outside the scope of 3GPP in this release of specification. NOTE 8: The S&F monitoring list may assist the UE in retrieving MT data. - The UE needs to be aware that a satellite supports S&F mode. NOTE 9: How the UE is aware that a satellite supports S&F mode of operation depends on RAN. - A UE may be rejected if the satellite cannot support the UE at this time. The attach reject may provide a timer for the time the UE should wait before reattempting and S&F monitoring list which the UE can attempt attach again.

In last SA2 meeting, SA2 has also provided the response in a LS reply [10] with respect to the support of multi-satellite for S&F operation:

Question 1b: Does SA2 consider any use cases where multi-satellites can be involved in the S&F operation? [SA2 answer]: Yes. S&F operation involving multiple satellites of the same PLMN is considered the general case. The use of a single serving satellite would be a particular case.

This architecture was reflected in RAN3#125 agreements:

RAN3 decides to work on Split MME architecture and Full CN on board architecture. Whether eNB only on board architecture is feasible or not can be discussed later.

In the last RAN2 meeting following agreements were made related to S&F Satellite operation.

Agreements:

1.	RAN2 adopts the SA2 study conclusions on the possible S&F architectures as the baseline for further discussion (RAN2 will only consider the full CN and spit-MME payload options)

2.	RAN2 will consider both single satellite pass and multiple satellite pass scenarios

3.	RAN2 will consider both MO and MT data within scope

4.	UE is informed whether its serving satellite is currently operating in S&F via System Information broadcast (FFS if we also need a static indication that in general the NW supports the feature)

5.	RAN2 does not further discuss whether legacy UEs will always need to be barred in a S&F network, at least not before further progress in SA2 discussion. If there will be a need for this, mechanisms to bar legacy UEs are already in place and no further impact on RAN2 specs in expected.

7.	RAN2 agrees there will be a way to bar legacy UEs (using legacy cellBarred and/or cellBarred-NTN bit) and still allow R19 S&F capable UEs. FFS on the exact solution (e.g. new barring bits or whether this is linked to some other indication)

In this contribution we discuss the store and forward operations with multiple satellite and the impact on S1.

# 2	Discussion

## 2.1 Architecture aspects for Store-and-Forward operations

Observation 1	 According to the agreements in RAN3 and other groups pointed in the introduction, support for S&F Satellite operation is based on:

- An architecture where the eNB and part of, or all, CN functions are onboard the satellite

- S&F operation involving multiple satellites of the same PLMN is considered the general case. The use of a single serving satellite would be a particular case

Figure 1 - Discontinuous coverage and Store-and-forward examples (extracted and modified from R2-2403337)

Store and Forward operations are particularly useful when the number of gateway to connect to the ground is limited. As illustrated in the Figure 1-c, satellites can operate in S&F because the feeder link is unavailable but connected to each other through ISL. It brings more resiliency to the system, since the data can be distributed between satellites in the constellation by routing and can then support longer feeder link interruption for non-delay tolerant data. Also, a satellite when its buffer is full can store data on another neighbour satellite. The need for a communication infrastructure resilient to feeder-link momentarily unavailable is also stated in [7], “the connection between the satellite and the rest of the NB-IoT network functional elements is not guaranteed over time. […] As a consequence, the whole architecture shall be designed in order to allow NTN terminals to exchange data with satellites, even in the temporary absence of the feeder-link”.

Note that the revisit time with a single satellite in typical LEO orbits may be around 12h or longer [9]. Therefore, restricting the solution to single-satellite would mean that a UE will only be able to send/receive data e.g. twice a day on average. This case is even reinforced for a moving UE.

Therefore, the support of a Store and Forward Satellite operation solution able to serve a UE via multiple satellites (in other words, not restricting the interaction of a UE and the network to a single satellite of the constellation) should be considered in the architecture from the beginning. This is referred in the following as multi-satellite S&F. SA2 already considered the support of multi-satellite [10] for the same PLMN.

- Proposal 1	RAN3 aligns with SA2 that S&F operation involving multiple satellites of the same PLMN is considered the general case. The use of a single serving satellite would be a particular case. RAN3 considers this to drive the work on the signaling.

## 2.2 Impact on S1 interface

The full CN onboard architecture (eNB, MME, SGW, PGW, HSS, E-SMLC, SMSC, etc…) supports both UP and CP CIoT service. The architecture is depicted on the Figure below [11]:

NTN payload

NTN payload

proxy

proxy

Figure 2 - EPC architecture with full CN onboard

The intermittent feeder link supports the SGi interface between the P-GTW and the IP network. This interface is outside of the RAN3 scope. SA2 recommend to handle the mobility on this interface using a proxy that is outside of the 3GPP scope.

Proposal 2		For the Store and Forward architecture with full CN onboard there is no impacts on S1

The split MME architecture for supporting S&F satellite operation for SMS and CP CIoT services is presented in Figure 3a [11].

Figure 3a: EPC architecture with MME split for supporting S&F satellite operation for SMS and CP CIoT services

Proposal 3.		For the Store and Forward architecture with split-MME and considering the possibility that S&F operation mode can be switched on and off depending on feeder link connection availability, there is a need to support the graceful removal of the S1 interface if the onboard eNB is switching between the MME-onboard (when the feeder link is not available) and a MME on the ground (when the feeder link is available).

# 3	Conclusion

In the previous sections we made the following observations and proposals:

Observation 1	 According to the agreements in RAN3 and other groups pointed in the introduction, support for S&F Satellite operation is based on:

- An architecture where the eNB and part of, or all, CN functions are onboard the satellite

- S&F operation involving multiple satellites of the same PLMN is considered the general case. The use of a single serving satellite would be a particular case

- Proposal 1	RAN3 aligns with SA2 that S&F operation involving multiple satellites of the same PLMN is considered the general case. The use of a single serving satellite would be a particular case. RAN3 considers this to drive the work on the signaling.

Proposal 2		For the Store and Forward architecture with full CN onboard there is no impacts on S1

Proposal 3.		For the Store and Forward architecture with split-MME and considering the possibility that S&F operation mode can be switched on and off depending on feeder link connection availability, there is a need to support the graceful removal of the S1 interface if the onboard eNB is switching between the MME-onboard (when the feeder link is not available) and a MME on the ground (when the feeder link is available).

# References

RP-242397, Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3, RAN1 Vice-Chair, RAN#105, 2024-09, Melbourne, AU.

TR 23.700-29, “Study on integration of satellite components in the 5G architecture; Phase 3”, SA#104, 2024-06, Shanghai, CN.

TR 22.865, Study on satellite access Phase 3, v19.2.0, 2023-12.

TR 22.261, Service requirements for the 5G system, v19.5.0, 2023-12.

TR 23.700-29, Study on integration of satellite components in the 5G architecture, v0.4.0, 2024-03.

R3-243949, “Support for Regenerative Payload in NR NTN”, RAN3#124, Ericsson et al., 2024-05, Fukuoka, JP.

Sciddurlo, Giancarlo, et al. "Looking at NB-IoT over LEO satellite systems: Design and evaluation of a service-oriented solution." IEEE Internet of Things Journal 9.16 (2021): 14952-14964.

Kellermann, Timo, et al. "Novel architecture for cellular IoT in future non-terrestrial networks: store and forward adaptations for enabling discontinuous feeder link operation." IEEE access 10 (2022): 68922-68936.

R2-2107453, “On LEO satellite flyover timing and discontinuous coverage”; Eutelsat, August 9th - 27th, 2021, RAN2#115-e

R3-247016	Reply LS on Support of Regenerative-based Satellite Access, SA2.

S2-2409411 Introduction to Split MME Architecture in 3GPP TS 23.401, SA#164.