3GPP TSG-RAN WG3 Meeting #126	R3-247611

# Orlando, USA, November 18 – 22, 2024

Agenda item:	16.3

Source: 	Samsung

Title: 	Remaining issues on RAN-CN interface impact for Ambient IoT

Document for:	Discussion and decision

Introduction

Last meeting continued to discuss the RAN-CN interface impact for Ambient IoT, and the open issues are provided as follows,

RAN3 starts with inventory-only procedure. Discuss the content in Inventory/Command.

The AIoT CN to select suitable reader (AIoT enabled gNB or AIoT enabled UE reader) for both topology 1 and topology 2?

Assistance information from AIoT CN to AIoT RAN?

In this contribution, we further discuss these open issues.

Discussion

The RAN-CN interface impact for Ambient IoT continued to be discussed last meeting. And several open issues are captured.

For the content in Inventory/Command, there was little discussion on this open issue during last several meetings due to the lack of time, which we believe is more important than the protocol stack. According to the outcome of the May meeting, there are several open issues captured as follows,

In the Inventory Request from the AIoT CN towards the AIoT RAS, it is FFS whether the following information may be included:

Inventory Session ID (e.g., to allow multiple Inventory procedure instances to run in parallel): the purpose, scope and handling of this information in AIoT RAS are FFS.

Estimated Number of expected responses from devices, or Minimum number of required responses from devices.

FFS the usage of these information.

Periodicity: to enable periodic inventory at AIoT RAS triggered by a single Inventory Request from the AIoT CN

Note that the previous meeting has agreed to introduce AIoT Device Identification as well as Scope (related to area) in the Inventory Request message, while no agreement has been achieved on the content in Command message.

For the remaining potential parameters that can be included in the Inventory Request message, we think the Inventory Session ID is also needed. Recall that RFID supports multiple sessions for inventory. In order to provide a comparable capability for Ambient IoT to RFID. We see no obstacle to support multiple inventory sessions. In our understanding, the inventory session is determined by AF/CN, and a session related identifier should be provided in the Inventory Request message.

While for other potential parameters, the Estimated Number of expected responses from devices can be further discussed depending on the use cases and solutions. In our understanding, the purpose of the Inventory is to let the server understand how many and which AIoT devices are in the scope. So whether the Server or AIoT CN could estimate the number of expected responses is actually depending on SA2 progress. On the other hand, the usage of the Estimated number of expected responses may be used for determining the Q value used for Random Access which is depending on RAN2 progress. So RAN3 may not be able to achieve consensus on whether to introduce such parameter in Inventory Request message until more progress can be achieved in other WGs.

In addition, we see no strong need to introduce Periodicity in Inventory Request message since the periodic inventory can be easily achieved by sending another Inventory Request message.

Proposal 1: Consider the Inventory Session/Transaction ID as the content of Inventory Request message.

From the other aspect, the content of Command sent from CN to RAN has not been discussed last meeting. In our understanding, at least the following information can be included in the Command message,

- The Command container which is provided at AIoT upper layer information level.

- AIoT Device Identification. If the Command message is sent by setting up device context after inventory and before the command, such device related information can be AP UE IDs.

- The information related to the session. If the information related to the targeted Ambient IoT device, e.g. an device identifier used over RAN-CN interface is not globally unique, then the session related information may be needed if the device identifier used over RAN-CN interface can be allocated per session.

- The information on whether a reply is needed in response to the Command. The Command may contain Read command, Write command, etc. at the NAS or application layer level to perform corresponding operations with the storage of AIoT devices. For some command such as Write command, a reply from the AIoT device may not be needed; while for some other command such as Read command, a reply from the AIoT device is needed. In order to leverage the AIoT RAN to perform resource allocation for AIoT radio, the AIoT RAN needs to know whether the UL resources for a transmission should be allocated. Therefore the information on whether a reply is needed in response to the Command should be signalled from AIoT CN to AIoT RAN.

Proposal 2: Consider the following information as the content of Command message,

- The Command container.

- AIoT device identification.

- The information on whether a reply is needed in response to the Command.

The next urgent thing is the call flow for T2, especially on solving the open issue on how to trigger the resource allocation by AIoT enabled gNB for NAS based solution. Before this meeting, at least two options are proposed for NAS based solution, i.e. UE requested option and CN requested option. However, in our companion paper submitted to another agenda item, we think there is another alternative which needs to be considered, i.e. the AIoT CN provides the assistance information (the assistance information can be AIoT device ID, Area Scope, Transaction/Session ID, and/or estimated number of devices) which triggers the AIoT enabled gNB to perform the resource allocation. And the basic idea can be provided as follows,

Proposal 3: For NAS based solution, it is also possible that the AIoT CN provides the assistance information which triggers the AIoT enabled gNB to perform resource allocation. Capture the call flow to reflect such mechanism.

Conclusion

In this paper, we provide primary discussion on RAN-CN interface impact for Ambient IoT. The following proposals are provided,

Proposal 1: Consider the Inventory Session/Transaction ID as the content of Inventory Request message.

Proposal 2: Consider the following information as the content of Command message,

- The Command container.

- AIoT device identification.

- The information on whether a reply is needed in response to the Command.

Proposal 3: For NAS based solution, it is also possible that the AIoT CN provides the assistance information which triggers the AIoT enabled gNB to perform resource allocation. Capture the call flow to reflect such mechanism.

Reference