3GPP TSG-RAN WG3 Meeting #126	R3-247623

Orlando, USA, 18 – 22 November, 2024

Agenda item:	11.4

Source:	Nokia, Qualcomm, Jio, Deutsche Telekom

Title:	Continuous Management-based MDT

Document for:	Discussion

# 1	Introduction

Rel-19 NG-RAN AI/ML study we had considered the topic of Continuous Management-based MDT Data Collection. In the TR 38.743 we captured in the use case description that a UE in the NG-RAN can be configured with management-based Logged MDT to log measurements when in RRC_Idle and RRC_Inactive states and with management-based Immediate MDT to report measurements when in RRC_Connected state. Differently from signalling-based MDT, in management-based MDT, a UE is not uniquely identified in the MDT activation. Therefore, when a UE transits to RRC_Connected state from RRC_Idle/RRC_Inactive (during which Logged MDT data have been collected) the network does not have standardized means to select again the same UE for subsequent continuous MDT data collection. Also, management-based MDT configuration is lost during handover events since management-based MDT configuration is not propagated during a handover.

In the study item phase, we also identified two tasks that need to be solved in order to achieve Data Collection continuity in this scenario (TR 38.743):

- Problem A (measurement continuity): how to ensure that the same UE collects MDT measurements during the same RRC state and across different RRC states.

- Problem B (trace correlation): how to ensure that the TCE which eventually receives the MDT reports can associate the received logged and immediate MDT measurements to a continuous data collection period from the same UE.

In RAN3 #125bis we made the following agreement with respect to the topic of Continuous Management-based MDT:

RAN3 assumes that the OAM configures Management Based MDT for Continuous MDT towards the NG-RAN.

Even though this agreement doesn’t have immediate RAN3 impacts, it confirms the understanding that Management-based Continuous MDT, like legacy MDT, is configured by the OAM and the produced trace (MDT) records are to be consumed by the OAM system. We also captured the following open points:

Further discussions are needed on how to define a temporary identifier for Continuous MDT.

Further discussions are needed on the solution for measurement correlation in Continuous MDT.

Even though the above open points relate to the problem of trace correlation (Problem B), we need to discuss further also issues pertaining to measurement continuity (Problem A).

During the Rel-19 study item phase, different companies brought up possible solutions to address Problem A and Problem B above. In this paper we provide our views on these solutions and we propose a way forward towards the Rel-19 Management-based Continuous MDT Data Collection.

# 2	Discussion

In management-based MDT, MDT continuity is not maintained when a UE switches between RRC_Connected and RRC_Idle states, or when a UE is handed over to another gNB. This is due to the following reasons:

- Immediate MDT configuration is erased by the UE every time it transits to RRC_Idle state.

- When a UE transits from RRC_Idle to RRC_Connected state the network does not have standardized means to select a “specific” UE for continuous MDT. This is because the management-based MDT Configuration does not uniquely characterize a UE.

- In legacy Management-based MDT, the management-based MDT configuration does not propagate during a handover. Therefore, the target node cannot know the configuration that a UE had in the source before the handover.

In order to address the above issues, corresponding to problem A (measurement continuity), the following categories of solutions can be considered:

- Solution A1: Upon transition from RRC_Idle or RRC_Inactive to RRC_Connected, the UE indicates to the gNB that it should be selected for continuous management-based immediate MDT. In case of handover, the source gNB sends this information to the target gNB during handover preparation.

- Solution A2: When UE transits to RRC_Idle, a context related to the MDT configuration is stored by the gNB serving the UE to the Core Network. Upon transition from RRC_Idle to RRC_Connected or after successful handover, the AMF indicates to the second gNB serving the UE that the UE should be selected for management-based immediate MDT. In case of handover, the source gNB sends the context related to the data collection to the target gNB during handover preparation.

- Solution A3: UE logs measurements across RRC states (RRC_Idle, RRC_Inactive, RRC_Connected), and hence also independently of any handover in RRC_connected, by enhancing management-based Logged MDT.

The proposed solutions have impacts to different entities. Solution A3 has impacts to the UE since the logged MDT mechanism needs to be redesigned also for logging in connected state. Solution A1 also has UE impacts since in legacy when UE transits from RRC_Idle or RRC_Inactive to RRC_Connected state it only indicates to the network the availability of logged measurements. This means that if we adopt these solutions the number of UEs that support the continuous MDT feature will be limited to Rel-19 UEs implementing the feature.

Proposal 1: We propose to consider continuous MDT solutions that do not impact the UE.

In one alternative of option A1 the network could possibly determine that this is a UE for continuous MDT after it sends to the UE a UEInformationRequest message and receives in the response the logged measurement report, with a special Trace Reference corresponding to continuous MDT. However, this approach has several shortcomings:

- The gNB cannot determine from the availability indication that this pertains to a continuous MDT UE; it therefore needs to request logs for all UEs from which it receives an availability indication.

- The gNB can only determine that this UE is for Continuous MDT when the UE has uploaded the whole log to the gNB so that the gNB can decode the special Trace Reference. This will delay the reconfiguration of the UE with the new configuration and continuity will in many cases be lost.

Observation 1: A solution where the gNB identifies that a UE is a continuous MDT UE by requesting logged measurement reports from all UEs that have reported log availability indication and by receiving the corresponding logged measurement reports in order to determine if they correspond to a special Trace Reference for continuous MDT has the following issues: It is inefficient, may compromise MDT continuity due to the delay to reconfigure the UE, and mandates consumption radio resources unnecessarily during the early phase of an RRC Connection.

On the other hand, Solution A2 is a network-based solution with minor impacts to the Core Network (AMF). This solution may work with simple modifications to the legacy management-based MDT procedures.

In Figure 1, we illustrate an example of maintaining Management-based MDT Data Collection continuity when UE transits to RRC idle state.

In this example it is assumed that the management system configures a gNB (gNB 1) with both management-based Logged and Immediate MDT configuration with an indication that this configuration is for Continuous MDT, according to the RAN3 agreement from RAN3 #125bis. The gNB 1 chooses the UEs to configure for continuous management-based activation. These UEs are then configured with both Logged and Immediate MDT. The UE may report the collected measurements related to the Immediate MDT configuration to gNB 1 while in RRC_Connected state. When the UE transits to RRC_Idle the gNB where the UE was connected can store MDT session information about this UE to the Core Network. We propose the following:

Proposal 2: When the UE transits to RRC_Idle state, the gNB stores the management-based MDT Configuration information in the Core Network.

When the UE later transits to RRC_Connected state in the same or in another gNB, upon connection establishment, the Core Network can provide the stored MDT session information to the gNB to which the UE connects to. In the example in the figure, the UE connects to a different gNB 2. After receiving the information from the Core Network, the gNB can configure continuously the UE with Immediate MDT.

Figure 1 An example of continuous management-based MDT based on solution A2 when UE transits to RRC_Idle state.

With this solution we observe that management-based MDT Continuity when UE reconnects from RRC_Idle can be ensured by allowing the gNB to which UE reconnects to, to initiate Immediate MDT according to MDT information stored in the Core Network and made available to the gNB receiving the UE.

Proposal 3: When the UE reconnects from RRC_Idle state to a gNB, the gNB receiving the UE ensures MDT Continuity by initiating management-based MDT according to MDT session information stored in the core network and made available to the gNB.

In case of a handover event, the management-based immediate MDT configuration could be propagated in handover preparation signalling.

Proposal 4: Management-based MDT Continuity is maintained when a UE is handed over from a gNB to another by propagating the Immediate MDT configuration in handover preparation signalling.

For problem B (trace correlation), several solutions have been discussed:

- Solution B1: Send TR and TRSR of the Logged MDT configuration to the TCE together with the Immediate MDT data records.

- Solution B2: Create an identifier at the AMF, e.g. generated when the UE registers to the network and UE context is created in the Core Network and maintained in the Core Network for the duration of the UE context.

- Solution B3: Introduce an indication from OAM that the MDT configuration is for Management-based MDT either by

  - Solution B3a: The same TR is used for both Logged and Immediate MDT Configuration using a new Job Type for “Logged and Immediate MDT”

  - Solution B3b: Different TRs may be used for Logged and Immediate MDT Configuration by reserving some Trace References to correspond to Continuous Management-based MDT

Solution B1 comes with the advantage of absence of impact on standardized interfaces (except towards the TCE), but requires the gNB to retrieve TR/TRSR of the Logged MDT configuration from the UE in order to be able to write MDT data records for immediate MDT to the TCE. However a legacy UE will not be able to provide this information to the network unless it has already logged MDT measurements available, so this solution will in many scenarios not work.

Solution B3 is related to the agreement from the previous meeting, that OAM configures Management Based Continuous MDT towards the NG-RAN. One approach that OAM achieves this indication is through option B3a where a single Job Type for “Logged and Immediate” MDT is created. Introducing a new Job Type for “Logged and Immediate MDT” is limiting since it forces a gNB to map a single Immediate MDT configuration to a Logged MDT configuration across an area of gNBs. It also requires SA5 changes to introduce a new Job Type. But furthermore, according to TS 32.422 Job Type clause 5.9a, e.g., to support anonymization is described as follows:

***************************************************

## 5.9a	Job Type (M)

The Job Type is a mandatory parameter. The condition is either MDT or RLF or RCEF data collection functionality is supported. It defines if a single trace job, a combined MDT and trace job or RLF report collection job or RCEF report collection job is activated. This parameter also defines the MDT mode.

<<unmodified text skipped>>

***************************************************

According to the above observation creating a new Job Type combining Logged MDT and Immediate MDT goes against the current use of Job Type where it defines the MDT mode. So it is our understanding that Job Type can characterize a single MDT mode only.

Observation 2: Job Type can characterize a single MDT mode only, i.e., either Logged MDT or Immediate MDT.

Solution B3b only solves the correlation problem if a single Continuous Immediate MDT configuration is allowed within a given area. This is because otherwise a gNB receiving a UE wouldn’t know which Immediate MDT Configuration to apply to the UE, matching the logged MDT configuration already configured. Hence, it does not allow flexibility in case different AI/ML models need to be trained using different Immediate MDT measurements and/or targeting different kind of UEs. In addition, for this solution a gNB doesn’t know the UEs that need to be configured with Immediate MDT unless there is an indication that a given UE needs to be chosen for Continuous MDT either a) through the UE itself or b) through the network (for this UE). Similarly to our earlier input, we support solutions that do not impact the UE.

Proposal 5: In case Solution B3b is adopted we need to additionally consider how the network determines which among a number of Immediate MDT configurations to apply for a given UE and how to identify for which UE without creating UE impacts.

Solution B2 also has limited impacts in terms of MDT configuration and handling in the AMF and TCE, while avoiding UE impacts, and this solution seems to be the most suitable.

Proposal 6: We propose that MDT records are correlated in the TCE based on an identifier provided by the AMF.

# 3	Conclusion

In this paper we make the following observations and proposals:

Proposal 1: We propose to consider continuous MDT solutions that do not impact the UE.

Observation 1: A solution where the gNB identifies that a UE is a continuous MDT UE by requesting logged measurement reports from all UEs that have reported log availability indication and by receiving the corresponding logged measurement reports in order to determine if they correspond to a special Trace Reference for continuous MDT has the following issues: It is inefficient, may compromise MDT continuity due to the delay to reconfigure the UE, and mandates consumption radio resources unnecessarily during the early phase of an RRC Connection.

Proposal 2: When the UE transits to RRC_Idle state, the gNB stores the management-based MDT Configuration information in the Core Network

Proposal 3: When the UE reconnects from RRC_Idle state to a gNB, the gNB receiving the UE ensures MDT Continuity by initiating management-based MDT according to MDT session information stored in the core network and made available to the gNB.

Proposal 4: Management-based MDT Continuity is maintained when a UE is handed over from a gNB to another by propagating the Immediate MDT configuration in handover preparation signalling.

Observation 2: Job Type can characterize a single MDT mode only, i.e., either Logged MDT or Immediate MDT.

Proposal 5: In case Solution B3b is adopted we need to additionally consider how the network determines which among a number of Immediate MDT configurations to apply for a given UE and how to identify for which UE without creating UE impacts.

Proposal 6: We propose that MDT records are correlated in the TCE based on an identifier provided by the AMF.