3GPP TSG-RAN WG3 Meeting #126	R3-247624

Orlando, USA, 18 – 22 November, 2024

Agenda item:	20.2

Source:	Nokia

Title:	AI/ML Positioning for Case 3a and Case 3b

Document for:	Other

# 1	Introduction

In RAN3 #125bis, we agreed the following:

The existing LCS framework is re-used for AI/ML positioning.

Specifically, for case 3b we agreed the following:

Re-using the Measurement procedure defined in NRPPa to support requesting and reporting the related measurements to LMF for Case 3b. The details of the related measurements are still pending and subject to further discussion in RAN1.

NG-RAN gets measurements (i.e., part A information as called by RAN1) from TRP and provides them to LMF.

The details of the measurements are still pending and subject to further discussion in RAN1.

Also, for case 3a the following was agreed:

The AI/ML model (inference and training functions) is at the gNB.

LMF provides ground truth label and related data (i.e., part B information as called by RAN1) to NG-RAN.

FFS on the content of the ground truth label and related data pending on RAN1 progress. FFS on the signalling design.

How the LMF obtains the ground truth label and related data information is outside of RAN3 scope.

NG-RAN obtains measurements from TRP (i.e., part A information as called by RAN1).

The details of the measurements are still pending and subject to further discussion in RAN1.

We also captured the following WA for case 3a:

WA: RAN3 will proceed with Option A: NG-RAN node performs monitoring metric calculation for its own model.

We find the following RAN1 agreements useful for our discussions:

RAN1 #116bis

Agreement

For training data collection of AI/ML based positioning, the collected data sample can include the following components:

Part A:

- channel measurement

- quality indicator of channel measurement

- time stamp of channel measurement

Part B:

- ground truth label (or its approximation)

- quality indicator of label

- time stamp of label

Note: “Part A” and “Part B” terminologies are only for RAN1 discussion purpose, and may not be used in specification.

Note: contents in Part A and Part B may or may not be generated by different entities.

Note: Part A and/or Part B, and their contents may or may not apply for each case

FFS: detailed definition of channel measurement

RAN1 #117

Agreement

For training data collection of AI/ML based positioning, if a training data sample contains both Part A and Part B, RAN1 assumes that Part A and Part B in one training data sample are: ​

• for a same UE (PRU or Non-PRU UE), and ​

• for a same location associated with Part B.​

Note: the association can be discussed

Working Assumption

For training data generation of AI/ML based positioning Case 3b, the label and its related data (e.g., time stamp) can be generated by:

- PRU

- Non-PRU UE with estimated location

- LMF

Note: transfer of label and its related data is out of RAN1 scope.

Note: It is assumed that user data privacy of non-PRU UE is preserved.

Note: Previous related working assumption made in RAN1#116bis for training data generation of AI/ML based positioning Case 3b will not need to be confirmed.

RAN1 #118

Agreement

For AI/ML positioning Case 3a, for performance monitoring metric calculation in label-based monitoring, from RAN1 perspective, Option A and Option B are feasible,

- Option A.	NG-RAN node performs monitoring metric calculation for its own model.

- Option B.	LMF performs monitoring metric calculation for the model located at the NG-RAN node.

Note: Final selection of Option A and Option B is out of RAN1 scope. Potential support of Option A and/or Option B is pending RAN3 confirmation.

Note: Exact method to perform monitoring metric calculation is up to implementation.

Note: For Option A, RAN1 assumes that user data privacy needs to be preserved.

RAN1 #118bis

Agreement

From RAN1 perspective, when timing information is reported for Rel-19 AI/ML positioning Case 3a, at least the following are mandatorily or optionally supported in a measurement report from gNB to LMF:

- (Mandatory) timing information;

- (Optional) Quality of the timing information;

  - Existing IE “Timing Measurement Quality” can be reused.

- (Mandatory) Time stamp.

FFS: LOS/NLOS indicator.

Note: The final decision of “mandatory” or “optional” presence of each field is up to RAN3.

In this contribution, we provide our views with respect to different considerations pertaining to Cases 3a and 3b.

# 2	Discussion

## 2.1 	AI/ML Assisted Positioning (Case 3a)

In the last meeting we agreed that AI/ML Model Training and AI/ML Model Inference functions are located at the gNB. Another possible option for the AI/ML Model Training is OAM, if OAM receives training data (measurements, ground truth label and related data) so that an AI/ML Model can also be trained offline. This would be in line with the Rel-18 framework of NG-RAN AI/ML described in TS 38.300 where AI/ML Model Training may be located in OAM or in the gNB. In addition, this would be in line with the TR 38.843 where the following is mentioned: “For gNB-side model, training data can be generated by the gNB, while the termination point for training data may include the gNB, or OAM.”. This means, that OAM may be a termination point receiving training data and hence it is able to also train the AI/ML model of a gNB.

Proposal 1: Besides the gNB, AI/ML Model Training can also be located in OAM.

Note that if AI/ML Model Training is in OAM we can also reuse the Model Deployment/Update functionality that SA5 has introduced to support the deployment of the AI/ML Model from OAM to RAN.

An aspect that remains open is related to performance monitoring. Before discussing technical details of this aspect we would like to point out a misalignment in the terminology when it comes to performance monitoring. Model Monitoring in TR 38.843 corresponds to Model Performance monitoring in RAN3 discussions, which is the process enabling the monitoring of the performance of an AI/ML model. The outcome of Model Performance Monitoring is provided by the Model Inference to the Model Training entity through Model Performance Feedback.

Proposal 2: Use the term Model Performance monitoring to refer to Model Monitoring for better alignment with RAN3 terminology.

In the last meeting, RAN3 agreed a WA related to the entity that performs the monitoring metric calculation. In particular we agreed the following WA:

WA: RAN3 will proceed with Option A: NG-RAN node performs monitoring metric calculation for its own model.

The other option identified by RAN1 is Option B: LMF performs monitoring metric calculation for the model located at the NG-RAN node. According to RAN1 agreements, for training data collection of AI/ML based positioning, if a training data sample contains both Part A and Part B, RAN1 assumes that Part A and Part B in one training data sample are:

- For a same UE (PRU or Non-PRU UE), and ​

- For a same location associated with Part B

RAN3 also agreed that LMF provides the ground truth label and related data (i.e., part B information as called by RAN1) to NG-RAN while NG-RAN obtains measurements from TRP (i.e., part A information as called by RAN1). From the above, it follows that a data sample containing both Part A (from gNB) and Part B (from the LMF) must be for the same UE and for the same location, but RAN1 has made no assumption on how to associate the two parts.

Observation 1: According to RAN1 agreement, Part A and Part B are from the same location and from the same UE, but there is no assumption that in order to associate the two parts UE location is known at the gNB.

As one possibility, the association between Part A and Part B could be achieved through a common timestamp. Given the above, it seemed possible to agree in the previous meeting that a gNB trains and executes inference of its own model. If a gNB trains its own AI/ML Model, it can also calculate its own model performance metrics, which are model-specific and dependent on the model in use. Having those metrics defined at the gNB, the gNB can choose the relevant model performance metrics depending on the model in use and perform model performance monitoring of its own AI/ML model. In addition, this will enable the gNB to autonomously trigger retraining of its AI/ML Model, if needed, which will simplify the gNB’s implementation with respect to Life Cycle Management. Thus, we think that option A can be agreed.

Proposal 3: RAN3 to confirm the WA: RAN3 will proceed with Option A: NG-RAN node performs monitoring metric calculation for its own model.

According to RAN1 agreements, in Case 3a, the output of Model Inference at a gNB is timing information, which may be at least UL RTOA and gNB Rx-Tx Time Difference, and which the gNB reports to the LMF. In addition, it is under discussion in RAN1 whether LoS/NLoS indicator will also be reported to the LMF. According to legacy positioning methods, an LMF requests measurements from a gNB and uses those measurements for the calculation of a UE positioning estimate. The LMF’s positioning algorithm does not depend on the method that gNB used to determine those measurements, while at the same time it is up to gNB implementation whether and how to perform those measurements.

Observation 2: For Case 3a, the gNB, by its implementation, will use its best available method to provide the requested timing information measurements to the LMF.

Looking at the TRP Measurement Result IE in TS 38.455, we observe that the timing information that a gNB can provide through AI/ML corresponds to measurements that can be reported already by a gNB to the LMF. From LMF point of view, there is no difference between measurements that are derived by AI/ML Model Inference or by legacy measurements, measured by the gNB/TRP. Thus, it should be transparent to the LMF what methods (AI/ML or legacy) the gNB uses to provide the requested measurements to the LMF. The gNB should respond to the LMF request by choosing the best measurement it has available by its implementation, either by deploying AI/ML or legacy procedures.

Proposal 4: gNB decides whether the measurements reported to LMF are outcomes of legacy measurements or produced by AI/ML operations.

Therefore, existing methods can be used to configure reporting by a gNB in Case 3a.

Proposal 5: Reuse existing NRPPa methods to configure gNB to report LoS/NLoS indication and timing information, such as UL RTOA and gNB Rx-Tx Time Difference.

One of the agreements of the previous meeting is that the LMF provides ground truth label and related data (i.e., part B information) to NG-RAN. This information will be used by a gNB to train its model and to perform model performance monitoring. An important question related to this agreement relates to the entity that selects the UE for which this ground truth label pertains. In addition, LMF can select UEs that are authorized to provide UE location. On the other hand, it is unclear if those UEs would be the “right” UEs for training a gNB-side model. The reason is that those UEs may be selected in a biased way and may not ensure diversity in the UE selection e.g., may not represent UEs chosen over different areas in a gNB. Only a gNB itself, knowing the details of its own ML Model, can determine the right UE selection to avoid training bias. Therefore, it seems that letting the choice of UE selection to the gNB has an advantage over UE selection at the LMF when it comes to optimizing the training and monitoring data collection related to an AI/ML Model at the gNB.

Proposal 6: Support UE selection for obtaining the ground truth information at the gNB.

## 2.2 	Direct Positioning (Case 3b)

In case of Direct AI/ML positioning according to Case 3b, the output of an AI/ML Model is a UE location. In the last meeting, we agreed that for Case 3a the Model Inference and Model Training functions are at the gNB. Similarly, for Case 3b we could agree that a possible location for Model Inference and Model Training functions is the LMF.

Proposal 7: For Case 3b, the Model Training and Model Inference functions can be at the LMF.

In addition, RAN1 had made a working assumption that the label and its related data (e.g., time stamp) for Case 3b can be generated by PRU, non-PRU UE with estimated location or by LMF. It therefore seems feasible that LMF can perform its own Model Performance Monitoring for Case 3b. This is also aligned with the SA2 TS 23.273.

Proposal 8: For Case 3b, the LMF performs Model Performance Monitoring of its own models.

In the last meeting we agreed that NG-RAN will send the measurements it gets from TRP to the LMF. For this, the Measurement and Measurement Report procedures defined in NRPPa can be used. However, the details related to those measurements are still under discussion in RAN1. For instance, regarding time domain channel measurements, RAN1 discusses sample-based and path-based methods without concrete conclusions. Therefore, we think that RAN3 cannot take any further agreements related to transferring of measurements from NG-RAN to the LMF before RAN1 progress.

Proposal 9: RAN3 to wait for RAN1 progress before proceeding with further discussions related to reporting of measurements from NG-RAN to the LMF.

We also provide a stage 2 TP to TS 38.300 in the Annex of this paper.

# 3	General Principles

Case 3a assumes the existence of an AI/ML Model in the gNB. We, therefore, see useful to agree for this case some high-level principles from TR 37.817, that were considered valid during Rel-18 AI/ML for NG-RAN normative phase. Specifically, even though in Case 3a, a gNB may train an AI/ML Model we should follow the same assumption as in NG-RAN AI/ML that AI/ML Models and Algorithms are not in scope of RAN3.

Proposal 10: Detailed of AI/ML Models and Algorithms for AI/ML positioning case 3a are implementation specific and out of RAN3 scope.

Another principle that we followed in Rel-18 NG-RAN AI/ML work is that an AI/ML Model used for Model Inference needs to have completed all the steps of Life Cycle Management, meaning AI/ML Model Training and AI/ML Testing.  Since in Case 3a, a gNB can provide the LMF with inferred measurements we thinking that the same principle should be valid for the AI/ML Air Interface work.

Proposal 11: An AI/ML model used in a Model Inference function has to be initially trained and tested before deployment.

Finally, another fundamental principle for Data Collection is related to respecting user data privacy and anonymization and it shall be valid for this work item also.

Proposal 12: User data privacy and anonymization need to be respected during AI/ML operation.

# 4	Conclusion

In this paper we make the following observation and proposals:

Proposal 1: Besides the gNB, AI/ML Model Training can also be located in OAM.

Proposal 2: Use the term Model Performance monitoring to refer to Model Monitoring for better alignment with RAN3 terminology.

Observation 1: According to RAN1 agreement, Part A and Part B are from the same location and from the same UE, but there is no assumption that in order to associate the two parts UE location is known at the gNB.

Proposal 3: RAN3 to confirm the WA: RAN3 will proceed with Option A: NG-RAN node performs monitoring metric calculation for its own model.

Observation 2: For Case 3a, the gNB, by its implementation, will use its best available method to provide the requested timing information measurements to the LMF.

Proposal 4: gNB decides whether the measurements reported to LMF are outcomes of legacy measurements or produced by AI/ML operations.

Proposal 5: Reuse existing NRPPa methods to configure gNB to report LoS/NLoS indication and timing information, such as UL RTOA and gNB Rx-Tx Time Difference.

Proposal 6: Support UE selection for obtaining the ground truth information at the gNB.

Proposal 7: For Case 3b, the Model Training and Model Inference functions can be at the LMF.

Proposal 8: For Case 3b, the LMF performs Model Performance Monitoring of its own models.

Proposal 9: RAN3 to wait for RAN1 progress before proceeding with further discussions related to reporting of measurements from NG-RAN to the LMF.

Proposal 10: Detailed of AI/ML Models and Algorithms for AI/ML positioning case 3a are implementation specific and out of RAN3 scope.

Proposal 11: An AI/ML model used in a Model Inference function has to be initially trained and tested before deployment.

Proposal 12: User data privacy and anonymization need to be respected during AI/ML operation.

# Annex -TP to TS 38.300

## <<< start of changes >>>

## 16.20	Support of AI/ML for NG-RAN

### 16.20.1	General

Support of AI/ML for NG-RAN, as a RAN function, is used to facilitate Artificial Intelligence (AI) and Machine Learning (ML) techniques in NG-RAN.

The objective of AI/ML for NG-RAN is to improve network performance and user experience, through analysing the data collected and autonomously processed by the NG-RAN, which can yield further insights, e.g., for Network Energy Saving, Load Balancing, Mobility Optimization.

The objective of NG-RAN support for AI/ML assisted positioning is to enable NG-RAN to provide output of Model Inference, e.g., intermediate parameters to the LMF.

### 16.20.2	Principles

Support of AI/ML for NG-RAN requires inputs from neighbour NG-RAN nodes (e.g., predicted information, feedback information, measurements) and/or UEs (e.g., measurement results).

NG-RAN support for AI/ML positioning requires inputs of ground truth and related data from LMF and measurements from UEs.

## <<< unmodified text skipped >>>

### 16.20.3	Data Collection and Reporting

The following information can be configured to be reported by an NG-RAN node:

-	Predicted resource status information;

-	UE performance feedback;

-	Measured UE trajectory;

-	Energy Cost (EC).

The collection and reporting are configured through the Data Collection Reporting Initiation procedure, while the actual reporting is performed through the Data Collection Reporting procedure.

The collection of measured UE trajectory and UE performance feedback is triggered at successful Handover.

Cell-based UE trajectory prediction, which can be used, e.g., for the Mobility Optimization use case, is transferred to the target NG-RAN node via the Handover Preparation procedure to provide information for, e.g., subsequent mobility decisions. Cell-based UE trajectory prediction is limited to the first-hop target NG-RAN node.

### 16.20.x	Positioning procedures (name FFS)

The following information can be configured to be reported by an NG-RAN node to an LMF:

-	Positioning intermediate parameters (e.g. LoS indicator, NLoS indicator, timing information).

NRPPa procedures are used for the configuration and reporting of intermediate parameters.

## <<< end of changes >>>