3GPP TSG-RAN WG3 #126	R3-247636

Orlando, US, 18 – 22 Nov. 2024

Agenda Item:	18.2. Support LP-WUS Indicating Paging Monitoring

Source:	NTT DOCOMO INC.

Title:	Further discussion on LP-WUS subgrouping

Document for:	Discussion, agreement

Introduction

Currently, RAN1, RAN2, and RAN4 are in the LP-WUS normative phase discussions. In past two meetings, RAN3 achieved following [1,2]:

[RAN3#125]

- New IEs should be introduced to support LP-WUS on NG, Xn and F1.

[RAN3#125bis]

- The LP-WUS assistance information IE, which includes the CN assigned subgroup ID for LP-WUS, is introduced in the NG paging message. FFS the number of the subgroups.

- The LP-WUS assistance information IE is introduced in the NGAP core Networking assistance information IE to assist anchor gNB to initiate RAN paging. FFS the number of the subgroups.

- The LP-WUS assistance information IE, which indicates the CN assigned subgroup ID for LP-WUS, is introduced in F1 paging message for CN paging and RAN paging. FFS the number of the subgroups.

- Introducing LP-WUS subgrouping capability related information into F1 paging message for UE_ID based subgrouping. FFS on per UE or per cell. FFS for CN based subgrouping.

- The gNB-DU is responsible to generate and broadcast the LP-WUS related configuration, including LP-SS configuration, LP-WUS configuration. FFS on entry/exit condition for LP-WUS monitoring.

And also, some points were discussed but not agreed:

- The anchor gNB provides the new IE (FFS) for CN assigned subgroup ID into the Xn paging message for RAN paging. This agreement pending on RAN2 decision that UE can use the LP-WUS outside the last serving cell.

- Introducing new IE to include extra bits for UE_ID based LP-WUS subgrouping into F1 paging. The extra bits are used for DU to determine the LP-WUS subgrouping ID of the paged UE which applies UE_ID based LP-WUS subgrouping.

- NG-RAN node can acquire the total number of CN assigned subgrouping via OAM.

In this paper, to resolve remaining open points and make progress, we have further discussion for LP-WUS support in RAN3.

Discussion

LP-WUS assistance information for RAN paging

In the last meeting, we agreed to include the CN assigned subgroup ID in NG paging message and F1 paging message. This supports CN paging with LP-WUS for RRC_IDLE/INACTIVE UE, but not RAN paging for RRC_INACTIVE UE. This is because RAN2 has not yet decided whether to support LP-WUS in other cells than the last serving cell. This issue involves how LP-WUS works outside the last serving cell, and since this is a RAN2 matter, RAN3 should wait for their decision before discussing it.

Proposal 1:	Regarding Xn paging, RAN3 to pending on RAN2 progress.

LP-WUS capability/subgrouping assignment

In the previous meeting, it was agreed to include LP-WUS capability related information in the F1 paging message for UE that is not assigned a CN based subgrouping ID, in order for the DU to decide whether to derive a UE_ID based subgrouping ID. As FFS, it was captured whether UE capability is defined per UE or per cell, which is a point to be discussed in RAN2. On the other hand, we recognize that the indication of capability with respect to CN based subgrouping is related to whether the DU derives a UE_ID based subgrouping ID and it seems RAN3 matter. Our understanding is that whether the UE capability for LP-WUS with UE_ID based subgrouping ID and (if assigned) CN based subgrouping ID is enough for the DU to decide on the use of LP-WUS with UE_ID based subgrouping.

Proposal 2:	UE capability on UE_ID based LP-WUS subgrouping and allocated CN based subgrouping ID (if any) are enough for the DU to determine whether and how to determine the UE_ID based subgrouping ID.

LP-WUS configuration generation

Whether the LP-WUS configuration and entry/exit condition are configured by the DU/CU is not the same issue as which SIB to transmit the configurations. On the other hand, it is already agreed in RAN3 that the LP-SS/LP-WUS configuration to be transmitted in the SIB is generated by the DU. The entry/exit condition may be generated by either the CU or the DU, since LP-SS related information can be acquired by the CU via F1 gNB-DU System Information IE. While we do not have a strong view, if the LP-SS related configuration is generated at the DU, it is more natural that the exit/entry condition is done at the DU.

Proposal 3:	The gNB-DU is responsible to generate and broadcast entry/exit condition for LP-WUS monitoring.

Emergency PDU session

Some companies, including ourselves, are concerned about the impact of LP-WUS paging delays on emergency PDU session service [3,4]. While we do not currently acknowledge a clear service impact, since LP-WUS has longer delay for wake-up than PEI (as discussed in [5]), RAN3 should consider the need for a solution to this issue and discuss a solution if necessary.

Basically, the UE is aware of whether it has an active emergency PDU session and can avoid entering the LP state based on its implementation.For LP-WUS with CN based subgrouping, the NW can control the non-use of LP-WUS by a CN that knows both the UE capability on LP-WUS and the existence of active emergency PDU sessions. This solution is supported for PEI and can be used as a baseline for the discussion on a solution for LP-WUS. For PEI with UE_ID based subgrouping, the solution was not specified, so the solution for LP-WUS may need to be discussed.

Proposal 4:	For CN based LP-WUS subgrouping, to avoid the use of LP-WUS when the UE has active emergency PDU session, RAN3 assume that the solution introduced for PEI can be a baseline.

Proposal 5:	For UE_ID based LP-WUS subgrouping, RAN3 analyze the need for the solution to avoid the use of LP-WUS when the UE has active emergency PDU session and discuss how to specify it, if needed.

Conclusions and proposals

Our proposals are summarized below.

Proposal 1:	Regarding Xn paging, RAN3 to pending on RAN2 progress.

Proposal 2:	UE capability on UE_ID based LP-WUS subgrouping and allocated CN based subgrouping ID (if any) are enough for the DU to determine whether and how to determine the UE_ID based subgrouping ID.

Proposal 3:	The gNB-DU is responsible to generate and broadcast entry/exit condition for LP-WUS monitoring.

Proposal 4:	For CN based LP-WUS subgrouping, to avoid the use of LP-WUS when the UE has active emergency PDU session, RAN3 assume that the solution introduced for PEI can be a baseline.

Proposal 5:	For UE_ID based LP-WUS subgrouping, RAN3 analyze the need for the solution to avoid the use of LP-WUS when the UE has active emergency PDU session and discuss how to specify it, if needed.

References

- RAN3#125 Chair Notes

- RAN3#125bis Chair Notes

- R3-244409, “Discussion of LPWUS impacts on NG-RAN architecture”, Ericsson

- R3-244513, “Discussion on potential RAN3 impacts of LP-WUS”, CATT

- R3-244424, “Support of LP-WUS”, Nokia